| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0260 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, USA, 11 – 17 Jan. 2014 | Document: JCT3V-G0224 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides some additional information on the POC design in JCTVC-P0041/JCT3V-G0031, some of which with comparison to the layer tree based POC design in JCTVC-P0056/JCT3V-G0042. Provided information includes 1) an analysis of error resilience compared to the POC design in JCTVC-P0056/JCT3V-G0042, 2) a point regarding using of POC and layer-tree POC in post-processing entities, 3) an analysis of how it works with multi-standard multi-layer coding designs, and 4) a showcase of whether it works with important use cases.

# Error resilience analysis

Generally, whenever the signalling and derivation of a variable depends on some other data, then there would be some error resilience problem when the dependent data is lost. The signalling and derivation of POC in JCTVC-P0041/JCT3V-G0031 is layer specific, does not depends on any information from another layer, thus loss of coded pictures in one layer would never affect the POC derivation of another layer.

For example, the signalling and derivation of POC in JCTVC-P0056/JCT3V-G0042 have the following two error resilience issues for, as described below.

The first issue is the access unit boundary detection issue due to the requirement for cross-layer alignment of POC LSB values, as shown in Figure 1.

Figure 1: The access unit boundary detection issue

The second issue is described in Figure 2, where video redundancy coding (VRC) is used in multi-layer context. The key point here is to demonstrate that, even when an IDR picture at the base layer is lost, the possibility of correct derivation of POC values of an enhancement layer that is indicated by the VPS depending on the base layer can be useful. Other examples of such coding scenarios include gradual decoding refresh.

Figure 2: An example with video redundancy coding

The second issue is mitigated by the possibility of signalling the POC MSB cycle in the enhancement layer pictures is the same AU as an IDR picture. With this possibility, the error resilience problem would only occur when a pre-encoded bitstream without the POC MSB cycle signalled is later transmitted over an error-prone network channel.

# One point regarding using of POC and layer-tree POC in post-processing entities

In the POC design in JCTVC-P0056/JCT3V-G0042, for SEI message semantics and for use by external post-processing entities, POC is used for the base layer and layer tree POC is used for enhancement layers. However, since POC of a base layer picture does not need to be equal to layer tree POC of an enhancement layer picture in the same access unit, and layer tree POC values of different enhancement layer pictures in the same access unit do not need to be equal, association of decoded pictures to access units is not possible.

# Working with multi-standard multi-layer coding designs

An analysis of how the POC design in JCTVC-P0041/JCT3V-G0031 works with different multi-standard multi-layer coding designs is provided below. The analysis applies even when multiple independent layers are coded based on AVC, e.g. as suggested in JCTVC-P0183.

Legend:

BL: base layer

BL: enhancement layer

output sync: base layer and enhancement layer picture output synchronization

EL decoder: the decoder for enhancement layers

- No encapsulation (decoded pictures provided externally), output sync by external means, and BL decoded pictures NOT in EL decoder's DPB (see JCTVC-P0184)

Herein, the POC value of a BL picture is set equal to the POC value of any EL picture, when present, in the same access unit. When no EL picture is present in the access unit, the BL picture, even provided by the external means, is not needed (as there is no EL picture using it for decoding, and its output is controlled externally to the EL decoder).

- No encapsulation (decoded pictures provided externally), output sync by EL decoder, and BL decoded pictures (at least conceptually) in EL decoder's DPB (see JCTVC-P0140)

  - BL output (including POC) and RPS parameters provided externally

Herein the POC value of a BL picture is provided by external means, and it is required to be the same as that of any EL picture in the same access unit.

  - BL output (including POC) and RPS parameters provided in the bitstream

Herein the POC value of a BL picture is derived by EL decoder based on syntax, as all the POC related syntax can be included in the slice segment header for the BL pictures.

- AVC NAL units encapsulated as HEVC NAL units

  - JCTVC-P0140: Output sync by EL decoder, and BL decoded pictures (at least conceptually) in EL decoder's DPB. BL output (including POC) and RPS parameters provided in the bitstream.

Same as 2.b above, herein the POC value of a BL picture is derived by EL decoder based on syntax, as all the POC related syntax can be included in the slice segment header for the BL pictures.

  - JCTVC-O1012: Output sync unspecified.

Herein output sync should be specified similarly as in JCTVC-P0140, and the POC value of a BL picture is derived by EL decoder based on syntax. Providing POC related information through external means does not seem a good option now that the pictures themselves are provided in the bitstream.

- HEVC NAL units encapsulated as AVC NAL units (see JCTVC-O1012 and JCTVC-P0203): Output sync unspecified.

Herein it is not sure whether it is better to specify this as an AVC extension or an HEVC extension, and not sure whether the DPB operations should follow AVC or HEVC. In any case, output sync should be specified similarly as in JCTVC-P0140, and the POC value of a BL picture is derived by EL decoder based on syntax. Providing POC related information through external means does not seem a good option now that the pictures themselves are provided in the bitstream.

# Implementation showcase

## Showcase design

- Goal: To demonstrate that the approach works for important use cases.

- Important use cases to be tested/demonstrated:

  - IRAPs are cross-layer aligned

  - Lower layers have more frequent random access points (RAPs) than higher layers

  - Higher layers have more frequent random access points (RAPs) than lower layers

  - Decoding of the entire multi-layer bitstream

  - Decoding of the base layer bitstream by legacy HEVCv1 decoders

  - Layer up-switching

  - Layer down-switching and then up-switching

  - Decoding of sub-bitstreams wherein the base layer contains only CRA pictures

- Common encoding configurations

  - Frame rate: 30 frames/second

  - POC LSB length: 5 bits

- Coding structures

  - Two layers, random access periods (CRA pictures) for the base layer and the enhancement layer are ABOUT 1 second. Hierarchical B coding structure. Only IRAP pictures in the base layer are used for inter-layer prediction.

  - Two layers, random access periods (CRA pictures) for the base layer and the enhancement layer are ABOUT 1 second and 2 seconds, respectively. Hierarchical B coding structure.

  - Two layers, random access periods (CRA pictures) for the base layer and the enhancement layer are ABOUT 2 seconds and 1 second, respectively. Hierarchical B coding structure.

  - Two layers, random access periods (IDR pictures) for the base layer and the enhancement layer are about 1 second and 2 seconds, respectively. Low-delay coding structure (IPPPP…, or IPBBB…).

  - Two layers, random access periods (IDR pictures) for the base layer and the enhancement layer are about 2 seconds and 1 second, respectively. Low-delay coding structure (IPPPP…, or IPBBB…).

- How to demonstrate/showcase

  - For each of the following 21 combinations, show that the decoding result for the decoded pictures, in output order, matches at the encoder and decoder sides.

    - 16 combinations of {4, 5, 6, 7} x {B, C, D, E}

    - 5 combination of {8}x{A, B, C, D, E}

## Showcase summary

We did the following implementations to the SHM version 4.1-dev:

- The proposal JCTVC-P0041/JCT3V-G0031 is being implemented, including the following aspects:

  - Syntax elements and decoding process for POC.

  - At a POC resetting picture, all pictures that precede the current access unit in decoding order are output in the increasing order of POC.

  - Encoder command line arguments to enable restriction of inter-layer prediction only for those pictures that are in the enhancement layer and that are contained in IRAP access units. This is done to enable test case 8.

  - Decoder command line arguments to enable test cases 6, 7, and 8 can be done using the SHM decoder by simply ignoring the pictures that would not be present in the bitstream.

  - A patch to HM-12.1-dev (version 1 decoder) is also provided to decode a multi-layer bitstream. The NAL units that have nuh_layer_id greater than 0 are discarded, and a few assert statements are commented that do not apply to a multi-layer bitstream. This is to demonstrate test case 5.

  - Few bugs in the SHM software are fixed with appropriately commented guard macros.

The source code and the showcase script are provided in the attachment of this document.

It can be shown that the POC design in in JCTVC-P0041/JCT3V-G0031 works with all the important use cases described above.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).