| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9 – 18 Jan. 2014 | Document: JCTVC-P0262 |
| --- | --- |

| Title: | Support for out-of-band signaling in VPS to enable future layer additions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information and Proposal |  |  |
| Author(s) or Contact(s): | Ajay Luthra, Sam Narasimhan Arris 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 1-858-404-3470 Sam.narasimhan@arrisi.com Ajay.luthra@arrisi.com |

# Abstract

The current VPS structure in HEVC requires a change to the VPS (in the underlying video stream) to signal layer specific parameters when a new scalable layer is added to pre-compressed SHVC content. Even though removal of layers is supported currently (with an SEI) addition of new layers without a change to the VPS in underlying video elementary stream is currently not possible. One of the examples of the use case is where lower layer corresponds to 60 fps and higher layer corresponds to 120 fps. Earlier contributions (JCTVC-N0048, JCTVC-K0206 and ISO/IEC WG 11/N 12956) highlighted adding new layers at re-distribution points without a change to the parameters (VPS, SPS, PPS) in the lower layers as a required feature to be included in SHVC specification.

In order to enable this, we propose to add a flag (VPS external means flag) to the VPS syntax to indicate  parameters (such as profile, level, layer sets) for new layers are transmitted via external means rather than in the VPS. MPEG-2 transport streams provide the capability to signal these parameters through a ‘descriptor’ associated with the new layer-specific video stream carried in a separate PID. The proposal uses one of the ‘reserved’ bits in the VPS for this flag.  Based on the setting of VPS external means flag the layer specific parameters are either signaled in-band in the VPS or are sent through external means. With this addition, VPS does not have to be altered at re-multiplexing or re-distribution points when a new layer is added.

# External means

The addition of a flag (compatible) in the VPS is proposed to indicate the use of external means for transmission of parameters for the new layer that is added at re-distribution point. In order to enable adding a new layer to pre-compressed HEVC content, it is recommended that the VPS of such stream include more layers (nuh_layer_id) as well as layer sets without the parameters for these new layers or layer sets as these will be conveyed by other means.

## Proposed modifications to VPS Syntax and Semantics

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_extension_offset //vps_reserved_0xffff_15bits | u(15) |
| vps_external_means_flag | u(1) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| vps_timing_info_present_flag | u(1) |
| if( vps_timing_info_present_flag ) { |  |
| vps_num_units_in_tick | u(32) |
| vps_time_scale | u(32) |
| vps_poc_proportional_to_timing_flag | u(1) |
| if( vps_poc_proportional_to_timing_flag ) |  |
| vps_num_ticks_poc_diff_one_minus1 | ue(v) |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_layer_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_alignment_bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_external_means_flag equal to 1 signals that some of the layer specific parameters (for layers that are added at re-distribution point) that are not present in the VPS in-band are conveyed by other means (such as a descriptor in MPEG-2 transport streams).

# Patent rights declaration(s)

Arris may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).