| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0271 |
| --- | --- |

| Title: | RCE4: Cross-check on Test 1 - Major-color-based screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | W. Pu J. Sole | Email: | wpu@qti.qualcomm.com joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Cross-checking results of RCE4 Test 1 on Major-color-based Screen Content Coding are presented in this contribution. It is confirmed that both the software implementation and the results match to the text description and test results provided by the proponent.

# Introduction

A palette-based coding method (JCTVC-O0182) [1] was presented in 15-th JCT-VC meeting and was included as test 1 in RCE4 for San Jose [2]. This contribution provides the cross checking results.

# Experimental results

The proposed method is implemented on HM-12.1+RExt-5.1. RCE4 common test conditions specified in [3] are used. Encoding time and decoding time is measured on a homogenous cluster.

Table 1 Summary of AHG 8 Lossless Coding Results

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.6% | -0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -12.6% | -10.4% | -9.7% |
| RGB 4:4:4 Animation | -0.2% | -0.2% | -0.2% |
| YCbCr 4:4:4 SC | -8.6% | -7.2% | -6.0% |
| YCbCr 4:4:4 Animation | -0.4% | -0.4% | -0.4% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -34.5% | -31.8% | -27.0% |
| YCbCr 4:4:4 SC (Optional) | -27.1% | -24.9% | -22.4% |
| Enc Time[%] | 109% | 106% | 102% |
| Dec Time[%] | 99% | 106% | 109% |

Table 2 Summary of AHG8 Lossy Coding Results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.9% | -2.5% | -3.5% | -3.0% | -2.9% | -3.0% | -3.0% | -2.7% | -2.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -8.3% | -8.1% | -8.4% | -10.4% | -10.2% | -10.2% | -12.2% | -12.8% | -12.7% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | -7.1% | -6.5% | -6.9% | -9.4% | -8.1% | -8.0% | -11.2% | -9.6% | -9.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -28.1% | -27.0% | -27.3% | -31.6% | -30.9% | -31.6% | -37.0% | -37.6% | -37.7% |
| YCbCr 4:4:4 SC (Optional) | -16.4% | -16.5% | -15.4% | -22.4% | -21.6% | -20.4% | -29.7% | -28.6% | -29.0% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 98% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.1% | -1.3% | -2.5% | -2.0% | -1.7% | -1.8% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -7.6% | -7.2% | -7.5% | -9.1% | -8.9% | -9.1% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -6.2% | -5.2% | -6.0% | -8.2% | -6.7% | -6.9% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -24.0% | -23.1% | -23.1% | -27.5% | -26.9% | -27.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -14.8% | -15.2% | -14.3% | -19.6% | -18.9% | -18.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 113% |  |  | 112% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.2% | -2.0% | -1.7% | -1.3% | -1.5% | -1.0% |  |  |  |
| Class B | 0.1% | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -6.4% | -6.2% | -6.4% | -7.8% | -7.6% | -7.8% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -4.6% | -4.7% | -5.5% | -6.2% | -5.6% | -6.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.2% | 0.3% | 0.2% | 0.1% | 0.2% | 0.1% |  |  |  |
| RangeExt | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -20.2% | -19.8% | -19.8% | -24.9% | -24.4% | -25.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -13.9% | -13.3% | -13.3% | -17.5% | -16.9% | -16.8% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 112% |  |  | 111% |  |  |  |  |  |

# References

- X. Guo, B. Li, J. Xu, Y. Lu, S. Li, F. Wu, “AHG8: Major-color-based screen content coding,” JCTVC-O0182, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- X. Guo, Y. Lu, and S. Li, “RCE4: Test 1. Major-color-based screen content coding,” JCTVC-P0108, San Jose, US, Jan. 2014.

- L. Guo, X. Guo and A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette Coding For Screen Content”, JCTVC-O1124, Geneva, Switzerland, Oct. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).