| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0277 |
| --- | --- |

| Title: | Motion Vector Resolution Control for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | You Zhou Bin Li Jizheng Xu Gary J. Sullivan Bruce Lin | Tel: Email: | +1 425 707 1170 youzhou@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

It is observed that when encoding video content captured from screens, most of the spatial displacements are full pixel displacements – few of them are at fractional pixel positions. This document proposes and reports performance measurements for disabling fractional pixel motion compensation for screen content coding. Considering the fact that no bits need to be spent on the fractional parts of motion vectors for such content, the proposed scheme can reduce the bit rate needed to encode such content. The proposed modification does not change the syntax or parsing process (except for adding a control indication at the header level), but modifies the decoding process by interpreting the motion vector differences as integers and rounding scaled MV predictors to integer values. The modification is shown to improve coding efficiency as much as 7% for screen content sequences (averaging 2% for the test set of YUV and RGB 4:4:4 screen content test sequences), and can also reduce the complexity of the encoding and decoding processes.

# Introduction

The idea is based on our observation that when encoding video that is captured from desktop, most of the motion vectors in the encoded bitstream point have integer values; very few of them are at fractional positions. While encoders normally interpret motion vectors in bitstreams as ¼ pixel offsets, we can often save bit rate by encoding these motion vectors for screen coding applications as 1 pixel offsets, reducing the precision of the motion vectors by a factor of four, and thereby reducing the number of bits needed to code them. Considering that fractional motion vector values are still very useful for camera-captured content, we propose to enable encoders to select the motion vector precision in a flexible way: when the video source is from a captured screen and there is no fractional pixel motion we transmit the motion vector in unit of 1 pixel and for camera-content video we still transmit motion vectors with fractional precision.

# Detailed proposal description

We propose to add a high-level indicator (at the SPS, PPS, and/or slice header level) to indicate the resolution for interpretation of the motion vectors.

## The encoding process

Before encoding a slice, encoder may decide the motion vector resolution based on historical statistical data, knowledge of its type of application, multi-pass analysis, or some other such technique. If the encoder decides to use full pixel motion only, the fractional pixel search is skipped. If a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value.

## The Decoding Process

If the motion vectors are indicated to be at full pixel resolution and a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value. Motion vector differences are simply interpreted as integer offsets rather than 1/4-sample offsets. All other decoding processes remain same. The parsing process (below the header level) is unchanged.

When the motion vectors are coded at full-sample precision and the video uses 4:2:2 or 4:2:0 sampling, the chroma motion vectors can be derived in the usual manner, which will produce 1/2-sample chroma motion displacements. Alternatively, it may be worth considering also rounding the chroma motion vectors to integer values.

## Syntax Change

We propose to transmit a two-bit indicator, namely motion_vector_resolution_control_idc, in the PPS extension to indicate the motion vector resolution control modes. Three modes are defined. When the mode is 0, the motion vectors are encoded at ¼ pixel precision and all decoding processes remain unchanged. When the mode is 1, all of the motion vectors in the slices that refer to the PPS are encoded at full pixel precision. And when the mode is 2, the motion vector resolution is controlled on a slice-by-slice basis by a flag in slice header. When the motion_vector_resolution_control_idc is not present, its value is inferred as 0.

When motion_vector_resolution_control_idc is equal to 2, an additional flag called slice_motion_vector_resolution_flag is signaled in the slice header. When the flag is zero, the motion vectors of this slice are encoded at ¼ pixel precision, and when the flag is 1, the motion vectors are encoded at full pixel precision. When the flag is not present, its value is inferred as equal to the value of motion_vector_resolution_control_idc.

The modified PPS syntax is illustrated as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| output_flag_present_flag | u(1) |
| num_extra_slice_header_bits | u(3) |
| sign_data_hiding_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| … |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension1_flag | u(1) |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| luma_chroma_prediction_enabled_flag | u(1) |
| motion_vector_resolution_control_idc | u(2) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| … |  |

The modified slice header syntax is illustrated as follows:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( motion_vector_resolution_control_idc = = 2) |  |
| slice_motion_vector_resolution_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| … |  |

# Simulation results

We tested the proposed method on several desktop-UI screen content video sequences with 4:4:4 chroma format. The anchor for comparison was an unmodified HM encoder that performs fractional-position search. The results are shown in the following table.

|  | BD-rate (piecewise cubic) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SlideEditing_1280x720_30.yuv | -0.4% | -0.5% | -0.5% |
| sc_web_browsing_1280x720_30_8bit_300_rgb.rgb | -1.3% | -1.2% | -1.2% |
| sc_wordEditing_1280x720_60_8bit.rgb | -0.5% | -0.4% | -0.5% |
| sc_cad_waveform_1920x1080_20_8bit_200_rgb.rgb | -0.6% | -0.6% | -0.5% |
| sc_pcb_layout_1920x1080_20_8bit_200_rgb.rgb | -0.5% | -0.6% | -0.5% |
| sc_cg_twist_tunnel_1280x720_30_8bit_rgb.rgb | -7.1% | -6.9% | -6.9% |
| sc_cad_waveform_1920x1080_20_8bit_200_444_r1.yuv | -0.7% | -0.7% | -0.6% |
| sc_pcb_layout_1920x1080_20_8bit_200_444_r1.yuv | -1.3% | -1.3% | -1.3% |
| sc_cg_twist_tunnel_1280x720_30_8bit_300_444_r1.yuv | -6.5% | -5.3% | -5.1% |

Obviously, the proposed method does not work for typical camera-content video or some animation content. It is not difficult to come up with an algorithm for encoder to turn on or off the proposed method. Here we simply manually turn it on or off on a whole-sequence basis, and the results are shown in the following tables.

| BD-rate Y | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- |
| Class F | -1.1% | -0.8% | -1.3% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.3% | -0.5% | -0.8% | -1.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.1% | -0.3% | -0.4% | -0.8% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% |
| SC(444) GBR Optional | -2.7% | -2.9% | -4.4% | -4.1% |
| SC(444) YUV Optional | -2.8% | -2.8% | -4.8% | -4.4% |

|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.1% | -0.9% | -1.0% | -0.8% | -0.8% | -0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.3% | -0.3% | -0.3% | -0.5% | -0.5% | -0.5% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | 0.0% | 0.0% | -0.3% | -0.2% | -0.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -2.7% | -2.7% | -2.6% | -2.9% | -2.8% | -2.8% |
| YCbCr 4:4:4 SC (Optional) | -2.8% | -2.4% | -2.4% | -2.8% | -2.6% | -2.6% |
| Enc Time[%] | 85% |  |  | 88% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.3% | -1.2% | -1.2% | -1.1% | -1.1% | -1.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.8% | -0.7% | -0.8% | -1.1% | -1.0% | -1.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.2% | -0.3% | -0.8% | -0.7% | -0.8% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -4.4% | -4.2% | -4.3% | -4.1% | -3.7% | -3.8% |
| YCbCr 4:4:4 SC (Optional) | -4.8% | -4.4% | -4.6% | -4.4% | -4.2% | -4.3% |
| Enc Time[%] | 82% |  |  | 85% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# Conclusions

We propose to enable the encoder selection of full-pixel precision instead of quarter pixel precision for motion vectors. It is useful for videos captured from desktop. The bit rate savings has been shown to range from 0.4% to 7.1% for screen content video.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).