| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0278 |
| --- | --- |

| Title: | AHG8: Cross-check report of Sample adaptive offset with multiple parameters (JCTVC-P0109) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-P0109, “AHG8: Sample adaptive offset with multiple parameters”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Runtimes however were much higher, from 103% to 105% for encoder, and 106% to 108% to decoder. These runtimes are not surprising considering the number of additional SAO passes performed.

# Study of the proposal

The proposal intent is to apply SAO several times, each pass occurring on all LCUs before the next pass occurs.

When SAO is activated, it adds a flag at the RExt SPS extension level to indicate whether multiple passes are performed. If this flag is present and activated, then:

- It adds at the slice level the following additional parameters:

  - The number of SAO passes performed;

  - For each pass, flags to indicate respectively whether luma and whether chroma SAO filtering are activated.

- Per LCU, for each pass, the SAO syntax is sent as usual (depending on the activation of the luma or chroma filtering).

The default configuration evaluated is:

- 4 passes per intra slice;

- 2 passes per inter slice.

Changes not related to the proposal were observed, and the proponents indicated that these changes corresponded to SAO fixes from JCTVC-N0230 and updated accordingly their contribution with the results of JCTVC-N0230 alone. Departing from RExt5.1 normal test conditions, the software disables the SAO cleanup (HM_CLEANUP_SAO macro) and the backward compatibility ticket 1159, and contains the following changes from JCTVC-N0230:

- SAO picture-level Rate/Distortion optimization (SH_SAO_RD_PIC)

  - This disables the SAO encoding choice from JCTVC-I0184 (picture early termination);

- merge when slice-level SAO luma flag is false (SAO_BUGFIX_LUMA_MERGE_FLAG macro);

- Rate calculation fix for RDO (SAO_BUGFIX_RDO_RATE macro);

- EO 0 statistics calculation on boundary (SAO_BUGFIX_EO_ZERO_BOUNDARY macro).

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the lossy test conditions of AhG8, using HM12.1+RExt5.1 software as anchor.

Those results match the ones reported by the proponents in Table 2 of the r2 revision of their contribution.

# Conclusion

This contribution reports cross-verification results on contribution JCTVC-P0109, “AHG8: Sample adaptive offset with multiple parameters”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Runtimes however were much higher, from 103% to 105% for encoder, and 106% to 108% to decoder. These runtimes are not surprising considering the number of additional SAO passes performed.