| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0279 |
| --- | --- |

| Title: | Non-RCE3: Cross-check of fast encoder search for RCE3 Subtest B.3 (JCTVC-P0151) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-P0151, “Non-RCE3: Fast encoder search for RCE3 Subtest B.3”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. Similar tendency on encoding and decoding time was also observed.

# Study of the proposal

Performance verification on JCTVC-P0151 was conducted. The verified method proposes a fast encoder search for NxN Intra Block Copy PUs. The proposal describes correctly the changes implemented, i.e.:

- Reuse of the SAD of the 4x4 blocks for a 8x8 CU;

- Skip NxN search:

  - if cost already low;

  - 8x8 2D search is not active;

  - The 4 NxN MVs inside an 8x8 CU are identical.

Overall, the proposal and RCE3 Subtest B.3 add more than 1300 lines to RExt5.1 software, mainly because it seems to duplicate the Intra Block matching for 4x4 PUs. Reducing this duplication would be advisable from a software perspective.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the test conditions of RCE3. The software was not compiling at first under the aforementioned environment (variable shadowing), and the authors suggested changing the compilation options, which was successfully.

The BD Rate results in this contribution match those of the proponents. The decoding times are slightly higher than reported by the proponents of JCTVC-P0151, but:

- The proposal is encoder-only;

- The proponents’ decoding time are closer to those of RCE3 Test B.3, which would more likely indicate runtime instabilities on our system.

Concerning the results:

- For lossless:

- For lossy:

# Conclusion

This contribution reports cross-verification results on a fast encoder search for RCE3 Subtest B.3. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. No major concern were raised about the software implementation.