| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0280 |
| --- | --- |

| Title: | Non-RCE4: Cross-check of Major color table (palette) sharing (JCTVC-P0153) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-P0153, “Non-RCE4: Cross-check of Major color table (palette) sharing”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent.

# Study of the proposal

Performance verification on JCTVC-P0153 was conducted. The verified method proposes to share CU-wise major color tables in the scope of RCE4 Test1 method. This work was conducted using modified HM12.1+RExt5.1 software provided by the proponent.

An additional change implied by the proposal has been noticed. When the “horizontal” or “no prediction” coding modes are used on a line of indexes, explicit indexes are transmitted using a number of bits depending on the color. This change consists in changing the number of bits for some of those colors (differences highlighted):

UChar bitNum[16] = {0,1,1,2,2,3,3,3,3,4,4,4,4,4,4,4};

to

UChar bitNum[16] = {0,1,2,2,3,3,3,3,4,4,4,4,4,4,4,4};

The proponents explained that the color tables from previous CUs may not cover all indices of the current one, therefore requiring the use of escape values that were previously used in fewer cases.

It was also noted that the encoder actually performs 2 evaluations, one being the regular RCE4 Test1, and a last one using the stored last major color tables.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the test conditions of RCE4. Table 1 and Table 2 show the results of the proposed scheme compared with RCE4 anchors for lossy and lossless configurations, respectively. The JCTVC-P0153 proposal reports results instead using RCE4 Test1 as anchor, but the BD rate results do perfectly match the results from the proponent. The encoding runtime measurements show an increase that probably corresponds to running the second estimation with stored last major tables. The decoding runtimes are less trustworthy as a higher instability has been observed on our system.

Table 1: results for lossless encoding

Table 2: results for lossy encoding

# Conclusion

This contribution reports cross-verification results on a variant of RCE4 Test1 that proposes the reuse of last major color tables. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches the results from the proponent. The slight encoding time increase can probably be attributed to the proposal running two evaluations of the palette technique instead of one in RCE4 Test1.