| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0281r1 |
| --- | --- |

| Title: | Non-RCE4: Cross-check of Palette prediction for palette coding (JCTVC-P0160) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-P0160, “Non-RCE4: Palette prediction for palette coding”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. The results are only partial due to multiple delays, but do match the proponents’ results.

# Study of the proposal

The concept of the proposal is to look up a previously palette-coded CU to reuse for the CU being encoded, thereby requiring the buffering of several palettes. Test A.2 basically performs a horizontal search (confirmed to be doing so in the software) and stops at the first CU found or as soon the previous palette is more than 1 CTB away in raster scan order, whichever happens first. This causes a worst case storage of at least 768 bytes, if we consider that one palette per existing vertical position can be available, thus 8 possible palettes.

Implementation of Test A.2 in RCE4 Test2 (JCTVC-P0198) was the first implementation studied. After we reported a mismatch in both the proposal studied and the results, the proponents provided us with a bugfixed code, arriving after the JCT-VC meeting start.

Implementation of Test A.2 in HM12.0+RExt5.1+JCTVC-O0218 was actually implemented on top of Test1 from P0113, with the proposal’s code disabled for the purpose of Test A.2 experiment. Again, little reviewing was effectively possible. It was in particular not feasible to establish whether the code disabling had unexpected consequences.

Effectively, the latest software for each implementation on which the simulations employ identical searches. The search code of proposal’s TestA.2 is confined to the function TComDataCU::getPLTPred (JCTVC-P0198 implementation) or TComDataCU::searchPrePLT (JCTVC-O0218 implementation). The JCTVC-O0218 implementation signals the use of prediction through a CABAC-coded flag, using a new context, while the JCTVC-P0198 simply modifies the existing palette predictor derivation.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the test conditions of RCE4. Only test A.2 of the JCTVC-P0160 proposal has been evaluated. There are 2 sets of results:

- JCTVC-P0160 implementation on top of RExt5.1 + JCTVC-O0218, using it as anchor for both lossless (cf. table 1) and lossy (cf. table 2) configurations;

- JCTVC-P0160 implementation on top of JCTVC-P0198, using it as anchor for both lossless (cf. table 3) and lossy (cf. table 4) configurations.

For Table1, only RA and AI lossy configurations are completed, but the results do match the proposal and the sequences are decoded correctly.

Table 1: results for JCTVC-O0218 + JCTVC-P0160, test A.2 for lossless configurations

For Table2, a few sequences in the AI configuration are finished, but the results do match the proposal, and these sequences are decoded correctly.

Table 2: results for JCTVC-O0218 + JCTVC-P0160, test A.2 for lossy configurations

Table 3: results for RCE4 Test2 + JCTVC-P0160, test A.2 for lossless configurations

Table 4: results for RCE4 Test2 + JCTVC-P0160, test A.2 for lossy configurations

- Conclusion

This contribution reports cross-verification results on contribution JCTVC-P0160, “Non-RCE4: Palette prediction for palette coding”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. The results are only partial due to multiple delays, but do match the proponents’ results.