| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0286 |
| --- | --- |

| Title: | Non-RCE4: Cross-check of JCTVC-P0249 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-P0249, “Non-RCE4: A combination of the four-neighbor major color index prediction in JCTVC-P0098 and a simplified transition copy mode from JCTVC-P0115 on top of RCE4 Test1”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. BD rate results matched, while runtimes were inaccurate during those experiments and should not be considered.

# Study of the proposal

In the “Normal” prediction mode for a line, JCTVC-P0098 proposes to add 4 spatial prediction candidates. This contribution adds a new candidate inspired from JCTVC-P0115, corresponding to the last transition observed.

For each possible index, its last occurrence is stored, in the shape of the index immediately following it. Therefore, the buffer size required is 16*4=64 bits. When processing an index, one candidate is obtained by using the one stored for the value of the index on the left of the index to process. To encode/decode said candidate, a CABAC-coded flag is added between the prediction direction and the signaling of the above-left/above-right candidate. It uses an additional context compared to P0098.

The implementation is an independent yet similar version of JCTVC-P0115. The proposal however performs more bookkeeping to check and use the availability of the transition candidate:

- If it is equal to the left or top candidate, it is marked as unavailable;

- If the above-left or above-right candidates are equal to any of the left, top or transition candidate, they are marked as identical.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the test conditions of RCE4. Table 1 and Table 2 show the results of the proposed scheme compared with RCE4 anchors for lossy and lossless configurations, respectively. The JCTVC-P0153 proposal reports results instead using RCE4 Test1 as anchor, but the BD rate results do perfectly match the results from the proponent. The encoding runtime measurements show an increase that probably corresponds to running the second estimation with stored last major tables. The decoding runtimes are less trustworthy as a higher instability has been observed on our system.

Table 1: results for lossless encoding

Table 2: results for lossy encoding

# Conclusion

This contribution reports cross-verification results on contribution JCTVC-P0249, “Non-RCE4: A combination of the four-neighbor major color index prediction in JCTVC-P0098 and a simplified transition copy mode from JCTVC-P0115 on top of RCE4 Test1”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. BD rate results matched, while runtimes were inaccurate during those experiments and should not be considered.