| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0288 |
| --- | --- |

| Title: | BoG report on Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | BoG report |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | C. Rosewarne K. Sharman | Email: | chris.rosewarne@cisra.canon.au karl.sharman@eu.sony.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

A break out group was held between Thursday the 9th and Saturday the 11th of January 2014 to review input documents relating to the Range Extensions.  Specifically, topics on RCE1 on entropy coder throughput, RCE2 on Rice parameter initialization and RCE3 on intra block refinement and other technical contributions were discussed.

In addition, contributions indicated that RExt is giving significant compression benefits over AVC High 444, with around to 8:1 compression for screen content, and around 30% for AI, and 40% to 50% for inter.

This report conveys a number of recommendations from the BoG.  These recommendations are summarised in section 1.  Throughout the report, the expression "Recommendation:" is used to express the consensus in the group when a positive action should be taken.

# Recommendations

The following recommendations are made by the BoG:

RCE1:

- Recommend option B3a (switch from bypass to raw coding if any coeff_abs_level_remaining codeword is present) for 16-bit profile with a super tier.

RCE2:

- Request plenary discussion in light of interaction of tested approaches with new AHG18 lossless test results.

RCE3:

- Padding approaches for intra block copy are not necessary.

- Initialization of the intra block copy vector predictor is left unchanged [changed to –W, 0 intializer in plenary].

- The search area for intra block copy is not adjusted.

- Recommending adoption of either of: “NxN only” or “NxN/2NxN/Nx2N” partitioning for iintra block copy.

  - An RCE is established if NxN intra block is adopted (i.e. either of the above options) to examine block vector coding and binarization, to include P0149.

(Note: RCE4 was discussed in the main track)

Others:

- Review the remaining 2 contributions under “Source video test material”.

- Recommending establishing a CE to study worst case code lengths for coeff_abs_level_remaining for high bit-depths and complexity aspects. [P0061]

- Adopt P0066 on bit depth correction for CCP.

- Discuss SEI for FPA at the plenary level

- Add lossless AHG18 to the test conditions

- Adopt change to SAO for high bit depths with an SPS flag to control. Plenary is requested to consider whether slice level control of luma/chroma independently is also warranted.

- Form a CE/AHG to examine encoder configuration for CCP, regarding two methods (predict cr from cb, and use of a programmable LUT) [CE still needs to be properly defined]

- Form a CE/AHG for motion vector precision [CE still needs to be properly defined]

- Adopt an additional context for CCP alpha signalling.

- Recommend further study on encoder searching following previous changes for CCP.

- Request plenary to revisit P0044 on high/super tier and MinCR.

- Plenary requested to consider moving of the rotation and single context enable from the SPS to the PPS conditional on transquant bypass or transform skip being enabled and request plenary and/or HLS experts to review tool control flag allocation between PPS/SPS [P0166].

# RCE contributions

## RCE1: High bit rate coding at high bit depths (9)

### RCE1 summary and general discussion

JCTVC-P0032 RCE1: Summary report on HEVC Range Extensions Core Experiment 1 (RCE1) on high bit rate coding at high bit depths

A.1 Bypass after significance map (i.e. gr1, gr2, …) if Rice parameter of previous sub-block >= 4.

A.2 Bypass all ctx bins for current sub-block if previous sub-block Rice parameter was >= 4.

A.3 Bypass all the context-coded flags in a transform block if the Golomb Rice parameter value at the end of the previous 4×4 subblock is greater than or equal to 4. In that case, the last significant coefficient position was sent as (blkSizeX – lastX, blkSizeY – lastY).

A.4 All bins bypassed.

If bypass bins are aligned in the bitstream, they can be read out more easily.  Such aligned bins are referred to as ‘raw bins’.  Two alignment methods were tested:

Method 1: Align bypass using range = 256.

Method 2: Align bypass using range = 256 or 384.

B.1: Always align to 256 before sign coding.

B.2: Align to 256/384 before sign coding.

B.3: Align to 256 if and coeff_abs_level_remaining present in current sub-block.

B.4: Not exist.

B.5: A.1 with 256 alignment.

B.6: A.2 with 256 alignment.

B.7: A.3 with 256 alignment.

Reported worst case analysis studied the maximum counts of context, bypass and raw bins per sub-set averaged over two consecutive sub-sets. Averaging over two sub-sets was performed to capture the impact of the adaption applied and not applied cases.

Results and reported worst case analysis:

|  | AHG18 average | Ctx bins | Bypass bins | Raw bins |
| --- | --- | --- | --- | --- |
| A.1 | -0.1 to 0.1 | 20.5 (average) | 647 | 0 |
| A.2 | -0.1 to 0.2 | 12.5 (average) | 647 | 0 |
| A.3 | 0.8% losses, a few multiple % losses | 12.5 (average) | 647 | 0 |
| A.4 | A few % for SVT and 5-20% for 12-bit. | 0 | 0 | 647 |
| B.1 | 0.8% losses | 25 | 0 | 647 |
| B.2 | 0.5% losses | 25 | 0 | 647 |
| B.3a | 0.6% losses | 25 | 16 (unaligned) 0 (aligned) | 0 (unaligned) 647 (aligned) |
| B.5 | 0.2% losses | 20.5 | 323.5* | 323.5* |
| B.6 | 0.2% losses | 12.5 | 323.5* | 323.5* |
| B.7 | 0.7% losses | 12.5 | 323.5* | 323.5* |

* First sub-block all bypass and second sub-block all raw bins.

Comment: For AHG8 lossless conditions, test A.1 had 0.1-0.2% losses mostly, for A.2 and A.3 for SC losses up to 10%, but for RExt negligible losses.  For AHG5 conditions, A.1 and A.2 have <0.1% change and A.3 has 0.1-0.2% (0.5% in SHT) losses.

Observed that as ctx bins are removed, the measured bypass bin count does not increase much, due to coeff_abs_level_remaining base decreasing e.g. from 3 to 0.

Worst case analysis (A.1-A.3, B.5-B.7):

When using the Rice parameter from the previous sub-block to change behavior, then the worst case for the previous sub-block is unchanged. If looking at two consecutive sub-blocks, then a worst case complexity reduction is observed because the second sub-block has a reduced worst case.

For each sub-set, the decision to adapt is made using a previous sub-set (which could belong to a different TU or CU or CTB, i.e. only break dependency at slice level) and applied to a current sub-set.

Comment: There appear two broad aspects to the tests conducted - At which point to switch from ctx bins to bypass bins, at which point (if any) to switch to raw bins.

The raw bins appear most applicable to high bit-rates, e.g. as observed in AHG18 conditions.

Also tested A.1 + B.3 (very constrained number of bypass bins): has advantage of reducing average case ctx bin count.

Do we have any idea how much the throughput is increased overall from this numbers? Throughput means accommodating some maximum number of bits.

Even if the bit-rate is increased slightly, if that is due to conversion of bypass bins to raw bins then the number of bins processed per cycle can increase significantly.

To decode bypass bins you need to do serial subtraction, so several bins per cycle is achievable.

Raw bins are read directly from the bitstream, so you can process much higher bins per cycle.

Comment: Switching from ctx to bypass bins increased throughput e.g. from 1-2 bins/cycle to 4 bins/cycle in some architecture.

Comment: Residual coefficient magnitudes are much higher at the AHG18 operating point.

Condition to switch is any residual coefficient magnitude exceeding 64.

Comment: Over what period of time do you measure the worst case. What is the time-frame over which the averaging happens? CTB is one option. If you keep in mind the number of bits per CTB already has a limit. Even within one CTB you could have localized nasty cases (e.g. among several sub-blocks). So, should take into account the maximum expansion ratio defined for one CTB.

To avoid that threshold condition, a CTB of all 63 magnitude coefficients could be used.

Recommendation: Revisit all, with analysis of worst case data over a CTB for A1-3, B5-7, considering RawCtuBits limit.

Subtest A.1/B.5 worst case complexity analysis:

Study of A.1 is beneficial considering the bits per CTU expansion limit.

For 16-bit, one 4x4 sub-block may not exceed (on average over the CTU) 425 bits.

Assume one bin is one bypass because most are bypass.

First sub-block could be hard to code, but threshold of A.1 not present, then second sub-block easy to not hit the limit for the third (hard) sub-block. (alternating hard and easy sub-blocks are needed to hit the worst case with A.1).

750 bins per pair of sub-blocks.

With maximum Rice parameter of 10, 647 worst case reduced by 48 bins per sub-block. Other proposals to cap maximum length of coeff_abs_level_remaining to 32 also reduce the worst case. With the 32-bit restriction, the maximum is ~600 bins, so the expansion factor reduced to 1.2.

A.1 with B3a: If any coeff_abs_level_remaining syntax element, then align to 256 in CABAC.

Proponent of B3a asserts a hardware benefit here due to less bypass bins.

Under what conditions should be enabled? Super tier is one option. No need for action at other tiers because they already have to support the existing design.

For B3a, the sign bins could be either bypass or raw, depending on whether any coeff_abs_level_remaining syntax elements were coded.

A.1 and B3a give the same performance.

A.1 is to conditionally bypass the gr1 and gr2 flags depending on the Rice parameter at the end of the previous sub-block.

B3 is conditionally aligning to 256 (i..e bypass to raw) for the coeff_abs_level_remaining elements and any sign bits.

The case of alignment when only two bypass bins is possible. A 0.6% loss in efficiency in AHG5 test conditions.

The real issue appears to be the sheer number of bins to be processed.

One suggestion is to align after a bypass bin count threshold is reached. A high threshold can be set because at that point you are still far from the worst case.

B.5 is to combine A.1 with 256 alignment on the same condition (previous Rice parameter >= 4).

Are the gains being presented under the present test conditions meaningful, considering for the operating point being discussed?

The proponent asserts that hardware can decode two coeff_abs_level_remaining per cycle if they use raw bins.

Want to reduce the worst case locally not over picture and sustain the data rates of the proposed super tier.

B3 was considered two meetings ago for adoption unless alternative ideas were put forward that should also be considered.

The condition under which the switch to raw bins happens is still contentious. If a super tier is adopted, then there appears to be a problem.

This proposal is only applicable to 16-bit profiles.

The small gains between the B options are not going to be a deciding factor for professional application.

A simple approach: switching all bypass bins to raw bins, or switching to raw when a coeff_abs_level_remaining is encountered. Between B1 or B3a. Risk of increasing losses at higher QPs with B1.

Recommendation: For the proposed 16-bit profile with super tier, B3a would be highly beneficial.

### RCE1 primary contributions (3)

JCTVC-P0060 RCE1: Results for tests B1, B2 and B3a [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-P0073 RCE1: Results for subtest A [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-P0074 RCE1: Results for subtests B5, B6, B7 [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

### RCE1 cross checks (6)

JCTVC-P0077 RCE1: A crosscheck report on subtest B.1 [R. Joshi (Qualcomm)] [miss]

JCTVC-P0194 RCE1: Cross-verification of subtest A.2 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0195 RCE1: Cross-verification of subtest B.6 [S. Lee, C. Kim (Samsung)] [miss]

JCTVC-P0220 Cross-check report of 'RCE1: Results for subtest A' test A4 (JCTVC-P0073) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0221 Cross-check report of 'RCE1: Results for subtests B5, B6, B7' test B5 (JCTVC-P0074) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0245 RCE1: Cross-verification of subtest B.7 [S.-H. Kim, A. Segall (Sharp)] [late] [miss]

### RCE1 related

JCTVC-P0075 non-[RCE1, RCE2]: Combination of RCE1 subtests B5 and B6 with RCE2 subtest A1 [R. Joshi, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

The presenter confirms that there is no need to present this contribution.

## RCE2: Rice parameter initialization (5)

### RCE2 summary and general discussion

JCTVC-P0033 RCE2: Summary report on HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialization [C. Rosewarne, M. Karczewicz, K. Sharman, S.-H. Kim]

Test 1 - disable O0206 update method from last time from last time. Software retains the maximum of 7 (in previous meeting the maximum rice parameter could reach MAX_TR_DYNAMIC_RANGE).

Gains in AHG5 and AHG8

Test 2 – In addition to the proposed initialization method, the maximum Rice parameter was increased from 7 to 10.

No gains in AHG5 and AHG8.

Both test 1 and test 2 have gains for AHG18.

No results presented from just increasing the Rice Parameter limit to 10, as used in test 2.

Why is O0206 (fast update of Rice parameter and no-reset between sub-blocks of a TU) disabled in Test1? The two proposals are competing for the same gain.

How much gain is achieved by increasing Rice parameter limit, rather than any of the changes being proposed? If we left the maximum at 7, what are the gains then for AHG18 results?

Why is there coding loss for A1 for 4:0:0 medical sequences? The number of conditions and sequences has increased, and there is one class (2 sequences) that is causing problems.

Is there a result with the anchor software modified with the maximum Rice parameter of 10? No, not available.

How many changes are required for test A1, complexity and syntax? Same as before. One statistic update per sub-block.

Why is limit currently 7 (9 was discussed). 9 was proposed, but 7 chosen as it was a compromise for all the test conditions. O0206 has maximum of 9 – comparing results of 7 and 9, 2% is possibly accounted for 16-bit SVT sequences by change in max rice parameter.

Higher values better for 16-bit only (observation from experiment).

Run A1 with max of 10? Why 10? 7 is a good trade off.

Can we generate results for RExt5.1 maximum Rice parameter of 10?

Qualcomm to generate results for RExt5.1+max rice param=10 and Test1+max rice param=10.

Options:

- Test 1

- Test 2

- Increase max rice parameter to 10 – but when? All conditions?

- Do nothing

From last time, between 4 and 7, not much difference except for AHG18.

Do we need to discuss complexity? 4 statistic counters that get updated once per sub-block (encoder and decoder side).

Revisit with results of maximum Rice parameter of 10. Results of Test A1 with Max=10 showed gains in SVT but losses were observed for the medical sequences.

Test A.1 with the Rice parameter clipping is 0.3% worse than the Nokia proposal for lossless coding of SVT sequences. With maximum Rice parameter of 10, the losses (compared to Nokia) are 2.2%.

Results presented with the maximum Rice parameter set to bit-depth (only change relative to RExt5.1)

With no clipping in place, the maximum Rice parameter rarely went above bit-depth. There is a natural limit of MAX_TR_DYNAMIC_RANGE. The clip does not add anything in terms of performance and does not give a meaningful complexity reduction.

Concern that these results are still worse than those presented by Nokia.

Increasing RExt5.1 max from 7 to 10 closes the gap with Nokia from 14.7% to 2.x%. But max of 10 gives lossless losses of 1% in class F, RGB 4:4:4 SC 2.4%, 2.3%, 2.0% in AI, RA, LB and YCbCr 4:4:4 SC of f1.5%, 1.1%, 1.0%.

Just increasing the maximum to 10 introduces losses under AHG8 lossless conditions in some classes ranging from 1-2%, so this option is ruled out.

Test A.1 with no clipping closes the gap with Nokia anchor from 14.7% losses to 0.3% losses for SVT sequences.

The Nokia anchor is with test A.1 included (used for each 8-bit path).

Fast update method asserted to be unstable because it is missing a log2 operation, making it equal to a scaled version of the value rather than of the lo2 of the value.

Concerns being expressed over whether this particular solution is the best one to the problem. New test conditions not well studied by other experts. Solution on the table for several meeting cycles though.

The fast update is applied between each adjacent instance of coeff_abs_level_remaining (the fast update is more complex than the method it displaced, so that added complexity is removed, but the initialisation which occurs once per sub-block becomes more complex).

SVT sequences have noise, so results on these sequences alone may not be sufficient to reach a conclusion.

If an RCE is conducted, this should be used as an anchor.

Recommendation: Revisit in plenary.

### RCE2 primary contributions (2)

JCTVC-P0168 RCE2: Test A2. Rice parameter initialization based on quantization parameter and bit-depth [S.-H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-P0199 RCE2: Results of Test 1 on Rice Parameter Initialization [M. Karczewicz, L. Guo, J. Sole (Qualcomm), K. Sharman, N. Saunders, J. Gamei (Sony)]

### RCE2 cross checks (3)

JCTVC-P0210 RCE2: Cross-check of test A2 (JCTVC-P0168) [L. Guo (Qualcomm)]

JCTVC-P0219 Cross-check report of ' RCE2: Results of Test 1 on Rice Parameter Initialization' (JCTVC-P0199) by Qualcomm and Sony [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0236 RCE2: Verification for Test 1 on Rice Parameter Initialization [E. Alshina (Samsung)] [late]

## RCE3: Intra block copy refinement (19)

### RCE3 summary and general discussion

JCTVC-P0034 RCE3: Summary report on HEVC Range Extensions Core Experiment 3 (RCE3) on Intra block copy refinement [J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng]

Test A: Search Area vs. Performance

Tests 1-4 uses region of 4, 8, 16, 32 pixels left of the current CTU (and includes current CTU) instead of 64 pixels to the left as used by the anchor.

Test 5 uses current CTU + 4 columns + above 4 rows; the above 4 rows may be available, depending on design.

All tests used the fast search configuration.

As search area increased, so did coding efficiency.

If the search area is increased beyond that used by the anchor, coding efficiency gains increase over RExt5.1, although there are no results of this presented within the scope of the RCE. It was indicated that JCTVC-P0200 includes a trial of larger areas during a complexity analysis of Intra BC (not presented at this time).

It would have been advantageous to see what happens if larger areas were used – allowing the trade-off between complexity and coding efficiency to be fully assessed.

Recommend: no action (no change for now for search area).

Test B: block partitioning

Currently only 2Nx2N intra BC tested (with CU level prediction).

B1: Nx2N/2NxN Intra BC. CU level prediction.

B2 was withdrawn from the RCE but presented as a Non-CE (P0218), due to late submission to RCE.

B3: NxN PU with TU level prediction.

B4: Sample masking (partial block copy, but filling in a region of 1, 2 or 3 columns/rows (left, right, above, top) to a constant value based upon a sample.

B1 & B3 show similar gains (eg -4%, -9%, -9% for AIMT Class F, SC 444 RGB & YUV respectively; -1.5%, -5%, -5% for LBMT); test D2 shows the gains are additive.

It was asked whether NxN can be achieved with CU level prediction (i.e.  no prediction between the TUs within the same CU)? It was responded that this is not advantageous.

Nx2N/2NxN can be done with and without TU level prediction.

It was asked what is the BD-rate difference between TU and CU level prediction? This was examined in B2 for 2Nx2N.

It was asked what the gains are if we could not reference from a TU just reconstructed within the PU for B3? (i.e. PU-level prediction?) It was questioned whether the answer to this would change our decision – the worst case is the same as for intra. It was stated that if this dependency was not present, then reconstruction of 4x4 blocks can be done simultaneously; if BD-rate doesn’t change much, then it might help for parallelization.

Encoder side – 4x4 is harder – but there is a choice to not do this.

Should intraBC be more like intra or inter, in terms of blocks? Last time we were pushing IntraBC to be more like intra. Nx2N split is not in the intra design.

Overall, results of the sub-tests all look similar. However, the combination of NxN and 2NxN gives a few percent gains, but encoder run-times increase. Test D2b improves speed over D2, by restricting when the NxN is searched, based upon the result of the 2NxN search.

4:2:0 chroma – are there 2x2 blocks? Yes, there are. Should 1 vector be used for a 4x4 area, not 4 vectors for a 2x2 sample area?

This led to the summarizing of the concerns:

- Should 1 vector be used for a 4x4 area, not 4 vectors for a 2x2 sample area (for NxN, 4:2:0; similar for 4:2:2) ?

- Should 1 vector be used for a 4x4 area, not 2 vectors for a 2x4/4x2 area (for 2NxN, 4:2:0; similar for 4:2:2) ?

- TU-level prediction for NxN was tested; how essential is the TU-level prediction to the results (would a CU level prediction produce similar results) ?

If we were to go for 1 change, NxN looks most in-keeping with current design methodology (i.e. similar to intra processing). Should we go for Nx2N as well? Difference between B3 and D2.b – 1-2% improvement, +10% encoder run-time for intra, small increase in run-time for randomaccess, similar improvement.

Should Nx2N be done for intra-only configuration or just for inter? Could we add a SPS flag/constraint?

It was commented that encoders should be given the option of 2NxN and NxN partitioning.

Lossless gains of D2b over B3 are <1%, but large increase in run-times +50%.

NxN looks promising although there is a requirement for some further work (see above list of concerns) –revisit later this week (B3).

Nx2N further work (see above list of concerns, and also statement of lossless run-time) – potentially revisit later this week (D2b).

Tests run are the following:

RCE3 B3, with 4x4 block size for chroma, previous cases with and without TU reconstruction

Using the lower right 4x4 luma block vector for the 4x4 chroma block vector.

What happens in the 4:2:2 case? In this case, the top two 4x4 luma blocks are collocated with one 4x4 chroma vector. For each pair, the block vector of the right luma block is used for the corresponding chroma block.

The TU level prediction process on the current (2Nx2N) design was also giving approximately 0.5% gains, similar to the gains observed from TU reconstruction with the NxN option here.

Also observed that to achieve this  gain the motion search has to be done sequentially.

These tools are technically applicable to any chroma format, due to the higher chroma format profiles also supporting lower chroma formats.

Of the NxN options presented here, the best looks to be No TU reconstruction with one vector for 4x4 chroma.

Combinations with 2NxN are being tested (with the 2NxN fast search are being tested.

Contribution JCTVC-P0301 presents further summary of the results and summary of TU/PU partitioning. Further discussion continues in the following section on this contribution.

JCTVC-P0301 On the TU process and PU partitioning in Intra Block Copy [C.-C. Chen, W.-H. Peng] [late]

None of these proposals incorporated changes for chroma blocks in non:4:4:4 chroma formats.

The RCE software was used.

One comment that no effect on worst case with TU reconstruction because it reaches the Intra 4x4 case, but another disputed this.

Suggestion that the specification is clarified that the CU level constraint is clearer.

How does 2NxN interact with deblocking? Do we follow intra or inter deblocking rules? Boundaries to TU level. Nx2N forces an RQT split (i.e. IntraSplitFlag) and so the behavior is OK (i.e. deblocking flags are set correctly).

Currently support for NxN splitting and will revisit to determine if 2NxN should also be present (with fast search interaction).

Incremental gains of adding 2NxN are 1.5% for lossy case. Decoding runtime not much changed (<3%). Lossless encoder runtime increase reduced from 180% to 140% by increasing the bar for doing 2D search.  0.4 to 0.5% gains were observed.

Lossy results:

Lossless results:

Is text available for each of these options? Text for 2NxN with NxN is not currently available because original 2NxN text assumed no NxN.

NxN text incorporating chroma format adaptation is not currently available.

Non 4:4:4 Chroma format block vector adaptation for NxN, 2NxN, Nx2N cases: For 4:2:0 the chroma block vector is the lowest/rightmost of the available block vectors (i.e. regardless of the partition mode). For 4:2:2, imagine splitting the CU into top and bottom halves (i.e. collocated with each 4x4 chroma block) and then applying the 4:2:0 rule in each half. AHG8 conditions include 4:2:0 and 4:2:2 so these cases are tested. No subjective testing was performed.

Recommendation: To adopt one of NxN or NxN with 2NxN.  Regardless of previous decision: No chroma blocks smaller than 4x4.  No TU level reconstruction, only CU level reconstruction, revisit on 2NxN point. Revisit in plenary (text to be checked offline before the revisit).

--

B4

Run-time increase is not noticeable – it utilizes a CU-level calculation, based upon SAD of a few samples, and the calculation has been optimized to minimize the number of SADs calculated.

Results show it is independent on sub-test C (padding) tests; it is affected by B3, but still showing gains.

Is there an interaction with Nx2N? This has not been tested. There should still be gains as this test gives finer control.

For NxN PUs, 4x4 blocks would have syntax.

Is this destabilizing the design? It only affects two screen content for lossless sequences by ~1%, but with negligible run time increase.

Recommendation: no action.

--

SubTest C: Padding/Unavailable samples pre-set

Relax restrictions on block vectors by allowing some level of overlap between reference block and current non-reconstructed samples, in which case, what values do we use in the overlapping areas?

C1 – use mid-grey.

No significant BD-rate or encoder run-times, lossy and lossless.

C2 – CU horizontal padding

Class F, SC RGB/YUV ~0.4% improvement, +2% run-time (due to more searching), no significant BD-rate change for other classes. Lossless similar, but reported run-times (and observed by cross-checkers) getting quicker, possibly because of fast-search.

C3 – CTU horizontal padding – use samples from outside the current CTU. Similar to C2, slightly less gains, slightly more run-time (but still negligible). Lossless similar, slightly less gain (-0.4% to -0.2%), run-times +/-4%.

C4 – padding horizontal or vertically, depending on the vector – CU level. Slightly higher gains for SC RGB/YUV 444 – 0.7%/0.6%, -0.3% for class F, run-time should be similar to C2, but not quite the same search. ~0% elsewhere. Lossless -0.2% for SC 444 only, 0% elsewhere. Run-times negligible changes.

Horizontal padding adds bias towards horizontal features.

Is this solving a problem in the system or is it just adding complexity to a decoder?

Destabilizing. Percentages are not of any significant interest. Not solving a problem.

Padding is not of interest, unless we see larger gains.

Recommendation: No action.

### RCE3 primary contributions (9)

JCTVC-P0053 RCE3: Subtest C.4 - Padding-based generation of unavailable samples in intra block copy [S. Lee, E. Alshina, C. Kim (Samsung)]

JCTVC-P0055 RCE3: Subtest B.4 - Sample masking for intra block copy [J. Lainema, K. Ugur (Nokia)]

JCTVC-P0145 RCE3: Subtest B.3 - Intra block copy with NxN PU [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0146 RCE3: Subtest C.2 - Intra block copy with CU-based padding [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0147 RCE3: Subtest C.3 - Intra block copy with CTU-based padding [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0176 RCE3: Results of Subtest B.1 on Nx2N/2NxN Intra Block Copy [T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0180 RCE3: Results of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy [T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0189 RCE3: Results of Subtest D.1 on Combining Nx2N/2NxN Intra Block Copy with TU process [C.-C. Chen, R.-L. Liao, T.-S. Chang, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou (NCTU/ITRI)] [late]

JCTVC-P0211 RCE3: intra block copy search range (tests A) [E.Alshina, A.Alshin (Samsung), C. Pang, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-P0212 RCE3:Subtest C-1 preset of un-available for intra block copy samples [E.Alshina, A.Alshin (Samsung)]

Reclassified

### RCE3 cross checks (10)

JCTVC-P0088 RCE3: Cross-check on Subtest C.4 - Padding horizontally or vertically [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-P0105 RCE3: Cross-verification of subtest C.2 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0148 RCE3: Crosscheck report of RCE3 Subtest C.1 [C. Pang (Qualcomm)] [late]

JCTVC-P0185 RCE3: Crosscheck report of B.4 on sample masking [J. Xu (Sony)] [late]

JCTVC-P0188 RCE3: Cross-check report of Subtest B.3 - Intra Block Copy with NxN PU (JCTVC-P0145) [R.-L. Liao, T.-S. Chang, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0191.RCE3: Cross-check report of Subtest B.1 on Nx2N/2NxN Intra Block Copy (JCTVC-P0176) [X. Xu, S. Liu (MediaTek)] [miss]

JCTVC-P0193 RCE3: Cross-check report of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy [X. Xu, S. Liu (MediaTek)] [miss]

JCTVC-P0196 RCE3: Cross-verification of subtest D.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0232 Cross-check report of 'RCE3: Subtest C.3 - Intra block copy with CTU-based padding' (JCTVC-P0147) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0233 Cross-check report of 'RCE3: intra block copy search range (tests A)' test A5 (JCTVC-P0211) by Qualcomm and Samsung [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

### RCE3 related (intra block copy refinement) (20)

#### Padding-related

General: the consensus is that padding is not necessary because the current system just prohibits the overlapping cases. Gains would have to be significant to warrant a change in this area.

JCTVC-P0087 Non-RCE3: On padding/unavailable samples pre-set for intra block copy [A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

Padding of a constant colour, determined by the most frequently appeared value in neighbouring samples of current CU.

CU-level modification -0.1/-0.2% for Class F, 444 SC AI, neglible elsewhere for lossless. -0.6%, -0.9%, -0.9% for lossy Class F, 444 SC AI; -0.4%, -0.6%, -0.4% for inter. No significant change in run-time.

CTU-level similar to CU-level.

Requires memory for histogram.

Therefore, used longest connected pixel value to fill the unavailable area. Similar results for lossless. Lossy not available.

Destabilizing. Not solving a problem. No significant gains.

Recommendation: No action.

JCTVC-P0089 Non-RCE3 : On unavailable sample preset for intra block copy [C. Park, S. Lee, C. Kim (Samsung)]

Not presented. (Destabilizing. Not solving a problem. No significant gains.)

Recommendation: No action.

- JCTVC-P0190 Non-RCE3: On padding of overlapped area for IntraBC [K. Zhang, X. Xu, J. An, X. Zhang, S. Liu, S. Lei (MediaTek)]

Note: Renamed/reclassified contribution from RCE4.

Two padding methods.

Method A similar to P0238. Very similar results as well.

Method B mixture of vertical and horizontal padding – about the vector. Gains are smaller.

Destabilizing. Not solving a problem. No significant gains.

Recommendation: No action.

JCTVC-P0237 Non-RCE3: Major Color Padding Algorithm for Intra Block Copy [C.-H. Hung, E.-C. Ke, H.-T. Chiao, W.-H. Peng] [late]

Histogram of the reconstructed pixels from the left 1 column, above 1 row, and top left sample around current CU. Calculations the determine padding value to use.

-0.2% for 444 SC AI. Negligible elsewhere. 13% increase in runtime for AI lossless

-0.6%, -0.9%, -0.6% for Class F, 444 SC AI; -0.5%, -0.6%, -0.6% for Class F, 444 SC inter. Negligible elsewhere. 6% increase in runtime for AI.

Destabilizing. Not solving a problem. No significant gains.

Recommendation: No action.

JCTVC-P0254 Crosscheck of JCTVC-P0237 on major color padding algorithm for intra block copy [??(?)] [late] [miss]

JCTVC-P0238 Non-RCE3: Padding schemes for intra block copy [Y.-J. Chang, H.-L. Tsai, C.-L. Lin, F.-D. Jou (ITRI), C.-C. Chen, R.-L. Liao, W.-H. Peng (NCTU)] [late]

Presentation not uploaded.

Padding is a repeated copy of the source data.

-0.3%, -0.9%, -0.9% for lossy Class F 444 SC AI.

-0.2%, -0.8%, -0.7% for the same for Class F & 444 SC RD & LD

-0.1%, -0.2%, -0.3% for AI, -0.1%, -0.4%, for RA -0.5%, -0.2%, -0.3% for LD

No noticeable run-time increase.

Very late contributions: P0255, P0256 also reviewed those results here. Very similar results to P0238.

Destabilizing. Not solving a problem. No significant gains.

Recommendation: No action.

#### Block vector initialization

Also included in this section is P0102 methods 1 and 2, described later within this BoG report.

General comment: Currently the block vector predictor is initialized to (0, 0), but this itself is an invalid vector. The methods aim to find an alternative solution that do not produce illegal initial block vectors.

JCTVC-P0111 Non-RCE3: Motion predictor initialization for Intra Block Copy [G. Laroche, P. Onno, C. Gisquet, T. Poirier (Canon)]

Initialize to (-2*CUsize, 0) instead of (0, 0) where the latter is invalid.

-0.1% for AI for Class F, 444 SC (some positives for RGB Opt)

RA on average slightly positive.

Lossless: no significant impact.

cf P0111=P0150=P0217Proposal1.1, P0178, P0102

Tried (-CUSize, 0), results slightly worse.

Not an argument for efficiency.

Positive values in RA due to one sequence.

Subset of P0217. Further review within the P0217 contribution notes.

JCTVC-P0150 Non-RCE3: Block vector predictor initialization for intra block copy [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

No need to present – same as P0111.

JCTVC-P0178 Non-RCE3: Bv coding with default predictor [J. Xu, A. Tabatabai, O. Nakagami (Sony)]

Same as P0102-Method1. Also tested other LCU sizes, showing increased gains as LCU size is decreased.

JCTVC-P0217 Initialization of block vector predictor for intra block copy [L. Zhu, J. Xu, G. J. Sullivan, S. Sankuratri, B. A. Kumar, F. Wu (Microsoft)]

Proposal 1.1 - use of (-2W,0).

Proposal 1.2 – use of (–Min(2W,LCU),0)

Concern over using LCU size as limit, and not 64 [see notes on discrepancies between software and text].

Proposal 2, condition of MVy.

Proposal 3 (vector prediction)

Two motion vector predictors, with a bypass-bin indicator to select.

-0.2, -0.2, -0.3 for intra F, 444 SC.

-0.1, -0.1, -0.3 for inter F, 444 SC.

Lossless results not presented, but indicated to be negligible.

What happens if new PMV is same as existing ones? Don’t update – ensure that both vectors are different.

Results may be influenced by NxN decision.

Concern over the addition of syntax for each MV predictor.

Further work – dependent on NxN adoption, where larger gains should be observed.

See RCE described in P0149.

Summary of initialization methods

| Method | Predictor | Comments | Status |
| --- | --- | --- | --- |
| Current | (0,0) | Invalid for all CU sizes |  |
| P0102-Method 1, P0178 | (MVy>-H?-W:0,0) | OK for all CU sizes |  |
| P0102-Method 2 | (-24,0) | Invalid for 32x32,64x64 sizes | Not considered. |
| P0111, P0150, P0217-1.1 | (-2W,0) | Invalid for 64x64 (also a later proposal to remove 64x64s) | Not considered. |
| P0217-1.2 | (-Min(2W,LCU),0) | OK for all CU sizes. Same result as P0217-1.1 due to CTC. |  |
| P0217-2 | (MVy>-H?-W:-2W,0) | Invalid for some 64x64 | Not considered. |
| P0111 (not part of presentation!) | (-W,0) | OK for all CU sizes |  |

The purpose of the change is to remove invalid vectors. The solution should therefore remove invalid vectors entirely.

For tile and picture and slice boundaries, all predictors can be invalid. But you cannot code IntraBC at these edges.

If the current block is within the first CTB of the picture, and it is the bottom left 16x16 CU in the LCU and it is the first IntraBC block in the CTB, all the above predictors would be either horizontally negative (out of the picture), or (0,0) which is overlapping. i.e. all the above predictors would be invalid.

Therefore no solution is ideal and there is no necessity for a change.

Recommendation: No action.

JCTVC-P0304 Text and results for block vector predictor for intra block copy

Results for the block vector initialization approach that was adopted in the plenary discussion were presented. Small improvements were observed and WD text was provided.

#### Vector prediction

P0217, method 3 is also included in this section.

JCTVC-P0102 Non-RCE3: On displacement vector coding for intra block copy [C. Park, S. Lee, C. Kim (Samsung)]

Powerpoint presentation not available on server, document out of date.

Method 1: Initialization of vector predictor for first IntraBC in CTU - condition on Vy to offset Vx.

Negligible BD-rate changes. (Reviewed in P0217 discussion above)

Method 2: Initialization of vector predictor, with (min width+max width)/2 [late addition to proposal]

Negligible BD-rate changes. (Reviewed in P0217 discussion above)

Method 3: General vector coding - condition and sign coding (was method 2 in previous revision of the proposal).

-0.8%, -0.7% for 444 SC AI, -0.4%, -0.5% for RA, -0.5%, -0.6% for LB. [optional case is positive for PCB-layout] These are relative to RCE3-B3.

If structure was:

CU 16x16 (A)   CU 16x16 (B)

CU IBC  (C)     CU 16x16 (D)

C can no longer refer to B. Currently software in fast search does not check B when doing C, so not a problem. What happens when FAST_INTRABC is disabled?

If condition true, then offset MVx and sign code, else as current design.

Results relative to anchor required.

Confusion over proposal document version.

Concern expressed over destabilizing the design.

Recommendation: No action.

#### Syntax related

JCTVC-P0149 Non-RCE3: Block vector signaling for intra block copy [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

Presentation not uploaded.

Method 1: Left or above BV from within current CTU is selected as predictor, where the first of both of (-2*CUWidth,0) and (-CUWidth,0) are used if predictors not available.

Binarization changed: one flag indicates whether the difference is zero. when non zero, exp Golomb-3 used to encode remaining absolute level of BVD, and one further flag for sign.

Memory requirement – one predictor to 15 for 2Nx2N, 31 for NxN case. No objections to this.

Method 2

Only binarization changed.

Concern over change in binarization.

Method 1 results:

(-0.5, -0.6, -0.8) for AI Class F, SC 444

(-0.2, -0.4, -0.6) for inter Class F, SC 444

No increase in run-time.

Lossless – no significant change, although optional GBR case is positive due to one sequence.

Method 2

(-0.4, -0.4, -0.6) for AI Class F, SC 444

(-0.2, -0.3, -0.3) for inter Class F, SC 444

No increase in run-time.

(-0.1, -0.1, -0.1) for AI Class F, SC444 (nothing for RA/LB)

Gains for NxN PU also presented:

Method 1 results for NxN (-0.9, -1.3, -1.6) for AI; (-0.7, -0.9, -1.2) for inter; nothing for lossless.

Method 2 results for NxN (-0,5, -0.6, -0.7) for AI; (-0.3, -0.5, -0.7) for inter.

Concern over destabilizing for small changes. Want to see gains if NxN is adopted.

Recommendation: RCE on block vector predictor and binarization requested, if NxN is adopted. Consideration of gains, and complexity.

JCTVC-P0250 Crosscheck of JCTVC-P0149 [J. Xu (Sony)] [late]

JCTVC-P0202 Non-RCE3: On Intra block copy [C. Rosewarne, M. Maeda (Canon)]

Prohibit 64x64 intra-block copy. No significant change in results.

Current CTC does not search 64x64. The results are only indicating the saving in signaling.

Is there a strong reason – benefit cannot be seen on decoder side.

Clarification (highlighted in later discussions): the search width in the software is Current+64 horizontally.

Further work required to show what happens when the 64x64 is being searched.

JCTVC-P0205 Intra block copy syntax clean up [X. Xu, K. Zhang, X. Zhang, S. Liu, S. Lei (MediaTek)]

Current decision for inter-slice for Intra BC in advance of intra/inter.

Propose to change so that Intra/Inter is chosen, and if intra, then decide if intraBC or not.

0% for intra slice (no change).

0% for lossless RA/LD, +/-0.2% for lossy RA/LD.

IntraBC is different to inter and intra – so there is no absolute necessity to fully align to either.

For screen content, IntraBC is used more often. Therefore current design looks to be more efficient.

Recommendation: No action.

JCTVC-P0230 Crosscheck of JCTVC-P0205: Intra block copy syntax clean up [Z. Ma (Huawei)] [late] [miss]

#### Searching

JCTVC-P0151 Non-RCE3: Fast encoder search for RCE3 Subtest B.3 [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

Fast encoder search applied to the NxN PU split of Subtest B3, where Intra BC NxN is not checked based upon three conditions.

For lossy: -2.8, -7.3, -6.8 for AI (+9% run-time), -2.0, -6.3, -5.7 for RA, -0.9, -4.2, -3.7 for LB (no change in run-time).

For lossless: -1.2, -5.3, -5.6 for AI (+15% run-time)  -0.6, -4.4 and -4.5 for RA, -0.2, -4.1, -4.3 for LB.

Information provided shows the tradeoff between encoder complexity and efficiency.

Recommendation: No action.

#### Misc

JCTVC-P0170 Non-RCE3: simplified difference vector coding for Intra block copy [S.-H. Kim, K. Misra, A. Segall (Sharp)] [miss] [withdrawn]

JCTVC-P0206 Non-RCE3: Improvements on non square intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)] [miss] [withdrawn]

JCTVC-P0218 Non-RCE3 subtest B.2: Search Methods for Intra block copying for CU-level block vectors with TU-level prediction processing [L. Zhu, J. Xu, G. J. Sullivan (Microsoft)]

The specification text includes TU level prediction.  This allows overlap in block vector search with the current CU.

Lossy results: -0.5, -0.6 for 444 SC AI; -0.6, -0.5 for SC 444 RA; run-time no significant change.

Is this padding? It is not, because the prediction is defined completely for each TU, i.e. no initialization of the reconstructed sample memory is required.

Relative to the current specification text, this is a software encoder enhancement.

In the encoder search, the source data in the overlapped region is used. Only 8x8 case analyzed in these results.

It was raised that the intention probably was not to allow this case of TU-level prediction.

Recommendation: A break-out group will document discrepancies between text and software and also various understandings/interpretations.

The report was made to the plenary. The following clarifications and decisions were reached:

- Clarification was requested regarding all discrepancies between the intra block copy software and the text.  Addressing (at least) the following:

  - The intended behaviour of the search area for different LCU sizes [specified as CTB size, software implements 64. Specification set to 64 in plenary].

  - Reconstruction process (TU or CU based) [specified as CU based and consistent with software].

# Project development, status, and guidance

## HEVC coding performance, implementation demonstrations and design analysis

### RExt performance and design aspects (3)

JCTVC-P0200AhG8: Performance and complexity analysis of Range Extensions tools [J. Sole, T. Hsieh, C. Pang, M. Karczewicz (Qualcomm)]

HEVC Range extensions tools compared with tools disabled. Very large gains (not summarized).

HEVC Range extensions tools compared with AVC High 4:4:4. Very large gains >30% (40-50% even) for RA and LD.

Then, intra block copy is studied for natural content. Found gains of 0.5 to 5% for some sequences. (5% on ‘SpinCalendar’ 720P) (-0.5% in Station2 1080P).

Complexity of intra block copy was studied. A hardware analysis reported that the additional requirements are very small.

Comparing Intra_BC to DBF/SAO, Intra_BC was 4% of decoder area and DBF/SAO was 16.5% of decoder area.  For encoder, these numbers were 1% and 4.4% respectively. This increase looks reasonable considering the gains being introduced by this tool.

Numbers would be a bit architecture specific.

Does this mean the gain is enough already?

1:8 compression for the same quality. With further screen content tools, how much additional improvement in this would be achieved.

Gains for Intra_BC seem to be mostly due to SpinCalendar.

Was subjective testing on these natural content sequences? No.

Regarding section 2.1, we can always find some sequences that benefit a lot from a particular tool. This contribution appears to show that decoder side complexity of Intra_BC is OK, but the encoder could still be an issue.

Your concern relates to search time? Yes. For SC applications we tend to need very fast encoder.

Internal tests by the presenter showed big encoder-side improvements are possible.

JCTVC-P0201 AhG8: Performance of encoder and parameter only changes for Screen Content Coding [J. Sole, C. Pang, L. Zhang, M. Karczewicz (Qualcomm)]

Performance of RExt software was studied with some encoder-side tools and configuration parameter changes.

Search area for intra_BC was increased, so this is a normative change because the memory requirement increased. The whole frame was used.

Transform skip size was increased.

Intra: Gains of up to 25% for RGB SC 4:4:4, 20% YUV SC 4:4:4. Encoder side increase up to 122%, decoder decrease down to 77%.

Inter:

Compared to some earlier contributions with small gains but large run-time increases, this contribution shows that small changes in the configuration and encoder algorithms can give big improvements.

Does this imply that the RA configuration should be modified? Normative changes were also performed.

It appears that much better searches could be performed.

How are the gains with just the TS size increase? ~1.5%.

JCTVC-P0213 Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

The QPs were adjusted to enable comparison between HM and AVC. Numbers are close to those presented in JCTVC-P0200.

## Source video test material (2)

JCTVC-P0228 Description of HDR sequences proposed by Technicolor [S. Lasserre, F. Le Leannec, E. François (Technicolor)] [late]

To be reviewed.

JCTVC-P0229 Quantitative quality evaluation of images for HDR and WCG coding [S. Lasserre, F. Le Leannec, E. François (Technicolor)] [late]

To be reviewed.

# Non-CE Technical Contributions

## Range extensions

### Implementation aspects of high bit rate and high bit depth (7)

JCTVC-P0044 On MinCR [T. Suzuki (Sony)]

Professional applications are targeting significantly higher  bit-rates than consumer applications.

Main 12 4:4:4 not operating correctly for negative QPs, 4x4 and 8x8 blocks are clipped.

Took 2 ctx bins/cycle and 4 EP bins/cycle as a hardware performance point and estimated the worst case cycles/sample. Then calculated number of entropy decoders required for HD and 4K. Alignment asserted to provide significant reduction in the number of required entropy decoders.

Current text sets MinCR to professional levels and below in some cases.

In some cases 50:1 is best, which is too high for professional applications.

A ‘super tier’ is requested, supporting high bit-rate and high throughput.

In version 1 definition, MinCR same for main and high tier.

High tier added mainly to support high bit rate for storage application.

Didn’t change MinCR for high tier.

In 4K 60P, level 5.1 the bit-rate of high tier is 160Mbps and MinCR is 8.  This condition is hit by Intra picture in a long GOP. In case of 4K and higher, MinCR is larger. MinCR could be a bottleneck for visual quality.

Want to include an operating point that reduces MinCR for Main and Main10 profiles, as well as for RExt profiles.

Could change high tier or add new tier and change MinCR in the new tier and applied in Main and Main10 profiles or can we change MinCR for high tier in the existing profiles.

Think that there are no existing high tier products for Main/Main10. But no proposal to change main tier for Main/Main10.

Three options are proposed that have been earlier proposed. Option 2 is currently presented which is to add super tier with reduced MinCR and leave main and high tier unchanged.

Range extension level definition:

Maximum compressed picture size was calculated and compare to the CPB size. In some levels the CPB is not sufficient to store one picture. Want to change the CPB size so in all cases at least one picture can always be stored.

Propose to reduce MinCR and increase CPB size.

Three options:

1. Add new super tier, also add a flag to affect main and high tier of version 1 (Main/Main10).

2. Also new super tier but not affect current Main/Main10 profile (signaling does not affect version 1).

To signal the tier one bit is available, but another is needed to signal super tier. In option 1, one bit is used from the general reserved zero flags. With this option, 2 bits are available to signal the tier, thus a fourth tier could be added in future.

3. For RExt profiles the high tier is redefined to provide the parameters of the super tier, so no signaling change is necessary.

Intra profiles are signaled through constraint flags rather than profile_idc values.

Recommendation: Revisit at the JCT-VC level.

JCTVC-P0061 AHG18: Worst-case Escape Code Length Mitigation [K. Sharman, N. Saunders, J. Gamei (Sony)]

Worst case escape code length in HM is 32-bits.

Normally the suffix length is equal to the prefix length. Propose that when extended precision processing is enabled, a mapping table is introduced. Result of this is the same worst case length of 32-bits is retained.

If you don’t use extended precision processing, can you still exceed 32? No.

Why does it matter being greater than 32? Originally a benefit was asserted for software. Also the worst case reduction (discussed in RCE1) is another benefit.

The 46 worst case is not sustained? Increasing Rice parameter reduces this worst case between consecutive instances.

The fast update and max 7 is a separate flag to the extended precision processing flag.

This feature is not independently configurable from extended precision processing because we are trying to reduce this particular worst case.

Interacts with the worst case analysis currently in RCE1.

How was the table obtained? Several mappings were tried, we observed that the initial 1:1 mapping was needed. Then larger values were so rare that removing them was causing no impact.

What happens if the prefix and suffix length match for prefix =5 (i.e. suffix = 5 instead of 6) how does it perform? Some losses were observed.

Are we solving a problem here? Reducing this worst case is a bit different to the worst case being discussed in RCE1 because this is for the immediate window into the bitstream, e.g. amount of prefetched data needed to guarantee to decode one codeword.

Very low impact on coding efficiency.

Do we understand the specific requirements for bit-rate at these operating points, can the current design meet that bit-rate in terms of bins/cycle.

Would want to start to reach 2:1 compression ratio at 4K.

How many bins/second? Large numbers such as 10G/s were being discussed.

This proposal is not needed if bypass bins (i.e. not raw bins) are used, because the high-throughput architecture discussed could not be realised.

Revisit when P0044 is reviewed, so we can better understand the real bin-rate requirement.

What happens when extended precision processing is disabled and the decoder is configured for 12-bit operation? The clipping still in place ensures that the 32-bit worst case remains. So the problem only exists when extended precision processing is enabled.

This interacts with RCE2, so maybe should discuss this.

Prefer a more regular table relationship, rather than the non-linear

This is well defined so testing in a CE is possible.

With the raw bypass approach, is this approach still necessary?

Benefit is mostly for software, as 32-bits is a natural word size. Some questioning about the extent of this benefit, e.g. due to bit alignment of the codeword.

Recommendation: Study in a CE.

JCTVC-P0066 AHG5: Luma-chroma prediction for different bitdepth [O. Nakagami, T. Suzuki (Sony)]

Relates to cross-component prediction.

When BitDepthY and BitDepthC are different, the current process of CCP is not optimal, because the ranges of input and output variables are not the same in eq. 8-298.

Two proposed solutions:

1/ Left/Right shift the luma residual to match the chroma bit depth prior to scaling by ResScaleVal.

2/ When bit depths differ, do not permit CCP.

Experiments conducted using modified RExt CTC.

Test 1 (BitDepthY=12;BitDepthC=8) & 2 (BitDepthY=8;BitDepthC=12) examine solution 1.

Test 3 (BitDepthY=12;BitDepthC=8) & 4 (BitDepthY=8;BitDepthC=12)  examine solution 2.

Note that SAO was disabled in tests 2 & 4 due to recent modifications/limitations to SAO in HM. See ticket #1219.

RGB results not available.

Solution 1:

Test 1(-1.4,-0.6,-0.4) luma for AI,RA,LB

Test 2 similar to Test 1

Solution 2:

Test 3 (-0.1,-0.2,-0.3)

Test 4 (+0.1,-0.2,-0.4, but UV are positive) for AI, RA, LB

Cross-checker: bug-fix for a special situation.

Comment: as long as the standard allows for different bit depths, this is a bug-fix.

Comment: was mentioned at previous meeting.

Comment: solution 2 seems extreme, solution 1 seems a logic BF.

Recommendation: adopt solution 1 (BF).

JCTVC-P0100 AHG5: cross-check results of luma-chroma prediction for different bitdepth (JCTVC-P0066) [K. Kawamura, S. Naito (KDDI)] [late]

- JCTVC-P0162 On 16-bit coding [A. Aminlou, K. Ugur (Nokia)] [late]

Specifically looking into 16-bit lossless coding.

16-bit source into two 8-bit sources and code losslessly.

Report 15% gains for AI. 13% gain for LD. Indicates that some issue could exist in the 16-bit lossless coding tools. Rice parameter might be one cause. Running tests with different Rice parameter for lossless case.

Second aspect: Take LSB, code MSB lossy but quite high quality. Then, have a residual in MSB, added to LSB and coded losslessly. Gives 18% gains in AI, 16% in LD.

Two suggested ways to implement: Auxiliary picture for LSB part. Otherwise FPA.

Benefit is reuse of low bit-depth coder and improved coding efficiency for 16-bit lossless coding.

Some of this could be due to the maximum Rice parameter of 7. But if this maximum was increased that that gap was not closed, could indicate further design issue.

We should consider adding a lossless AHG18 test condition may be useful.

SEI/Auxiliary picture aspect.

Results reported on SVT sequences.

Strong indication that current design has issue.

How do you ensure the error would be within the overlap bits (e.g. for a 12/12 split).

Have you looked at the LSB picture? Yes, very noisy.

Noise is also in the 16-bit source.

Is there any compression happening on the LSB portion? Even expansion can happen because it is basically noise.

What resolution is being compressed? These are 4:4:4, 1080P 50fps.

Expansion is observed in the 8-bit LSB results, as average 2.7Mbps greater than raw bit-rate of ~2.5Mbps.

Recommendation: Discuss SEI for FPA at plenary level, add lossless AHG18, design experiment (e.g. using some combination of proposed methods) to establish a ‘high bar’ for performance for RExt to match the performance of these approaches.

JCTVC-P0173 AHG 5 and 18: Coding of high bit-depth source with lower bit-depth encoders and a continuity mapping [C. Auyeung, J. Xu (Sony)]

Take 16-bit source and split into two parts: LSB part and MSB part. In this proposal, but more bits into the LSB part.  LSB part is lossy coded and MSB part is lossless coded.

Overlap between LSB part and MSB part referred to as a ‘continuity mapping’. If the LSB bits of the MSB part is equal to 0, no output is directly assigned as the LSB part. Otherwise

SEI message to add an auxiliary picture is proposed.

Using the 5 test sequences from CGS: Red curve is with continuity mapping, black without. For class B AI, 50% gain in luma, RA and LB are 93-94%. Also tested 8-bits, improvement still approximately 60% (result not presented).

Is there any need for such techniques?

Proponent wants to use low bit-depth codec to encode high bit-depth

This proposal is another FPA type of proposal.

Anchor is without continuity mapping.

Why not use AHG18 conditions as there are many higher bit-depth sequences.

Recommendation: No action for now. Possibly should be considered at a separate level such as track/plenary/SEI BoG.

JCTVC-P0287 AhG 5 and 18: Cross-check of coding of high bit-depth source with lower bit-depth encoders and a continuity mapping in JCTVC-P0173 [W.-S. Kim, W. Pu (Qualcomm)] [late]

JCTVC-P0222 AhG 18: On SAO performance at high bit-depth and high bit-rates [E. Alshina, A. Tourapis, W.-S. Kim, Y.-W.Huang] [late]

AHG8 lossy test indicates that SAO is beneficial.

4.9% for Screen Content

0.8% for camera captured 8b

0.7% for camera captured 10b

AHG18 SAO on/off shows no significant changes. Why?

For 16-bit test condition, PSNRs are very high – no significant error to correct.

For 12-bit test conditions, some modification may help. But 12-bit still only being tested at high PSNRs.

After quantization rounding, offsets end up almost always as 0, so encoder disables SAO.

Solution 1: removal of SAO quantization shift. Range +/-31 allowed, but only observed +/-5.

Small changes for 16, as expected.

For 12-bit, some more significant improvements.

Solution 2: change classification rule.

Summary:

SAO disabled 0.0, 0.0, 0.0, 0.0

Solution 1: 0.0, -0.5, -0.2, -2.1

Solution 2.1: 0.0, -0.3, 0.0, -1.0

Solution 2.2: -0.0, -0.2, ?, -1.4

Solution 2.1 and 2.2 not recommended for any further action.

1.3 avg for 12-bit

0.2 avg for 16-bit

Reason for quantization in HM was due to concern over number of bins in the offset at higher bit depths.

Can we run with Solution 1 with AHG18 test conditions using AHG5 QPs? To address whether we need a flag.

There is no backwards compatibility issue – the quantization only affects beyond 10-bit.

Concern over making change to definition. However, this quantization only affects beyond 10-bit.

What happens if MSBs added to 8-bit, or LSBs added to 8-bit. At a certain point it will cost too much to signal SAO parameters.

Potential CE.

Discussed about using a 2-bit field instead of a flag, as this would allow further extensions.

Current quantization is harmful to the current test conditions. Flag is least risk method to remove quantization.

Revisit following results for QP=22->37 for AHG18 to determine whether flag is not necessary.

Additional tests for the specified QPs gave 27 to 61dB PSNR in 12 bits and 16 bits were performed. At these QPs, a bug in RD cost estimation was found due to a type conversion (unnecessary intermediate rounding to 32-bits on architectures with 32-bit INT). AHG18 tests not affected due to lower QPs. Also no effect for 12-bit but large effect for 16-bit (e.g. 60% loss).

Removing the quantization results in losses at the 12-bit and 16-bit operating points at the higher QP range. However gains at lower QP range.

Therefore, the flag to conditionally disable the quantisation flag appears beneficial. SPS flag is specified.

Slice level control for SAO quantisation was then considered.

Testing other shift amounts was also considered for signalling at the slice level. The additional gains from having variable shift amounts appear small.

Decoder impact of variable shift amount is negligible.

Concern over subjective impact of misconfiguring such parameters.

Even if a CE was established, how would we fully test the impact of such change? (i.e. subjectively at these bit depths).

The flag was put into the SPS because this is the level at which the operating point is decided.

Does additional flexibility beyond just controlling the quantization with a flag benefit other (e.g. intermediate) operating points?

Just having the flag: Restricting quantization always for LD-16 bit AHG18 conditions gave up to 1.2% loss, 0.3% loss in AI-16.

0.6 to 0.7% gains of RExt 5.0.1 RD in LD12 and LD16 respectively cost estimation bug-fix with SAO adaptive quantization at the slice level and for luma/chroma independently signalled.

Recommendation: Adopt this proposal with a flag in the SPS. Revisit at plenary to consider whether slice level control of luma/chroma independently is also warranted.

### Lossless and screen content coding related contributions (5)

JCTVC-P0097 Extended Cross-Component Decorrelation for Animated Screen Content [A. Khairat, T. Nguyen, D. Marpe (Fraunhofer HHI)]

Presentation not uploaded.

Current RExt:

CCP uses luma as predictor for 1st and 2nd chroma with fixed set of possible alpha (+/-1,2,4,8).

Two proposed extensions

1: Allow luma or 1st chroma as predictor for 2nd chroma.

Signal only if both corresponding residual blocks inherit residuals. i.e. (cbfy=1 and cbfcb=1) or (cbfy=1 and alphacb!=0), and also controlled by PPS.

Improvement for YCbCr animated screen content (-0.9%, -0.6%, -0.4% for AI/RA/LB) and for all 4:4:4 content under AHG8 CTC. No benefit for RGB content under AHG5.

Same cost as for CCP without extension and same encoder run time – there is additional searching, but bit rate is reduced.

2: Allow limitation of alpha to a specific range, with the signaling limitation in the PPS.

Range for alpha designed under constraint of natural YCbCr content. Distribution centred around 0.

Observation: for RGB content, alpha is often centred around 8. For some content the inverse order for alpha is more beneficial.

Limiting the alpha range in PPS: keeping current quantization as default option, but signal lower and upper bound.

CCP default (-3, +3) = LUT(1,2,3)+sign

RGB content (+2,+3) = LUT(2,3)

Inverse signaling (+3,-3) = LUT(3,2,1) + sign

Implementation: code sign before absolute value of alpha. Expand the PPS values to an array. Apply LUT operation to map bit stream value to real alpha value.

i.e. LUT is programmable via the PPS.

AHG5 RGB (LUT=+2,+3), (-1.1, -1.6, -2.4) (AI/RA/LB)

AHG8 ASC RGB (LUT=+2,+3) (-1, -1.1, -1.6) (AI/RA/LB)

Inverse signaling AHG5 and AHG8 ASC RGB small gains

Method 1+2 are reported to be additive (no results presented)

Comments:

Method 2 gives control.

How does the encoder make the choice?

If the setting is correct, there is a good gain for RGB, but if the wrong setting is chosen, there is loss.

What about mixed content? Could you default, but the results indicate that the choice is not affected by content type.

Should signaling by in slice header instead of PPS – allows more flexibility. But would the content change

Concern over:

Missing text for method 2. (Method 1 is the same as last time)

Results show big losses (when the incorrect setting is chosen for a sequence) – perhaps the results should have been presented in a different way.

Relative to the gains of CCP, this is giving a small gain or giving an opportunity to be detrimental.

Method 1

Concern over encoder complexity – has to decide whether to use CCP for luma or for chroma_cb.

Serializes the reconstruction process – Cb and Cr cannot be done in parallel.

Gains variable over individual RGB444 sequences.

No support from the floor. More questions and concerns from the floor.

Method 2

Design more flexible, which is good. Decode side probably not an issue. How to configure encoder?

Combined Method1+2 looks interesting.

Recommendation: form a CE/AHG to examine encoder configuration for Method1, Method2 and Method1+2 (concern about whether the definition is too vague for a CE).

Related to P0154 in another section.

JCTVC-P0112 AHG8: Chroma interpolation filters for Lossless compression [G. Laroche, T. Poirier, P. Onno, C. Gisquet (Canon)]

Aim to reduce memory bandwidth for lossless compression.

Inter luma interpolation filter changed to use the chroma interpolation filter.

Gains for lossless compression - -0.2%, -0.3%, -0.1%, -0.1%, -0.2% for F, B, RGB 444 Anim, YCbCr 444 Anim, RangeExt respectively for RA.

Two tap showed further compression efficiency, although there is no two-tap filter in the current design.

Some form of interpolation filter is required.

This technique would be applicable if there were to be a lossless only profile.

Cross-checker confirmed response and commented about discussions at previous meetings on the choice of interpolation filters for high-resolution lossy conditions.

No action for now (information only), as this is dependent on profile discussions.

JCTVC-P0214 Screen content coding using dictionary based mode [B. Li, J. Xu, F. Wu (Microsoft)]

Presented revision not available.

Related to O0357.

1D and 2D dictionary. On average 22% -> 30% observed for CfP sequences.

Animation sequences are not showing improvement.

AHG8 lossless and lossy showing large changes (up to -60%), but again Animation sequences showing little change.

CRC 16-bit being used to generate hash. CU is raster scanned to produce the CRC. Multiple candidates are kept for a single hash value. Number of candidates limited, can be hundreds for one hash value.

Hash value only useful for matching exact, how to handle lossy case? The encoder, for each block, performs SAD to get best match.

Two methods:

- Encoder transmits hash, index and residual.

No source code available.

Would need more information, e.g. cross-check, for study in an AhG.

Recommendation: Further study.

JCTVC-P0277 Motion Vector Resolution Control for Screen Content Coding [Y. Zhou, B. Li, J. Xu, G. J. Sullivan, B. Lin (Microsoft)] [late]

For screen content, sub-fractional motion vectors are rare. Disable sub-fractional scaling.

Change to decoding process: header control bit.

-1.1%, -0.3%, -0.1% for Class F, SC 444 (RGB,YUV) RAMT;

-1.3, -0.8, -0.4% for LBMT.

Does not work for typical camera-content video – turned on/off on a per-sequence basis.

Encoder decision at sequence level.

JCTVC-P0283 Adaptive MV precision for Screen Content Coding [X. Li, J. Sole, M. Karczewicz (Qualcomm)] [late]

Results not in proposal document.

Related to P0277.

CU-level flag to indicate whether integer precision MV is used for the CU.

Predictors rounded to integer precision. Signal the MV integer difference.

-1.5, -0.5, -1.1, -1.3, -0.8, -1.2, -0.4 for RAMT, EncTime=177%

-1.6, -0.3, -2.0, -2.3, -1.5, -0.8, -0.2 for LBMT. EncTime=176%

This and P0277 are screen-content coding motivated tools. Except P0283 includes AHG8-ClassB improvements.

What are the affects with Class A (high resolution) – the benefits of subfractional bits may be reduced and these tools may be applicable.

This contribution should not be specifically targeted for screen content.

The scope should be increased, not limited to screen content, not limited to just integer or ¼ pel. What about ½ pel etc.

Is a CE possible? Unknown encoder methods at this time.

For this contribution with encoder configured to always use integer, the gains are halved. No signaling

Recommendation: AHG/CE depending on revisit (off-line discussions between proponents and proponents of P0277)

### Other (4)

JCTVC-P0109 AHG8: Sample adaptive offset with multiple parameters [S.-T. Hsiang, S. Lei (MediaTek)]

Objective to improve coding performance by the more effective use of the existing SAO tools.

Enable more than one SAO filtering operations performed on each CTU.

Additional syntax.

-0.7, -1.2, -2.2 for F, SC AIMT AHG8 lossy

-0.6, -0.8, -1.2 for RAMT; Class B and (RGB,YCbCr) 444 Anim are +0.1/+0.2.

-0.6, -1.0, -1.0 for LBMT

Set to use SAO 4 times for intra slice, 2 times for inter slice.

Each has different parameter sets.

Is overall run-time appropriate measure here for SAO?

It would be increase in hardware.

Visual comparison needed because SAO is a visual coding performance tool.

SAO is an area RExt has not adjusted before.

Contributor indicates that this can be treated as an informative document.

JCTVC-P0278 AHG8: Cross-check report of Sample adaptive offset with multiple parameters (JCTVC-P0109) [Christophe Gisquet (Canon)] [late]

JCTVC-P0154 AHG5 and AHG8: Alpha parameter coding methods for inter-component residual prediction in HEVC range extension [X. Zhang, K. Zhang, J. An, S. Lei (MediaTek)]

Related to P0097 in a different section.

Presentation not uploaded.

Current binarization:

Sign +

0 0

1 10

2 110

4 1110

8 1111

3 contexts (1 each for first two bins, 1 for latter two).

Observation 1: lots of sources select 8,4 as alpha values more often, but 2 bits used to identify bigger values and 4,8 share one context model.

Observation 2: RGB video has different average settings from Observation 1

Method 1 – add a context for 4th bin

Method 2 – alpha value predictor coding, where pred_value = 5 bins to identify 0, 8, 4, 2 1 and error, with 5 context model

Alpha

0  0

8 10

4  110

2 1110

1 11110

-1 11111 + 00

-2 11111 + 01

-4 11111 + 10

-8 11111 + 11

Where the first 5 bins are context-encoded, and the latter two are bypass encoded.

Method 1: -0.2, -0.4, -0.3, -0.2 for 444 SC+Anim, 0% elsewhere for AI

-0.4, -0.2 for Anim for RA

-0.2, -0.6, -0.3, -0.1 for LB for 444 SC+Anim

Run-time of results not reliable.

Method 1+2 for RGB

AI -0.5 for RGB 444 Anim, otherwise same as Method 1

RA -0.5, -0.6 for RGB 444 SC+Anim, otherwise same as Method 1

LB -0.6, -1.0 for RGB 444 SC+Anim, otherwise same as Method 1

AHG5 RGB: -0.4, -0.6, -0.9 for Method 1 AI, RA, MT

AHG5 RGB: -0.5, -0.8, -1.1 for Method 1+2 AI, RA, MT.

Gains are consistent for Method 1.

AHG8 Lossless results not presented.

Incremental gains for Method 2 over Method 1 seem small.

Method 1 AHG5 is not being damaged.

Method 2 does not look interesting.

Method 1 – has CABAC initial probability been tuned? No. Can these gains be achieved by tuning the current contexts.

Context tuning generally required. There is an ‘illegal’ initial value for RDPCM inter.

Concern over AHG8 word editing – it is giving degraded performance, but class-average results are OK.

Recommendation: adopt method 1 (addition of a context)

- JCTVC-P0282 AHG5 and AHG8: Cross-check of parameter coding methods for inter-component residual prediction in JCTVC-P0154 [W.-S. Kim, W. Pu (Qualcomm)] [late]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (1)

### Encoder optimization

JCTVC-P0059 Unifying HM and RExt Inter-Prediction Search [K. Sharman, N. Saunders, J. Gamei (Sony)]

At last meeting adopted CCP, inter search function backward compatibility issue with HM was present in the provided software.

The proponent’s inter search function was provided as a macro. Currently have a bug-free HM compatible search and a new search.

Propose either:

- Patch HM to be compatible with modified search

- Remove modified search

Changes are additional CABAC resets.

HM-12.1 compatible search: For each component the entropy coder is reset. Then transform and quantize to prodice estimate. Then, for each component do reset, check null block, to dequantise and inverse transform. [refer contrib for method description].

Proposed inter search uses a single loop. Proponent prefers this, but couldn’t introduce without affecting HM backward compatibility.

Issue for CCP is that decision making needs to know in advance if luma null block or not, because affects chroma.  In HM search, the chroma decisions are made on a luma block, which could in a subsequent loop be set to a null (cbf=0) block.

HM search is introducing losses in AHG5 conditions compared to the proposed search (0.2 to 0.3% losses).

For SC, sometimes the SC search is better.

Main aspect appears to be modifying HM for introducing the new function. No impact on class F. Due to SC losses, maybe some future modification would also be required. Big concern is having two methods of the search, and that the best search depends on your test sequences.

Proponents preferred solution is to have the new search.

The new code is much easier to work with, due the cleaner structure.

Does not need to be constrained by the RExt time-frame.

Recommendation: Further study to reduce the SC losses.

JCTVC-P0166 Some Syntax Modifications for HEVC Range Extension [Y. Yu, L. Wang Arris)]

Aspect 1: Propose to move the rotation and single ctx flags from the SPS to the PPS.

What happens in the lossless case (i.e. transquant bypass enabled)?

These flags could be renamed to make clear that they are applicable to both transform skip and transquant bypass.

One suggestion was to rename the flags to make clear the scope of these flags.

[revisit requested]

Transquant bypass enable and transform skip enable are both signalled in the PPS.

Transform skip minimum block size is in the PPS as is chroma QP offset signalling.

Parameters signalled in the PPS are those expected to change during the sequence.

So why isn’t intra block copy enabled signalled in the PPS?

Would like to review the allocation of enable signalling for all RExt tools between PPS/SPS/both.

This change seems to be HLS related.

Recommendation: Consider moving of the rotation and single context enable from the SPS to the PPS conditional on transquant bypass or transform skip being enabled.

Revisit in plenary.

Aspect 2: Intra block copy

When intra block copy flag is signalled it is tested unnecessarily in two places in the coding unit syntax. This is also an editorial change.

This was suggested to the editors as a possible change.

Recommendation: Highlight this editorial change to the editors.