| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0289 |
| --- | --- |

| Title: | Non-RCE1: Combination of subtests A1 and B3.a of RCE1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm, Inc. |  |  |

_____________________________

# Abstract

This document reports a combination of subtests A1 and B3a of HEVC Range Extensions core experiment 1. In this combination, as proposed in subtest A1, the coding of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for the current 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold. In addition, as proposed in subtest B3a, alignment of the ivlCurrRange to 256 is performed depending on the condition that some escape data (coeff_abs_level_remaining) is present in the current 4×4 subblock

It is asserted that compared to subtest A1, the combination more tightly bounds the number of bins coded using bypass CABAC mode for each 4×4 subblock. It is also reported that under AHG18 test conditions, the average bin count reduction for intra 16-bit and lowdelay 16-bit configurations is 18% and 11%, respectively.

# Introduction

This document reports a combination of subtests A1 and B3a of HEVC Range Extensions core experiment 1 (RCE1). In this combination, the coding of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for the current 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold. In addition, as proposed in subtest B3a, alignment of the ivlCurrRange to 256 is performed depending on the condition that some escape data (coeff_abs_level_remaining) is present in the current 4×4 subblock.

# Simulation results

The proposal is implemented on top of HM12.1_Rext5.1. Simulations are performed under AHG18, AHG5 and AHG8 (lossless) test conditions. The performance is compared to the anchor in terms of BD-rate savings.

## AHG18 results

Table 1: BD-rate results for the proposed combination of subtests A1 and B3a under AHG18 test conditions (threshold 4)

Table 2: BD-rate results for the proposed combination of subtests A1 and B3a under AHG8 test conditions (threshold 4)

Table 3: BD-rate results for the proposed combination of subtests A1 and B3a under AHG5 test conditions (threshold 4)

# Average Bin counts

Table 4 presents the average bin counts for AHG18 test conditions. It can be seen that for 16-bit test conditions, the average bin count reduction for intra and lowdelay configurations is 18% and 11%, respectively.

Table 4: Average bin counts for the proposed combination of subtests A1 and B3a under AHG5 test conditions (threshold 4)

# Worst-case complexity analysis

|  | Total number of context bins | Total number of bypass bins with CABAC | Total number of bypass bins (raw) |
| --- | --- | --- | --- |
| Base | 16+8+1 = 25 | (1+46) + (1+39)*15 = 647 | 0 |
| coeff_abs_level_greaterX bypass, no bypass alignment | 16 | 16 (sign flags) | 0 |
| No coeff_abs_level_greaterX bypass + bypass alignment | 16+8+1 = 25 | 0 | (1+46) + (1+39)*15 = 647 |
| coeff_abs_level_greaterX bypass + bypass alignment | 16 | 0 | (1+46) + (1+39)*15 = 647 |

This assumes fast Golomb Rice parameter adaptation with a maximum of 7.

# Conclusions

We have proposed a combination of subtests A1 and B3a of HEVC Range Extensions core experiment 1. In this combination, as proposed in subtest A1, the coding of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for the current 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold. In addition, as proposed in subtest B3a, alignment of the ivlCurrRange to 256 is performed depending on the condition that some escape data (coeff_abs_level_remaining) is present in the current 4×4 subblock.

Worst case complexity analysis indicates that the combination more tightly bounds the number of bins coded using bypass CABAC mode for each 4×4 subblock to 16. It is also reported that under AHG18 test conditions, the average bin count reduction for intra 16-bit and lowdelay 16-bit configurations is 18% and 11%, respectively.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).