| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0291 |
| --- | --- |

| Title: | Cross-check report for P0091: On palette update for palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for the proposal described in Document JCTVC-P0091. No palette update option is cross-checked and it is reported the all the results released by the proponent are confirmed. Analysis of the source code reveals perfect matching between the algorithm and the implementation.

# Introduction

JCTVC-P0091 addresses the memory size while updating the dictionary in palette mode coding. Five different options are provided by the proponents and this cross-check report only tests Option 0, i.e. no palette update. The experimental results and source code analysis are reported in the following sections.

# Experimental results

The proposed no palette update method has been tested over RCE3 coding conditions described in document JCTVC-O1123. It should be noted that given the participation of the cross-checker to RCE3 activities rather than RCE4, the results reported in the following differ from the ones included in JCTVC-P0091 because of different anchors. However, it is assessed that the obtained coding results are identical to those released by the proponent. In the following the results obtained by the cross-checker are reported in Table 1 and Table 2 for lossy and lossless coding, respectively.

Table 1: BD-rates obtained using RCE3 anchors and lossy coding.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -1.7% | -1.1% | -1.1% | -1.8% | -1.3% | -1.4% | -1.7% | -1.5% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -7.9% | -7.9% | -8.2% | -10.7% | -10.6% | -10.7% | -13.8% | -14.8% | -14.7% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -5.5% | -7.4% | -8.4% | -9.0% | -10.6% | -11.0% | -12.5% | -14.4% | -15.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -23.0% | -22.0% | -22.8% | -26.3% | -26.2% | -26.8% | -32.9% | -33.0% | -34.1% |
| YCbCr 4:4:4 SC (Optional) | -18.3% | -24.0% | -23.6% | -25.2% | -29.8% | -30.2% | -33.0% | -38.6% | -39.3% |
| Enc Time[%] | 113% |  |  | 112% |  |  | 110% |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  | 92% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -2.5% | -1.1% | -0.5% | -1.9% | -1.1% |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -7.5% | -7.4% | -7.7% | -9.6% | -9.7% | -9.8% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | -5.3% | -7.8% | -8.7% | -8.6% | -10.5% | -10.8% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -19.4% | -18.4% | -19.0% | -22.3% | -21.8% | -22.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -16.8% | -25.8% | -26.2% | -21.3% | -28.0% | -29.1% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -2.3% | -0.6% | -0.2% | -1.6% | -0.4% |  |  |  |
| Class B | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -3.3% | -3.3% | -3.5% | -4.5% | -4.6% | -4.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -2.3% | -3.1% | -3.5% | -3.7% | -4.2% | -4.5% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.2% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -9.3% | -8.9% | -9.3% | -13.2% | -12.8% | -13.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -9.9% | -14.5% | -14.6% | -13.3% | -17.6% | -17.5% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

Table 2: BD-rates obtained using RCE3 anchors and lossless coding.

# Source code inspection

The proposed no update method has been implemented on top of the HM-12.1-RExt-5.1 source code with palette coding mode as released to RCE4 participants. All source code changes are marked with the following macro:

- NO_PALETTE_UPDATE

This macro disables the update of the dictionary used for palette coding in the member function “getPLTPred” of the TComDataCU class. The source code analysis performed by the cross-checker reveals perfect matching between the algorithmic description and the provided implementation.

# Draft text changes

Not provided at time of cross-checking