| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0292v9 |
| --- | --- |

| Title: | JCT-VC BOG report: Color Gamut Scalability (CGS) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Duenas | Email: | alberto@ngcodec.com |
| Source: | BOG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the break out group (BoG) on Color Gamut Scalability during the 16th JCT-VC meeting.

This  BoG was held during from Friday the 10th to  Tuesday 14th of January 2014 to work on Color Gamut Scalability activities.

This report conveys a number of recommendations from the BoG.  These recommendations are throughout the report, the expression "Recommends:" is used to express the consensus in the group when a positive action should be taken.

# Mandates

This BoG will further discuss the setup of the CE and complexity assessments:

Items to be investigated (mandates):

- Define the test conditions for a potential CE

- Define the methodology for assessment of complexity of different CGS techniques

# BoG Agenda

The following agenda was proposed and agreed by the participants to be used during this BoG.

- Starting with the creation of a methodology for assessment of the complexity.

- Reviewing the current methodology for complexity analysis for SHVC.

- Creating a new spreadsheet for the analysis of complexity and reviewing it during the meeting (if possible using one proposal as an example).

- Reviewing the current proposals and identifying similarities between the current proposed techniques.

- Defining the test conditions and CE, starting with the last SCE1 conditions.

# CGS Complexity analysis

As agreed, the BoG started by trying to define a methodology for assessment of the complexity of different color gamut scalability techniques.

The description of the current complexity assessment used for SHVC (JCTVC-L0440) was presented as it was agreed to use it as a good starting point to create the complexity assessment for CGS.

One of the proponents indicated that one of the goals of SHVC could be to keep the complexity of the SHVC approach not bigger than the Simulcast approach, but it was not agreed by the group that this is a necessary element when measuring the complexity assessment for CGS.

It was indicated by one of the proponents that we could evaluate the results on the 2x case, mostly for the worse case algorithmic analysis. The group considered that it was relevant to review the use cases for color gamut scalability, because of that the group decided that the color gamut use cases described on JCTVC-P0127 were presented to the BoG.

From the presentation two important use cases were identified as most relevant for this work:

1. HEVC HD (1080p50/60) with Rec.709 and 8 bits to: HEVC UHD-1 (2160p50/60) with Rec.2020 and 10 bits

2. HEVC UHD-1 (2160p50/60) with Rec.709 and 10 bits to: HEVC UHD-1 (2160p50/60) with Rec.2020 and 10 bits. (The resolutions, bit depth and frame rate are the same in both layers. The only differences between the two layers will be just the Gamut. In this second use case, the base layer and the enhancement layer will be 10 bits or above.)

The BoG recommends further reviewing of contribution JCTVC-P0127 On a CGS profile for SHVC.

It was noted that it could be possible that in the future we may have other different use cases, but it is unclear how important those use cases would.

Because of these use cases the BoG recommends testing the 1x and the 2x spatial scalability cases.

The BoG recommends continuing using the current 2160p sequences for 1x and using for the 2x case a new set of 1080p down sampled version of the existing 2160p.

In relation to bit depth the BoG recommends using 8 bits on the base layer for the 2x approach and 10 bit only for the 1x approach.

The BoG continued reviewing the spatial aspects on the current JCTVC-L0440 (used in past for the complexity assessment of SHVC), extensive analysis of the different items was conducted.

Based on the review and the analysis of JCTVC-L0440 it was noted that the following items are important to take into account to evaluate complexity of Color Gamut Scalability. The BoG recommends using the following items when analyzing the algorithmic complexity of each of the techniques:

- Consider the number of multipliers and their type (i.e. 8 bits or 16 bits or any other type). As we are now considering 10 Bits input we should consider the different cost for different type of multipliers. It was noted that use cases may have a mixed type of operation and this may be affecting some implementations. The BoG recommends that when we do worse case analysis we should consider the different types of operations and those should be reported independently

- Reporting the potential sizes of LUT in number of table entries contained on the LUT

- Reporting the number of stages and a short summary of each stage (reporting how many passes of the data or pipeline stages are needed)

- Reporting if re-sampling is used when reporting the number of multiplications

- Reporting if cross color dependency is being used

It was decided that it is not necessary to report the memory access for each of the proposals.

It was mentioned that we might want to report the description of the encoder optimizations being use on the different schemes and based on the discussion the BoG recommends that proposals should include descriptions of the encoder optimizations being use.

It was noted that we should create a complexity analysis during this meeting and further work will be done on a future session.

# Color Gamut Scalability CE preparation

The group suggested allowing some time to identify similarities between the different schemes and potentially reducing the number of techniques to be tested. It was agreed to allow some time for the next session of this BoG.

It was noted that we currently have two test cases and we may want to consider reviewing if we cant to continue with this in the next CE.

After reviewing this, the BoG recommends that we should only use one test and it is preferred to test with only updating of the parameters at every picture, the BoG recommends only using the current picture to calculate the parameters.

The group briefly reviewed the current contributions under consideration:

Those include: P0063, P0124, P0197:

-	Only investigating 8x2x2 LUT configuration (P0063/P0197) in combination with entropy coding elements from P0128 and P0186.

-	Investigating P0124 configurations 2, 3, 4. To be discussed in BoG whether investigation of configuration 1 is also of benefit.

The group agrees to recommends allowing configuration 1 of P0124 to be used on the CE.

Currently the list of proposals that the group recommends to tested on the CE is as follows:

- P0063, this is one test.

- P0197, these are two tests (with and without additional phase alignment filters).

- P0128, with  8x2x2 LUT configuration only. This is one test.

- P0124 with configurations 1, 2, 3, 4. These are 4 test.

This will be 8 individual test.

The proponents of these techniques indicated that there are similarities between the proposed techniques and that harmonization could be considered to simplify the CE.

The BoG reviewed the JCTVC-O1011 used on the previous SCE1.

The group recommends to add the additional 1080p BT.709 versions of the current sequences to the test as described above, these are downscaled version versions of the previously used 2160p sequences.

The groups recommends using an updated version of the software what we used on the previous test. The groups recommends using SHM5.0, we will update the anchor to use SHM5.0 as it is currently configured.

These will lead to the following Two tests (A and B) will be conducted one for 1x and another for 2x.

Test A: The test conditions for 2x scalability will be with the following contents:

Base layer	Enhancement layer

Test A	1920 x 1080, BT. 709, 8-bit to	3840 x 2160, BT.2020, 10-bit

Test B: The test conditions for 1x scalability will be with the following contents:

Base layer	Enhancement layer

Test B	3840 x 2160, BT. 709, 10-bit	3840 x 2160, BT.2020, 10-bit

The common SHVC test conditions (QPs) will be used for AI and RA configurations, 1x and  2x scalability.

# CGS Complexity analysis continuation

The updated spreadsheet proposed to analyze the complexity of each of the proposals was created, presented and reviewed by the members of the BoG, It was agreed using this spreadsheet and perform further reviewing offline and this spreadsheet was uploaded with an updated version of the notes. The members of the BoG would like to thank E. Alsina for creating this spreadsheet and for its clear presentation.

The BoG recommends using this spreadsheet to evaluate the complexity of color gamut scalability proposals.

The BoG considered that no further topics needed discussion and the BoG agreed presenting the recommendations to plenary after verifying if the proponents could harmonize the proposals to reduce the number of test to be considered on the CE.

# CE preparation continuation

The group reviewed the notes from the BoG and founds them accurate.

The groups reviewed the process being used for the creation of the test sequences being proposed on the BoG and agrees that the creation process being used does not seems to be favoring any of the techniques being investigated and it is accurately representing the actual workflow being used in the industry for the creation of multiple color gamut representations.

The group reviewed the set of QPs being consider for the testing. One recommendation was to use the same set of QPs for the 1x and 2x. Another proponent indicated that we need to avoid to use the same QPs that are used for current SNR test in SHVC as the conditions are very different.

It was indicated that for the 1x case the QPs may be preferable to be the same for the 2 layers. Another proponent indicated that it is preferable to use the current QPs being use for SNR scalability (that would be -4 and -6). Some proponents suggested that using the same QPs for SNR as CGS may not be a good approach. One of the proponents indicated that the use cases may required similar quality for the base layer and the enhancement layer and it was agreed that this may a good starting point to define the QPs to be used on the two layer. After reviewing the available data from previous test it was felt that the QPs should be more similar than in the SNR common test condition case and -6 may not be producing relevant results for the CGS used cases being considered. It was felt by the BoG that we currently may not have sufficient data to make a decision on the QPs used and the group agreed to run some simple test with QPs of 0, -2 and -4 to try to decide the QPs to be used on the CE and make a decision at an additional session of the BoG to decide the QPs to be used for the 1x.

The group discussed the QPs to be used for the 2x case and the BoG recommends to keep the existing Qps used on the test conditions used in the past for the 2x testing.

The BoG reviewed the comments in relation to update of the parameters and memory bandwidth complexity analysis expressed during the plenary and felt that it was not needed to modify the previous recommendations.

We reviewed the previously available test conditions for previous color gamut scalability CE and made a number of modifications to reflect the recommendations from the BoG.

The group met again to work on additional items based on the feedback from the JCT-VC plenary and the join meeting with Requirements, JCT-VC and VCEG.

The conclusions from this Joint meeting were reviewed on the BoG.

The group discussed the QPs to be used for the 1x case based on preliminary data obtained.

With the previously agreed goals of delivering similar quality by the Base layer and Enhancement layer, one of the proponents suggested to use 0 and -2, another proponents indicated that he would prefer to also add -4 to the test to potentially use other use cases in the future. The BoG recomends to use only two deltas QPs being used and the BoG suggest to use delta QPs to be 0 and -4.

The BoG recommends to use for the base layer the same QPs as in the 2x case.

The groups agrees that we have fully define the Core Experiment Conditions.

Technicolor updated the group that all the sequences are now available on the Hannover ftp and they are working to address the comments being expressed before in relation to the Copyright statement of these sequences.

One of the experts expressed that some SIMD implementations may have difficulties to support some of the available options to support CGS and this needs to be taken into account when evaluation the complexity of the different options.

One of the proponents indicated that this is implementation depend and it is possible to implement the available proposals in a SIMD implementation efficiently. One of the proponents indicated that that is possible to used a SIMD implementation to perform LUT operations and proposed the following option.

General color conversion process: (In general, following formulas applies. Minor variations may be observed in different proposals. With additional techniques, such as vertex coding, dynamic range during calculation may be further reduced.)

input: (y,u,v),  base layer reconstruction

output (Y,U,V), converted samples.

Color conversion formulas.

Y  = LUT[yuvIdx][0][0] * y + LUT[yuvIdx][0][1] * u + LUT[yuvIdx][0][2] * v + LUT[yuvIdx][0][3]

U  = LUT[yuvIdx][1][0] * y + LUT[yuvIdx][1][1] * u + LUT[yuvIdx][1][2] * v + LUT[yuvIdx][1][3]

V  = LUT[yuvIdx][2][0] * y + LUT[yuvIdx][2][1] * u + LUT[yuvIdx][2][2] * v + LUT[yuvIdx][2][3]

Please note that yuvIdx is derived from (y,u,v) with simple operations like shift. More details may be related proposals.

An example of SIMD calculation for Y is as follows. U and V calculation is similar.

Y  = LUT[yuvIdx][0][0] * y + LUT[yuvIdx][0][1] * u + LUT[yuvIdx][0][2] * v + LUT[yuvIdx][0][3] * 1

Step1: Load (LUT[yuvIdx][0][0] , LUT[yuvIdx][0][1] , LUT[yuvIdx][0][2] , LUT[yuvIdx][0][3] ) into one SIMD register Reg0. Please note that the four coefficients above share the same table entry yuvIdx.

Step2: Load (y, u, v, 1) into another SIMD register Reg1.

Step3: Multiply Reg0 and Reg1 and accumulate with instructions such as _mm_madd_epi16 (SIMD intrinsics)

Technicolor has updated the licensing terms for the sequences that they have provided to perform testing on SGC. This new license is uploaded with the BoG report.