| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0297 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0237 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Cross-layer POC anchor picture derivation (follow-up of JCTVC-P0056/JCT3V-G0042) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Nokia, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The contribution follows up part 1 of JCTVC-P0056/JCT3V-G0042 (version 2), which proposed a cross-layer POC anchor picture derivation on top of the so-called POC reset approach proposed in JCTVC-P0041/JCT3V-G0031. It is asserted that the specification text of this contribution includes the modifications agreed by JCT-VC (as documented in the JCT-VC meeting notes related to JCTVC-P0056 on 10th January, 2014).

# Summary of the proposal

The following changes relative to JCTVC-P0041/JCT3V-G0031 are proposed:

- A VPS-level flag that indicates whether slice_pic_order_cnt_lsb values are the same in all slice headers of an access unit and specifies whether cross-layer POC anchor picture derivation is in use.

- When cross-layer POC anchor picture derivation is in use, a POC anchor picture is selected to be the previous picture with TemporalId equal to 0 and discardable_flag equal to 0 that is not a RASL picture, a RADL picture or a sub-layer non-reference picture. This change is implemented in the specification text with the following changes:

  - PrevPicOrderCnt[ lId ] is updated for the lId values equal to the nuh_layer_id value of the current picture (as earlier) and the nuh_layer_id values of the layers for which the current layer is a direct or indirect reference layer.

  - The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture or to any layer for which the current layer is a direct or indirect reference layer is decremented by DeltaPocVal.

- When cross-layer POC anchor picture derivation is in use, poc_msb_val is present for CRA and BLA pictures of independent layers and may be present for any pictures.

The detailed proposal is indicated with change-marked text in the accompanying specification text document. The changes relative to the specification text in version 2 of JCTVC-P0056/JCT3V-G0042 are indicated by user identities "(v2)" and "Ye-Kui Wang".

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).