| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0300 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0238 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On alt_output_layer_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution responds to the comments on the alt_output_layer_flag design presented in JCTVC-P0132/JCT3V-G0133.

Two options are presented to modify the current alt_output_layer_flag design:

- Keep the current alt_output_layer_flag syntax.

Derive AltOutputLayerFlag[ outputLayerSetIdx ] for each output layer set. AltOutputLayerFlag[ outputLayerSetIdx ] is equal to alt_output_layer_flag, if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise AltOutputLayerFlag[ outputLayerSetIdx ] is equal to 0.

- Include alt_output_layer_flag for each output layer set that has only one output layer.

Derive AltOutputLayerFlag[ outputLayerSetIdx ] for each output layer set. AltOutputLayerFlag[ outputLayerSetIdx ] is equal to alt_output_layer_flag[ outputLayerSetIdx ], if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise AltOutputLayerFlag[ outputLayerSetIdx ] is equal to 0.

An alternative output layer is proposed to be selected among the direct and indirect reference layers of the output layer. In the current MV-HEVC/SHVC text an alternative output layer could be any layer with smaller nuh_layer_id as that of the output layer. It is asserted that limiting the selection of alternative output layers to the direct and indirect reference layers of the output layer provides better support for including auxiliary picture layers in the same output layer set as quality/spatial scalability layers containing primary pictures.

Additionally, a VPS VUI indication is proposed to indicate whether a bitstream that is formed by removing, from another conforming bitstream, any coded picture that is not used as a reference for prediction for any other picture and is not the only coded picture of an access unit remains conforming.

# Introduction

## Summary of related MV-HEVC/SHVC concepts and features

The MV-HEVC/SHVC design includes output layer sets specified in the VPS extension, each of which specifies the layers that are output by the decoder when that output layer set is in use. The selection of the output layer set in use is intended to be performed by external means, similarly to selecting the target temporal level for example. In order to enable conformance checking in the HRD the selected output layer set must remain unchanged for the entire CVS.

The MV-HEVC/SHVC draft specification derives a default output layer set for each layer set specified in the VPS. The VPS extension flag default_one_target_output_layer_flag, when equal to 0, specifies that each layer is an output layer, and when equal to 1, specifies that only the highest layer is an output layer. In addition, to the default output layer sets, the VPS extension enables to specify additional output layer sets with selected layers indicated to be output layers.

The MV-HEVC/SHVC draft specification includes alt_output_layer_flag in the VPS extension. When alt_outputLayer_flag is equal to 0, pictures that are not at the target output layers are not output. When alt_outputLayer_flag is equal to 1 and a picture at the target output layer is not present in an access unit, a picture with highest nuh_layer_id among those pictures of the access unit for which PicOutputFlag is equal to 1 and which are not among the target output layers is output.

## Use cases and motivation for alt_output_layer_flag

The MV-HEVC/SHVC design allows access units in which the pictures may represent a subset of the layers present in the bitstream. This characteristic can be used for example for the following purposes:

- Different picture rate in different layers, e.g. the base layer at a lower spatial resolution with 60 Hz and the enhancement layer at a higher spatial resolution with 30 Hz.

- Use SHVC for enabling adaptive resolution change of otherwise single-layer bitstream (see e.g. JCTVC-M0040)

- Layer-wise start-up, where the number of layers is step-wise increased after the start of the bitstream (see e.g. JCTVC-N0066)

alt_output_layer_flag can be used to support use cases where the highest layer changes from one access unit to another.

It can be content-dependent whether the output of lower spatial and/or SNR layers is desired in the absence of the specified output layer. In an example, the base layer is coded at a lower quality or spatial resolution with 60 Hz and the enhancement layer at a higher quality or spatial resolution with 30 Hz (Figure 1). The content provider can intend the enhancement layer to be consumed at 30 Hz (not interleaving the base layer pictures into the output e.g. due to too heavy quality fluctuation, Figure 2), while in other cases the content provider can intend the enhancement layer decoding to produce a 60-Hz output (Figure 3). With alt_output_layer_flag, encoders specify whether it is required or disallowed to output a non-output-layer picture in the absence of a picture in the output layer in the same access unit.

Figure 1. BL at a lower spatial resolution or quality with 60 Hz and the enhancement layer at a higher spatial resolution or quality with 30 Hz

Figure 2. Output behavior of the 2-layer output layer set where the EL is the output layer and alt_output_layer_flag is equal to 0. Content creator has considered the "green" pictures to cause too much fluctuation of quality.

Figure 3. Output behavior of the 2-layer output layer set where the EL is the output layer and alt_output_layer_flag is equal to 1. Content creator has considered the "green" pictures to be suitable for output with the EL pictures.

It is asserted that in many architectures the interface between the decoder and the display process consists of the output pictures as specified in the HEVC (extension) standard. To support use cases where the highest layer might not be present in all access units in all conforming decoders, the use of the alternative output layer is specified in the HRD.

It is noted that the decoder has no means to know if sub-bitstream extraction has been applied previously while VPS has not been updated to reflect the result of the sub-bitstream extraction. Hence, the decoder may select an output layer set to be used in decoding and playback for which the output layer is not present in the bitstream. When alt_output_layer_flag is equal to 1 for the selected output layer set from which the output layer has been removed, a standard-conforming decoder still outputs a useful sequence. Without the alternative output layer mechanism being specified or used, a standard-conforming decoder outputs nothing in the presented case.

# Response to comments in JCTVC-P0132/JCT3V-G0133

The following enumerated bullets correspond to comments presented in JCTVC-P0132/JCT3V-G0133 on the alt_output_layer_flag design.

- Burden for encoders

  - More than one output layer, e.g. multiview, in an output layer set

We agree that the adopted alternative output layer set design was not intended to cover cases with more than one output layer in the same output layer set. We propose that the use of the feature is turned off when there are more than one output layer in an output layer set in either of the following ways:

    - Keep the current alt_output_layer_flag. Derive a variable for each output layer set AltOutputLayerFlag[ outputLayerSetIdx ] which is equal to alt_output_layer_flag, if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise is equal to 0.

    - Include alt_output_layer_flag for each output layer set that has only one output layer. Derive a variable for each output layer set AltOutputLayerFlag[ outputLayerSetIdx ] which is equal to alt_output_layer_flag[ outputLayerSetIdx ], if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise is equal to 0.

  - Determining of parameters

Having looked at this asserted issue, we found that only MaxDecPicBufferingMinus1 (for sub-DPBs of alternative output layers) is potentially affected, if pictures at the output layer of an output layer set are accidentally lost or individually removed (with a process other than the sub-bitstream extraction). The other parameters are access unit based and are hence not affected.

When determining MaxDecPicBufferingMinus1 for an alternative output layer, encoders should consider the worst case, i.e. that the output layer has been entirely removed. This does not typically cause additional burden to encoders. For example, if there are two layers, the encoder derives the value for MaxDecPicBufferingMinus1 of the base layer for the output layer set consisting of the base layer only. This same value of MaxDecPicBufferingMinus1 suits the base layer also in the output layer set consisting of both layers and the base layer being the alternative output layer.

We note that after removal of individual pictures (with a process other than the sub-bitstream extraction) may cause HRD parameters (particularly CPB-related parameters for CBR mode) to become invalid regardless of whether the alternative output layer feature is used or not.

To indicate whether MaxDecPicBufferingMinus1 is indicated robustly when alt_output_layer_flag is equal to1 and whether other HRD parameters are tolerant against removal of individual pictures that are not used as reference for prediction, we propose a new VPS VUI indication.

- JCTVC-P0132/JCT3V-G0133 asserts that decoder implementation can try to achieve the same behavior without the need of such flag, as it can choose to output any picture or perform any other course of action when a picture at a target output layer is not present in an access unit.

It can be content-dependent whether the output of lower spatial and/or SNR layers is desired in the absence of the specified output layer. In an example, the base layer is coded at a lower quality or spatial resolution with 60 Hz and the enhancement layer at a higher quality or spatial resolution with 30 Hz. The content provider can intend the enhancement layer to be consumed at 30 Hz (not interleaving the base layer pictures into the output e.g. due to too heavy quality fluctuation), while in other cases the content provider can intend the enhancement layer decoding to produce a 60-Hz output. With alt_output_layer_flag, encoders specify whether it is required or disallowed to output a non-output-layer picture in the absence of a picture in the output layer in the same access unit. Without alt_output_layer_flag and the suggested decoder implementation behavior in JCTVC-P0132/JCT3V-G0133, decoders could output (and display) pictures that the content creator did not intend to be displayed.

As discussed above, the output of pictures from a non-output layer might cause an increase in the MaxDecPicBufferingMinus1 for those layers. Hence, decoder implementations trying to achieve the same behavior as caused by the alt_output_layer_flag must prepare for this increased MaxDecPicBufferingMinus1 for non-output layers. However, there is no mechanism to indicate how much the increased MaxDecPicBufferingMinus1 requirement is for the suggested decoder implementation behavior in JCTVC-P0132/JCT3V-G0133 without alt_output_layer_flag.

Additionally, the suggested decoder implementation behavior in JCTVC-P0132/JCT3V-G0133 would break the design principle has been to disallow the output (and display) of pictures with PicOutputFlag equal to 0.

- JCTVC-P0132/JCT3V-G0133 asserts that when a picture with a different resolution from a lower layer is output in between pictures from the target output layer, all in the same coded video sequence, the sequence of output pictures, if displayed, would look odd without interpolation and so on. That means, some implementation-specific trick would be needed anyway for typical use cases such as spatial scalability to make the solution complete.

We note that typical display processes are capable of scaling the video to meet the resolution of the displaying window.

An option would be to require the use of the in-loop resampling process to be applied when a picture of an alternative output layer is output (at the stage when the picture is cropped and output). However, that option would limit the implementation freedom and hence not preferred by us.

- JCTVC-P0132/JCT3V-G0133 asserts that when there is more than one target output layer, the behaviour of the flag could lead to wrong order of output of pictures within an access unit. One could signal alt_output_layer_flag for each output layer set, and restrict the value of the flag to be equal to 0 when the corresponding output layer set has more than one target output layer.

As responded to bullet 1a above, we agree that the feature was not designed for the case for multiple output layers and propose that the use of the feature is turned off when there is more than one output layer in an output layer set.

- JCTVC-P0132/JCT3V-G0133 asserts that when multiple views are coded, user experience would be affected when a picture from one view is output instead of a picture from another view. Thus, the alt_output_layer_flag should not be applied to multi-view scenarios.

As responded to bullet 1a above, we agree that the feature was not designed for the case for multiple output layers and propose that the use of the feature is turned off when there is more than one output layer in an output layer set.

# Overview of the proposal

Two options are presented to modify the current alt_output_layer_flag

- Keep the current alt_output_layer_flag syntax. Derive a variable for each output layer set AltOutputLayerFlag[ outputLayerSetIdx ] which is equal to alt_output_layer_flag, if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise is equal to 0.

This option is indicated with change marks of user identity "(option1)" in the accompanying specification text.

- Include alt_output_layer_flag for each output layer set that has only one output layer. Derive a variable for each output layer set AltOutputLayerFlag[ outputLayerSetIdx ] which is equal to alt_output_layer_flag[ outputLayerSetIdx ], if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise is equal to 0.

This option is indicated with change marks of user identity "(option2)" in the accompanying specification text.

The common parts for both options are indicated with change marks of user identity "Miska Hannuksela" in the accompanying specification text.

An alternative output layer is proposed to be selected among the direct and indirect reference layers of the output layer. In the current MV-HEVC/SHVC text an alternative output layer could be any layer with smaller nuh_layer_id as that of the output layer. It is asserted that limiting the selection of alternative output layers to the direct and indirect reference layers of the output layer provides better support for including auxiliary picture layers in the same output layer set as quality/spatial scalability layers containing primary pictures.

Additionally, a VPS VUI indication is proposed to indicate whether a bitstream that is formed by removing, from another conforming bitstream, any coded picture that is not used as a reference for prediction for any other picture and is not the only coded picture of an access unit remains conforming.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).