| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0301 |
| --- | --- |

| Title: | On the TU process and PU partitioning in Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, 1Wen-Hsiao Peng 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution investigates the combined effects of the TU process (TU-level prediction) and PU partitioning on IntraBC’s compression performance, to address a concern raised in the RCE BoG meeting (JCTVC-P0288) regarding how essential the TU process is to IntraBC, especially when combined with the NxN and/or 2NxN/Nx2N PUs. Several tests involving various combinations of the TU process (on vs. off) and PU partitioning (NxN vs. 2NxN/Nx2N vs. NxN/2NxN/Nx2N) were conducted, with the performance and runtime results reported under the RCE3 common test conditions (JCTVC-01123). It was found that the coding efficiency losses from disabling the TU process are in the range of 0-0.5% in the lossy configuration, and are slightly higher, ranging from 0-1%, in the lossless configuration.

# Experimental Results

Experiments were conducted using the HM-12.1+RExt-5.1 software [3] and the RCE3 common test conditions [1]. The configuration for each test is detailed as follows. Note that some of the tests are part of the RCE3 while the others are new. For those in RCE3, the results are from the summary report [3].

- Test A: 2NxN/Nx2N (w/o TU process) [JCTVC-P0176; part of RCE3]

- Test B: 2NxN/Nx2N (w/ TU process) [JCTVC-P0189; part of RCE3]

- Test C: NxN (w/o TU process) [cross-checked by Samsung]

- Test D: NxN (w/ TU process) [JCTVC-P0145; part of RCE3]

- Test E: 2NxN/Nx2N (w/o TU process), NxN (w/o TU process) [cross-checked by Microsoft]

- Test F: 2NxN/Nx2N (w/ TU process), NxN (w/ TU process) [cross-checked by Microsoft]

## Performance Summary for Lossy Coding

|  |  | AI-RExt, Main-tier |  |  |  |  | AI-RExt, Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -3.9% | -8.8% | -8.7% | 124% | 99% | -3.2% | -8.7% | -8.6% | 127% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B | Nx2N/2NxN | -4.0% | -9.3% | -9.0% | 128% | 100% | -3.3% | -9.1% | -9.0% | 131% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| C | NxN | -3.5% | -8.7% | -8.4% | 130% | 99% | -3.1% | -9.6% | -9.4% | 127% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D | NxN | -3.5% | -9.2% | -8.8% | 130% | 98% | -3.1% | -10.0% | -9.9% | 128% | 97% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -4.6% | -10.8% | -10.3% |  |  | -4.0% | -11.1% | -11.0% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -4.7% | -11.2% | -10.8% | 142% | 100% | -4.1% | -11.6% | -11.4% | 149% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |

|  |  | RA-RExt, Main-tier |  |  |  |  | RA-RExt, High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -2.9% | -7.2% | -7.1% | 102% | 99% | -2.7% | -7.1% | -7.4% | 102% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B | Nx2N/2NxN | -2.9% | -7.7% | -7.4% | 106% | 99% | -2.7% | -7.7% | -7.7% | 107% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| C | NxN | -2.5% | -7.4% | -7.1% | 101% | 103% | -2.4% | -7.8% | -7.7% | 102% | 102% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D | NxN | -2.5% | -7.8% | -7.5% | 102% | 96% | -2.4% | -8.1% | -8.0% | 104% | 97% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -3.3% | -9.1% | -8.4% |  |  | -2.4% | -9.2% | -8.8% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -3.5% | -9.5% | -9.0% | 105% | 100% | -3.2% | -9.7% | -9.4% | 107% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |

|  |  | LB-RExt, Main-tier |  |  |  |  | LB-RExt, High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -1.5% | -5.4% | -5.1% | 102% | 100% | -1.5% | -5.7% | -5.5% | 103% | 100% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| B | Nx2N/2NxN | -1.6% | -5.5% | -5.1% | 104% | 101% | -1.6% | -5.9% | -5.6% | 104% | 101% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| C | NxN | -1.3% | -5.2% | -4.7% | 101% | 103% | -1.3% | -5.9% | -5.3% | 101% | 102% |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| D | NxN | -1.3% | -5.5% | -4.8% | 102% | 101% | -1.3% | -6.2% | -5.7% | 104% | 101% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -1.8% | -6.6% | -5.9% |  |  | -1.9% | -7.2% | -6.7% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -1.8% | -6.8% | -6.2% | 104% | 100% | -1.9% | -7.4% | -6.9% | 105% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |  |  |  |  |  |

### TU process on vs. off

The tables below summarize the BD-rate improvements from enabling the TU process (TU-level prediction) for 2NxN/Nx2N or NxN IntraBC, or their combination. Figures shown are computed as

(BD-rate number with TU process on - BD-rate number with TU process off).

A negative value means an (delta) improvement in BD-rate saving obtained from additionally applying the TU process.

It can be seen that the additional coding efficiency benefits offered by the TU process are mostly in the range of 0-0.5%. Moreover, the results appear to be independent of the PU type used for IntraBC and hold true across different configurations.

|  | AI-RExt, Main-tier |  |  | AI-RExt, Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | -0.1% | -0.4% | -0.4% | -0.1% | -0.4% | -0.4% |
| w/ vs. w/o |  |  |  |  |  |  |
| NxN | -0.0% | -0.4% | -0.4% | -0.0% | -0.4% | -0.4% |
| w/ vs. w/o |  |  |  |  |  |  |
| Nx2N/2NxN/NxN | -0.0% | -0.3% | -0.2% | -0.0% | -0.4% | -0.3% |
| w/ vs. w/o |  |  |  |  |  |  |

|  | RA-RExt, Main-tier |  |  | RA-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | 0.0% | -0.5% | -0.3% | 0.0% | -0.5% | -0.3% |
| w/ vs. w/o |  |  |  |  |  |  |
| NxN | 0.0% | -0.4% | -0.4% | 0.0% | -0.4% | -0.3% |
| w/ vs. w/o |  |  |  |  |  |  |
| Nx2N/2NxN/NxN | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.5% |
| w/ vs. w/o |  |  |  |  |  |  |

|  | LB-RExt, Main-tier |  |  | LB-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.2% |
| w/ vs. w/o |  |  |  |  |  |  |
| NxN | 0.0% | -0.3% | -0.1% | 0.0% | -0.3% | -0.2% |
| w/ vs. w/o |  |  |  |  |  |  |
| Nx2N/2NxN/NxN | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| w/ vs. w/o |  |  |  |  |  |  |

### (2NxN/Nx2N vs. NxN) with TU process on/off

The tables below summarize the differences in BD-rate numbers between 2NxN/Nx2N and NxN IntraBC, both with the TU process on or off. Figures shown are computed as

(BD-rate number of 2NxN/Nx2N - BD-rate number of NxN) with the TU process on or off.

As such, a negative (respectively, positive) value suggests 2NxN/Nx2N performs better (respectively, worse) than NxN.

It is observed that these two IntraBC types perform very close to each other, with the average BD-rate differences in the tested sequences in Class F, RGB SC and YUV SC being less than 1%. It is also found that 2NxN/Nx2N performs slightly better in Main-tier configuration and in Class F sequences while NxN is slightly more advantageous in High-tier and Super-High-tier configurations, which is in line with the general observation that a larger partition size is likely to be more beneficial at lower rates whereas a smaller partition size is more preferred at higher rates. This observation appears to hold true across different configurations and regardless of whether the TU process is on or off.

|  | AI-RExt, Main-tier |  |  | AI-RExt, Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.3% | -0.1% | -0.3% | -0.1% | 0.9% | 0.8% |
| w/ TU proc. | -0.4% | -0.1% | -0.3% | -0.2% | 0.9% | 0.8% |

|  | RA-RExt, Main-tier |  |  | RA-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.4% | 0.2% | 0.0% | -0.3% | 0.6% | 0.4% |
| w/ TU proc. | -0.5% | 0.0% | 0.1% | -0.3% | 0.5% | 0.4% |

|  | LB-RExt, Main-tier |  |  | LB-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.2% | -0.2% | -0.4% | -0.2% | -0.2% | 0.0% |
| w/ TU proc. | -0.3% | 0.0% | -0.3% | -0.3% | 0.4% | 0.0% |

### (2NxN/Nx2N/NxN vs. NxN) with TU process on/off

The tables below summarize the differences in BD-rate numbers between the combination of 2NxN/Nx2N and NxN and the single use of NxN, both with the TU process on or off. Figures shown are computed as

(BD-rate number of 2NxN/Nx2N/NxN - BD-rate number of NxN) with the TU process on or off.

Therefore, a negative value means an (delta) improvement in BD-rate saving obtained from the combination.

It is observed that relative to the single use of NxN IntraBC, additional 1-2% coding efficiency benefits can be achieved from the incorporation of 2NxN/Nx2N IntraBC. Turing on and off the TU process causes a slight change (+/-0.2%) in most cases.

|  | AI-RExt, Main-tier |  |  | AI-RExt, Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -1.1% | -2.0% | -1.9% | -0.9% | -1.5% | -1.5% |
| w/ TU proc. | -1.2% | -2.0% | -2.0% | -1.0% | -1.6% | -1.5% |

|  | RA-RExt, Main-tier |  |  | RA-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.8% | -1.7% | -1.3% | -0.8% | -1.5% | -1.1% |
| w/ TU proc. | -1.0% | -1.7% | -1.5% | -0.8% | -1.6% | -1.4% |

|  | LB-RExt, Main-tier |  |  | LB-RExt, High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.5% | -1.4% | -1.2% | -0.6% | -1.3% | -1.2% |
| w/ TU proc. | -0.5% | -1.3% | -1.4% | -0.6% | -1.2% | -1.2% |

## Performance Summary for Lossless Coding

The following sections present results and analyses for the lossless configuration. The figures shown are computed in the same way as their counterparts in the lossy configuration, except that the BD-rate is replaced by the bit rate to achieve lossless compression.

|  |  | AI-RExt, Lossless |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -1.4% | -4.8% | -5.4% | 140% | 98% |
|  | (w/o TU Proc) |  |  |  |  |  |
| B | Nx2N/2NxN | -1.4% | -5.1% | -5.7% | 143% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |
| C | NxN | -1.3% | -5.9% | -6.3% | 141% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |
| D | NxN | -1.3% | -6.2% | -6.6% | 132% | 93% |
|  | (w/ TU Proc) |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -1.8% | -6.6% | -7.2% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -1.9% | -6.9% | -7.6% | 179% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |

|  |  | RA-RExt, Lossless |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -0.7% | -4.2% | -4.4% | 105% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |
| B | Nx2N/2NxN | -0.8% | -4.2% | -4.5% | 105% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |
| C | NxN | -0.8% | -5.1% | -5.0% | 105% | 100% |
|  | (w/o TU Proc) |  |  |  |  |  |
| D | NxN | -0.8% | -5.4% | -5.3% | 104% | 97% |
|  | (w/ TU Proc) |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -1.0% | -5.4% | -5.8% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -1.0% | -5.7% | -6.1% | 111% | 98% |
|  | (w/ TU Proc) |  |  |  |  |  |
| G | Nx2N/2NxN + NxN | -1.0% | -5.7% | -6.0% | 110% | 99% |
|  | (Hybrid) |  |  |  |  |  |

|  |  | LB-RExt, Lossless |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Class F | RGB SC | YUV SC | Enc. | Dec. |
| A | Nx2N/2NxN | -0.3% | -3.7% | -4.0% | 104% | 99% |
|  | (w/o TU Proc) |  |  |  |  |  |
| B | Nx2N/2NxN | -0.3% | -3.9% | -4.3% | 105% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |
| C | NxN | -0.4% | -4.6% | -4.7% | 104% | 101% |
|  | (w/o TU Proc) |  |  |  |  |  |
| D | NxN | -0.4% | -4.8% | -4.9% | 96% | 99% |
|  | (w/ TU Proc) |  |  |  |  |  |
| E | Nx2N/2NxN + NxN | -0.5% | -5.1% | -5.4% |  |  |
|  | (w/o TU Proc) |  |  |  |  |  |
| F | Nx2N/2NxN + NxN | -0.5% | -5.3% | -5.6% | 109% | 100% |
|  | (w/ TU Proc) |  |  |  |  |  |
| G | Nx2N/2NxN + NxN | -0.5% | -5.3% | -5.5% | 108% | 100% |
|  | (Hybrid) |  |  |  |  |  |

### TU process on vs. off

|  | AI-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | -0.1% | -0.3% | -0.3% |
| w/ vs. w/o |  |  |  |
| NxN | 0.0% | -0.3% | -0.3% |
| w/ vs. w/o |  |  |  |
| Nx2N/2NxN/NxN | 0.0% | -0.3% | -0.3% |
| w/ vs. w/o |  |  |  |

|  | RA-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | 0.0% | 0.0% | -0.2% |
| w/ vs. w/o |  |  |  |
| NxN | 0.0% | -0.3% | -0.3% |
| w/ vs. w/o |  |  |  |
| Nx2N/2NxN/NxN | 0.0% | -0.3% | -0.2% |
| w/ vs. w/o |  |  |  |

|  | LB-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| Nx2N/2NxN | 0.0% | -0.2% | -0.2% |
| w/ vs. w/o |  |  |  |
| NxN | 0.0% | -0.2% | -0.2% |
| w/ vs. w/o |  |  |  |
| Nx2N/2NxN/NxN | 0.0% | -0.2% | -0.1% |
| w/ vs. w/o |  |  |  |

### (2NxN/Nx2N vs. NxN) with TU process on/off

|  | AI-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | 0.0% | 1.1% | 0.9% |
| w/ TU proc. | -0.1% | 1.2% | 0.9% |

|  | RA-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | 0.1% | 0.9% | 0.6% |
| w/ TU proc. | 0.0% | 1.2% | 0.8% |

|  | LB-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | 0.1% | 0.9% | 0.7% |
| w/ TU proc. | 0.1% | 0.9% | 0.6% |

### (2NxN/Nx2N/NxN vs. NxN) with TU process on/off

|  | AI-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.5% | -0.7% | -0.9% |
| w/ TU proc. | -0.6% | -0.7% | -1.0% |

|  | RA-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.2% | -0.3% | -0.8% |
| w/ TU proc. | -0.2% | -0.3% | -0.8% |

|  | LB-RExt, Lossless |  |  |
| --- | --- | --- | --- |
| Test | Class F | RGB SC | YUV SC |
| w/o TU proc. | -0.1% | -0.5% | -0.7% |
| w/ TU proc. | -0.1% | -0.5% | -0.7% |

# Conclusion

This contribution presented a study on the combined effects of the TU process and PU partitioning on IntraBC’s compression performance. It was found that the coding efficiency losses from disabling the TU process are in the range of 0-0.5% in the lossy configuration, and are slightly higher, ranging from 0-1%, in the lossless configuration.

# References

- C. Rosewarne, “BoG report on Range Extensions,” JCTVC-P0288, San José, US, Jan. 2014.

- J. Sole, E. Alshina, D.-K. Kwon, and W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” JCTVC-O1123, Geneva, Oct. 2013.

- JCT-VC, “HM-12.1+RExt-5.1,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.1/

# Patent rights declaration(s)

NCTU/ITRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).