| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0303 |
| --- | --- |

| Title: | Suggested Software for the AHG on Investigation of Palette Mode Coding Tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Xun Guo Patrice Onno PoLin Lai Jun Xu | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com Xun.Guo@microsoft.com Patrice.Onno@crf.canon.fr Polin.Lai@mediatek.com jun.xu@am.sony.com |
| Source: | Qualcomm Incorporated Microsoft Corp. Canon Research Centre France Sony Electronics Inc. MediaTek Inc. |  |  |

_____________________________

# Abstract

RCE4 Test 1 and Test 2 evaluated two color palette coding methods and their respective software implementations was released. This contribution suggests a combined software as the basis for AHG investigation of color palette coding. It is asserted that the suggested basis addresses the issues expressed in the RCE4 discussions. Better coding performance than both methods in RCE4 is reported.

# Introduction

RCE4 [1] tested two palette modes, the so called Test 1 [2] and Test 2 [3]. During the palette coding BoG [4], it was agreed to establish a base software for further investigation on palette coding tool.

This document provides a software and the Working Draft text to facilitate further work in the AhG on investigation for color palette coding. The discussions in RCE4 and the BoG provided feedback on the shortcomings of RCE4 Test 1 and Test 2. The suggested palette design addresses those issues and achieves higher coding gains, thus providing a better solution for the work on palette mode in the AhG.

# Method Description

The suggested software is based on RCE4 Test 2 [2] with the following modifications:

- Removal of the prediction of escape pixel [JCTVC-P0108, JCTVC-P0231]

To reduce the coding complexity and to make the syntax cleaner, escape pixel prediction is disabled in ‘pixel mode’. The quantized escape pixel value is directly coded using the fixed length coding described below.

- Removal of the throughput issue for escape value [JCTVC-P0108, JCTVC-P0231]

To code ‘pixel mode’, fixed length coding is used for each color component with one CABAC context for each bit-plane. To increase the entropy coding throughput, only the MSB is context-coded using a single context while the other bits are coded using bypass CABAC coding. This refinement reduces the worst-case of context-coded bins and also, the number of contexts for the palette method.

- Previous CU palette prediction [P0114, P0153]

In order to limit the impact of the palette prediction on the memory needed to store the palette predictor, only the palette of the last palette-coded CU is used as the predictor. In addition, to offer palette estimation in parallel at CTB line level at encoder side, the palette is only reset at the beginning of each CTB line.

- Copy-from-above syntax redundancy removal [P0095, P0116]

This improvement consists in avoiding coding two consecutive flags of the copy-from-above mode (since the second one could effectively be merged into the first one).

- Removal of the copy-from-above for first line [P0116, P0179]

This improvement consists in avoiding signalling the copy-from-above mode for the first row in the CU (since there is no palette index to be copied from above).

- Adaptive bit length to code the quantized escape value [P0108, P0231]

To code an escape pixel, each component is fixed length coded using the input bit-depth. For lossy coding, due to quantization and depending on the quantization step size, the necessary bit-depth to represent a quantized pixel value might be less than the input bit-depth. This fact is used to reduce the number of signalled bits.

- Escape pixel flag signaling [P108]

This method avoids explicitly signaling of the escape pixel flag by assigning escape a special index (equals to the size of palette).

# Experimental Results

## RExt 5.1 As Anchor

RCE4 common test conditions specified in [1] are used.

Table 1 Summary of AHG8 Lossy Coding Results (Proposed v.s. RExt-5.1 Anchor)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.3% | -3.4% | -2.6% | -1.5% | -2.7% | -2.3% | -1.7% | -2.1% | -1.9% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -14.6% | -13.5% | -13.9% | -16.8% | -16.2% | -16.3% | -19.2% | -19.7% | -19.5% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -10.6% | -13.7% | -15.3% | -14.8% | -17.0% | -17.8% | -18.0% | -20.3% | -21.4% |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -32.6% | -31.4% | -32.7% | -36.5% | -35.9% | -36.9% | -43.0% | -42.9% | -43.9% |
| YCbCr 4:4:4 SC (Optional) | -26.9% | -33.3% | -31.9% | -33.6% | -37.3% | -37.4% | -40.4% | -45.6% | -46.4% |
| Enc Time[%] | 115% |  |  | 115% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  | 97% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -3.6% | -2.0% | -0.7% | -2.4% | -1.5% |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -12.6% | -11.6% | -11.9% | -13.7% | -13.1% | -13.3% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | -9.6% | -12.6% | -14.1% | -12.5% | -14.8% | -15.7% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | -0.2% | -0.1% | 0.1% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -24.5% | -23.2% | -24.4% | -27.4% | -26.5% | -27.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -22.5% | -30.5% | -30.4% | -26.8% | -32.4% | -33.5% |  |  |  |
| Enc Time[%] | 111% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -3.0% | -1.5% | -0.2% | -1.8% | -0.9% |  |  |  |
| Class B | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -5.4% | -5.2% | -5.4% | -6.5% | -6.3% | -6.4% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -4.1% | -5.2% | -6.2% | -5.4% | -6.1% | -6.9% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.2% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -13.0% | -12.3% | -12.7% | -17.6% | -17.0% | -18.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -13.2% | -17.8% | -18.1% | -17.1% | -20.7% | -21.2% |  |  |  |
| Enc Time[%] | 111% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |  |  |  |

Table 2 Summary of AHG8 Lossless Coding Results (Proposed v.s. RExt-5.1 Anchor)

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.3% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -17.4% | -12.4% | -9.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -16.7% | -12.7% | -9.4% |
| YCbCr 4:4:4 Animation | -0.6% | -0.1% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -41.2% | -31.1% | -22.1% |
| YCbCr 4:4:4 SC (Optional) | -40.1% | -32.7% | -26.5% |
| Enc Time[%] | 130% | 111% | 109% |
| Dec Time[%] | 100% | 112% | 115% |

## RCE4 Test 2 As Anchor

Table 3 Summary of AHG8 Lossy Coding Results (Proposed v.s. RExt-5.1 Anchor)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -1.7% | -1.6% | -0.5% | -1.0% | -1.0% | -0.4% | -0.4% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -4.8% | -3.8% | -3.9% | -4.2% | -3.7% | -3.7% | -3.3% | -2.9% | -2.9% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | -3.7% | -5.0% | -5.7% | -3.9% | -4.9% | -5.5% | -3.6% | -4.3% | -4.7% |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -7.7% | -7.4% | -7.7% | -8.2% | -8.0% | -8.1% | -8.4% | -8.2% | -8.3% |
| YCbCr 4:4:4 SC (Optional) | -6.4% | -6.0% | -5.7% | -6.1% | -5.6% | -5.6% | -5.9% | -5.5% | -5.6% |
| Enc Time[%] | 97.6% |  |  | 98.7% |  |  | 99.4% |  |  |
| Dec Time[%] | 91.3% |  |  | 91.5% |  |  | 92.0% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -1.1% | -1.0% | -0.2% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -3.5% | -2.6% | -2.7% | -2.5% | -2.0% | -2.0% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -2.9% | -3.6% | -4.3% | -2.6% | -3.3% | -3.8% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -4.3% | -4.1% | -4.4% | -4.8% | -4.6% | -4.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -4.5% | -2.8% | -2.2% | -4.7% | -3.3% | -3.5% |  |  |  |
| Enc Time[%] | 102.2% |  |  | 102.7% |  |  |  |  |  |
| Dec Time[%] | 102.8% |  |  | 103.1% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.8% | -0.8% | -0.1% | -0.2% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -1.3% | -1.1% | -1.2% | -1.3% | -1.1% | -1.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.2% | -1.5% | -2.2% | -1.2% | -1.3% | -1.8% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -3.0% | -2.6% | -2.6% | -4.1% | -4.0% | -4.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -3.6% | -3.1% | -3.3% | -4.2% | -3.5% | -3.9% |  |  |  |
| Enc Time[%] | 103.0% |  |  | 103.3% |  |  |  |  |  |
| Dec Time[%] | 99.7% |  |  | 99.8% |  |  |  |  |  |

Table 4 Summary of AHG8 Lossless Coding Results (Proposed v.s. RExt-5.1 Anchor)

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.3% | -0.6% | -1.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.7% | -0.5% | -1.4% |
| YCbCr 4:4:4 Animation | 0.2% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -4.2% | -5.1% | -6.1% |
| YCbCr 4:4:4 SC (Optional) | -4.7% | -5.2% | -6.2% |
| Enc Time[%] | 104% | 100% | 106% |
| Dec Time[%] | 96% | 107% | 107% |

# References

- L. Guo, X. Guo, and A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” JCTVC-O1124, Geneva, Switzerland, Oct. 2013.

- X. Guo, Y. Lu, and S. Li, “RCE4: Test1. Major-color-based screen content coding,” JCTVC-P0108, San José, US, Jan. 2014.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, and F. Zou, “RCE4: Results of test 2 on palette mode for screen content coding,” JCTVC-P0198, San José, US, Jan. 2014.

- C. Rosewarne, “BoG report on palette mode,” JCTVC-P0298, San José, US, Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).