| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0304_r1 |
| --- | --- |

| Title: | Text and results for block vector predictor for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Patrice Onno Tangi Poirier Lihua Zhu Jizheng Xu Gary J. Sullivan Sridhar Sankuratri B. Anil Kumar Feng Wu Chao Pang Joel Sole |  |  |
| Source: | Canon Microsoft Corp. Qualcomm |  |  |

_____________________________

# Abstract

Currently the predictor for the block vector of the first CU using intra block copy is (0, 0), which is not a valid block vector. In discussions during the meeting, it was agreed to replace the prediction value (0, 0) with (-W, 0), where W is the width of the CU. Text and results to verify the effectiveness of the change are provided in this document.

# Introduction

In the current intra block copy design, the block vector (BV) prediction value (BVx, BVy) is initalized to (0, 0) at the beginning of each CTU, and the BV is subsequently predicted based on the value of the vector for the preceding CU within the same CTU. It is noted that the initialization value (0, 0) is not actually an allowed value for the BV, and it is thus always necessary for a non-zero BV difference to be sent for the first CU in each CTU. This contribution proposes alternatives for initialization of the BV predictor. The proposal is suggested to be motivated primarily as a clean-up fix.

A simple solution is to always initialize the BV value to (−W, 0), where W is the width of the CU. The coding efficiency impact of this change is reportedly generally slightly beneficial, although acknowledged to be very small.

# Proposed text changes for the simplest approach

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the block vector bvIntra.

The variable nCbS is set equal to 1  <<  log2CbSize.

The variable InitBvIntra[0] = −nCbS;

The variable InitBvIntra[1] = 0;

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

The block vector bvIntra is derived by the following ordered steps, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xCb ][ yCb ][ compIdx ] + InitBvIntra[ CompIdx ]

- Otherwise, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xCb ][ yCb ][ compIdx ] + BvpIntra[ compIdx ]	(8-1)

# Simulation results

Table 1 shows the summary results. The anchor is HM-12.1+RExt5.1. Detailed results can be found in the accompanying Excel file.

Table 1. Result summary of block vector predictor initialization with (-W, 0).

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).