| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0316 |
| --- | --- |

| Title: | Comments on Hybrid Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document contains comments and proposed changes for hybrid scalability with respect to the document JCTVC-P0184v4_atttachment_based_on_P1008v4.docx.

In r1 revision of this document only the document number in the “Document: ”  header section above is corrected with no other changes.

# Introduction

This document contains comments and proposed changes for hybrid scalability for HEVC Extensions [1] with respect to the document JCTVC-P0184v4_atttachment_based_on_P1008v4.docx [2].

Following signaling modifications, bitstream constraints and decoding process modifications are proposed.

- Item1: In VPS extension a bug-fix is proposed for signaling bit rate and pic rate information when base layer is externally specified.

- Item2: In VPS extension parameters min_spatial_segment_offset_plus1[ i ][ j ],  ctu_based_offset_enabled_flag[ i ][ j ], min_horizontal_ctu_offset_plus1[ i ][ j ] are not signaled when base layer is externally specified and is one of the direct reference layers for layer i (i.e. layer_id_in_nuh[ LayerIdxInVps[ RefLayerId[ layer_id_in_nuh[ i ][ j ] ] ] ]  == 0 ).

- Item3: The cross-layer alignment constraint on TSA_N and TSA_R nal_unit_type is relaxed with respect to the base layer when the base layer is externally specified and is a direct or indirect reference layer.

- Item4: The cross-layer alignment constraint on STSA_N and STSA_R nal_unit_type is relaxed with respect to the base layer when the base layer is externally specified and is a direct or indirect reference layer.

- Item5: Modification of the semantics for nuh_temporal_id_plus1 is proposed for pictures of externally specified base layer.

Additionally an inference rule for TemporalId value of externally specified base layer decoded picture is proposed in F 8.1 General decoding process.

Two options for this item5 (option 1 and option 2 are described).

- Item6: Semantics modification and changes are proposed related to hrd_layer_set_idx[ i ]. Item6 applies to both the cases - with the base layer internally or externally specified.

- Item7: Modification to the range of valid values for the index profile_level_tier_idx[ i ] for output layer sets is proposed when base layer is externally specified.

- Item8: It is proposed to signal by external means the value of a variable BlRepFormatIdx which specifies an index into the list of rep_format() syntax structures in the VPS of the rep_format() structure that applies to the  decoded picture with nuh_layer_id equal to 0 specified by external means. This is because without this modification when any of the representation format information for the externally specified layer with nuh_layer_id equal to 0 changes (e.g. change in picture height or width), a new VPS will need to be activated since currently the 0'th representation format structure is always selected to indicate representation format of the base layer.

# Proposal

The proposed text for item1-item8 is marked in the accompanying specification text with respect to the document JCTVC-P0184v4_atttachment_based_on_P1008v4.docx.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 5,” JCTVC-P1008, San Jose, January 2014..

[2] Y.-K. Wang, J. Chen, Y. Chen, Hendry, A. K. Ramasubramonian, ‘Support of AVC base layer in SHVC,” JCTVC-P0184v4, February 2014.