| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0021r1 |
| --- | --- |

| Title: | SCE1: Summary Report of Colour Gamut and Bit Depth Scalability |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE summary |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Alberto Duenas, Elena Alshina Yan Ye Kemal Ugur, Xiang Li, | Emails: | pierre.andrivon@technicolor.com alberto@ngcodec.com elena_a.alshina@samsung.com yan.ye@interdigital.com kemal.ugur@nokia.com lxiang@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE1 on Colour Gamut and Bit Depth Scalability. The evaluated tools and the test conditions are defined in document JCTVC-P1101.

# Introduction

First works ([3]) related Colour Gamut and Bit Depth Scalability (CGS) in the scalable extensions of HEVC were presented in JCT-VC in October 2012. An Ad Hoc Group (AHG14, [2]) and a Core Experiment (SCE1, [1]) have been established to assess different technologies.

The main challenge of CGS is to define a pertinent color and bit-depth predictor (an inter-layer prediction mechanism) by predicting the Enhancement Layer (EL) samples from the reconstructed Base Layer (BL) samples (Cf. figure 1).

Figure 1: Colour Gamut and Bit Depth Scalability principle

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| X. Li J. Chen M. Karczewicz V. Seregin | lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com | Qualcomm | X | X |
| Y. Ye Y. He J. Dong | yan.ye@interdigital.com yuwen.he@interdigital.com jie.dong@interdigital.com | InterDigital | X |  |
| P. Andrivon P. Bordes E. François F. Hiron | pierre.andrivon@technicolor.com philippe.bordes@technicolor.com edouard.françois@technicolor.com franck.hiron@technicolor.com | Technicolor | X |  |
| K. Ugur A. Aminlou | kemal.ugur@nokia.com ext-alireza.aminlou@nokia.com | Nokia | X | X |
| E. Alshina E. Alshin A. Saxena | elena_a.alshina@samsung.com alexander_b.alshin@samsung,com asaxena@sta.samsung.com | Samsung |  |  |
| K. Misra J. Zhao S.-H. Kim | misrak@sharplabs.com jzhao@sharplabs.com kimse@sharplabs.com | Sharp Labs |  | X |
| K. Sato C. Auyeung | kazushi.sato@jp.sony.com cheung.auyeung@am.sony.com | Sony |  | X |
| A. Duenas | alberto.duenas@ngcodec.com | NGCodec |  |  |
| K. Minoo | koohyar.minoo@gmail.com | Arris |  | X |

# Tests conditions

## Test material

Test material is provided by AHG14 ([2]) and the test conditions are described in SCE1 description ([1]).

The sequences with the associated license conditions are available at the Hannover FTP site: ftp.tnt.uni-hannover.de/scalable/sequences/CGS with scalability use credentials. An MD5 file per file is provided to verify that the locally down scaled version of the sequences are identical to the sequences on the FTP site.

Table 1: List of test sequences

| Test Sequences | Size | fps | Frames Nb | Bit-depth |
| --- | --- | --- | --- | --- |
| BT709_Birthday_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart1_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart2_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Parakeets_1920x1080_50_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 8 |
| BT709_TableCar_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT709_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT2020_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Birthday_3840x2160_60_10bit.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart1_3840x2160_60_10bit.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart2_3840x2160_60_10bit.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_Parakeets_3840x2160_50_10bit.yuv | 3840 x 2160 | 50 | 250 | 10 |
| BT2020_TableCar_3840x2160_60_10bit.yuv | 3840 x 2160 | 60 | 300 | 10 |

## Reference software (anchors)

The SCE1 reference software (anchors) is SHM5.0 with a SAO bug-fix provided by Vadim Seregin to support 10 bit bit-depth. It includes bit-depth scalability and the option to set the weighted prediction (WP) restricted to inter-layer prediction (ILP) (as adopted in JCTVC-O0194). SHM5.0_irap, built on top of SHM5.0, supports the insertion of regular Intra Random Access Point (IRAP) with corresponding SPS and PPS on BL and EL in order to emulate more realistic broadcast/broadband use cases. A patch on top of SHM5.0  (revision 598) was provided by Technicolor to the SCE1 participants ten days after the SHM5.0 release. The anchors are based on SHM5.0 with WP ILP. The anchors preclude the IRAP insertion modification.

The IRAP insertion modifications to SHM5.0 are enabled with the macro SPS_PPS_CRA_PERIODICITY. The parameter SpsPpsPeriod (or “-SpsPpsPeriod” in the command line argument) allows to set the IRAP periodicity as a multiple of the frame rate (modulo 8 as for IntraPeriod), for RA or AI configurations. For example, “-SpsPpsPeriod 1” will insert a IRAP every 48 frames for 50Hz, every 64 frames for 60Hz, etc… It is noted that  “-SpsPpsPeriod 0” (default value) corresponds to SHM5.0 release with the SAO bug-fix and only one IRAP at the beginning of the bitstream. The IRAP periodicity has been set to 1 second, which corresponds to realistic TV broadcast IRAP periodicity value.

When using different BL and EL bit-depths a decoder crash was observed for regular SHM5.0 revision 598. The bug-fix (in “TAppDecTop.cpp”) as well as the IRAP insertion modifications have been provided to the SCE1 participants by Technicolor.

## Description of experiments

Two test cases have been performed: 2x and 1x scalability with Random Access (RA) and All Intra (AI) configurations.

Table 2: Test conditions

|  | Base layer | Enhancement layer |
| --- | --- | --- |
| 2x: RA and AI | 1080p 8-bit BT.709 QP_BL={22, 26 ,30, 34} | 2160p 10-bit BT.2020 QP_EL1={22, 26, 30, 34} QP_EL2={24, 28, 32, 36} |
| 1x: RA and AI | 1080p 10-bit BT.709 QP_BL={22, 26 ,30, 34} | 1080p 10-bit BT.2020 QP_EL1={22, 26, 30, 34} QP_EL2={18, 22, 26, 30} |

For the 2x scalability, the common SHVC test conditions (QPs) have been used for AI and RA configurations.

For the 1x scalability, they have been used the same QPs for the Base Layer as on the SHVC test conditions and for the enhancement layer delta QPs are 0 and -4.

At the encoder side, the computation of the CGS information does not utilize information other than current picture. However encoder may signal CGS information for each picture if necessary.

Table 3: Summary of SCE1 and non-SCE1 Colour Gamut and Bit-Depth Scalability documents

| Test number | Method | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- |
| 1.1 | Linear matrix mapping (N=8) | JCTVC-Q0072 (Nokia) | JCTVC-Q0143 (Sharp) |
| 1.2 | Linear matrix mapping (N=1) | JCTVC-Q0072 (Nokia) | JCTVC-Q0123 (Arris) |
| 1.3 | Region based GO mapping (N=8) | JCTVC-Q0072 (Nokia) | JCTVC-Q0196 (Sony) |
| 1.4 | Polynomial based mapping (N=1) | JCTVC-Q0072 (Nokia) | JCTVC-Q0097 (Qualcomm) |
| 2.1 | Asymmetric 3D LUT mapping Phase alignment filter enabled | JCTVC-Q0048 (Technicolor, Interdigital, Qualcomm) | JCTVC-Q0059, JCTVC-Q0222 (Nokia) |
| 2.2 | Asymmetric 3D LUT mapping Phase alignment filter disabled | JCTVC-Q0048 (Technicolor, Interdigital, Qualcomm) | JCTVC-Q0144, JCTVC-Q0222 (Sharp) (Nokia) |

# Summary of results

## Test 2x: BL 1080p 8-bit Rec. 709, EL 2160p 10-bit Rec. 2020

| Test 2x | AI HEVC 2x 8-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test number | Y | U | V | Y | U | V |
| 1.1 | -8.5% | -12.3% | -16.7% | -6.4% | -10.2% | -14.2% |
| 1.2 | -3.2% | -3.2% | -4.4% | -2.1% | -1.8% | -3.5% |
| 1.3 | -5.1% | -4.4% | -9.8% | -4.3% | -4.8% | -9.4% |
| 1.4 | -6.7% | -7.8% | -8.0% | -4.6% | -4.8% | -5.7% |
| 2.1 | -8.2% | -10.3% | -13.2% | -6.0% | -7.8% | -11.5% |
| 2.2 | -7.6% | -10.0% | -12.8% | -5.7% | -7.6% | -11.2% |

  - Test 1x: BL Rec. 709, EL Rec. 2020 (1080p 10-bit)

| Test 1x | AI HEVC 1x 10-bit base |  |  | RA HEVC 1x 10-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test number | Y | U | V | Y | U | V |
| 1.1 | -8.1% | -15.3% | -18.4% | -9.7% | -15.9% | -19.4% |
| 1.2 | -3.3% | -6.1% | -5.7% | -3.2% | -5.1% | -5.7% |
| 1.3 | -5.2% | -6.1% | -12.0% | -6.9% | -8.3% | -13.8% |
| 1.4 | -7.7% | -11.1% | -10.0% | -8.1% | -10.4% | -10.8% |
| 2.1 | -9.9% | -15.9% | -18.8% | -10.4% | -16.3% | -21.0% |
| 2.2 | -8.9% | -15.6% | -18.5% | -9.9% | -16.2% | -20.8% |

## Complexity evaluation

| Test nb |  |  | 1.1 | 1.2 | 1.3 | 1.4 | 2.1 | 2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multipliers number per EL pixel | 2x case |  | 4.5 | 4.5 | 1.5 | 13.5 | 1.125 | 1.125 |
|  | 1x case |  | 4.5 | 4.5 | 1.5 | 13.5 | 4.5 | 4.5 |
| Multiplier bit-depth |  |  | 15-bit x EL bitdepth | 15-bit x EL bitdepth | 15-bit x EL bitdepth | 15-bit x EL bitdepth | 12-bit x BL bitdepth | 12-bit x BL bitdepth |
| Table entries |  |  | 6144 (8x8x8x4x3) | 12 (1x1x1x4x3) | 3072 (8x8x8x3x2) | 21 (1x1x1x3x7) | 384 (8x2x2x4x3) | 384 (8x2x2x4x3) |
| Cross-color dependency |  | partitioning | Yes | N/A | Yes | N/A | Yes | Yes |
|  |  | processing | Yes | Yes | No | Yes | Yes | Yes |
| Colour mapping/upsampling order |  |  | after | after | after | after | before | before |
| Multiplier/sample value dependency* |  |  | Yes | No | Yes | No | Yes | Yes |

# *multiplier of each pixel depends on the value of the pixel

# Appendix: non-SCE1

| Technology/Document type | Document |
| --- | --- |
| Denoising of ILR | JCTVC-Q0043 |
| Encoder improvements for WP | JCTVC-Q0056 |
| Regions number reduction (test 1.1-based) | JCTVC-Q0129 |
| Coefficients coding (test 1.1 & 1.3-based) | JCTVC-Q0141 |
| Cross-check of Q0129 | JCTVC-Q0199 |
| FRNB Comment on CGS | m33412 |

- References

- A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li , K. Ugur, K. Sato, S.-H. Kim (CE coordinators), “Description of HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability”, JCTVC-P1101, January 2014, San José, USA.

- P. Bordes, A. Duenas, E. Alshina, S. Deshpande, Y. He, D.-K Kwon, X. Li, “JCT-VC AHG report: Colour gamut scalability (AHG14)”, JCTVC-P0014, January 2014, San José, USA.

- L. Kerofsky et al, “Color Gamut Scalable Video Coding”, JCTVC-K0241, October 2012, Shanghai, CN

- A. Luthra et al, “Requirements of the scalable enhancement of HEVC”, w12956, July 2012, Stockholm, SE