| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0022 |
| --- | --- |

| Title: | RCE1: Summary Report of adaptive motion precision |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | X. Li J. Xu Y. Zhou | Emails: | lxiang@qti.qualcomm.com jzxu@microsoft.com youzhou@microsoft.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in RCE1 on adaptive motion precision. The evaluated tools and the test conditions are defined in document JCTVC-P1121.

# Introduction

Adaptive motion precision techniques were proposed in JCTVC-P0277 and JCTVC-P0283. To further investigate these techniques, this core experiment was established.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| Y. Zhou J. Xu | jzxu@microsoft.com youzhou@microsoft.com | Microsoft | X | X |
| X. Li J. Sole | lxiang@qti.qualcomm.com joels@qti.qualcomm.com | Qualcomm | X | X |
| A. Saxena C. Park S. Lee E. Alshina | a.saxena1@samsung.com cy1205.park@smasung.com sunil.lee@samsung.com elena_a.alshina@samsung.com | Samsung |  |  |
| P. Onno G. Laroche | Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr | Canon |  |  |
| Y. Huang S.-T. Hsiang P. Chuang | Yuwen.huang@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com | MediaTek |  |  |
| X. Xiu | Xiaoyu.Xiu@InterDigital.com | InterDigital |  |  |

# Document list

|  | Proposal | Crosscheck |
| --- | --- | --- |
| Test 1 | JCTVC-Q0155 (Microsoft) | JCTVC-Q0099 (Qualcomm) |
| Test 2 | JCTVC-Q0049 (Qualcomm) | JCTVC-Q0156 (Microsoft) |

# Tools to be tested

## JCTVC-Q0155

It is proposed to add a high-level indicator (at the SPS, PPS, and/or slice header level) to indicate the resolution for interpretation of the motion vectors.

Before encoding a slice, encoder may decide the motion vector resolution based on historical statistical data, knowledge of its type of application, multi-pass analysis, or some other such technique. If the encoder decides to use full pixel motion only, the fractional pixel search is skipped. If a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value.

At decoder side, if the motion vectors are indicated to be at full pixel resolution and a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value. Motion vector differences are simply interpreted as integer offsets rather than 1/4-sample offsets. All other decoding processes remain same. The parsing process (below the header level) is unchanged. When the motion vectors are coded at full-sample precision and the video uses 4:2:2 or 4:2:0 sampling, the chroma motion vectors can be derived in the usual manner, which will produce 1/2-sample chroma motion displacements. Alternatively, it may be worth considering also rounding the chroma motion.

## JCTVC-Q0049

It is proposed to signal a flag at CU level to indicate whether all PUs in the CU have integer-precision MVs. When the flag is on, MV predictors of non-skip and non-merge PUs are rounded to integer precision and MV differences are signaled in integer precision. For PUs in merge or skip mode, MVs are rounded only before motion compensation so that fractional MVs are saved and can be used as MV predictors for neighboring PUs. When the flag is off, sub-pixel motion compensation is allowed.

# Summary of results

The tests were conducted according to the RCE1 description JCTVC-P1121. The results are summarized as follows.

## RA-MT

|  | JCTVC-Q0155 Seq flag with full search |  |  | JCTVC-Q0155 Seq flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -1.0% | -1.1% | -0.9% | -0.7% | -0.8% | -1.1% | -1.2% | -1.2% | -0.6% | -0.7% | -0.8% |
| Class B | -0.3% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% |
| RGB 4:4:4 SC | -0.7% | -0.7% | -0.8% | 0.0% | 0.0% | 0.0% | -0.9% | -0.9% | -0.9% | -0.6% | -0.6% | -0.6% |
| RGB 4:4:4 Animation | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% | -1.0% | -1.3% | -1.4% | -0.6% | -0.8% | -0.9% |
| YCbCr 4:4:4 SC | -0.7% | -0.6% | -0.6% | 0.1% | 0.1% | 0.1% | -0.6% | -0.5% | -0.5% | -0.4% | -0.3% | -0.3% |
| YCbCr 4:4:4 Animation | -0.5% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | -0.9% | -1.4% | -1.5% | -0.3% | -0.7% | -0.7% |
| RangeExt | -0.3% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |
| RGB 4:4:4 SC (Optional) | -2.4% | -2.4% | -2.4% | -2.0% | -2.0% | -2.0% | -1.8% | -1.7% | -1.8% | -0.5% | -0.5% | -0.5% |
| YCbCr 4:4:4 SC (Optional) | -2.6% | -2.3% | -2.2% | -2.0% | -1.7% | -1.6% | -1.7% | -1.5% | -1.5% | -0.4% | -0.4% | -0.3% |
| Enc Time[%] | 197% |  |  | 97% |  |  | 142% |  |  | 147% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  | 101% |  |  |

## RA-HT

|  | JCTVC-Q0155 Seq flag with full search |  |  | JCTVC-Q0155 Seq flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.0% | -0.9% | -0.9% | -0.8% | -0.7% | -0.7% | -1.2% | -1.3% | -1.3% | -0.7% | -0.8% | -0.9% |
| Class B | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% |
| RGB 4:4:4 SC | -0.8% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% | -1.2% | -1.2% | -1.2% | -0.8% | -0.7% | -0.8% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -1.3% | -1.6% | -1.6% | -0.9% | -1.1% | -1.1% |
| YCbCr 4:4:4 SC | -0.6% | -0.6% | -0.6% | 0.3% | 0.3% | 0.3% | -0.7% | -0.7% | -0.7% | -0.5% | -0.5% | -0.5% |
| YCbCr 4:4:4 Animation | -0.1% | -0.3% | -0.2% | 0.0% | 0.0% | 0.0% | -0.9% | -1.4% | -1.4% | -0.4% | -0.9% | -0.8% |
| RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.3% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -2.3% | -2.2% | -2.2% | -2.0% | -1.9% | -1.9% | -1.8% | -1.7% | -1.7% | -0.7% | -0.6% | -0.6% |
| YCbCr 4:4:4 SC (Optional) | -2.8% | -2.6% | -2.5% | -2.2% | -2.0% | -1.9% | -1.9% | -1.7% | -1.7% | -0.6% | -0.6% | -0.5% |
| Enc Time[%] | 198% |  |  | 98% |  |  | 142% |  |  | 147% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  | 96% |  |  | 101% |  |  |

## LB-MT

|  | JCTVC-Q0155 Seq flag with full search |  |  | JCTVC-Q0155 Seq flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.5% | -1.3% | -1.5% | -0.9% | -0.6% | -0.6% | -1.1% | -0.5% | -1.0% | -0.9% | -0.4% | -0.7% |
| Class B | -0.4% | 0.1% | 0.2% | 0.2% | 0.2% | 0.4% | -0.1% | 0.1% | 0.4% | -0.2% | -0.6% | -0.4% |
| RGB 4:4:4 SC | -1.0% | -1.1% | -1.1% | 0.0% | 0.0% | 0.0% | -1.1% | -1.1% | -1.2% | -0.7% | -0.8% | -0.8% |
| RGB 4:4:4 Animation | -0.3% | -0.4% | -0.5% | 0.0% | 0.0% | 0.0% | -2.0% | -2.1% | -2.0% | -1.3% | -1.4% | -1.4% |
| YCbCr 4:4:4 SC | -1.2% | -1.2% | -1.3% | -0.1% | -0.1% | -0.1% | -0.7% | -0.8% | -1.0% | -0.4% | -0.5% | -0.5% |
| YCbCr 4:4:4 Animation | -0.2% | -0.5% | -0.3% | 0.0% | 0.0% | 0.0% | -0.5% | -1.4% | -1.0% | -0.4% | -1.0% | -0.9% |
| RangeExt | -0.2% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -5.0% | -5.0% | -4.9% | -3.3% | -3.3% | -3.3% | -3.0% | -3.0% | -3.0% | -1.8% | -1.8% | -1.7% |
| YCbCr 4:4:4 SC (Optional) | -5.5% | -5.3% | -5.2% | -2.0% | -1.9% | -1.8% | -3.0% | -2.9% | -2.8% | -1.2% | -1.1% | -0.9% |
| Enc Time[%] | 197% |  |  | 97% |  |  | 142% |  |  | 148% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 93% |  |  | 100% |  |  |

## LB-HT

|  | JCTVC-Q0155 Seq flag with full search |  |  | JCTVC-Q0155 Seq flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.3% | -1.1% | -1.2% | -0.7% | -0.5% | -0.5% | -1.1% | -0.9% | -1.1% | -0.9% | -0.7% | -0.8% |
| Class B | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.3% | -0.1% | -0.1% | -0.3% | -0.4% | -0.3% |
| RGB 4:4:4 SC | -1.2% | -1.2% | -1.2% | 0.0% | 0.0% | 0.0% | -1.5% | -1.5% | -1.5% | -1.0% | -1.1% | -1.1% |
| RGB 4:4:4 Animation | -0.3% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% | -2.6% | -2.6% | -2.4% | -1.5% | -1.5% | -1.5% |
| YCbCr 4:4:4 SC | -1.3% | -1.2% | -1.3% | -0.1% | -0.1% | -0.1% | -1.1% | -1.1% | -1.2% | -0.7% | -0.7% | -0.8% |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.8% | -1.8% | -1.4% | -0.5% | -1.1% | -1.0% |
| RangeExt | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -4.1% | -4.1% | -4.0% | -2.9% | -2.9% | -2.9% | -1.7% | -1.7% | -1.7% | -1.7% | -1.7% | -1.7% |
| YCbCr 4:4:4 SC (Optional) | -5.1% | -5.1% | -4.9% | -2.1% | -2.2% | -2.0% | -2.7% | -2.7% | -2.6% | -1.0% | -0.9% | -0.8% |
| Enc Time[%] | 198% |  |  | 97% |  |  | 143% |  |  | 149% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  | 94% |  |  | 100% |  |  |

## RA-lossless

|  | JCTVC-Q0155 Seq flag with full search |  | JCTVC-Q0155 Seq flag with fast search |  | JCTVC-Q0049 CU flag |  | JCTVC-Q0049 Enc Only |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Aver | Total | Aver | Total | Aver | Total | Aver |
| Class F | -0.1% | -0.2% | -0.1% | 0.0% | -0.5% | -0.5% | -0.3% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.5% | -1.1% | 0.0% | 0.0% | -1.3% | -0.9% | -1.2% | -0.9% |
| RGB 4:4:4 Animation | -0.2% | -0.2% | 0.0% | 0.0% | -0.8% | -0.9% | -0.7% | -0.8% |
| YCbCr 4:4:4 SC | -0.8% | -0.7% | 0.0% | 0.0% | -0.9% | -0.9% | -0.7% | -0.7% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.5% | -0.4% | -0.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.8% | -1.4% | -1.7% | -0.7% | -1.4% | -0.5% | -0.5% | -0.2% |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.7% | -1.6% | -0.6% | -1.5% | -1.4% | -0.6% | -1.0% |
| Enc Time[%] | 195% |  | 98% |  | 137% |  | 142% |  |
| Dec Time[%] | 95% |  | 95% |  | 92% |  | 98% |  |

## LB-lossless

|  | JCTVC-Q0155 Seq flag with full search |  | JCTVC-Q0155 Seq flag with fast search |  | JCTVC-Q0049 CU flag |  | JCTVC-Q0049 Enc Only |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Aver | Total | Aver | Total | Aver | Total | Aver |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | -0.4% | -0.4% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.4% | -1.6% | -0.7% | -0.2% | -1.6% | -1.7% | -1.3% | -1.4% |
| RGB 4:4:4 Animation | -0.2% | -0.3% | 0.0% | 0.0% | -0.8% | -0.9% | -0.6% | -0.7% |
| YCbCr 4:4:4 SC | -0.8% | -0.9% | 0.0% | 0.0% | -0.7% | -0.7% | -0.6% | -0.6% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.5% | -0.4% | -0.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.9% | -0.3% | -1.8% | 0.2% | -1.5% | 0.4% | -0.5% | 1.0% |
| YCbCr 4:4:4 SC (Optional) | -2.0% | -3.3% | -1.8% | -0.6% | -1.5% | -2.6% | -0.6% | -1.8% |
| Enc Time[%] | 193% |  | 95% |  | 138% |  | 141% |  |
| Dec Time[%] | 93% |  | 94% |  | 91% |  | 95% |  |

# Conclusions

It is recommended to discuss the proposals in RCE 1 and the non-RCE1proposals.

# Appendix: Non-RCE1 proposals

| Document Number | Proposal | Crosscheck |
| --- | --- | --- |
| JCTVC-Q0067 | Non-RCE1: On MV resolution and motion vector predictor number | JCTVC-Q0098 (Qualcomm) |
| JCTVC-Q0092 | Non-RCE1: Simplification of RCE1 Test2 | JCTVC-Q0173 (Qualcomm) |