| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0031 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Ying Chen, Ted Hsieh, Rajan Joshi, Marta Karczewicz, Woo-Shi Kim, Xiang Li, Chao Pang, Wei Pu, Krishna Rapaka, Joel Sole, Li Zhang, Feng Zou 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document presents the details of Qualcomm’s response to the Joint Call for Proposals for Coding of Screen Content issued by MPEG and ITU-T. The proposed solution is developed based on the HEVC Range Extensions Draft 6 and the HM13.0-RExt6.0 software. The main changes with respect to the base are the following:

- Extension of the current tools: intra BC, deblocking filter and explicit RDPCM

- New tools: palette and color transform

- Encoder modifications and subjective enhancement for screen content

The proposed screen content video codec reportedly achieves BD-rate savings summarized as follows:

For RGB sequences, G-component, lossy condition:

- text & graphics with motion, 1080p : (AI) 52.7%         (RA) 45.4%         (LB) 41.7%

- text & graphics with motion,720p    : (AI) 39.9%         (RA) 42.2%         (LB) 39.7%

- mixed content, 1440p                       : (AI) 35.4%         (RA) 42.3%         (LB) 48.3%

- mixed content, 1080p                       : (AI) 31.1%         (RA) 36.6%         (LB) 38.1%

- animation, 720p                                : (AI) 23.4%         (RA) 26.5%         (LB) 28.4%

For YUV sequences, Y-component, lossy condition:

- text & graphics with motion, 1080p : (AI) 47.5%         (RA) 36.9%         (LB) 30.7%

- text & graphics with motion,720p    : (AI) 27.7%         (RA) 29.5%         (LB) 26.0%

- mixed content, 1440p                        : (AI) 21.8%         (RA) 22.8%         (LB) 29.5%

- mixed content, 1080p                        : (AI) 20.2%         (RA) 20.6%         (LB) 20.9%

- animation, 720p                                : (AI) 0.6%           (RA) 3.8%           (LB) 6.1%

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Inter-component prediction	4

2.3	Inter-picture prediction	5

2.4	Intra-picture prediction	5

2.4.1	Explicit RDPCM	5

2.4.2	Intra block copying	5

2.5	In-loop filtering	6

2.5.1	Deblocking filter	6

2.5.2	SAO	7

2.5.3	Other	7

2.6	Spatial transforms	7

2.7	Other specific compression tools for screen content	7

2.7.1	Palette	7

2.7.2	1-D Dictionary (for lossless coding)	9

2.8	Quantization	10

2.9	Motion/mode/parameter coding	10

2.9.1	Merge/skip	10

2.9.2	Motion vector coding	10

2.9.3	Intra prediction mode coding	10

2.10	Entropy coding of residual / transform coefficients	10

2.11	Deflickering for All-Intra	10

2.12	Scene Change Detection	10

3	Syntax and semantics description	10

4	Compression performance discussion	10

4.1	Lossy test condition	10

4.2	Mathematically lossless test condition	13

5	Complexity analysis	14

5.1	Encoding time and measurement methodology	14

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	14

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	15

5.4	Expected memory usage of encoder	15

5.5	Expected memory usage of decoder	15

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	15

5.7	Complexity characteristics of decoder motion compensation	15

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	15

5.8.1	Intra block copying	15

5.8.2	Deflickering	16

5.8.3	Fast decision for All-Intra	16

5.9	Complexity characteristics of decoder intra-frame prediction operation	16

5.9.1	Deflickering	16

5.10	Complexity characteristics of encoder inter-component prediction tools	16

5.11	Complexity characteristics of decoder inter-component prediction tools	16

5.12	Complexity characteristics of transforms specific in screen content coding	17

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	17

5.13.1	Palette	17

5.13.2	1-D Dictionary	17

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	17

5.14.1	Palette	17

5.14.2	1-D Dictionary	17

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	18

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	18

5.17	Degree of capability for encoder parallel processing	18

5.18	Degree of capability for decoder parallel processing	18

6	Software implementation description	18

7	Highlighted aspects discussion	18

8	Closing remarks	18

9	References	18

10	Patent rights declaration(s)	19

# Introduction

This document presents the details of Qualcomm’s response to the Joint Call for Proposals for Coding of Screen Content issued by MPEG and ITU-T [1]. This response builds upon the HEVC Range Extensions Draft 6 specification (HEVC RExt Draft 6) [2] and the corresponding reference software HM13.0-RExt6.0 [3].

The summary of the main aspects incorporated in this response are the following:

- Full frame intra block copying (intra BC): a simple extension of the search area for intra BC provides large performance gains for screen content coding (SCC).

- Adaptive color-space transform: since RGB sequences are part of the Call for Proposals (CfP) and out-of-loop color transform is not allowed, a CU-based adaptive color transform is incorporated.

- Explicit RDPCM is extended to all the intra modes.

- Deblocking filter: screen content (SC) sequences typically have large flat areas that show visible blocking artifacts at the low rates. The blockiness is alleviated by extending the deblocking filter. Also, due to the 4:4:4 sampling, the chroma components in SC may have similar characteristics to the luma component, so the luma deblocking filter is also applied to the chroma components.

- Palette mode, which has shown potential in coding of SC [4], is further improved.

- A 1-D dictionary method (related to the one in [5]) is implemented for the lossless category, where it shows some potential to provide further gains on the top of intra BC and palette.

- Deflickering: for the CfP, subjective tests of all-intra configuration at low bit-rate are performed. In this situation, the temporal flickering across frames seems to be the dominant artifact at very high QPs.  A deflickering method is applied to alleviate this artifact.

- Additional encoder-only enhancements are implemented to deal with the specific characteristics of SC.

# Algorithm description

Below is a brief description of the tools added or changed with respect to HEVC Range Extensions. For a more detailed description, please refer to the appendix where syntax and semantics are provided.

## CTU/TU/PU partitioning

Partitioning is the same as in HEVC RExt Draft 6 except for intra BC, in which 2N×N and N×2N partitions are disabled. At the encoder, transform skip 8×8 is enabled and the maximum RQT depth is set to 2.

## Inter-component prediction

In addition to the cross-component prediction (CCP) method in HEVC RExt Draft 6, a CU-level tool named Adaptive Color-space Transform (ACT) is included in this response. The ACT converts one color-space to another one wherein the correlations among the three color components are further reduced. ACT can be applied jointly with CCP to minimize the redundancy among the color components.

In ACT, each CU adaptively enables or disables the color-space transform (CST) depending on an R-D criterion. For intra BC and inter CUs, the CST is performed in the residual domain, i.e., just before invoking the CCP and conventional transform. For intra modes, the CST is applied to both the prediction and the original block, i.e., before the residual generation process. A flag is signaled to indicate the usage of the CST. For intra BC and inter modes, such a flag is signaled only when there is at least one non-zero coefficient in current CU.

Two different CSTs are applied depending on whether the CU is coded losslessly or in a lossy manner (depending on an SPS flag). The forward and the inverse CST for lossy coding are modified YCoCg transforms defined as follows:

Forward: C'0C'1C'2=12120−2−12−1C0C1C2 	Inverse:	C0C1C2=11−11011−1−1C'0C'1C'24

wherein the original color space (C0, C1, C2) may correspond to (R, G, B) or (Y, U, V). For lossless coding, the YCoCg-R transform in [6] is used.

The forward CST used in lossy coding is not normalized, which results in signal amplification when the transform is applied. Considering that the norm of the forward CST is roughly equal to square root of 6, a delta QP of 8 is used to compensate such amplification. In other words, when the CST is applied, the quantization parameter is set equal to QP+8, where QP is the ‘normal’ QP value of the current CU. Meanwhile, the bit-depth is increased by 2 when color-space transform is applied.

## Inter-picture prediction

Inter-picture prediction is unchanged from HEVC RExt Draft 6.

## Intra-picture prediction

Intra prediction is modified in two ways:

- Extension of explicit RDPCM to angular modes.

- Extension of intra BC: full frame search area and other enhancements.

### Explicit RDPCM

The RDPCM mode is explicitly signaled for all blocks (intra, inter and intra BC), thus removing the implicit RDPCM mode for intra. Explicit RDPCM has three modes: RDPCM_OFF, RDPCM_VER, and RDPCM_HOR. The mode is selected based on the SAD of the residuals for each mode. The contexts for the intra explicit RDPCM flags are shown in Table 1 and Table 2.

| Context | Intra prediction mode |
| --- | --- |
| 1 | Intra prediction mode in VER±3 or HOR±3 |
| 0 | Remaining |

Table 1. Context for the first flag (RDPCM ON/OFF)

| Context | Intra prediction mode |
| --- | --- |
| 1 | Intra prediction mode greater than 18 (diagonal direction) |
| 0 | Remaining |

Table 2. Context for the second flag (RDPCM VER/HOR)

### Intra block copying

Intra BC is modified in three ways:

- Larger search area: full frame.

- Refinement of block vector coding.

- Intra N×N is used as in HEVC RExt Draft 6, but the partitions 2N×N and N×2N are disabled, since better performance / encoding time trade-off is obtained by using full frame search.

      - Intra BC full frame search

The search region for intra BC is enlarged to be the whole frame. The encoder uses this larger area for 8×8 and 16×16 block sizes. For 16×16, only 1-D search is performed. For the other block sizes, the encoder search region is limited to the left CTU as in HEVC RExt Draft 6.

A hash-based search is used to speed up the full frame search for 8×8 blocks. The bit-length of the hash table entry is 16. Each node in the hash table records the position of one pixel in the frame. With the hash table, only the blocks having the same hash entry value as the current block are examined using SAD. The block with the minimum SAD is selected as the reference block.

The 16-bit hash entries for the current block and the reference block are calculated by using the original pixel values. Specifically, let Grad denote the gradient of one 8×8 block and let DC0, DC1, DC2 and DC3 denote the DC values of the four 4×4 blocks inside the 8×8 block. Then, the 16-bit hash entry H is calculated as follows:

H = MSB(DC0, 3) << 13 + MSB(DC1, 3) << 10 + MSB(DC2, 3) << 7 + MSB(DC3, 3) << 4 + MSB(Grad, 4)

where MSB(X, n) is the function of extracting the n most significant bits of X.

      - Block vector coding for intra BC

In the block vector (BV) coding, the left or the above block vector is selected as predictor. If one of them is unavailable, (−2*CUwidth, 0) is used instead. If none of them are available, then (−2*CUwidth, 0) and (2*CUwidth, 0) are used instead.

With the selected BV predictor, the resulting block vector difference (BVD) is coded as follows: first, a flag is signaled to indicate whether the difference is zero. When BVD is not zero, an exponential-Golomb code of order 3 is used to code the remaining absolute level of the BVD and a flag to code its sign.

## In-loop filtering

### Deblocking filter

To improve the deblocking filter performance two modifications are made on top of the deblocking filter in HEVC RExt Draft 6. First, luma filtering process is applied to the chroma components as well. Secondly, an additional boundary strength (Bs=3) is introduced to enable stronger filtering. The new filtering is enabled when the corresponding PPS flag is equal to 1.

The procedure to apply the luma filtering process to the chroma components, how to set Bs to 3, and the stronger filtering process are described below.

#### Deblocking chroma as luma

The chroma components are deblocked in the same way as the luma component when the PPS flag deblock_chroma_as_luma is 1. This means that the strong filter and two types of weak filters are available for the chroma blocks and they are applied according to the conditions defined for the luma component (but checking them using the chroma samples). This deblocking filter is applied to both intra- and inter-coded blocks.

#### Boundary strength setting

The boundary strength is set to 3 when the TU boundaries satisfy the following conditions:

- The maximum transform unit size (MaxTrafoSize) is greater than or equal to 16×16;

- The larger transform size between the two adjacent TUs is equal to MaxTrafoSize and one of the following conditions is satisfied:

  - The related CU is intra-coded;

  - The related CU is not intra-coded and the partition size is 2N×2N and cbf of the TU is equal to 1;

- The smaller transform size between the two adjacent TUs is greater than or equal to 16×16;

- The Gradient of the two adjacent 8×8 blocks on the TU boundary is smaller than a certain threshold (set to 48 for I-frame and 16 for B/P-frame in the proposed scheme);

The Gradient of an 8×8 block is calculated as:

GradientH = ∑i,j abs ( P( i, j ) – P( i, min( 7, j + 1 ) ) )

GradientV = ∑i,j abs ( P( i, j ) – P( min( i+1, 7 ), j ) ) )

Gradient = GradientH + GradientV

where P( i, j ) denotes the luma sample value in the 8×8 block and with (i = 0, 2, 4, 6; j = 0, 2, 4, 6) and (i = 1, 3, 5, 7; j = 1, 3, 5, 7).

#### Filtering process when Bs is 3

If Bs is equal to 3 and all the pixels at the TU boundaries are to be processed as strong filtering, the following filtering process is applied. Otherwise the HEVC RExt Draft 6 filtering process is applied.

Let pi and qi be the pixel values across the block boundary before filtering (as specified in 8.7.2.5.7) but extending the indices to i = 0..7.

Then, d is the difference between the pixels across the boundary, d = q0 – p0.

Finally, the modified pixel values pi′ and qi′ with i = 0..7 are calculated as:

pi′ = Clip1( pi + ( ( d * ( 8 – i ) + 8 ) >> 4 ) )

qi′ = Clip1( qi – ( ( d * ( 8 – i ) + 8 ) >> 4 ) )

### SAO

SAO is unchanged from HEVC RExt Draft 6.

### Other

No in-loop filtering is added with respect to HEVC RExt Draft 6.

## Spatial transforms

Spatial transforms are unchanged from HEVC RExt Draft 6.

## Other specific compression tools for screen content

Two specific tools for compression of screen content have been added. A palette coding mode related to the one in [4] and, for lossless coding, a 1-D dictionary technique based on the one in [5].

### Palette

A CU palette mode very similar to the one in [1] is used. For an input CU, the palette encoding process is composed of two stages:

- Palette derivation: Palette is a data structure that stores (index, pixel value) pairs. To derive the palette, the histogram of the pixel values in the current CU is calculated. The peak values in the histogram are added into the palette. As an example, in Fig. 1, there are two peak values ‘r’ (red) and ‘b’ (blue), so two pairs (0, r) and (1, b) are added into the palette. There is an implicit quantization in this operation. The maximum palette size is 32. Low frequency pixel values (e.g. the white pixel in Fig. 1) may not be included in the palette due to the palette size restriction. The quantization step for palette increases with QP.

- CU coding using palette: The pixels in the CU are encoded in a raster scan order using one of the following three modes.

  - “Copy from Left” run mode (CL): in this run mode, one palette index is first signaled followed by a non-negative value n indicating the run length, which means that the following n pixels have the same pixel index as the first signaled one.

  - “Copy from Top” run mode (CT): in this run mode, a positive run length value m is transmitted to indicate that for the following m pixels, the palette indexes are the same as their neighbors in the row above.

  - “Escape” mode: escape mode is used to code pixels that are not in the palette. The quantized pixel values are directly coded.

Fig. 2 is an example to illustrate the palette mode coding. The input CU block has three different colors, i.e. red, white, and blue. As the blue pixel occurs only once, it is not included in the palette (this is encoder decision).  Therefore, there are two entries in the palette: white pixels are mapped to index 0 and red pixels are mapped to index 1. The input CU after mapping is specified as “index block” in Fig. 2.

Following this example, in the second stage, the CT mode has m = 5 and the CL mode has n = 4. This is shown in Fig. 3, where the blue pixel is coded in “escape” mode.

Figure 1. Palette derivation.

Figure 2. Illustration of palette coding process.

Figure 3. Illustration of ‘Copy from Top’ run and ‘Copy from Left’ run in palette mode.

#### Palette Signaling

- Palette entries: neighboring CUs palette tend to have strong correlation. To exploit such correlation and to avoid an extra line buffer, the current CU palette entries are predicted from the last CU coded in palette mode. Specifically, a 0/1 vector is signaled to indicate whether the pixel values in the reference palette are reused by the current palette or not. The number of palette entries that cannot be predicted is binarized and signaled using unary code. The palette prediction vector and the unary coded remaining palette size are coded using CABAC with one context each to improve coding efficiency.

- Run: the method to code the run length values is aligned to the coefficient coding method in HEVC, i.e. coding ‘greater than 0’, ‘greater than 1’, and ‘greater than 2’. Then, if necessary, Golomb-Rice code with parameter equal to 3 is used to code the remaining values.

- Escape pixel: only the MSB of the quantized escape pixel is coded using CABAC context while other bits are coded as bypass.

- Index coding: to reduce the number of context coded bins, index is coded using CABAC bypass. As the size of palette is adaptive and cannot be guaranteed to be a power of 2, truncated binary codes (a special case of Golomb-Rice codes) are used to binarize the index.

### 1-D Dictionary (for lossless coding)

A 1-D dictionary algorithm is used to provide an arbitrary shape matching scheme for lossless coding. The 1-D dictionary algorithm is an alternative coding mode competing with the other HEVC coding modes. An R-D criterion is used to select the best mode in terms of the minimum R-D cost.

The method splits a CTU into multiple pixel strings, where each pixel contains the three color component samples. For each CTU, the pixels are raster scanned into 1-D pixel array. When the string has a matching string in the reconstructed area (as shown in Fig. 4) two syntax elements are coded to indicate the position and length of the match. The syntax element matching_string_offset denotes the relative distance between the current string and the reference string, while the syntax element matching_string_run denotes the matching length.

The matching is applied in units of pixels. In other words, if the three color components (samples) of the reference pixel are equal to the three samples of the current pixel, it is asserted that the current pixel has a matching pixel; otherwise, it is asserted that the reference pixel is not matched with the current pixel. When the pixel has no match in the reconstructed area, the original pixel is directly coded. A typical matching example is shown in Fig. 5.

To enable fast matching at the encoder, a hash value is calculated by a simple concatenation of MSBs of the three samples of a pixel. The number of the bits (nBitHash) for the hash value is configurable at the encoder. The number of MSBs of each component i is derived as follows: (nBitHash+2-i)/3. Seven MSBs from each component are used, and thus a 21 bit hash value is used for each pixel. The hash value is calculated using the reconstructed pixels.

A linked list is defined to store the reference pixel locations with the same hash value. Given the current pixel value, the reference pixel with the same value is found using a linked list implemented in the form of look-up table. After a match is identified between a current and a reference pixel, the string run continues until a mismatch is identified. In case there are multiple reference pixels with the same hash value, the one that has the longest run is selected.

To code the matching string offset, 8 recently coded offsets are used to predict the current one. The predictors are reset to 0 at the beginning of each slice or if the CTU is not coded using 1-D dictionary. If the matching string offset is equal to one the predictors, a predictor flag is set to 1 and a predictor index of 3 bit is coded; otherwise, the predictor flag is set to 0 and the matching string offset is coded with an exponential Golomb like codeword. There is no limitation on the range of the offset, that is, the reference pixel can be anywhere within the reconstructed area.

To code the matching string run, a concatenation of Golomb-Rice and exponential Golomb is employed with the Rice parameter equal to 4. The run is in the range of 1 to 64×64.

Figure 4. 1-D matching example

Figure 5. Example of pixel matching in 1-D dictionary

## Quantization

Quantization is unchanged from HEVC RExt Draft 6. At the encoder, uniform quantization (rounding) is used for the RDPCM residue (as in [7]).

## Motion/mode/parameter coding

### Merge/skip

Merge / skip coding is unchanged from HEVC RExt Draft 6.

### Motion vector coding

Motion vector coding is unchanged from HEVC RExt Draft 6.

### Intra prediction mode coding

Intra prediction coding is unchanged from HEVC RExt Draft 6.

## Entropy coding of residual / transform coefficients

Entropy coding of residual / transform coefficients is unchanged from HEVC RExt Draft 6.

## Deflickering for All-Intra

Flickering artifacts are observed mainly for the All-intra configuration and at low bitrates (high QPs). To mitigate the flickering, the following deflickering method is used.

Before encoding a block in an intra frame, a decision is taken whether the block is likely to flicker. This decision is based on the luma gradient of the original block, the number of major colors of the block, and the SAD between the original block and the collocated block from the next frame. If the block is determined to be in a flickering region, a flag is signaled at the CU level. When the flag is on, all the spatially neighboring blocks of the CU are marked as unavailable for spatial intra prediction. When the flag is off, the coding process in HEVC RExt Draft 6 is used. An SPS flag enables the deflickering method, which is only applied for all-intra configurations and high QPs.

## Scene Change Detection

A scene change detection algorithm is included for low delay configuration to better compress the frames that follow a new scene in the sequence. The picture that starts a new scene is coded as a CRA picture. A scene change is detected using empirical thresholds for the following similarity metrics computed with respect to the current and the closest temporal past frame:

- Sum of absolute difference of the normalized pixel histogram.

- Sum of absolute difference of the average DC.

The scene detection algorithm does not require looking ahead and processing future pictures.

# Syntax and semantics description

See attached appendix for the syntax and semantics description.

# Compression performance discussion

## Lossy test condition

A summary of lossy BD-Rate reduction for both with PSNR clipping for each frame at 58.92 dB and without PSNR clipping in comparison to the RExt 5.1 is shown below. The results without clipping also select points with different PSNR/bitrates (even though a single point might satisfy the rate condition imposed in the CfP) in order to make the BD-rate function able to compute a value. More detailed data is enclosed in the accompanying spreadsheets.

Summary of BD-Rate reduction (without PSNR clip)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -52.7% | -51.9% | -51.6% |
| RGB, text & graphics with motion,720p | -39.9% | -33.4% | -36.0% |
| RGB, mixed content, 1440p | -35.4% | -29.4% | -29.4% |
| RGB, mixed content, 1080p | -31.1% | -26.3% | -25.6% |
| RGB, Animation, 720p | -23.4% | -20.4% | -17.1% |
| YUV, text & graphics with motion, 1080p | -47.5% | -49.3% | -48.9% |
| YUV, text & graphics with motion,720p | -27.7% | -31.3% | -33.8% |
| YUV, mixed content, 1440p | -21.8% | -25.1% | -25.0% |
| YUV, mixed content, 1080p | -20.2% | -22.5% | -22.1% |
| YUV, Animation, 720p | -0.6% | -6.7% | -3.5% |
| Enc Time[%] | 175% |  |  |
| Dec Time[%] | 79% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -45.4% | -43.1% | -42.6% |
| RGB, text & graphics with motion,720p | -42.2% | -34.7% | -38.9% |
| RGB, mixed content, 1440p | -42.3% | -34.4% | -34.6% |
| RGB, mixed content, 1080p | -36.6% | -28.3% | -26.6% |
| RGB, Animation, 720p | -26.5% | -23.2% | -17.1% |
| YUV, text & graphics with motion, 1080p | -36.9% | -40.2% | -39.9% |
| YUV, text & graphics with motion,720p | -29.5% | -33.4% | -38.2% |
| YUV, mixed content, 1440p | -22.8% | -29.3% | -29.1% |
| YUV, mixed content, 1080p | -20.6% | -26.8% | -25.9% |
| YUV, Animation, 720p | -3.8% | -15.1% | -13.1% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 116% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -41.7% | -38.9% | -38.4% |
| RGB, text & graphics with motion,720p | -39.7% | -30.2% | -34.4% |
| RGB, mixed content, 1440p | -48.3% | -38.5% | -38.3% |
| RGB, mixed content, 1080p | -38.1% | -28.2% | -26.0% |
| RGB, Animation, 720p | -28.4% | -19.8% | -12.8% |
| YUV, text & graphics with motion, 1080p | -30.7% | -33.7% | -33.6% |
| YUV, text & graphics with motion,720p | -26.0% | -29.4% | -34.6% |
| YUV, mixed content, 1440p | -29.5% | -38.7% | -38.2% |
| YUV, mixed content, 1080p | -20.9% | -28.7% | -27.8% |
| YUV, Animation, 720p | -6.1% | -18.2% | -17.5% |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 111% |  |  |

Summary of BD-Rate reduction (with PSNR clip)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, text & graphics with motion,720p | -48.3% | -33.4% | -36.0% |
| RGB, mixed content, 1440p | -35.4% | -29.4% | -29.4% |
| RGB, mixed content, 1080p | -31.1% | -26.3% | -25.6% |
| RGB, Animation, 720p | -23.4% | -20.4% | -17.1% |
| YUV, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion,720p | -27.6% | -31.1% | -33.4% |
| YUV, mixed content, 1440p | -21.8% | -25.1% | -25.0% |
| YUV, mixed content, 1080p | -20.2% | -22.5% | -22.1% |
| YUV, Animation, 720p | -0.6% | -6.7% | -3.5% |
| Enc Time[%] | 175% |  |  |
| Dec Time[%] | 79% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, text & graphics with motion,720p | -51.1% | -42.1% | -46.5% |
| RGB, mixed content, 1440p | -42.3% | -34.4% | -34.6% |
| RGB, mixed content, 1080p | -36.6% | -28.3% | -26.6% |
| RGB, Animation, 720p | -26.5% | -23.2% | -17.1% |
| YUV, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion,720p | -29.5% | -33.3% | -37.9% |
| YUV, mixed content, 1440p | -22.8% | -29.3% | -29.1% |
| YUV, mixed content, 1080p | -20.6% | -26.8% | -25.9% |
| YUV, Animation, 720p | -3.8% | -15.1% | -13.1% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 116% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, text & graphics with motion,720p | -39.7% | -30.3% | -34.4% |
| RGB, mixed content, 1440p | -48.3% | -38.5% | -38.3% |
| RGB, mixed content, 1080p | -38.1% | -28.2% | -26.0% |
| RGB, Animation, 720p | -28.4% | -19.8% | -12.8% |
| YUV, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion,720p | -25.9% | -29.3% | -34.3% |
| YUV, mixed content, 1440p | -29.5% | -38.7% | -38.2% |
| YUV, mixed content, 1080p | -20.9% | -28.7% | -27.8% |
| YUV, Animation, 720p | -6.1% | -18.2% | -17.5% |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 111% |  |  |

## Mathematically lossless test condition

The tables below summarize the performance for the mathematically lossless test condition:

|  | AI |  |  |  |
| --- | --- | --- | --- | --- |
| Bit-rate savings | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 51.4% | 51.8% | 44.7% | 65.6% |
| RGB, text & graphics with motion,720p | 23.9% | 25.4% | 5.2% | 46.9% |
| RGB, mixed content, 1440p | 18.5% | 18.1% | 16.6% | 19.7% |
| RGB, mixed content, 1080p | 22.0% | 21.3% | 19.7% | 22.9% |
| RGB, Animation, 720p | 5.2% | 5.2% | 5.2% | 5.2% |
| YUV, text & graphics with motion, 1080p | 49.9% | 50.3% | 40.7% | 66.6% |
| YUV, text & graphics with motion,720p | 21.7% | 24.5% | 3.6% | 52.7% |
| YUV, mixed content, 1440p | 16.2% | 16.0% | 15.0% | 17.0% |
| YUV, mixed content, 1080p | 15.9% | 16.6% | 14.9% | 18.2% |
| YUV, Animation, 720p | 2.2% | 2.2% | 2.2% | 2.2% |
| Enc Time[%] | 238% |  |  |  |
| Dec Time[%] | 78% |  |  |  |
|  | RA |  |  |  |
| Bit-rate savings | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 41.4% | 49.4% | 40.7% | 65.7% |
| RGB, text & graphics with motion,720p | 10.6% | 21.0% | 7.7% | 48.1% |
| RGB, mixed content, 1440p | 13.0% | 13.0% | 12.8% | 13.2% |
| RGB, mixed content, 1080p | 19.3% | 16.5% | 13.2% | 19.8% |
| RGB, Animation, 720p | 9.5% | 9.5% | 9.5% | 9.5% |
| YUV, text & graphics with motion, 1080p | 34.4% | 43.5% | 33.4% | 61.5% |
| YUV, text & graphics with motion,720p | 7.3% | 17.9% | 2.9% | 52.9% |
| YUV, mixed content, 1440p | 5.6% | 5.7% | 4.6% | 6.7% |
| YUV, mixed content, 1080p | 10.9% | 8.7% | 6.1% | 11.2% |
| YUV, Animation, 720p | 2.7% | 2.7% | 2.7% | 2.7% |
| Enc Time[%] | 178% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | LB |  |  |  |
| Bit-rate savings | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 41.5% | 49.0% | 40.6% | 65.3% |
| RGB, text & graphics with motion,720p | 10.3% | 20.8% | 7.7% | 46.7% |
| RGB, mixed content, 1440p | 12.7% | 12.7% | 12.6% | 12.8% |
| RGB, mixed content, 1080p | 19.0% | 15.9% | 12.5% | 19.4% |
| RGB, Animation, 720p | 10.2% | 10.2% | 10.2% | 10.2% |
| YUV, text & graphics with motion, 1080p | 33.3% | 41.0% | 32.9% | 56.3% |
| YUV, text & graphics with motion,720p | 6.1% | 16.3% | 2.4% | 51.1% |
| YUV, mixed content, 1440p | 4.1% | 4.2% | 3.3% | 5.1% |
| YUV, mixed content, 1080p | 10.6% | 7.5% | 4.0% | 11.0% |
| YUV, Animation, 720p | 2.7% | 2.7% | 2.7% | 2.7% |
| Enc Time[%] | 172% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

# Complexity analysis

## Encoding time and measurement methodology

The table below is a summary of encoding times compared to the anchor, which is the ratio of the geometric means for each test configuration. The software is compiled using gcc 4.1.2 and simulation is performed on a Unix cluster. Detailed platform information can be found in section 5.3.

Encoding is performed twice both for the anchor and the tested method, and the average encoding time of two runs for each simulation point is used to generate the ratio of the geometric mean of the tested method over that of the anchor, which is summarized in the tables below.

| Lossy | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 145% | 136% | 143% |
| RGB, text & graphics with motion,720p | 164% | 104% | 116% |
| RGB, mixed content, 1440p | 179% | 98% | 112% |
| RGB, mixed content, 1080p | 239% | 136% | 165% |
| RGB, Animation, 720p | 210% | 136% | 170% |
| YUV, text & graphics with motion, 1080p | 169% | 137% | 145% |
| YUV, text & graphics with motion,720p | 152% | 97% | 111% |
| YUV, mixed content, 1440p | 178% | 98% | 102% |
| YUV, mixed content, 1080p | 221% | 125% | 136% |
| YUV, Animation, 720p | 206% | 118% | 137% |
| All | 175% | 115% | 128% |

| Lossless | AI | RA | LB |
| --- | --- | --- | --- |
| All | 238% | 178% | 172% |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

The table below is a summary of decoding time compared to the anchor, which is the ratio of the geometric means for each test configuration. The software is compiled using gcc 4.1.2 and simulation is performed on a Unix cluster. Detailed platform information can be found in section 5.3.

Decoding is performed five times both for the anchor and the tested method, and the average decoding time of five runs for each simulation point is used to generate the ratio of the geometric mean of the tested method over that of the anchor, which is summarized in the tables below.

| Lossy | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 64% | 117% | 113% |
| RGB, text & graphics with motion,720p | 87% | 114% | 109% |
| RGB, mixed content, 1440p | 99% | 122% | 120% |
| RGB, mixed content, 1080p | 101% | 125% | 115% |
| RGB, Animation, 720p | 109% | 123% | 115% |
| YUV, text & graphics with motion, 1080p | 59% | 110% | 110% |
| YUV, text & graphics with motion,720p | 71% | 111% | 105% |
| YUV, mixed content, 1440p | 76% | 119% | 109% |
| YUV, mixed content, 1080p | 83% | 116% | 113% |
| YUV, Animation, 720p | 92% | 120% | 113% |
| All | 79% | 116% | 111% |

| Lossless | AI | RA | LB |
| --- | --- | --- | --- |
| All | 78% | 104% | 97% |

Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The encoding/decoding test was conducted on a 64-bit Linux cluster with Intel Xeon E5 2680 16-core CPUs (2.7 GHz). Each cluster host has 64 GB memory. All hosts share the same network drive. The encoder and decoder binaries were compiled with gcc 4.1.2 64-bit compiler. Both the proposed codec and the anchor are tested on the same platform.

Expected memory usage of encoder

The encoder stores temporal reference pictures in the decoded picture buffer in a way similar to HM13.0-RExt6.0. In addition, unfiltered samples that are used in intra BC are also stored. For lossless coding, additional memory is needed when 1-D dictionary is used. Further details for each tool are given in the following sections.

## Expected memory usage of decoder

The decoder stores temporal reference pictures in the decoded picture buffer in a way similar to HM13.0-RExt6.0. In addition, unfiltered samples that are used in intra BC and 1-D dictionary are also stored. Further details for each tool are given in the following sections.

## Complexity characteristics of encoder motion estimation and partitioning selection

The motion estimation is modified to better capture the spatial and temporal characteristics in screen content sequences. These sequences exhibit non-monotonic error surface and high spatial frequency characteristics contrary to the assumptions of fast motion estimation algorithms designed for natural camera-captured sequences. The implemented modifications are the following:

- Multi-stage approximate error cost computation with early exit at each stage

- Modified initial search method with a fixed diamond search traversing uniformly across the initial search space

- Modified early skip detection so that merge motion vectors are not involved in the skip decision

The complexity in terms of encoding time of the proposed modifications is an increment of the overall encoding time by around 15% to 20% for low delay and random access configurations.

## Complexity characteristics of decoder motion compensation

Inter motion compensation is unchanged from HM13.0-RExt6.0.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection

### Intra block copying

Hash-based encoder search is employed to speed up the 8×8 block full frame search for intra BC. In current implementation, the bit-length of each hash index is selected to be 16. The memory used to store the hash table depends on the resolution, being the total memory 12×W×H bytes, where W and H are the frame width and height, respectively. For instance, the total memory cost for the hash table is about 23.7 MB for 1080p sequences. In addition, the search region for the 16×16 block is also extended to full frame. However to speed-up the encoder search, its block vector is restricted to be 1-D. For the other block sizes, the search region as in HM13.0-RExt6.0 is used.

Additionally, the encoder search considers the chroma component in the R-D decision (as in [8]). The best 4 block vectors with lowest luma SAD are used in the second step in which the chroma SAD is taken into account.

### Deflickering

Spatial intra prediction is used to code a CU in a flickering region with the modification that all the spatially neighboring blocks are marked as unavailable for spatial prediction. Therefore, the worst-case encoding complexity remains the same as spatial intra prediction in HM13.0-RExt6.0.

For the determination of whether a block is likely to flicker, the number of major colors in the block, SAD between the original current block and the collocated block from the next frame and edge gradient magnitude need to be calculated. The number of major colors in the block is already being calculated for palette mode and can be reused. The edge gradient is determined using the Sobel edge detector.

### Fast decision for All-Intra

For a CU, the further split process during encoding is skipped if all the following conditions are met:

- All the pixels in the blocks up to and including the current CU are identical to the collocated pixels in the previous frame.

- The CU depth of the collocated block in the previous frame is the same as the current depth of the CU.

When this feature is enabled, the encoding speed of all intra case is improved with negligible coding loss.

## Complexity characteristics of decoder intra-frame prediction operation

### Deflickering

Spatial intra prediction is used for a CU in a flickering region except that all the spatially neighboring blocks are marked as unavailable for spatial prediction. Therefore, the worst-case decoding complexity is the same as spatial intra prediction in HM13.0-RExt6.0.

## Complexity characteristics of encoder inter-component prediction tools

The additional encoder complexity for inter-component prediction is due to the introduction of the ACT.

One straightforward way for ACT implementation is to try all the possible modes twice for each CU: one without CST and the other with CST. Such two-pass process doubles the complexity in terms of encoding time. To reduce the complexity, following methods are employed:

- For intra modes, the best luma and chroma modes are shared between the two passes (with or without CST).

- For intra BC and inter modes, the motion information and predictors are shared between the two passes.

- The order of checking the R-D cost of enabling / disabling CST is dependent on the original color format. For RGB sequences, the R-D cost for enabling CST is first checked.  This order is switched for YUV sequences. In addition, the second pass is only invoked when there is at least one non-zero coefficient in the first pass.

With above methods, the encoding complexity of ACT is limited within 20% in terms of additional encoding time.

## Complexity characteristics of decoder inter-component prediction tools

The decoder complexity of ACT involves the inverse CST (after the inverse transform and CCP). The additional complexity is asserted to be minor since only addition and shift operations are required.

## Complexity characteristics of transforms specific in screen content coding

No changes have been made in transforms.

## Complexity characteristics of encoder related to other tools specific in screen content coding

### Palette

At the encoder side, the palette mode complexity mainly comes from three procedures:

- Palette entries calculation: this procedure requires sorting of the histogram, which in our implementation has a linear complexity with the number of pixels, n, in the CU. Note that it is possible to use other sorting algorithms to reduce the per pixel calculation to O(log(n)).

- Derivation of run mode and run length: only integer comparisons are required at this stage. R-D based optimization is not used. As the encoder needs to determine the three run modes, it roughly requires 2n pixel by pixel comparison. The memory access pattern in this process is highly localized, so very high cache hit ratio could be expected.

- Syntax coding: the syntax encoding and decoding have similar complexity, which is detailed in 5.14.1.

Overall, compared to HEVC, the palette mode does not use motion search, spatial prediction, residual transform (iterative), cross component prediction, RDOQ, or RDPCM. Therefore, the encoder complexity of the palette mode is much lower compared to the other HEVC modes. The memory access requirements are also lower compared to the other HEVC modes.

### 1-D Dictionary

In the 1-D dictionary coding mode, the search of matching pixels is achieved by table look-up. If there are multiple reference pixels with the same hash value, a maximum of 2048 reference pixels are tested at the encoder starting from the most recent one. In our implementation, a 21 bit hash value is used and thus, a table with 221 elements is created to store the positions corresponding to each hash value. The dictionary table stores all the previously coded positions, where the reference pixels are linked through the pixel positions. The longest run is selected during the 1-D encoder decision. However, if the string starting from the next pixel has a longer run, then the current pixel is coded in the escape mode.

## Complexity characteristics of decoder related to other tools specific in screen content coding

### Palette

The syntax parsing has comparable complexity to an HEVC decoder because the syntax elements in palette are coded with Golomb-Rice, unary, fixed length, and truncated binary codes. Also, palette is a run-length based method, so basically, it uses copying to reconstruct a pixel.

Palette decoding does not require any buffer to store intermediate decoding data besides the palette entries (which is 3×32 bytes in the worst case). Therefore, it is very likely that both data and instructions in palette mode stay in the cache, which can potentially improve decoder throughput.

Overall, compared to the other HEVC intra modes, spatial prediction, inverse transform, RDPCM, and cross component prediction processes are bypassed for palette. Therefore, palette mode has lower decoding complexity than the other HEVC modes.

### 1-D Dictionary

In the 1-D dictionary coding mode, if the current pixel is escape coded, the pixel value is directly decoded and reconstructed. If the current string has a reference string in the reconstructed area, the reference pixel address calculation and access is needed to reconstruct the current pixel. There is no hash table needed at the decoder, only the storage related to the reconstructed pixels in the current frame.

## Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding

Similar to HM13.0-RExt6.0.

## Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding

Similar to HM13.0-RExt6.0.

## Degree of capability for encoder parallel processing

Degree of capability for decoder parallel processing is similar to HEVC RExt Draft 6.

## Degree of capability for decoder parallel processing

Degree of capability for decoder parallel processing is similar to HEVC RExt Draft 6.

# Software implementation description

The proposed codec was developed based on HM13.0_RExt6.0 and implemented with standard C++ programming language.

# Highlighted aspects discussion

The GOP structures for low delay and random access are the same as the anchor, so the structural delay due to frame reordering and buffer is the same.

# Closing remarks

This document provides details of the implementation, complexity and performance of the screen content coding proposal produced by Qualcomm in response to the Call for Proposals for coding of Screen Content.

It is suggested that HM13.0-Rext6.0 with encoder improvements and full frame search area for intra BC would be a strong candidate to be used as a basis for a Test Model for future work on Screen Content Coding. New tools such as palette, 1-D dictionary, etc. should be developed on top of and compared against this benchmark.

# References

- ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/16, “Joint Call for Proposals for Coding of Screen Content”, ISO/IEC JTC 1/SC 29/WG 11 (MPEG) Doc. N14175, San Jose, USA, Jan. 2014.

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 6”, JCTVC-P1005, San Jose, USA, Jan. 2014.

- HM13.0-RExt6.0, reference software for HEVC Range Extensions, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0/

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “Suggested software for the AHG on investigation of palette mode coding tools,” JCTVC-P0303, San Jose, USA, Jan. 2014.

- T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG8: P2M based dual-coder extension of HEVC,” JCTVC-L0303, Geneva, CH, Jan. 2013.

- Henrique S. Malvar, Gary J. Sullivan, and Sridhar Srinivasan, “Lifting-based reversible color transformations for image compression,” SPIE Proceedings, Vol. 7073, DOI: 10.1117/12.797091, Oct. 2008.

- F. Zou, R. Joshi, J. Sole, M. Karczewicz, “Quantization rounding for RDPCM”, JCTVC-Q0148, Valencia, ES, March 2014.

- C. Pang, J. Sole, M. Karczewicz, “Intra block copy with encoder search using chroma component”, JCTVC-Q0175, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).