| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0031 |
| --- | --- |

| Title: | Draft text of screen content coding technology proposal by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document presents syntax, semantics and decoding process details of the Qualcomm response to the joint Call for Proposals on Screen Content Coding issued by MPEG and ITU-T.

# Contents

Abstract	1

Contents	1

1	High-level syntax and semantics	2

2	Draft text for specific tools	4

2.1	Inter-component prediction	4

2.2	Intra-picture prediction	5

2.2.1	Explicit RDPCM	5

2.2.2	Modifications to Intra block copying	5

2.3	Other specific compression tools for screen content	7

2.3.1	Palette	7

2.3.2	1-D dictionary	11

2.4	Deflickering	13

# High-level syntax and semantics

This section provides the syntax and semantics combined for all the tools in the Qualcomm CfP response down to coding_tree_block. Section 2 provides the text at CTU-level and below for each tool separately. This organization of the text has been selected for the sake of clarity.

Compared to JCTVC-P1005_v1, the added parts are highlighted in yellow and the deleted parts are marked as strikethrough.

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| color_transform_enabled_flag | u(1) |
| lossless_ color_transform_enabled_flag | u(1) |
| dictionary_1d_enable_flag | u(1) |
| deflicker_enabled_flag | u(1) |
| reserved_bit | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … | ue(v) |
| if( pps_extension_flag[ 0 ] ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| log2_sao_offset_scale_luma | ue(v) |
| log2_sao_offset_scale_chroma | ue(v) |
| if( !deblocking_filter_control_present_flag ¦ ¦ !pps_deblocking_filter_disabled_flag ) { |  |
| deblock_chroma_as_luma_flag | u(1) |
| deblock_smooth_filter_flag | u(1) |
| deblock_smooth_th_intra2 | se(v) |
| deblock_smooth_th_inter2 | se(v) |
| } |  |
| } |  |
| if( pps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Sequence parameter set RBSP semantics

color_transform_enabled_flag equal to 1 specifies that in-loop color-space transform process may be invoked in the decoding process. color_transform_enabled_flag equal to 0 specifies that the in-loop color-space transform is not applied. When not present, the value of color_transform_enabled_flag is inferred to be equal to 0.

lossless_color_transform_enabled_flag equal to 1 specifies that when the in-loop color-space transform process is invoked it uses the lossless color transform. lossless_color_transform_enabled_flag equal to 0 specifies that when the in-loop color-space transform is invoked it uses the lossy color transform. When not present, the value of color_transform_enabled_flag is inferred to be equal to 0.

dictionary_1d_enable_flag equal to 1 specifies that dictionary coding may be invoked for coding units of the coded video sequence. dictionary_1d_enable_flag equal to 0 specifies that dictionary coding is not invoked for any coding units of the coded video sequence. When not present, the value of dictionary_1d_enable_flag is inferred to be equal to 0.

deflicker_enabled_flag equal to 1 specifies that deflickering flag is signaled at CU level. deflicker_enabled_flag equal to 0 specifies that deflickering flag is not signaled at CU level. When deflicker_enabled_flag is not present, it is inferred to be equal to 0.

#### Picture parameter set RBSP semantics

deblock_chroma_as_luma_flag equal to 1 specifies that the chroma components are deblocked as the luma component. When deblock_chroma_as_luma_flag is not present, it is inferred to be equal to 0.

deblock_smooth_filter_flag equal to 1 specifies that the smooth deblocking filter is enabled. When deblock_smooth_filter_flag is not present, it is inferred to be equal to 0.

# Draft text for specific tools

## Inter-component prediction

#### 7.3.5.8	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( color_transform_enabled_flag ) |  |
| color_transform_flag[ x0 ][ y0 ] | ae(v) |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| … |  |

#### 7.4.9.5	Coding unit semantics

color_transform_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded with color-space transform. color_transform_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded without color-space transform. When not present, the value of color_transform_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

## Intra-picture prediction

### Explicit RDPCM

#### 7.3.8.11 Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ) && explicit_rdpcm_enabled_flag && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| explicit_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( explicit_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| explicit_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_sig_coeff_x_prefix | ae(v) |
| … |  |
| if( cu_transquant_bypass_flag ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && implicit_rdpcm_enabled_flag && transform_skip_flag[ x0 ][ y0 ][ cIdx ] && ( predModeIntra = = 10 ¦ ¦ predModeIntra = = 26 ) ) ¦ ¦ explicit_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 ) |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| … |  |

### Modifications to Intra block copying

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| intra_bc_mv_coding ( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| intra_bc_mv_coding ( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| intra_bc_mv_coding ( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| intra_bc_mv_coding ( x0 + ( nCbS / 2 ), y0, 2) |  |
| intra_bc_mv_coding ( x0, y0 + ( nCbS / 2 ), 2) |  |
| intra_bc_mv_coding ( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { |  |
| … |  |

#### Intra BC block vector syntax

| intra_bc_mv_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| intra_bc_bvp_flag[ x0 ][ y0 ] | ae(v) |
| intra_bc_abs_bvd_greater0_flag[ 0 ] | ae(v) |
| intra_bc_abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if(intra_bc_abs_bvd_greater0_flag[ 0 ] ) { |  |
| intra_bc_abs_bvd_minus1[ 0 ] | ae(v) |
| intra_bc_bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(intra_bc_abs_bvd_greater0_flag[ 1 ] ) { |  |
| intra_bc_abs_bvd_minus1[ 1 ] | ae(v) |
| intra_bc_bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

intra_bc_bvp_flag[ x0 ][ y0 ] equal to 1 specifies that above block vector is used as the block vector predictor. intra_bc_bvp_flag[ x0 ][ y0 ] equal to 0 specifies that left block vector is used as the block vector predictor.

intra_bc_abs_bvd_greater0_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 0.

intra_bc_abs_bvd_minus1[ compIdx ] plus 1 specifies the absolute value of a block vector component difference.

When intra_bc_abs_bvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

intra_bc_bvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

–	If intra_bc_bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

–	Otherwise (intra_bc_bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When intra_bc_bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

## Other specific compression tools for screen content

### Palette

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| if(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY)) && ( !palette_mode_flag[ x0 ][ y0 ] ) |  |
| part_mode | ae(v) |
| } |  |
| if( !palette_mode_flag[ x0 ][ y0 ] ){ |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| palette_coding(x0, y0, CbWidth, CbHeight) { | Descriptor |
| --- | --- |
| if( ChromaArrayType = = 3 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 3 ) |  |
| } else if( ChromaArrayType = = 2 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| palette_coding_component( x0, y0, CbWidth >> 1, CbHeight, 2 ) |  |
| } else if( ChromaArrayType = = 1 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| palette_coding_component( x0, y0, CbWidth >> 1, CbHeight >> 1, 2 ) |  |
| } else if( ChromaArrayType = = 0 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| } |  |

| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteSize; i++ ) { |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| if ( previous_palette_entry_flag[ i ] ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ i ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| if( numPredPreviousPalette < 32 ) |  |
| palette_num_signalled_entries | ae(v) |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| for( i = 0; i < palette_num_signalled_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + palette_entries |  |
| previousPaletteSize = palette_size |  |
| previousPaletteEntries = palette_entries |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| palette_mode_type_flag[ xC ][ yC ] | ae(1) |
| if ( palette_mode_type_flag[ xC ][ yC ] == 1 ){ |  |
| scanPos++ |  |
| for (cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++){ |  |
| palette_escape_val | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| palette_run_type_flag[ xC ][ yC ] = NONE_RUN_MODE |  |
| } else if ( yC != 0 && previous_run_type_flag != COPY_ABOVE_RUN_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(1) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = COPY_LEFT_RUN_MODE |  |
| previous_run_type_flag = palette_run_type_flag |  |
| if(palette_mode_type_flag[ xC ][ yC ] != ESCAPE_MODE && palette_run_type_flag[ xC ][ yC ] == COPY_LEFT_RUN_MODE ) { |  |
| palette_index | ae(v) |
| palette_run | ae(v) |
| runPos = 0 |  |
| while ( runPos <= palette_run ) { |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| for (cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++){ |  |
| samples_array[ cIdx ][ xC ][ yC ] = palette_entries[ cIdx ][palette_index] |  |
| } |  |
| scanPos++ |  |
| runPos++ |  |
| } |  |
| } |  |
| if(palette_mode_type_flag[ xC ][ yC ] != ESCAPE_MODE && palette_run_type_flag[ xC ][ yC ] == COPY_ABOVE_RUN_MODE) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| while ( runPos <= palette_run + 1 ) { |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| samples_array[ cIdx ][ xC ][ yC ] = samples_array[ cIdx ][ xC ][ yC - 1 ] |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding unit block relative to the top-left luma sample of the picture.

palette_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded using the palette mode.

When palette_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

previous_palette_entry_flag[ i ] equals to 1 specifies that the palette entry from the last palette mode coded CU is copied. previous_palette_entry_flag [ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The variable previousPaletteSize is the size of the palette of the last palette mode coded CU. The variable previousPaletteSize is set equal to 0 for the first decoded palette CU of the current CTU line.

palette_num_signalled_entries specifies the number of palette entries that are explicitly signalled for the current coding unit. When palette_num_signalled_entries is not present, it is inferred to be equal to 0.

palette_entries[ cIdx ][ i ] specifies the i-th element in the palette for the color component cIdx.

The array previousPaletteEntries is equal to the array palette_entries of the last CU decoded in palette mode.

palette_mode_type_flag equal to 1 specifies that the current pixel samples_array[ cIdx ][ xC ][ yC ] is  coded using escape mode. palette_mode_type_flag equal to 0 specifies that the current pixel is not coded as escape mode.

palette_escape_val specifies the escape pixel value for one color component.

palette_run_type_flag equal to 1 specifies that the run mode is COPY_ABOVE_RUN_MODE where the decoded pixel value is equal to the pixel at the same location in the above row. palette_run_type_flag equal to 0 specifies that the run mode is COPY_LEFT_RUN_MODE where the pixel value is equal to the corresponding value of the index in the palette. When palette_run_type_flag is not present, it is inferred to be equal to COPY_LEFT_RUN_MODE.

palette_index is an index to the palette entries.

palette_run specifies the number of consecutive locations with the same palette index as the position in the above row minus one when palette_run_type_flag is equal to COPY_ABOVE_RUN_MODE or represents the number of consecutive locations with the same palette index when palette_run_type_flag is equal to COPY_LEFT_RUN_MODE.

### 1-D dictionary

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( dictionary_1d_enable_flag ) |  |
| dictionary_coded_flag | ae(v) |
| if( dictionary_coded_flag ){ |  |
| dictionary_syntax() |  |
| } else { |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| } |  |
| } |  |

| dictionary_syntax( ) { |  |
| --- | --- |
| for( i=0; i < (1<<(2* log2CbSize); ) { |  |
| matching_string_flag | ae(v) |
| if( matching_string_flag = = 1 ) { |  |
| matching_string_offset_use_recent_8_flag | ae(v) |
| if( matching_string_distance_use_recent_8_flag ) { |  |
| matching_string_offset_recent_8_idx | ae(v) |
| } |  |
| else { |  |
| matching_string_offset_minus1 | ae(v) |
| } |  |
| matching_string_length_minus1 | ae(v) |
| i += (matching_string_length_minus1 + 1) |  |
| } |  |
| else { |  |
| unmatchable_sample_value_component0 | ae(v) |
| unmatchable_sample_value_component1 | ae(v) |
| unmatchable_sample_value_component2 |  |
| i ++ |  |
| } |  |
| } |  |
| } |  |

7.4.9.2 Coding tree unit semantics

dictionary_coded_flag equal to 1 specifies that dictionary coding is used for the coding tree unit. dictionary_coded_flag equal to 0 specifies that dictionary coding is not used for the coding unit. When not present, the value of dictionary_coded_flag is inferred to be equal to 0.

1-D dictionary block table semantics

matching_string_flag equal to 1 indicates that the current pixel starts a matching string. matching_string_flag equal to 0 indicates the current pixels does not start a matching string and its sample values are explicitly signalled.

matching_string_offset_use_recent_8_flag equal to 1 indicates the current matching string offset is equal to one of the eight previously decoded matching string offsets and the string offset is specified by matching_string_offset_recent_8_idx. matching_string_offset_use_recent_8_flag equal to 0 indicates the current matching string offset is explicitly signalled by the syntax matching_string_offset_minus1.

matching_string_offset_recent_8_idx specifies the index to the eight previously coded matching string offsets. When not present, the value of matching_string_offset_recent_8_idx is inferred to be equal to 0.

matching_string_offset_minus1 plus 1 specifies the matching string offset between the current string and the reference string. When not present, the value of matching_string_offset_minus1is inferred to be equal to 0.

matching_string_length_minus1 plus 1 specifies the matching string run (the number of pixels that the current string match the reference string). When not present, the value of matching_string_length_minus1is inferred to be equal to 0.

unmatchable_sample_value_component0 is specifies the value of the 0-th component sample of the current pixel.

unmatchable_sample_value_component1 is equal to pixel value of the 1-st component sample the current pixel.

unmatchable_sample_value_component2 is equal to pixel value of 2-nd component sample of the current pixel.

      - Derivation process of 1-D dictionary mode

- If dictionary_1d_enable_flag is equal to 1, the following 1-D dictionary decoding process is applied:

  - A pixel counter decPixelCnt of the current CU is set to 0

  - If decPixelCnt is smaller than (1<<(2* log2CbSize), the following loop continues

    - A parameter matchingStringOffset is set to 0;

    - A parameter matchingStringRun is set to 0;

    - If matching_string_flag is equal to 1, the following procedure is applied:

      - If matching_string_offset_use_recent_8_flag is equal to 1,  matchingStringOffset is set equal to recent8offsets[matching_string_offset_recent_8_idx]  plus 1;

      - Otherwise, matchingStringOffset  is set equal to matching_string_offset_minus1 plus 1 and the following procedure is applied to update recent8offsets[]

        - For (i=1;i<8;i++)	recent8offset[i+1] = recent8offset[i]

        - recent8offsets[0] is set equal to matching_string_offset_minus1.

      - matchingStringRun is set equal to matching_string_length_minus1 plus 1,

    - Otherwise (matching_string_flag equal to 0) the escape pixel is derived as follows:

      - The 0th component of the current pixel is set equal to unmatchable_sample_value_component0;

      - The 1st component of the current pixel is set equal to unmatchable_sample_value_component1;

      - The 2nd component of the current pixel is set equal to unmatchable_sample_value_component2;

      - matchingStringRun is set equal to 1.

    - decPixelCnt is increased by matchingStringRun plus 1.

## Deflickering

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| if(deflicker_enabled_flag && CuPredMode[ x0 ][ y0 ] == MODE_INTRA && ( !palette_mode_flag[ x0 ][ y0 ]) |  |
| cu_deflicker_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| } |  |
| … |  |

cu_deflicker_flag equal to 1 specifies that the CU is in a flickering region so that all spatially neighboring blocks of the CU are marked as unavailable for spatial intra prediction. cu_deflicker_flag equal to 0 specifies that the CU is not in a flickering region. When cu_deflick_flag is not present, it is inferred to be equal to 0.