| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0032 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by NCTU and ITRI International |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, Tsui-Shan Chang, Ru-Ling Liao, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw, 2wpeng@cs.nctu.edu.tw |
|  | Yao-Jen Chang, Chao-Hsiung Hung, Ching-Chieh Lin, Jih-Sheng Tu, Erh-Chung Ke, Jung-Yang Kao, Chun-Lung Lin, 3Fan-Di Jou, Fang-Chu Chen 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan |  | +886-3-582-0100 3Fandi@itri.org.tw |
| Source: | National Chiao Tung University (NCTU) ITRI International |  |  |

_____________________________

# Abstract

This document describes the technologies, jointly proposed by NCTU and ITRI International, in response to the joint call for proposals for coding of screen content. It extends the HEVC RExt (JCTVC-P01005) with three additional types of intra modes (palette coding, combined palette coding and intra block copy, and line-based intra block copy), adaptive motion vector precision, and two block vector coding schemes (block vector initialization and second block vector predictor). The average G BD-rate reduction for RGB sequences in lossy coding is reported as follows:

- text & graphics with motion, 1080p : (AI) 28.6%,	(RA) 17.3%,	(LB) 13.7%;

- text & graphics with motion,720p  : (AI) 14.9%,	(RA) 12.3%,	(LB) 10.4%;

- mixed content, 1440p            : (AI) 9.5%,	(RA) 7.7%,	(LB) 10.0%;

- mixed content, 1080p            : (AI) 11.1%,	(RA) 8.7%,	(LB) 8.0%;

- animation, 720p                : (AI) 0.8%,	(RA) 1.5%,	(LB) 2.3%.

The average Y BD-rate reduction for YUV sequences is reported as follows:

- text & graphics with motion, 1080p : (AI) 26.3%,	(RA) 15.6%,	(LB) 10.5%;

- text & graphics with motion,720p  : (AI) 12.7%,	(RA) 10.6%,	(LB) 8.2%;

- mixed content, 1440p            : (AI) 9.2%,	(RA) 6.6%,	(LB) 7.6%;

- mixed content, 1080p            : (AI) 10.5%,	(RA) 7.8%,	(LB) 6.5%;

- animation, 720p                : (AI) 0.6%,	(RA) 0.6%,	(LB) 0.8%.

The average runtime relative to the HM-12.1+RExt-5.1 anchors is reported as follows:

- encoding time                  : (AI) 169%,	(RA) 122%,	(LB) 121%;

- decoding time                  : (AI) 103%,	(RA) 86%,	(LB) 115%.

- Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Inter-component prediction	4

2.3	Inter-picture prediction	4

2.4	Intra-picture prediction	4

2.4.1	Palette coding	4

2.4.2	Combined palette coding and intra block copy	7

2.4.3	Line-based intra block copy	8

2.5	In-loop filtering	8

2.5.1	De-blocking filter	8

2.5.2	SAO	8

2.6	Spatial transforms	8

2.7	Other specific compression tools for screen content	8

2.7.1	Second block vector predictor	8

2.7.2	Initialization of block vector predictor initialization	9

2.8	Quantization	9

2.9	Motion/mode/parameter coding	9

2.9.1	Merge/skip	9

2.9.2	Motion vector coding	9

2.9.3	Intra prediction mode coding	10

2.10	Entropy coding of residual / transform coefficients	10

2.11	High-layer syntax	10

3	Syntax and semantics description	10

3.1	Sequence parameter set RBSP syntax	10

3.2	Coding unit syntax	10

3.3	Intra block vector difference syntax	12

3.4	SCM information coding syntax	12

3.5	Sequence parameter set RBSP semantics	14

3.6	Coding unit semantics	14

4	Compression performance discussion	16

4.1	Lossy test condition	16

4.1.1	Summary	16

4.1.2	Per-sequence BD-rate	18

4.2	Mathematically lossless test condition	21

4.2.1	Summary	21

4.2.2	Per-sequence bit-rate saving	22

5	Complexity analysis	22

5.1	Encoding time and measurement methodology	22

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	23

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	23

5.4	Expected memory usage of encoder	23

5.5	Expected memory usage of decoder	23

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	23

5.7	Complexity characteristics of decoder motion compensation	23

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	24

5.9	Complexity characteristics of decoder intra-frame prediction operation	24

5.10	Complexity characteristics of encoder inter-component prediction tools	24

5.11	Complexity characteristics of decoder inter-component prediction tools	24

5.12	Complexity characteristics of transforms specific in screen content coding	24

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	24

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	25

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	25

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	25

5.17	Degree of capability for encoder parallel processing	25

5.18	Degree of capability for decoder parallel processing	25

6	Software implementation description	25

7	Closing remarks	25

8	Reference	25

9	Patent rights declaration(s)	25

# Introduction

This document is a technical description of the proposal submitted by NCTU in response to the joint call for proposals for coding of screen content [1].

# Algorithm description

## CTU/TU/PU partitioning

CTU/TU/PU partitioning is the same as that of the HEVC RExt draft specification [2].

## Inter-component prediction

Inter-component prediction is the same as that of the HEVC RExt draft specification [2].

## Inter-picture prediction

Inter-picture prediction is the same as that of the HEVC RExt draft specification [2].

## Intra-picture prediction

Besides the intra-picture prediction specified in the HEVC RExt draft specification [2], this contribution presents three types of intra-picture prediction techniques: palette coding, combined palette coding and intra block copy, and line-based intra block copy.

### Palette coding

This contribution uses a palette-based coding tool based on JCTVC-P0108. Pixels within the current CU will be represented by major color indices or escape index instead of their pixel intensities, followed by a reconstruction process which maps these indices with reference to the associated major color table or escaped pixel values to reconstruct the pixel intensities. A flag is sent at CU level to indicate whether the palette coding mode is enabled. If yes, the major colors and the indices corresponding to each pixel in the current CU will be transmitted.

The proposed coding process for the palette tool is described as follows. Major colors are selected based on the histogram algorithm proposed in JCTVC-P0108. In specific, the most significant N peak values in the histogram are selected as major colors. For lossy coding, the pixel values that are close to a major color will be quantized to it with a quantization level 2q. As shown in Figure 1(a), there are two major colors, for example--a pixel being quantized to the nearest major color if the difference between the nearest major color and the pixel value is not larger than a pre-defined threshold, q. However, if the quantization regions of two major colors overlap with each other, as the overlapping region shown in Figure 1(b), the pixels in the overlapping region will be quantized to the most significant major color, i.e., 1st significant major color in this example. Other pixels which are not quantized to any major color will be escape pixels, and will also be quantized before entropy coding. While for lossless coding, both of the quantization processes will not be used. If N major colors are decided by encoder, the color indices 0 ~ N-1 will correspond to the N major colors and the index N will correspond to the escape pixel. Each pixel is assigned an index to indicate which major color it is quantized to or it belongs to escape pixels.

| (a) | (b) |
| --- | --- |

- - Examples of quantization method for palette mode (a) no overlapping region between two major colors; (b) overlapping region between two major colors

The palette tool requires to transmit the value of each major color to decoder. To reduce the transmission bits, this contribution applies the major color coding method in JCTVC-P0152 to the proposed palette tool. The encoder selects a best mode based on RD cost from the following three major color coding modes:

- Left CU merge mode: the major colors of the left CU are entirely selected as the major colors of the current CU as shown in Figure 2. In this case, a flag ‘10’ is transmitted.

- Above CU merge mode: the major colors of the above CU are entirely selected as the major colors of the current CU as shown in Figure 2. In this case, a flag ‘11’ is transmitted.

- Normal mode: a flag ‘0’ is transmitted. Major colors are selected based on the histogram algorithm proposed in JCTVC-P0108. In this case, the major colors are coded in an element-by-element manner. The following coding modes are selected to code the i-th major color:

- Left copy: the i-th major color of the current CU is the same with the i-th major color of the left CU. In this case, a flag ‘10’ is transmitted.

- Above copy: the i-th major color of the current CU is the same with the i-th major color of the above CU. In this case, a flag ‘11’ is transmitted.

- No copy: if the mode is not ‘left copy’ or ‘above copy’, it is ‘no copy’ mode. In this case, a flag ‘0’ and the i-th major color itself are transmitted.

- - Above CU merge mode and left CU merge mode

However, if a neighboring CU is not coded by palette tool, the major color coding modes mentioned above cannot be used. To fulfill the cases of non-palette neighboring CU in the major color coding, the major-colors propagation through non-palette CUs in JCTVC-P0096 is also used in the proposed palette tool. As shown in Figure 3, if the left CU is a non-palette CU, the major colors propagated from the last palette CU relative to the left CU are copied to be the major colors of the left CU. Similarly, if the above CU is a non-palette CU, the major colors propagated from the last palette CU relative to the above CU are copied to be the major colors of the above CU. By using the propagation scheme, the major color coding modes (Left CU merge mode, Above CU merge mode and Normal mode) are still practical even if the left CU or the above CU is a non-palette CU.

- - Propagation of major colors to the next non-palette CU

If the palette tool is enabled, the color index corresponding to each pixel in CU is also transmitted. Similar to the concept of JCTVC-P0108, the proposed color index coding is designed in a row-by-row manner. The color indices are encoded as follows:

- For each pixel line, a flag is signaled to indicate the coding mode as shown in Table 1. There are four coding modes including horizontal, vertical, vertical-above and normal modes as follows:

- Horizontal mode: the flag ‘100’ is signaled. The whole line shares the same color index. In this case, the first color index of the line is transmitted.

- Vertical mode: the flag ‘11’ is signaled. The color indices of the line are the same with those of the above line. In this case, nothing is transmitted.

- Vertical-above mode: the flag ‘101’ is signaled. The color indices of the line are the same with those of the above-above line, and nothing is transmitted.

- Normal mode: the flag ‘0’ is signaled. If the mode is not horizontal mode or vertical mode, it is normal mode. In this mode, the current pixel line is partitioned into two equal sub-lines as shown in Figure 4(a). Figure 4(b) shows a detailed flag-level flow chart for the i-th sub-line (i = 1 or 2). For the i-th sub-line, a flag is first signaled to indicate whether to copy above line or not:

- If enabled: the i-th sub-line is the same with the reference line y as shown in Figure 5. In this case, the reference line index y is transmitted.

- If disabled: no above line is the same with the sub-line i. In this case, there are five modes to code each pixel in the sub-line i. The five modes are above-pixel prediction, left-pixel prediction, above-right-pixel prediction, above-left-pixel prediction and no prediction. The first four prediction modes, based on JCTVC-P0098, represent the current pixel is the same with the above pixel, left pixel, above-right pixel and above-left pixel, respectively. For the no prediction mode, the color index itself is transmitted.

- If the pixel is escape pixel, the pixel value is quantized and then transmitted.

- - Binarization of the four line modes in color index coding

| Line mode | Horizontal | Vertical | Vertical-above | Normal |
| --- | --- | --- | --- | --- |
| Mode index | 100 | 11 | 101 | 0 |

| (a) | (b) |
| --- | --- |

- – (a) The current pixel line (2Nx1) is partitioned into two equal sub-lines (Nx1) in normal mode; (b) syntax tree for the i-th sub-line

- - An example of copy above line for each sub-line

### Combined palette coding and intra block copy

The main idea for the combined palette coding and intra block copy is to divide the pixels in the current CU into two groups: spatially matched pixels and spatially unmatched pixels. For each CU, the spatially matched pixels are coded by intra block copy (IntraBC) while the spatially unmatched pixels are processed by palette coding.

For the example illustrated in Figure 6, an index map is transmitted to represent the current CU just like that of palette coding but one certain index (e.g. 0 in this case) is used to indicate the pixels which are categorized as the spatially matched pixels. Then, the predictor of the spatially matched pixels (i.e., the blue ones in Figure 6) in the current CU is generated through copying the matching blocks obtained from PU-based IntraBC which is applied in the HM-13.0+RExt-6.0. Otherwise, the remaining pixels will be classified as spatially unmatched ones (e.g. the green and orange ones in Figure 6) and the predictor of these pixels will be produced by the major color representation at CU level. Besides, for those spatially unmatched pixels which cannot be clustered into any major color sets, are processed as escape pixels instead. Therefore, the current CU can be totally reconstructed by IntraBC and palette coding.

In the encoding stage, the predictor of the current CU is generated by IntraBC first. Then, the difference between each original pixel in the current CU and the co-located one in the matching block is calculated. For those pixels of which difference mentioned before is larger than a QP-dependent threshold (Table 2), are categorized as spatially unmatched ones and handled by palette coding to replace the predictor. Accordingly, the pixels of which predictor is obtained by IntraBC are identified as the spatially matched pixels.

- - Example for combined palette coding and intra block copy

- - QP-to-thresholds mapping for combined palette coding and intra block copy

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Threshold | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| Threshold | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 9 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
| Threshold | 10 | 10 | 10 | 11 | 12 | 13 | 13 | 14 | 15 | 16 | 17 | 17 | 18 | 19 | 20 | 21 |  |  |

### Line-based intra block copy

The notion of the line-based intra block copy is to provide an estimate of pixel intensities based on the intra block copy as specified in HEVC RExt draft specification [2]. This contribution divides a prediction unit equally into multiple rows or columns, termed as lines in general. A flag is present at PU level to specify whether the row-wise splitting or the column-wise splitting is in use. For each line, pixels within are predicted in the same way as the intra block copy except that the processing unit is not a block but a line (Figure 7), and the search range is the same as specified in [2] and pixels within the current CU is not allowed for reference. The block vector prediction for each line is specified in Section 2.7.1 and 2.7.2, and the block vector difference is coded in the same way as the intra block copy specified in [2].

A fast encoding option is enabled for this mode to be evaluated only at the smallest CU level.

- - BV search for lines within PUs

## In-loop filtering

### De-blocking filter

De-blocking filtering is the same as that of the HEVC RExt draft specification [2].

### SAO

SAO is the same as that of the HEVC RExt draft specification [2].

## Spatial transforms

Spatial transforms are the same as that of the HEVC RExt draft specification [2].

## Other specific compression tools for screen content

### Second block vector predictor

Besides taking the previously decoded block vector as predictor, this contribution also takes the second previously decoded block vector as an additional block vector predictor (Figure 8). A flag is sent to choose adaptively between the two block vector predictors as follows:

- when intra block copy mode or the combined palette coding and intra block copy is in use, this flag is sent at PU level;

- when line-based intra block copy is in use, this flag is sent line-by-line.

Any block vector predictor, which belongs to a block outside the current coding tree unit, is marked as invalid. When the number of valid block vector predictors is less than 2, the procedure specified in Section 2.7.2 is invoked to make sure that there are always two block vectors to be used for block vector prediction.

- – The block vector predictors of the current block is from, for example, the 7th and the 4th block, where the orange block are previously decoded intra block copy blocks.

### Initialization of block vector predictor initialization

When (a) both of the block vector predictors are not valid or (b) only the second one is invalid, the initialization procedure is invoked as follows: (a) the first predictor as (-w, 0) and the second as (-2w, 0), and (b) the second predictor as (-w,0), where the w denotes the width of the current coding unit.

## Quantization

Quantization is the same as that of the HEVC RExt draft specification [2].

## Motion/mode/parameter coding

### Merge/skip

The derivation process of merge candidates is the same as that of the HEVC RExt draft specification [2], except that one additional set of merge candidates is introduced for each PU when coded by using the merge mode. The candidates in the new set are derived from the merge candidates in [2], followed by a truncation process which sets the two least significant bits of each candidate’s motion vectors in the new set equal to 0. A flag, adaptive_mv_precision flag, is present at PU level for the proposed encoder to switch adaptively from the two sets of merge candidates.

Two fast encoding options are enabled to speed up the encoding, one of which skips evaluating this mode when the PU is of size 2Nx2N while the other, as illustrated in Figure 9(a), optimizes this mode flag with reference to the motion vector precision of the first PU’s motion vectors. If the resulting motion vectors of the first PU are all integer-precision, adaptive_mv_precision flag is set equal to 1 for the current coding unit; otherwise, it is set equal to 0.

### Motion vector coding

In this contribution, an alternative motion vector difference coding mode is introduced. When this mode is in use, the two least significant bits of the motion vector difference are both inferred as 0, so there is need for them to be signaled. Besides, the candidate set of motion vector predictors (MVp) is formed by copying the MVp candidates as specified in [2], followed by a truncation process which sets the two least significant bits of each candidate’s motion vectors equal to 0. The same flag, adaptive_mv_precision flag, as presented in Section 2.9.1 is sent at PU level for the proposed encoder to choose adaptively from the alternative motion vector difference coding mode and the [2]’s.

A fast encoding flow is illustrated in Figure 9(b). This mode flag is optimized with reference only to the precision of the first PU’s motion vectors. If the resulting motion vectors of the first PU are all integer-precision, adaptive_mv_precision flag is set equal to 1 for the current coding unit; otherwise, it is set equal to 0.

| (a) | (b) |
| --- | --- |

- – Fast encoding process for optimizing the adaptive_mv_precision flag when performing (a) merge mode and (b) motion estimation

### Intra prediction mode coding

Intra prediction mode coding is the same as that of the HEVC RExt draft specification [2].

## Entropy coding of residual / transform coefficients

This part is the same as that of the HEVC RExt draft specification [2].

## High-layer syntax

The high-layer syntax is the same as that of the HEVC RExt draft specification [2], except that an additional flag, intra_screen_coding_enabled_flag, is introduced in the sequence parameter set to indicate the use of palette coding mode.

# Syntax and semantics description

## Sequence parameter set RBSP syntax

This table is written on top of Section 7.3.2.2 specified in [2]. The modifications are marked in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| intra_screen_coding_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| … |  |

## Coding unit syntax

This table is written on top of Section 7.3.8.5 specified in [2]. The modifications are marked in yellow.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && intra_screen_coding_enabled_flag ) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_scm_flag[ x0 ][ y0 ] ){ |  |
| scm_info_coding(x0, y0, nCbS, nCbS) |  |
| }else{ |  |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| line_intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| mbvd_coding( x0, y0, 2 nCbS ) |  |
| if( PartMode = = PART_2NxN ) |  |
| mbvd_coding( x0, y0 + ( nCbS / 2 ), 2 nCbS ) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mbvd_coding( x0 + ( nCbS / 2 ), y0, 2 nCbS) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mbvd_coding( x0 + ( nCbS / 2 ), y0, 2 nCbS) |  |
| mbvd_coding( x0, y0 + ( nCbS / 2 ), 2 nCbS) |  |
| mbvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2 nCbS) |  |
| } |  |
| if( !line_intra_bc_flag[ x0 ][ y0 ] ) |  |
| combine_ibc_palette_flag[ x0 ][ y0 ] | ae(v) |
| if( combine_ibc_palette_flag[ x0 ][ y0 ] ) |  |
| scm_info_coding(x0, y0, nCbS, nCbS) |  |
| } else { |  |
| … |  |
| } |  |
| } else { |  |
| adaptive_mv_precision flag[ x0 ][ y0 ] | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| … |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] && !intra_scm_flag[ x0 ][ y0 ] && !combine_ibc_palette_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| … |  |
| } |  |
| } |  |
| } |  |

## Intra block vector difference syntax

| bvd_coding (x0, y0, nCbSize) { |  |
| --- | --- |
| numBv = 1 |  |
| if( line_intra_bc_flag[ x0 ][ y0 ] ) { |  |
| line_intra_bc_type[ x0 ][ y0 ] | ae(v) |
| numBv = ( PartMode = = PART_2NxN && !line_intra_bc_type[ x0 ][ y0 ] ) ¦ ¦ ( PartMode = = PART_Nx2N && line_intra_bc_type[ x0 ][ y0 ] ) ¦ ¦ PartMode = = PART_NxN ? nCbSize / 2 : nCbSize |  |
| } |  |
| for( i = 0; i < numBv ; i += 1 ) { |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| mvd_coding( xPb, yPb, 2 ) |  |
| } |  |
| } |  |

## SCM information coding syntax

| scm_info_coding(x0, y0, CbWidth, CbHeight) { |  |
| --- | --- |
| if( !combine_ibc_palette_flag[ x0 ][ y0 ] ) |  |
| major_color_merge_flag[ x0 ][ y0 ] | ae(v) |
| if(!combine_ibc_palette_flag[ x0 ][ y0 ] && major_color_merge[ x0 ][ y0 ]) { |  |
| major_color_merge_dir_flag[ x0 ][ y0 ] | ae(v) |
| if(major_color_merge_dir[ x0 ][ y0 ]) { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] = major_color_num[ x0 - CbWidth ][ y0 ] [ cIdx ] |  |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] = major_color_value[ x0 - CbWidth ][ y0 ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } else { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] = major_color_num[ x0 ][ y0 - CbHeight ][ cIdx ] |  |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] = major_color_value[ x0 ][ y0 - CbHeight ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType = = 0 )?1:3; cIdx++) { |  |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( major_color_pred[ cIdx ] ) { |  |
| major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType = = 3 )?1:3; cIdx++) { |  |
| for( j = 0; j < CbHeight; j ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][ j ] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ j ] = = HOR_MODE) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType = = 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ 0 ][ j ] | ae(v) |
| } |  |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][j] = = VER_MODE ¦ ¦ line_mode[ x0 ][ y0 ][ cIdx ][j] = = VER_ABOVE_MODE) { |  |
| lineOffset = line_mode[ x0 ][ y0 ][ cIdx ][j] = = VER_ABOVE_MODE ? 2 : 1 |  |
| for(i = 0; i < CbWidth;i++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType = = 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] = color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j - lineOffset ] |  |
| } |  |
| } |  |
| } else { //NOR_MODE |  |
| for(k = 0; k < 2; k++) { |  |
| copy_above_half_line_mode[ x0 ][ y0 ][ cIdx ][ j ]k | ae(v) |
| copy_above_half_line_shift_vector[ x0 ][ y0 ][ cIdx ][ j ]k, | ae(v) |
| if(copy_above_half_line _shift_vector[ x0 ][ y0 ][ cIdx ][ j ]k == 0) { |  |
| for(i = k*(CbWidth/2); i < (k+1)*(CbWidth/2); i++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(i = k*(CbWidth/2); i < (k+1)*(CbWidth/2); i++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] = = NO_PREDICTION) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType = = 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| for(i = k*(CbWidth/2); i < (k+1)*(CbWidth/2); i++) { |  |
| if( ( intra_scm_flag[ x0 ][ y0 ] && color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][j] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ] ) ¦ ¦ ( combine_ibc_palette_flag[ x0 ][ y0 ] && color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ] + 1 ) ) { |  |
| quant_escape_value[ cIdx + cSubIdx ][ x0 ][ y0 ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } // if(copy_above_half_line _shift_vector[ x0 ][ y0 ][ cIdx ][ j ]k == 0) |  |
| } // for(k = 0; k < 2; k++) |  |
| } // else { //NOR_MODE |  |
| } // for( j = 0; j < CbHeight; j ++) |  |
| } // for(cIdx = 0; cIdx < ( ChromaArrayType == 3 )?1:3; cIdx++) |  |

## Sequence parameter set RBSP semantics

This section is written on top of Section 7.4.3.2 specified in [2]. The modifications are marked in yellow.

…

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, intra_screen_coding_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

intra_screen_coding_enabled_flag equal to 1 specifies that intra screen coding may be used for intra blocks. intra_screen_coding_enabled_flag equal to 0 specifies that intra screen coding is not applied. When not present, the value of intra_screen_coding_enabled_flag is inferred to be equal to 0.

…

## Coding unit semantics

This section is written on top of Section 7.4.9.5 specified in [2]. The proposed changes are as follows.

…

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in screen coding mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in screen coding mode. When intra_scm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_merge_flag[ x0 ][ y0 ] equals to 1 specifies the major color table of current CU is copied from left or above CU. major_color_merge_flag[ x0 ][ y0 ] equals to 0 specifies major color table of current CU will be coded then transmitted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_merge_dir_flag[ x0 ][ y0 ] equals to 1 specifies the major color table of current CU is copied from left CU, otherwise from above CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the above or left CU. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] specifies the prediction direction of the n-th major color for color component cIdx. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies current major color is the same as the corresponding major color in the above CU. When it equals to 0, current major color is the same as the corresponding major color in the left CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_mode[ x0 ][ y0 ][ i ] specifies the line mode used in color index coding according to Table 3. line_mode[ x0 ][ y0 ][ i ] equal to HOR_MODE specifies that the line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][ i ] equal to VER_MODE specifies that the line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ i ] equal to VER_ABOVE_MODE specifies that the line should be decoded in vertical-above line mode. line_mode[ x0 ][ y0 ][ i ] equal to NOR_MODE specifies that the line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

- - Specification of the four line mode in color index coding

| line_mode[ x0 ][ y0 ][ i ] | Name of line_mode[ x0 ][ y0 ][ i ] |
| --- | --- |
| 0 | NOR_MODE |
| 1 | VER_MODE |
| 2 | HOR_MODE |
| 3 | VER_ABOVE_MODE |

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current pixel in position ( i, j ) according to Table 4. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to INDEX_PRED_ABOVE, INDEX_PRED_LEFT, INDEX_PRED_ABOVE_ABOVE, INDEX_PRED_ABOVE_LEFT, INDEX_PRED_ABOVE_RIGHT, specifies that index value of current pixel is the same as the above one, left one, above above one, above left one and above right one. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to 0 specifies that index value of current pixel is 0. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to NO_PREDICTION specifies that index value of current pixel cannot be derived from the above or the left pixel.

- - Specification of the five color index mode in color index coding

| index_pred_symbol[ x0 ][ y0 ][ i ][ j ] | Name of index_pred_symbol[ x0 ][ y0 ][ i ][ j ] |
| --- | --- |
| 0 | INDEX_PRED_ABOVE |
| 1 | INDEX_PRED_LEFT |
| 2 | INDEX_PRED_ABOVE_ABOVE |
| 3 | INDEX_PRED_ABOVE_LEFT |
| 4 | INDEX_PRED_ABOVE_RIGHT |

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape pixel value for color component cIdx in at the location ( i, j ). The variable bitEscapeQuant is define as Table

- escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] is set equal to

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ]<<(bitDepth – bitEscapeQuant)

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the pixel at the location( i, j ) for color component cIdx.

copy_above_half_line_mode[ x0 ][ y0 ][ cIdx ][ j ]k speficies the copy above mode of kth half part, k = 0 or 1, of jth line (row). The kth half part will search the matched part vertically in the reconstructed lines above current line.

copy_above_half_line_shift_vector[ x0 ][ y0 ][ cIdx ][ j ]k speficeis the matched position above the kth half part, k = 0 or 1, of jth line (row). If it equals 0, current half part have no matched part in the reconstructed lines above current line.

line_intra_bc_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in line-based intra block copy mode. line_intra_bc_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded according to intra_bc_flag. When not present, the value of line_intra_bc_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_intra_bc_type[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is split equivalently into columns. line_intra_bc_type[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is split equivalently into rows.

combine_ibc_palette_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in combination of intra block copy and palette coding coding mode. combine_ibc_palette_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in combination of intra block copy and palette coding coding mode. When not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

adaptive_mv_precision_flag[ x0 ][ y0 ] equals to 1 specifies that all the motion vectors in the current coding unit are in integer-pixel precision. cu_ADMVP_flag[ x0 ][ y0 ] equals to 0 specifies that all the motion vectors in the current coding unit are in quarter-pixel precision. When not present, this flag is set equal to 0.

…

# Compression performance discussion

## Lossy test condition

### Summary

Experiments were conducted using the test conditions which are specified in the CfP document. In order to meet the target file size, the QP value is changed into QP+1 at a specified frame, and the value QP+1 is applied to the subsequent frames. The QP settings are listed in Table 5. The average BD-rate saving of the proposed codec achieves 28.6%, 17.3%, 13.7% in the All Intra, Random Access, Low Delay B configurations, respectively. Per-class coding performance is provided in Table 6.

- - Change QP setting

| Change QP Settings - All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate Point |  | R4 |  | R3 |  | R2 |  | R1 |  |
| Sequence |  | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame |
| RGB | S01 | 22 | 536 | 28 | 362 | 34 | 208 | 40 | 124 |
|  | S02 | 5 | 287 | 10 | 150 | 26 | 551 | 39 | 511 |
|  | S03 | 12 | 210 | 23 | 578 | 32 | 476 | 40 | 181 |
|  | S04 | 20 | 295 | 25 | 207 | 31 | 590 | 36 | 358 |
|  | S05 | 20 | 538 | 25 | 579 | 30 | 501 | 35 | 259 |
|  | S06 | 15 | 90 | 21 | 84 | 33 | 151 | 39 | 83 |
|  | S07 | 20 | 51 | 26 | 558 | 31 | 236 | 36 | 236 |
|  | S08 | 19 | 78 | 25 | 452 | 30 | 239 | 36 | 500 |
|  | S09 | 26 | 286 | 31 | 303 | 36 | 255 | 41 | 76 |
|  | S10 | 21 | 12 | 26 | 6 | 31 | 4 | 36 | 8 |
|  | S11 | 20 | 35 | 25 | 108 | 30 | 104 | 35 | 34 |
|  | S12 | 21 | 183 | 26 | 211 | 31 | 189 | 36 | 150 |
| YUV | S01 | 23 | 217 | 29 | 107 | 35 | 97 | 41 | 237 |
|  | S02 | 14 | 49 | 23 | 216 | 32 | 111 | 40 | 164 |
|  | S03 | 18 | 443 | 25 | 177 | 33 | 160 | 40 | 563 |
|  | S04 | 20 | 116 | 25 | 40 | 31 | 427 | 36 | 225 |
|  | S05 | 20 | 548 | 25 | 507 | 30 | 258 | 36 | 562 |
|  | S06 | 19 | 270 | 27 | 125 | 35 | 139 | 41 | 115 |
|  | S07 | 21 | 462 | 26 | 342 | 31 | 276 | 36 | 208 |
|  | S08 | 20 | 483 | 25 | 275 | 30 | 40 | 36 | 501 |
|  | S09 | 26 | 263 | 31 | 238 | 36 | 187 | 41 | 110 |
|  | S10 | 21 | 6 | 26 | 8 | 31 | 9 | 36 | 6 |
|  | S11 | 20 | 118 | 25 | 147 | 30 | 49 | 36 | 189 |
|  | S12 | 21 | 185 | 26 | 178 | 31 | 147 | 36 | 93 |

| Change QP Settings – Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate Point |  | R4 |  | R3 |  | R2 |  | R1 |  |
| Sequence |  | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame |
| RGB | S01 | 26 | 595 | 31 | 415 | 36 | 248 | 41 | 50 |
|  | S02 | 6 | 504 | 14 | 0 | 30 | 597 | 42 | 528 |
|  | S03 | 21 | 193 | 28 | 177 | 35 | 196 | 42 | 193 |
|  | S04 | 21 | 212 | 26 | 180 | 31 | 135 | 36 | 96 |
|  | S05 | 21 | 480 | 26 | 585 | 30 | 14 | 35 | 6 |
|  | S06 | 16 | 160 | 21 | 25 | 33 | 113 | 39 | 121 |
|  | S07 | 21 | 196 | 26 | 138 | 31 | 104 | 36 | 74 |
|  | S08 | 21 | 249 | 26 | 204 | 31 | 171 | 36 | 148 |
|  | S09 | 26 | 150 | 31 | 161 | 36 | 105 | 41 | 33 |
|  | S10 | 21 | 17 | 26 | 1 | 32 | 290 | 37 | 289 |
|  | S11 | 21 | 48 | 26 | 105 | 31 | 143 | 36 | 177 |
|  | S12 | 21 | 89 | 26 | 80 | 31 | 55 | 36 | 57 |
| YUV | S01 | 26 | 429 | 31 | 244 | 36 | 84 | 42 | 508 |
|  | S02 | 14 | 185 | 25 | 575 | 33 | 315 | 41 | 214 |
|  | S03 | 23 | 515 | 29 | 249 | 36 | 537 | 41 | 181 |
|  | S04 | 21 | 156 | 26 | 103 | 31 | 84 | 36 | 51 |
|  | S05 | 21 | 561 | 25 | 57 | 30 | 0 | 36 | 482 |
|  | S06 | 19 | 121 | 27 | 89 | 35 | 68 | 41 | 161 |
|  | S07 | 21 | 120 | 26 | 110 | 31 | 105 | 36 | 100 |
|  | S08 | 21 | 215 | 26 | 185 | 31 | 165 | 36 | 160 |
|  | S09 | 26 | 157 | 31 | 143 | 36 | 135 | 41 | 65 |
|  | S10 | 21 | 2 | 26 | 0 | 31 | 0 | 37 | 289 |
|  | S11 | 21 | 96 | 26 | 155 | 31 | 185 | 36 | 121 |
|  | S12 | 21 | 80 | 26 | 57 | 31 | 53 | 36 | 36 |

| Change QP Settings – Low Delay B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate Point |  | R4 |  | R3 |  | R2 |  | R1 |  |
| Sequence |  | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame | Init. QP | Change frame |
| RGB | S01 | 26 | 500 | 31 | 0 | 36 | 155 | 42 | 300 |
|  | S02 | 4 | 530 | 16 | 0 | 32 | 113 | 40 | 518 |
|  | S03 | 20 | 32 | 28 | 262 | 36 | 599 | 40 | 227 |
|  | S04 | 21 | 243 | 26 | 0 | 31 | 0 | 36 | 0 |
|  | S05 | 21 | 346 | 26 | 402 | 31 | 397 | 36 | 373 |
|  | S06 | 16 | 288 | 23 | 288 | 34 | 254 | 40 | 8 |
|  | S07 | 21 | 140 | 26 | 100 | 31 | 68 | 36 | 68 |
|  | S08 | 21 | 173 | 26 | 159 | 31 | 142 | 36 | 81 |
|  | S09 | 26 | 152 | 31 | 117 | 36 | 87 | 42 | 400 |
|  | S10 | 21 | 27 | 26 | 8 | 31 | 0 | 36 | 0 |
|  | S11 | 21 | 25 | 26 | 34 | 31 | 36 | 36 | 32 |
|  | S12 | 21 | 97 | 26 | 76 | 31 | 40 | 36 | 37 |
| YUV | S01 | 26 | 313 | 31 | 148 | 37 | 598 | 42 | 508 |
|  | S02 | 15 | 508 | 26 | 198 | 35 | 560 | 42 | 537 |
|  | S03 | 23 | 358 | 29 | 42 | 36 | 499 | 41 | 79 |
|  | S04 | 21 | 154 | 26 | 114 | 31 | 60 | 36 | 24 |
|  | S05 | 21 | 343 | 26 | 372 | 31 | 402 | 36 | 148 |
|  | S06 | 20 | 69 | 29 | 68 | 36 | 288 | 41 | 68 |
|  | S07 | 21 | 75 | 26 | 68 | 31 | 64 | 36 | 21 |
|  | S08 | 21 | 155 | 26 | 135 | 31 | 111 | 36 | 100 |
|  | S09 | 26 | 94 | 31 | 92 | 36 | 72 | 42 | 488 |
|  | S10 | 21 | 10 | 26 | 0 | 32 | 296 | 37 | 292 |
|  | S11 | 21 | 27 | 26 | 36 | 31 | 34 | 36 | 0 |
|  | S12 | 21 | 44 | 26 | 39 | 31 | 36 | 36 | 32 |

- - BD-rate of the proposed codec in lossy test condition

|  | All Intra BD-rate (%) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -28.57 | -28.42 | -28.20 |
| RGB, text & graphics with motion,720p | -14.89 | -14.35 | -14.93 |
| RGB, mixed content, 1440p | -9.52 | -8.79 | -8.88 |
| RGB, mixed content, 1080p | -11.15 | -10.81 | -10.79 |
| RGB, Animation, 720p | -0.84 | -0.92 | -0.92 |
| YUV, text & graphics with motion, 1080p | -26.33 | -24.70 | -25.27 |
| YUV, text & graphics with motion,720p | -12.68 | -12.54 | -14.43 |
| YUV, mixed content, 1440p | -9.16 | -9.96 | -9.76 |
| YUV, mixed content, 1080p | -10.49 | -10.34 | -10.24 |
| YUV, Animation, 720p | -0.61 | -0.81 | -0.73 |

|  | Random Access BD-rate (%) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -17.27 | -17.17 | -17.06 |
| RGB, text & graphics with motion,720p | -12.26 | -11.55 | -12.04 |
| RGB, mixed content, 1440p | -7.66 | -6.88 | -6.97 |
| RGB, mixed content, 1080p | -8.68 | -8.54 | -8.52 |
| RGB, Animation, 720p | -1.49 | -1.92 | -1.77 |
| YUV, text & graphics with motion, 1080p | -15.65 | -15.58 | -16.80 |
| YUV, text & graphics with motion,720p | -10.62 | -10.17 | -12.21 |
| YUV, mixed content, 1440p | -6.60 | -8.32 | -8.35 |
| YUV, mixed content, 1080p | -7.75 | -8.84 | -8.73 |
| YUV, Animation, 720p | -0.62 | -1.62 | -1.27 |

|  | Low delay B BD-rate (%) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -13.66 | -13.69 | -13.66 |
| RGB, text & graphics with motion,720p | -10.43 | -9.64 | -9.96 |
| RGB, mixed content, 1440p | -9.99 | -8.57 | -8.73 |
| RGB, mixed content, 1080p | -8.04 | -7.54 | -7.62 |
| RGB, Animation, 720p | -2.33 | -2.72 | -2.34 |
| YUV, text & graphics with motion, 1080p | -10.45 | -10.97 | -12.36 |
| YUV, text & graphics with motion,720p | -8.21 | -9.37 | -11.07 |
| YUV, mixed content, 1440p | -7.57 | -9.71 | -9.65 |
| YUV, mixed content, 1080p | -6.45 | -8.00 | -7.79 |
| YUV, Animation, 720p | -0.83 | -2.17 | -1.74 |

### Per-sequence BD-rate

Table 7 shows per-sequence coding performance of the proposed codec. In All Intra coding conditions, the highest BD-rate reduction (34-35%) is observed in S02 (Desktop, RGB colour format) and the lowest (0.6-0.8%) in S10 (Robot, YUV colour format). In Random Access and Low Delay B coding conditions, S06 (Web_browsing, RGB colour format) achieves 28.8% and 24.8% BD-rate saving, respectively.

- - Per-sequence BD-rate saving in lossy test condition

|  |  | All Intra BD-rate (%) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| RGB, text & graphics with motion, 1080p | S01 | -21.43 | -21.82 | -21.57 |
|  | S02 | -35.14 | -34.29 | -34.26 |
|  | S03 | -29.14 | -29.15 | -28.78 |
|  | Avg. | -28.57 | -28.42 | -28.20 |
| RGB, text & graphics with motion,720p | S06 | -29.41 | -30.01 | -31.70 |
|  | S07 | -8.47 | -8.34 | -8.74 |
|  | S08 | -13.52 | -11.63 | -11.76 |
|  | S09 | -8.14 | -7.43 | -7.50 |
|  | Avg. | -14.89 | -14.35 | -14.93 |
| RGB, mixed content, 1440p | S11 | -11.92 | -10.86 | -10.93 |
|  | S12 | -7.11 | -6.71 | -6.83 |
|  | Avg. | -9.52 | -8.79 | -8.88 |
| RGB, mixed content, 1080p | S05 | -14.87 | -14.18 | -14.19 |
|  | S04 | -7.43 | -7.43 | -7.40 |
|  | Avg. | -11.15 | -10.81 | -10.79 |
| RGB, Animation, 720p | S10 | -0.84 | -0.92 | -0.92 |
|  | Avg. | -0.84 | -0.92 | -0.92 |
| YUV, text & graphics with motion, 1080p | S01 | -17.30 | -18.45 | -19.15 |
|  | S02 | -31.68 | -27.89 | -29.50 |
|  | S03 | -30.02 | -27.76 | -27.16 |
|  | Avg. | -26.33 | -24.70 | -25.27 |
| YUV, text & graphics with motion,720p | S06 | -23.78 | -25.12 | -29.32 |
|  | S07 | -6.58 | -7.55 | -9.18 |
|  | S08 | -13.97 | -11.17 | -11.70 |
|  | S09 | -6.39 | -6.32 | -7.51 |
|  | Avg. | -12.68 | -12.54 | -14.43 |
| YUV, mixed content, 1440p | S11 | -11.86 | -12.33 | -12.40 |
|  | S12 | -6.47 | -7.58 | -7.12 |
|  | Avg. | -9.16 | -9.96 | -9.76 |
| YUV, mixed content, 1080p | S05 | -14.23 | -14.04 | -13.90 |
|  | S04 | -6.76 | -6.65 | -6.59 |
|  | Avg. | -10.49 | -10.34 | -10.24 |
| YUV, Animation, 720p | S10 | -0.61 | -0.81 | -0.73 |
|  | Avg. | -0.61 | -0.81 | -0.73 |

|  |  | Random Access BD-rate (%) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| RGB, text & graphics with motion, 1080p | S01 | -12.16 | -13.06 | -13.03 |
|  | S02 | -22.77 | -21.58 | -21.49 |
|  | S03 | -16.88 | -16.88 | -16.65 |
|  | Avg. | -17.27 | -17.17 | -17.06 |
| RGB, text & graphics with motion,720p | S06 | -28.76 | -28.85 | -29.77 |
|  | S07 | -5.38 | -5.54 | -5.67 |
|  | S08 | -8.40 | -5.78 | -6.60 |
|  | S09 | -6.50 | -6.02 | -6.14 |
|  | Avg. | -12.26 | -11.55 | -12.04 |
| RGB, mixed content, 1440p | S11 | -9.81 | -8.76 | -8.73 |
|  | S12 | -5.51 | -5.00 | -5.21 |
|  | Avg. | -7.66 | -6.88 | -6.97 |
| RGB, mixed content, 1080p | S05 | -12.57 | -11.88 | -11.95 |
|  | S04 | -4.80 | -5.19 | -5.09 |
|  | Avg. | -8.68 | -8.54 | -8.52 |
| RGB, Animation, 720p | S10 | -1.49 | -1.92 | -1.77 |
|  | Avg. | -1.49 | -1.92 | -1.77 |
| YUV, text & graphics with motion, 1080p | S01 | -7.64 | -11.27 | -12.54 |
|  | S02 | -21.96 | -20.65 | -22.19 |
|  | S03 | -17.34 | -14.82 | -15.68 |
|  | Avg. | -15.65 | -15.58 | -16.80 |
| YUV, text & graphics with motion,720p | S06 | -23.73 | -24.15 | -28.28 |
|  | S07 | -4.06 | -5.67 | -7.33 |
|  | S08 | -9.07 | -5.87 | -7.36 |
|  | S09 | -5.60 | -4.98 | -5.88 |
|  | Avg. | -10.62 | -10.17 | -12.21 |
| YUV, mixed content, 1440p | S11 | -8.79 | -10.56 | -10.69 |
|  | S12 | -4.41 | -6.08 | -6.01 |
|  | Avg. | -6.60 | -8.32 | -8.35 |
| YUV, mixed content, 1080p | S05 | -11.90 | -12.60 | -12.61 |
|  | S04 | -3.60 | -5.07 | -4.85 |
|  | Avg. | -7.75 | -8.84 | -8.73 |
| YUV, Animation, 720p | S10 | -0.62 | -1.62 | -1.27 |
|  | Avg. | -0.62 | -1.62 | -1.27 |

|  |  | Low Delay B BD-rate (%) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| RGB, text & graphics with motion, 1080p | S01 | -9.26 | -9.83 | -9.70 |
|  | S02 | -17.07 | -16.61 | -16.58 |
|  | S03 | -14.66 | -14.64 | -14.71 |
|  | Avg. | -13.66 | -13.69 | -13.66 |
| RGB, text & graphics with motion,720p | S06 | -24.75 | -23.96 | -24.12 |
|  | S07 | -5.14 | -4.62 | -5.07 |
|  | S08 | -6.74 | -5.09 | -5.53 |
|  | S09 | -5.07 | -4.89 | -5.12 |
|  | Avg. | -10.43 | -9.64 | -9.96 |
| RGB, mixed content, 1440p | S11 | -14.59 | -12.25 | -12.54 |
|  | S12 | -5.39 | -4.90 | -4.91 |
|  | Avg. | -9.99 | -8.57 | -8.73 |
| RGB, mixed content, 1080p | S05 | -11.42 | -10.24 | -10.53 |
|  | S04 | -4.66 | -4.83 | -4.71 |
|  | Avg. | -8.04 | -7.54 | -7.62 |
| RGB, Animation, 720p | S10 | -2.33 | -2.72 | -2.34 |
|  | Avg. | -2.33 | -2.72 | -2.34 |
| YUV, text & graphics with motion, 1080p | S01 | -4.88 | -8.48 | -10.49 |
|  | S02 | -12.62 | -12.65 | -13.57 |
|  | S03 | -13.85 | -11.79 | -13.01 |
|  | Avg. | -10.45 | -10.97 | -12.36 |
| YUV, text & graphics with motion,720p | S06 | -17.19 | -20.52 | -22.29 |
|  | S07 | -3.95 | -5.61 | -9.66 |
|  | S08 | -8.04 | -5.93 | -6.79 |
|  | S09 | -3.68 | -5.42 | -5.55 |
|  | Avg. | -8.21 | -9.37 | -11.07 |
| YUV, mixed content, 1440p | S11 | -10.82 | -13.45 | -13.58 |
|  | S12 | -4.32 | -5.98 | -5.72 |
|  | Avg. | -7.57 | -9.71 | -9.65 |
| YUV, mixed content, 1080p | S05 | -10.02 | -11.40 | -11.28 |
|  | S04 | -2.88 | -4.59 | -4.30 |
|  | Avg. | -6.45 | -8.00 | -7.79 |
| YUV, Animation, 720p | S10 | -0.83 | -2.17 | -1.74 |
|  | Avg. | -0.83 | -2.17 | -1.74 |

## Mathematically lossless test condition

### Summary

The coding performance of the proposed codec in lossless test condition is shown in Table 8. The proposed codec can averagely achieve 35.0%, 34.5%, 34.6% bit-rate saving in All Intra, Random Access, and Low Delay B test condition, respectively.

- - Bit-rate saving in lossless test condition

|  | All Intra Bit-rate Saving (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 33.07 | 35.02 | 24.22 | 47.70 |
| RGB, text & graphics with motion,720p | 12.82 | 15.05 | 2.29 | 33.34 |
| RGB, mixed content, 1440p | 4.88 | 5.54 | 2.69 | 8.39 |
| RGB, mixed content, 1080p | 10.08 | 9.37 | 7.62 | 11.13 |
| RGB, Animation, 720p | 0.89 | 0.89 | 0.89 | 0.89 |
| YUV, text & graphics with motion, 1080p | 28.05 | 29.33 | 20.00 | 39.26 |
| YUV, text & graphics with motion,720p | 10.79 | 13.05 | 1.67 | 31.24 |
| YUV, mixed content, 1440p | 3.75 | 4.09 | 2.12 | 6.07 |
| YUV, mixed content, 1080p | 8.37 | 7.81 | 6.49 | 9.14 |
| YUV, Animation, 720p | 0.46 | 0.46 | 0.46 | 0.46 |

|  | Random Access Bit-rate Saving (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 25.73 | 34.54 | 24.72 | 48.51 |
| RGB, text & graphics with motion,720p | 8.60 | 15.37 | 6.44 | 32.53 |
| RGB, mixed content, 1440p | 8.91 | 8.85 | 7.70 | 10.01 |
| RGB, mixed content, 1080p | 12.70 | 11.17 | 9.40 | 12.94 |
| RGB, Animation, 720p | 7.70 | 7.70 | 7.70 | 7.70 |
| YUV, text & graphics with motion, 1080p | 17.49 | 24.55 | 16.57 | 35.76 |
| YUV, text & graphics with motion,720p | 4.83 | 10.83 | 2.43 | 30.44 |
| YUV, mixed content, 1440p | 2.46 | 2.42 | 2.00 | 2.85 |
| YUV, mixed content, 1080p | 8.39 | 5.62 | 2.47 | 8.77 |
| YUV, Animation, 720p | 2.31 | 2.31 | 2.31 | 2.31 |

|  | Low Delay B Bit-rate Saving (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Average | Min | Max |
| RGB, text & graphics with motion, 1080p | 25.33 | 34.59 | 24.61 | 49.61 |
| RGB, text & graphics with motion,720p | 9.01 | 15.84 | 6.62 | 30.61 |
| RGB, mixed content, 1440p | 9.80 | 9.77 | 9.01 | 10.53 |
| RGB, mixed content, 1080p | 12.57 | 11.40 | 10.06 | 12.73 |
| RGB, Animation, 720p | 8.24 | 8.24 | 8.24 | 8.24 |
| YUV, text & graphics with motion, 1080p | 16.38 | 22.82 | 15.77 | 32.41 |
| YUV, text & graphics with motion,720p | 4.37 | 9.86 | 2.40 | 27.95 |
| YUV, mixed content, 1440p | 2.29 | 2.26 | 1.85 | 2.67 |
| YUV, mixed content, 1080p | 8.28 | 5.22 | 1.79 | 8.65 |
| YUV, Animation, 720p | 2.24 | 2.24 | 2.24 | 2.24 |

### Per-sequence bit-rate saving

- - Bit-rate saving in lossless test condition

| Sequence |  | Bit-rate saving (%) |  |  |
| --- | --- | --- | --- | --- |
|  |  | All Intra | Random Access | Low Delay B |
| RGB, text & graphics with motion, 1080p | S01 | 24.22 | 24.72 | 24.61 |
|  | S02 | 47.70 | 48.51 | 49.61 |
|  | S03 | 33.14 | 30.38 | 29.55 |
|  | Total | 33.07 | 25.73 | 25.33 |
|  | Avg. | 35.02 | 34.54 | 34.59 |
| RGB, text & graphics with motion,720p | S06 | 33.34 | 32.53 | 30.61 |
|  | S07 | 14.63 | 12.85 | 14.00 |
|  | S08 | 9.96 | 6.44 | 6.62 |
|  | S09 | 2.29 | 9.66 | 12.13 |
|  | Total | 12.82 | 8.60 | 9.01 |
|  | Avg. | 15.05 | 15.37 | 15.84 |
| RGB, mixed content, 1440p | S11 | 8.39 | 10.01 | 10.53 |
|  | S12 | 2.69 | 7.70 | 9.01 |
|  | Total | 4.88 | 8.91 | 9.80 |
|  | Avg. | 5.54 | 8.85 | 9.77 |
| RGB, mixed content, 1080p | S05 | 7.62 | 9.40 | 10.06 |
|  | S04 | 11.13 | 12.94 | 12.73 |
|  | Total | 10.08 | 12.70 | 12.57 |
|  | Avg. | 9.37 | 11.17 | 11.40 |
| RGB, Animation, 720p | S10 | 0.89 | 7.70 | 8.24 |
|  | Total | 0.89 | 7.70 | 8.24 |
|  | Avg. | 0.89 | 7.70 | 8.24 |
| YUV, text & graphics with motion, 1080p | S01 | 20.00 | 16.57 | 15.77 |
|  | S02 | 39.26 | 35.76 | 32.41 |
|  | S03 | 28.73 | 21.33 | 20.28 |
|  | Total | 28.05 | 17.49 | 16.38 |
|  | Avg. | 29.33 | 24.55 | 22.82 |
| YUV, text & graphics with motion,720p | S06 | 31.24 | 30.44 | 27.95 |
|  | S07 | 11.78 | 6.34 | 4.90 |
|  | S08 | 7.49 | 4.13 | 4.21 |
|  | S09 | 1.67 | 2.43 | 2.40 |
|  | Total | 10.79 | 4.83 | 4.37 |
|  | Avg. | 13.05 | 10.83 | 9.86 |
| YUV, mixed content, 1440p | S11 | 6.07 | 2.85 | 2.67 |
|  | S12 | 2.12 | 2.00 | 1.85 |
|  | Total | 3.75 | 2.46 | 2.29 |
|  | Avg. | 4.09 | 2.42 | 2.26 |
| YUV, mixed content, 1080p | S05 | 6.49 | 2.47 | 1.79 |
|  | S04 | 9.14 | 8.77 | 8.65 |
|  | Total | 8.37 | 8.39 | 8.28 |
|  | Avg. | 7.81 | 5.62 | 5.22 |
| YUV, Animation, 720p | S10 | 0.46 | 2.31 | 2.24 |
|  | Total | 0.46 | 2.31 | 2.24 |

# Complexity analysis

## Encoding time and measurement methodology

Table 10 summarizes the encoding time of the proposed screen content coding technology for all intra (AI), random access (RA), and low delay B (LB) configurations. All the simulations were conducted using the PCs with Intel Xeon E5540 8-core CPUs (2.53GHz) and 32GB installed memory (RAM). Otherwise, the operating system of each PC is 64-bit Windows 7. To check detailed data for each sequence, please refer to the attached file named JCTVC-Q0032-lossy.xlsm.

- - The encoding time of the proposed software compared with the anchor

|  | AI | RA | LB |
| --- | --- | --- | --- |
| Lossy | 169% | 122% | 121% |
| Lossless | 187% | 128% | 125% |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

Table 11 summarizes the decoding time of the proposed screen content coding technology for all intra (AI), random access (RA), and low delay B (LB) configurations. All the simulations were conducted using the PCs with Intel Xeon E5540 8-core CPUs (2.53GHz) and 32GB installed memory (RAM). Otherwise, the operating system of each PC is 64-bit Windows 7. To check detailed data for each sequence, please refer to the attached file named JCTVC-Q0032-lossless.xlsm.

- - The decoding time of the proposed software compared with the anchor

|  | AI | RA | LB |
| --- | --- | --- | --- |
| Lossy | 103% | 86% | 115% |
| Lossless | 99% | 111% | 114% |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

The description of the computing platform used to determine encoding and decoding times are provided in Section 5.1 and 5.2.

## Expected memory usage of encoder

In the palette-based coding and CIBCPC, one additional buffer of which size is the same as that of current CU is needed to copy the above sub-lines in the index map coding. Therefore, the worst case is using a buffer of which size is the same as that of a CTU.

For the line-based IntraBC, the size of the memory usage is increased by four times when the finest granularity of BV storage is of 4x4.

When the algorithm described in Section 2.7.1 is enabled, one extra BV predictor should be stored so the extra buffer is needed as well.

## Expected memory usage of decoder

The expected memory usage of the decoder is the same as that of the encoder side which is described in Section 5.4.

## Complexity characteristics of encoder motion estimation and partitioning selection

For each block partition, one additional R-D cost check for inter blocks is needed when the adaptive MV precision described in Section 2.9.1 and 2.9.2 is in use.

## Complexity characteristics of decoder motion compensation

The complexity characteristics of decoder motion compensation is the same as that of the HM-13.0+RExt-6.0 software.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection

The palette coding, combined palette coding and intra block copy, and line-based intra block copy are proposed in this contribution, so the complexity will be increased due to the additional estimation and process in the encoder stage.

The palette coding introduced in Section 2.4.1 is based on the algorithm described in JCTVC-P0108. Accordingly, the complexity of the palette-based coding is similar to that of the mechanism introduced in JCTVC-P0108.

In combined palette coding and intra block copy, the BVs are obtained by reusing that of the intra block copy mode in [2]. Consequently, the encoding time is similar to that of the palette-based coding and the complexity overhead is negligible as well.

The complexity of the line-based intra block copy is also close to that of the intra block copy in [2]. This is due in that the total number of the pixels involved in block vector search is almost identical to that of the intra block copy in [2].

Besides, the partitioning selection procedures are unchanged so the complexity characteristic of this part is similar to that of the HM-13.0+RExt-6.0 software.

## Complexity characteristics of decoder intra-frame prediction operation

When the current CU is coded in palette coding, the complexity characteristics of decoder operation are similar to those of the mechanism introduced in JCTVC-P0108.

For the combined palette coding and intra block copy, the current CU is reconstructed by copying the matching block and decoding the major colors as well as index map, and no residuals should be reconstructed. The complexity is nearly identical to that of intra block copy.

The complexity of the line-based intra block copy is also close to that of the intra block copy in [2]. This is due in that the total number of the pixels involved in block vector search is almost identical to that of the intra block copy in [2].

## Complexity characteristics of encoder inter-component prediction tools

The complexity characteristics of encoder inter-component prediction tools is the same as that of the HEVC RExt draft specification [2].

## Complexity characteristics of decoder inter-component prediction tools

The complexity characteristics of decoder inter-component prediction tools is the same as that of the HEVC RExt draft specification [2].

## Complexity characteristics of transforms specific in screen content coding

This part is the same as that of the HEVC RExt draft specification [2].

## Complexity characteristics of encoder related to other tools specific in screen content coding

The additional complexity produced by checking the best from the two block vector predictors is negligible; hence the complexity characteristics of encoder related to other tools specific in screen content coding is the same as that of the HEVC RExt draft specification [2].

## Complexity characteristics of decoder related to other tools specific in screen content coding

This part is the same as that of the HEVC RExt draft specification [2].

## Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding

The index map coding introduced in Section 2.4.1 is based on the algorithm provided in JCTVC-P0108. Thus, in the index map coding, the complexity is the same as that of JCTVC-P0108.

## Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding

The complexity characteristics of the index map coding introduced in Section 2.4.1 are similar to those of the mechanism introduced in JCTVC-P0108.

## Degree of capability for encoder parallel processing

This part is similar to that of the HEVC RExt draft specification [2].

## Degree of capability for decoder parallel processing

This part is similar to that of the HEVC RExt draft specification [2].

# Software implementation description

The proposed technologies are implemented on top of the HM-13.0+RExt-6.0 software using standard C++ programming language. The macros and configuration files provided in the HM-13.0+RExt-6.0 software package are unchanged. Modifications to the HM-13.0+RExt-6.0 software are properly marked by using macros.

# Closing remarks

In this contribution, we proposed palette coding with major color propagation and sub-line index coding, combined palette coding and intra block copy, and line-based intra block copy for intra block coding, adaptive motion vector precision for motion vector coding, and block vector initialization and second block vector predictor for block vector coding. In summary, the results show BD-rate reductions of 0.6-35.1% for all-intra settings, 0.6-28.7% for random access settings, and 0.8-24.7% for low delay B settings, when compared with the anchors.

# Reference

- MPEG2014/N14175, “Joint Call for Proposals for Coding of Screen Content,” ISO/IEC JTC1/SC29/WG11, Jan. 2014

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan and T. Suzuki, “HEVC Range Extensions Draft 6,” JCTVC-P1005-v1, 16th JCT-VC meeting, San Jose, Jan., 2014.

# Patent rights declaration(s)

NCTU may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ITRI International may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).