| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0033 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Tzu-Der Chuang, Yu-Chen Sun, Xiaozhong Xu, Jing Ye, Shih-Ta Hsiang, Yi-Wen Chen, Kai Zhang, Xianguo Zhang, Shan Liu, Yu-Wen Huang, Shawmin Lei | Email: | {polin.lai, peter.chuang, yc.sun, xiaozhong.xu, jing.ye, shih-ta.hsiang, yiwen.chen, kai.zhang, xianguo.zhang, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents MediaTek’s response to the Joint Call for Proposals for Coding of Screen Content issued by ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG). The goal of this proposal is to provide screen content coding technologies on top of the HEVC standard. In order to achieve this goal, a number of coding tools are proposed. These include intra block copying (IntraBC) with extended search range, triplet palette mode, line-based (2N×1/1×2N) intra copying, modified coding of inter-component residual prediction, single color mode. With all the proposed tools enabled, the proposed screen content video codec reportedly achieves BD-rate savings summarized as the following:

For RGB sequences, G-component, lossy condition:

- text & graphics with motion, 1080p	: (AI) 46.0%,	(RA) 31.4%,	(LB) 26.0%;

- text & graphics with motion,720p 	: (AI) 28.6%,	(RA) 23.7%, 	(LB) 20.3%;

- mixed content, 1440p           	: (AI) 20.2%,	(RA) 14.1%, 	(LB) 13.0%;

- mixed content, 1080p           	: (AI) 18.8%,	(RA) 13.1%, 	(LB) 8.7%;

- animation, 720p               	: (AI) 1.7%, 	(RA) 1.3%, 	(LB) 1.1%.

For YUV sequences, Y-component, lossy condition:

- text & graphics with motion, 1080p	: (AI) 44.1%,	(RA) 27.4%,	(LB) 21.2%;

- text & graphics with motion,720p 	: (AI) 25.1%,	(RA) 20.0%, 	(LB) 14.7%;

- mixed content, 1440p           	: (AI) 20.0%,	(RA) 14.0%, 	(LB) 11.5%;

- mixed content, 1080p           	: (AI) 17.9%,	(RA) 13.4%, 	(LB) 8.1%;

- animation, 720p               	: (AI) 0.3%, 	(RA) 0.1%, 	(LB) 0.3%.

The complete data, including lossless results, can be found in the accompanying spread sheets. The average encoding time for the proposed encoder compared against the anchor RExt 5.1 are 314%, 151%, 141% for lossy AI, RA, and LD configurations, and 472%, 175%, 163% for lossless AI, RA, and LD configurations, respectively. The corresponding average decoding time compared against anchor are 81%, 99%, and 101% for lossy and 87%, 93%, and 96% for lossless, respectively.

# Contents

Abstract	1

Contents	1

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Inter-component prediction	3

2.2.1	Modified alpha parameter coding for inter-component residual prediction	3

2.3	Inter-picture prediction	4

2.4	Intra-picture prediction	4

2.4.1	Boundary filter disabling	4

2.5	In-loop filtering	5

2.5.1	De-blocking filter	5

2.5.2	SAO	5

2.5.3	Other	5

2.6	Spatial transforms	5

2.7	Other specific compression tools for screen content	5

2.7.1	PU-based Intra block copying with extended search range	5

2.7.2	Line-based (2N×1/1×2N) Intra copying	6

2.7.3	Triplet palette mode	7

2.7.4	Single color mode	12

2.8	Quantization	12

2.9	Motion/mode/parameter coding	13

2.9.1	Merge/skip	13

2.9.2	Motion vector coding	13

2.9.3	Intra prediction mode coding	13

2.10	Entropy coding of residual / transform coefficients	13

3	Syntax and semantics description	13

3.1	Sequence parameter set RBSP syntax	13

3.2	Picture parameter set RBSP syntax	15

3.3	Slice segment header syntax	16

3.4	Coding unit syntax	16

3.5	Extended search range for Intra block-copying	19

3.6	Line-based intra copying syntax	19

3.7	Palette syntax (scm_info_coding)	19

3.8	Cross-component prediction syntax	23

4	Compression performance discussion	25

4.1	Lossy test condition	25

4.2	Mathematically lossless test condition	26

5	Complexity analysis	27

5.1	Encoding time and measurement methodology	27

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	27

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	27

5.4	Expected memory usage of encoder	28

5.5	Expected memory usage of decoder	28

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	28

5.7	Complexity characteristics of decoder motion compensation	29

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	29

5.9	Complexity characteristics of decoder intra-frame prediction operation	29

5.10	Complexity characteristics of encoder inter-component prediction tools	29

5.11	Complexity characteristics of decoder inter-component prediction tools	29

5.12	Complexity characteristics of transforms specific in screen content coding	29

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	29

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	30

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	30

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	30

5.17	Degree of capability for encoder parallel processing	30

5.18	Degree of capability for decoder parallel processing	30

6	Software implementation description	30

7	Highlighted aspects discussion	30

8	Closing remarks	30

9	Patent rights declaration(s)	30

References	31

# Introduction

In response to the “Joint Call for Proposals for Coding of Screen Content” [1], MediaTek has submitted the required coded test materials. This contribution is a technical description of the MediaTek’s proposal.

Several coding tools were developed and integrated together to achieve significantly better coding efficiency than the Call for Proposals (CfP) anchors. The integrated tools include intra block copying (IntraBC) with extended search range, triplet palette mode, line-based intra copying, modified coding of inter-component residual prediction, and single color mode.

The software has no mismatch between the encoder and the decoder throughout all the test cases required by the CfP. With the presented complexity-efficiency and complexity tradeoff, it is asserted that this software can be a good starting framework for the development of this standard.

# Algorithm description

## CTU/TU/PU partitioning

The basic structure of coding unit / transform unit / prediction unit (CU/TU/PU) partitioning is the same as that in the HEVC Main profile [2]. Specific implementation is the same as HM-12.1_RExt-5.1 software [3]. Specific parameter settings are the same as the following files from HM-12.1_RExt-5.1 software package, which were used to generate the anchors as well:

encoder_intra_main_rext.cfg

encoder_randomaccess_main_rext.cfg

encoder_lowdelay_main_rext.cfg

## Inter-component prediction

### Modified alpha parameter coding for inter-component residual prediction

In HEVC Range Extensions Draft 5 [4], an alpha parameter in the inter-component residual prediction is coded with a sign flag and an absolute value. This alpha parameter is signalled by syntax elements of res_scale_sign_flag and log2_res_scale_abs_plus1. The latter represents the absolute values of alpha and is binarized as a truncated unary code as shown in Table 1 [4]. log2_res_scale_abs_plus1 utilizes 3 context models in total. Two context models are used by the first two bins separately, while one context model is shared by the last two bins.

Table 1 [4]. Binarization for res_scale_abs_val and the corresponding alphas.

| Abosulte ResScaleVal | log2_res_scale_abs_plus1 [ c ] |
| --- | --- |
| 0 | 0(0) |
| 1 | 1(10) |
| 2 | 2(110) |
| 4 | 3(1110) |
| 8 | 4(1111) |

However, it is observed that alpha being equal to 4 or 8 takes quite a large proportion, especially in RGB sequences. Since the current coding method does not match with such a distribution very well, two methods are proposed to code the alpha parameter more efficiently [5].

Method 1: To code log2_res_scale_abs_plus1, four context models are used by the four bins separately. Note that this method is the same as the one adopted in HEVC Range Extensions Draft 6 [6].

Method 2: A flag scale_parameter_coding_mode in PPS extension is signalled to indicate whether Method 1 or the following new signalling scheme is applied. In the new signalling, the alpha parameter is represented by log2_res_scale_pred_4minus and log2_res_scale_neg as depicted in Table 2. The binarization for the two syntax elements and the corresponding alphas are demonstrated in Table. 3. Five context models are used by the five bins separately for log2_res_scale_pred_4minus. And log2_res_scale_neg is coded in the by-pass mode simply. In summary, alpha parameter equals to 1<<(4–log2_res_scale_4minus) when scale_parameter_coding_mode equal to 1 and log2_res_scale_4minus is 0 or 5. If log2_res_scale_4minus equals to 5, alpha parameter equals to – (1<< log2_res_scale_neg). In this proposal, the flag scale_parameter_coding_mode is set to be 1 for RGB sequences, and set to be 0 for YUV sequences.

Table 2. Alpha parameter syntax in method 2

| cross_comp_pred ( x0, y0, c ) { | Descriptor |
| --- | --- |
| if(scale_parameter_coding_mode = = 0) { |  |
| log2_res_scale_abs_plus1[ c ] | ae(v) |
| if( log2_res_scale_abs_plus1[ c ] != 0 ) |  |
| res_scale_sign_flag[ c ] | ae(v) |
| } |  |
| else { |  |
| log2_res_scale_pred_4minus[ c ] | ae(v) |
| if(log2_res_scale_4minus [ c ] == 5) |  |
| log2_res_scale_neg [ c ] | ae(v) |
| } |  |
| } |  |

Table 3. Binarization and corresponding alphas in method 2

| Alpha | log2_res_scale_pred_4minus[ c ] | log2_res_scale_neg [ c ] |
| --- | --- | --- |
| 0 | 0(0) | Na |
| 8 | 1(10) | Na |
| 4 | 2(110) | Na |
| 2 | 3(1110) | Na |
| 1 | 4(11110) | Na |
| -1 | 5(11111) | 0(00) |
| -2 | 5(11111) | 1(01) |
| -4 | 5(11111) | 2(10) |
| -8 | 5(11111) | 3(11) |

## Inter-picture prediction

This proposal does not modify the inter-picture prediction aspect as compared to anchor HM-12.1_RExt-5.1.

## Intra-picture prediction

### Boundary filter disabling

For Intra DC mode in the HEVC-based video coding standard including 3D-HEVC, a boundary filter (or smoothing filter) is applied. The boundary prediction samples of DC mode will be smoothed by applying an [1,3] or [1,2,1] filter to reduce the blocking artifact, as shown in Figure 1. For Vertical and Horizontal intra prediction directions, a gradient based boundary filter is applied. Figure 2 shows an example for the gradient based boundary smoothing filter for vertical intra prediction direction.

Note that, the above boundary filters work under the assumption that the PU border is smooth. However, due to the special characteristics of screen content videos, such assumption is not strictly followed, leading to reduced coding efficiency. Therefore, this proposal suggests selectively disabling these boundary filters through a flag transmitted in SPS. In this proposal, this flag is always set to 1, i.e., these boundary filters are always disabled.

Figure 1. Boundary smoothing filter for Intra DC mode in HEVC

Figure 2. Gradient filter for Intra Vertical mode in HEVC

## In-loop filtering

### De-blocking filter

This proposal does not modify the de-blocking filter aspect as compared to anchor HM-12.1_RExt-5.1.

### SAO

This proposal does not modify the SAO aspect as compared to anchor HM-12.1_RExt-5.1.

### Other

This proposal does not contain any additional filtering operation as compared to anchor HM-12.1_RExt-5.1.

## Spatial transforms

This proposal does not modify the spatial transforms aspect as compared to anchor HM-12.1_RExt-5.1.

## Other specific compression tools for screen content

### PU-based Intra block copying with extended search range

On top of the CU-based intra block copying (2N×2N only) utilized in the anchor based on HM-12.1_RExt-5.1 [3] and HEVC Range Extensions Draft 5 [4], in this proposal PU-based intra block copying for 2N×N / N×2N and N×N partitions are also used. The partition and signalling of these PU-based intra block copying modes are the same as those included in HEVC Range Extensions Draft 6 [6] and HM13.0_RExt6.0 [7]. The largest CU size for both CU-based and PU-based intra block copying is set to 16×16, which is the same as the anchor setting. The precision of the block vectors (BV) are integer-pel only, no subpel interpolation is allowed.

In this proposal, the search range for both CU-based and PU-based intra block copying are extended to cover more previous reconstructed areas. The corresponding search range is explained and illustrated as in the following figure.

Figure. 3: The extended search area for PU-based intra block copying

Besides the already reconstructed area in the current CTU, the allowed area for intra block copying compensation also covers the left, top and top right directions. More specifically, the newly defined search area includes two CTUs to the left, two CTUs on top and two CTUs to the right. Altogether, there are 12 CTUs plus the already reconstructed CUs in current CTU that can be used for PU-based intra block copying.

### Line-based (2N×1/1×2N) Intra copying

This proposal introduces a line-based (2N×1/1×2N) intra copying modes to further improve the coding efficiency on screen content. In the line-based (2N×1/1×2N) intra copying modes, an CU is partitioned into 2N×1 row-wise lines or 1×2N column-wise lines, each line has its own intra block vector (BV). For example, for an 8×8 CU, line-based intra copying modes contain 8 row-wise lines or 8 column-wise lines. One flag is used to indicate whether a CU is being coded as line-based (2N×1/1×2N) intra copying modes, and another flag is used to indicate whether it is partitioned with horizontal lines (2N×1) or vertical lines (1×2N).

It was asserted that the line-based block copying modes exhibit special characteristics in their BVs, such that this proposal designs the BV signalling and prediction as the following:

- BV representation for line-based (2N×1/1×2N) intra copying

For horizontal lines of 2N×1, only vertical BV exists (i.e. the BV horizontal component is always zero). Correspondingly, such a vertical BV can only point to a region that is above the current horizontal line. As for the encoding algorithm, the search range is limited to the reconstructed region in the current CTU and above CTU.

For vertical lines of 1×2N, only horizontal BV exists (i.e. the BV vertical component is always zero). Correspondingly, such a horizontal BV can only point to the region that is to the left of the current vertical line. As for the encoding algorithm, the search range is limited to the reconstructed region in the current CTU and left CTU.

Figure 4 below illustrates the BV and search range for the proposed line-based (2N×1/1×2N) intra copying modes.

Figure 4. The 1D BV representation and search range for line-based (2N×1/1×2N) intra copying

As compared to the signalling of MVD in HEVC, the differentially-coded BVs of line-based (2N×1/1×2N) intra copying are signalled with only one component, and without the sign bit. Note that it is completely doable to also signal such constrained BV using exactly the same signalling of 2D-MVD in HEVC [2].

- BV prediction for line-based (2N×1/1×2N) intra copying:

To signal BVs of line-based (2N×1/1×2N) intra copying efficiently, a predictive coding scheme is applied. For BV[i], a BVd[i] is first calculated as BVd[i] = BV[i] - p, where p equals to BV[i-1]. BV[-1] is set to be 0 when i equals to 0, i.e. the first line of the CU. To code BVd[i] without the sign-bit, a non-negative codeword C[i] is mapped from BVd[i] according to a mapping table as depicted in Table 4. The mapped C[i] is coded in the same manner as the absolute value of MVD for i from 0 to 2N-1 sequentially.

Table 4. The mapping from BVd to C, which is then coded as the absolute value of MVD

| BVd[i] | 0 | 1 | -1 | 2 | -2 | … | p | -p | p+1 | p+2 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C[i] | 0 | 1 | 2 | 3 | 4 | … | 2p-1 | 2p | 2p+1 | 2p+2 | … |

### Triplet palette mode

A flag is transmitted at CU level to signal if the palette mode is used. In the palette mode, samples in each CU are represented using major sample values and escape sample values. Major sample values are the most representative sample values in a CU. Escape sample values are the sample values that do not belong to the major sample values.

Major sample values and escape sample values are described by a triplet palette representation method. Based on this representation, a sample of a color component (e.g., Y or G) and its two corresponding samples of different color components (e.g., Cb or B, and Cr or R) in the same position are represented by a color index. The color index can be predicted and corresponds to three sample value indices.

#### Triplet palette representation method

##### Triplet palette representation

In each CU, for each color component, the encoder first calculates the histogram of sample values and chooses several most representative major sample values. The major sample values of each color component are indexed by a palette of the color component. Figure 5 shows an example. There are three component-wise palettes, and each palette records major sample values of a color component. In this example, there are N1, N2, and N3 major sample values for the three color components respectively. Each sample value index of a palette indicates a major sample value unless it is equal to N1, N2, and N3 for the 1st, 2nd, and 3rd color component, respectively, for representing an escape sample value.

Figure 5. An example of component-wise palettes for three color components.

Then, based on the sample values in the three component-wise palettes, the encoder chooses several most representative combinations from each of the three color components. A triplet palette records the combinations and is a color index table of triplets of sample value indices of different color components, as shown in Figure 6. Note that a color index can denote a combination of sample value indices representing not only major sample values but also escape sample values. In this example, there are N major colors. When the color index is equal to N, all the sample values of the three color components belong to escape.

Figure 6. An example of triplet palette (or called color index table).

The N1, N2, N3, N, major sample values of the three component-wise palettes, and the sample value indices of the triplet palette are transmitted to the decoder. At the decoder side, for each sample, the color index of the triplet palette is first decoded. Next, with the decoded color index, the three sample value indices of the three color components are derived from the triplet palette. Finally, with the three derived sample value indices, the three sample values are derived from the component-wise palettes.

In this proposal, the maximum sample value indices of the component-wise palettes (i.e., maximum value of N1, N2, and N3) are set to 15 (i.e., N1max=N2max=N3max=15), and the maximum color index of the triplet palette (i.e., maximum value of N) is set to 63 (i.e., Nmax=63).

##### Coding of component-wise palettes and triplet palette

The component-wise palettes and the triplet palette are first predicted and then coded. During decoding, the decoder maintains three component-wise palette predictors of (N1max-1), (N2max-1), and (N3max-1) sample values from previous CUs for predicting the three component-wise palettes of the current CU respectively. The decoder also maintains a triplet palette predictor of (Nmax-1) colors from previous CUs for predicting the triplet palette of the current CU. The N1, N2, N3, and N values of the last palette coded CU are also stored as N1last, N2last, N3last, and Nlast, respectively.

There are two palette prediction modes: palette copy mode and palette modification mode. In this proposal, the three component-wise palettes share one selection between the two prediction modes, and the triplet palette has its own selection between the two prediction modes.

###### Palette copy mode

Basically, the palette copy mode is to copy the palette predictor. Figure 7 shows an example of palette copy on. component-1. The palette copies for other component-wise palettes and the triplet palette can be derived by analogy. In this example, the last component-1 palette has N1last (=10) major sample values, and the component-1 palette predictor always has N1max (=15) sample value predictors regardless of the value of N1last. The palette size (here defined as the number of major sample values for a component-wise palette or the number of major colors for a triplet palette) of the current CU, N1, is set to N1last, the first N1last sample value predictors are copied as the first N1last major sample values, and the sample value index equal to N1last represents an escape sample value.

Figure 7. An example of palette copy mode for component-1.

###### Palette modification mode

In the palette modification mode, information of palette modifications is signaled to indicate how to derive a palette from the palette predictor. Figure 8 shows an example of palette modification on component-1. The palette modifications for other component-wise palettes and the triplet palette can be derived by analogy. In this example, the palette predictor is first created as the palette copy mode. Next, the new palette size, N1 (=3), is signaled to indicate the number of the major sample values for the current CU. Then, N1 flags are signaled to indicate if the major sample value is the same as the sample value predictor, and the corresponding new major sample values (e.g., P’[0] and P’[2]) are further signaled. Finally, the N1 major sample values for the current CU are copied from either P[] or P’[] according to the to-be-modified-flags, and the sample value index equal to N1 represents an escape sample value.

Figure 8. An example of palette modification mode for component-1.

###### Palette predictor update

After palette coding for the current CU, each of the component-wise palettes and the triplet palette are updated. Figure 9 shows an example of palette predictor update on component-1. The palette predictor updates for other component-wise palettes and the triplet palette can be derived by analogy. In this example, the updated last palette size, N1last, is set to the current palette size, N1 (=3), and the first N1 sample value predictors are copied from the N1 sample value of the current palette. Note that regardless of the updated value ofN1last, the component-1 palette predictor always has N1max (=15) sample value predictors. We call this property as full palette propagation, which can enhance the palette prediction efficiency across different CU sizes.

Figure 9. An example of palette predictor update for component-1.

#### Predictive coding of color index map

For a palette coded CU, samples are converted into color indices and represented by the triplet palette representation described in Section Error: Reference source not found. Basically speaking, the color indices of all samples in the CU are coded line by line. For each sample line, an “index line mode” is signaled to indicate the coding mode. The index line mode includes vertical mode, horizontal mode, and normal mode:

- Vertical index line mode: The color indices of the current line are copied from the above line.

- Horizontal index line mode: One color index is signaled and the color indices of all the samples in the current line are set to the signaled color index.

- Normal index line mode: The color indices of the line are signaled sample by sample. For each sample, a color prediction index is signaled to specify how to predict the color index, and a color index of the sample is transmitted directly when the color prediction index indicates no prediction.

- In this proposal, escape sample values can only occur when the normal line mode is selected. After the color indices of each line with the normal index line mode are coded, escape sample values, if exist, are fixed-length coded, where the codeword length of each escape sample value depends on the sample bit depth and the quantization parameter.

##### Coding of index line mode

The last index line mode is used as a predictor for the current index line mode as follows.

If the last index line mode is vertical or normal, one flag is signaled to indicate whether the current index line mode is the same as the last index line mode. If the current index line mode is not the same as the last index line mode, another flag is signaled to select between vertical and normal.

Else (if the last index line mode is horizontal), one flag is signaled to indicate whether the current index line mode is normal. If the current index line mode is not normal, another flag is signaled to select between vertical and horizontal.

For the first line of the CU, its last index line mode is set to vertical.

##### Coding of color index (only for samples of the normal index line mode)

When the normal mode is selected as the current index line mode, the color index of each sample is signaled one by one using color indices of coded samples as color index predictors. For each sample, a color index candidate list is constructed and a color prediction index is signaled to specify which color predictor is used. The related details are specified as follows.

###### Four-neighbor color index prediction

Color indices of four neighboring samples are considered in the color index candidate list. The four neighboring samples are the left, the above, the above-left, and the above-right samples.

###### Transition copy prediction

A transition copy (TC) candidate is considered in the color index candidate list. In each CU, at the first sample, the TC table is reset. The TC table is maintained as the samples in the CU are coded. As shown in Table 5, for each to-be-coded non-first-column sample, the input of the TC table is called a pilot color index and is set to the color index of the left sample, and the outputs of the TC table are two TC color indices, where the two TC color indices are likely to occur right after an occurrence of the pilot color index according to the causal area of the CU. As for each to-be-coded first-column sample, the input of the TC table is unavailable, and the two outputs of the TC table are set to 0. The TC table is updated after each non-first-column sample in the CU is coded as follows. If the current color index is not the same as any of the pilot color index and the two TC color indices derived from the TC table using the pilot color index as the input, the 2nd TC color index is replaced with the 1st TC color index, and the 1st TC color index is replaced with the current color index.

Table 5. Illustration of a transition copy table.

An example of the TC candidate is shown in Figure 10. The current sample, C, has a pilot color index equal to 1. The shaded squares with numbers labeled are coded samples. From the causal area of coded samples, it can be found that when a color index equal to 1 is coded, the next color index is likely to be 2 or 3. These transition patterns (i.e., 1=>3 and 1=>2) can be recorded in the TC table to enhance color index prediction efficiency.

Figure 10. An example of deriving the 1st TC color index and the 2nd TC color index. Each square denotes a sample, C denotes the current sample, and the numbers in the squares denote the coded color indices

###### Color index candidate list construction with a fixed candidate list size

An initial color index candidate list that consists of the color indices of the above, left, above-left, and above-right samples, the two TC color indices, and four default color index values equal to 0-3 is first created. The order of color indices in the initial candidate list is {above, left, TC1, TC2, above-left, above-right, 0, 1, 2, 3}. Next, a pruning process is applied to remove redundancy in the candidate list. The first four non-equal color index candidates and a new color index candidate form the final candidate list. The new color index candidate indicates that the current color index is not predictive coded and the color index value is directly signaled. The final candidate list size is fixed to 5. The codewords of the selected candidate index are shown in Table 6.

Table 6.  The codeword design of the selected candidate index

| Selected Candidate Index | Codeword |
| --- | --- |
| 0: Color index candidate 0 | 1 |
| 1: Color index candidate 1 | 01 |
| 2: Color index candidate 2 | 001 |
| 3: Color index candidate 0 | 0000 |
| 4: New color index candidate | 0001 |

###### Fixed-length coding of color index with skipped bins

When the new color index candidate is selected (i.e., the candidate index is equal to 4 in Table 6), a color index is first fixed-length binarized, where the length is equal to ceil(log2N). Then, the binarization codeword is coded from the most significant bin toward the least significant bin, and each bin is assigned a single context. The bins may lead to a value greater than the maximum codeword value, N, are inferred to be equal to 0 and are not coded.

Note that the same scheme is also applied to signaling sample value indices in the triplet palette (Figure 6).

##### Index map Rotation

###### For a palette coded CU, a color index rotation flag is signaled. If the flag is equal to 1, the sample position axes for the color index map will be swapped. In other words, color_index[ x0 ][ y0 ] originally means x position is x0 and y position is y0. If the flag is equal to 1, it means y position is x0 and x position is y0, and the color index becomes color_index[ y0 ][ x0 ].

### Single color mode

This proposal also introduces a coding mode termed as “Single Color Mode” to efficiently code the smooth area within a picture. The concept of Single Color Mode is to simply reconstruct the current CU as a smooth area with single sample value.

To reconstruct a CU coded as Single Color Mode, a sample color candidate list is first constructed by inserting the sample candidates derived from the neighboring samples of current CU in a predefined order into the candidate list. As shown in Figure 11 (each square represents a sample), the spatial neighboring samples are those five reconstructed samples around the current CU and are inserted into the candidate list in the order: (Am, Bn, B0, A0 and D). To achieve higher coding efficiency, a redundancy pruning process is applied to the construction of sample candidate list. A sample candidate will not be inserted into the candidate list if it is identical to any of the previous derived sample candidates. In the proposed scheme, the size of the sample candidate list is fixed to 2. If empty entry still exists in the candidate list after the derivation of candidates from the spatial neighboring samples, a default candidate with value equal to 1<<(g_bitDepth-1) will be used to fill in those empty entries. After the sample candidate list is constructed, a sample candidate index which is signaled in the bitstream is used to indicate which sample candidate is used to fill the current CU.

Figure 11. The spatial sample candidates used to construct the sample candidate list.

## Quantization

This proposal does not modify the quantization aspect as compared to anchor HM-12.1_RExt-5.1.

## Motion/mode/parameter coding

### Merge/skip

This proposal does not modify the merge/skip aspect as compared to anchor HM-12.1_RExt-5.1.

### Motion vector coding

This proposal does not modify the motion vector coding aspect as compared to anchor HM-12.1_RExt-5.1.

### Intra prediction mode coding

This proposal does not modify the intra prediction mode coding aspect as compared to anchor HM-12.1_RExt-5.1.

## Entropy coding of residual / transform coefficients

This proposal does not modify the entry coding if residual / transform coefficients aspect as compared to anchor HM-12.1_RExt-5.1.

# Syntax and semantics description

## Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| intra_boundary_filter_disable_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| intra_screen_coding_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, intra_boundary_filter_disable_flag,  intra_screen_coding_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

intra_boundary_filter_disable_flag equal to 1 specifies disabling the boundary filtering process when predModeIntra is INTRA_DC, 26 (vertical) or 10 (horizontal). intra_boundary_filter_disable_flag equal to 0 specifies that the boundary filtering process when predModeIntra is INTRA_DC, 26 (vertical) or 10 (horizontal) is not diabled. When not present, the value of intra_boundary_filter_disable_flag is inferred to be equal to 0.

intra_screen_coding_enabled_flag equal to 1 specifies that intra screen coding may be used for intra blocks. intra_screen_coding_enabled_flag equal to 0 specifies that intra screen coding is not applied. When not present, the value of intra_screen_coding_enabled_flag is inferred to be equal to 0.

The following decoding process is changed according to this intra_boundary_filter_disable_flag:

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

- Depending on the value of the colour component index cIdx, the following applies:

- If intra_boundary_filter_disable_flag is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-1)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-2)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-3)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-4)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-5)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If intra_boundary_filter_disable_flag is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

## Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_extension_present_flag | u(1) |
| if( pps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| pps_extension_flag[ i ] | u(1) |
| pps_extension_7bits | u(7) |
| } |  |
| if( pps_extension_flag[ 0 ] ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| cross_component_prediction_enabled_flag | u(1) |
| if ( cross_component_prediction_enabled_flag ) |  |
| scale_parameter_coding_mode | u(1) |
| line_based_intra_copying_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| log2_sao_offset_scale_luma | ue(v) |
| log2_sao_offset_scale_chroma | ue(v) |
| } |  |
| if( pps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

cross_component_prediction_enabled_flag equal to 1 specifies that log2_res_scale_abs_plus1 and res_scale_sign_flag or log2_res_scale_pred_4minus and log2_res_scale_neg may be present in the transform unit syntax for pictures referring to the PPS. cross_component_prediction_enabled_flag equal to 0 specifies that log2_res_scale_abs_plus1, res_scale_sign_flag, log2_res_scale_pred_4minus and log2_res_scale_neg are not present for pictures referring to the PPS. When not present, the value of luma_chroma_prediction_enabled_flag is inferred to be equal to 0. When ChromaArrayType is not equal to 3, it is a requirement of bitstream conformance that the value of luma_chroma_prediction_enabled_flag shall be equal to 0.

scale_parameter_coding_mode When not present, the value of scale_parameter_coding_mode is inferred to be equal to 0. If scale_parameter_coding_mode equal to 0, log2_res_scale_pred_4minus and log2_res_scale_neg are not present in the transform unit syntax for pictures referring to the SPS; If scale_parameter_coding_mode equal to 1, log2_res_scale_abs_plus1 and res_scale_sign_flag are not present for pictures referring to the PPS.

line_based_intra_copying_enabled_flag equal to 1 specifies that line-based block copying may be invoked in the decoding process for intra prediction. line_based_intra_copying_enabled_flag equal to 0 specifies that line-based block copying is not applied. When not present, the value of line_based_intra_copying_enabled_flag is inferred to be equal to 0.

## Slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| slice_single_sample_mode_enable_flag | ae(v) |
| … |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

slice_ single_sample_mode_enable_flag specifies whether single sample mode can be used for intra mode coding. If slice_single_sample_mode_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no single sample mode is used in decoding of the current picture. Otherwise (slice_single_sample_mode_enabled_flag is equal to 1), single sample mode may be used in decoding of the current picture. When not present, the value of slice_single_sample_mode_enabled_flag is inferred to be equal to 0.

## Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_single_sample_mode_enabled_flag ) |  |
| single_sample_flag[ x0 ][ y0 ] | ae(v) |
| if( single_sample_flag[ x0 ][ y0 ]) { |  |
| single_sample_idx[ x0 ][ y0 ] | ae(v) |
| } else { // if( ! single_sample_flag ) |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && intra_screen_coding_enabled_flag ) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_scm_flag[ x0 ][ y0 ] ) { |  |
| scm_info_coding(x0, y0, nCbS, nCbS) |  |
| } else { // if(!intra_scm_flag) |  |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else if ( line_based_intra_copying_enabled_flag && PartMode = = PART_2Nx2N ) { |  |
| line_based_intra_copying_flag[ x0 ][ y0 ] | ae(v) |
| if ( line_based_intra_copying_flag[ x0 ][ y0 ] ) { |  |
| line_based_intra_copying_dir[ x0 ][ y0 ] | ae(v) |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| libc_bvd_coding( i ) |  |
| } |  |
| } else { // INTRA syntax |  |
| … |  |
| } |  |
| } else { // ! MODE_INTRA |  |
| … |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && ( intra_bc_flag[ x0 ][ y0 ] ¦¦ line_based_intra_copying_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| … |  |
| } |  |
| } |  |
| } // if(!intra_scm_flag) |  |
| } // if( ! single_sample_flag ) |  |
| } |  |
| } |  |

single_sample_flag [ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, no more syntax elements except the single sample candidate index single_sample_idx[ x0 ][ y0 ] are parsed after single_sample_flag[ x0 ][ y0 ]. single_sample_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not single sample coded. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When single_sample_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

single_sample_idx[ x0 ][ y0 ] specifies the single sample candidate index of the single sample candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When single_sample_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in screen coding mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in screen coding mode. When intra_scm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_based_intra_copying_flag [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded by line-based intra copying mode. line_based_intra_copying_flag [ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded by line-based intra copying mode. When line_based_intra_copying_flag [ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_based_intra_copying_dir [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded by line-based intra copying mode with vertical-line partitions. The semantics of corresponding syntax libc_bvd_coding(i) represent the horizontal component of intra block vector. line_based_intra_copying_dir [ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded by line-based intra copying mode with horizontal-line partitions. The semantics of corresponding syntax libc_bvd_coding(i) represent the verical component of intra block vector.

## Extended search range for Intra block-copying

The syntax and semantics for PU level IntraBC operations are the same as those integrated into HEVC Range Extensions Draft 6 [6]. No new changes are presented except for the follow in 8.4.4 (Derivation process for block vector components in intra block copying prediction mode):

It is a requirement of bitstream conformance that all of the the following conditions are true:

–	The value of bvIntra[ xPb ][ yPb ][ 0 ] shall be greater than or equal to – ( xPb % CtbSizeY + CtbSizeY*2 ).

–	The value of bvIntra[ xPb ][ yPb ][ 0 ] shall be smaller than or equal to  ( CtbSizeY–(xPb % CtbSizeY) – nPbSw + CtbSizeY*2 ).

–	The value of bvIntra[ xPb ][ yPb ][ 1 ] shall be greater than or equal to – ( yPb % CtbSizeY + CtbSizeY*2).

## Line-based intra copying syntax

Line-based intra copying block vector difference syntax

| libc_bvd_coding( lineIdx ) { | Descriptor |
| --- | --- |
| coded_ libc_bvd _greater0_flag[lineIdx] | ae(v) |
| if( abs_ libc_bvd _greater0_flag ) |  |
| coded_ libc_bvd _greater1_flag[lineIdx] | ae(v) |
| if( abs_ libc_bvd _greater0_flag ) { |  |
| if( abs_ libc_bvd _greater1_flag ) |  |
| coded_ libc_bvd _minus2[lineIdx] | ae(v) |
| } |  |
| } |  |

coded_ libc_bvd_greater0_flag[ lineIdx] specifies whether the coded value of a line-based intra block vector difference BvdC[lineIdx] is greater than 0.

coded_ libc_bvd _greater1_flag[ lineIdx] specifies whether the coded value of a line-based intra block vector difference BvdC[lineIdx] is greater than 1.

When coded_ libc_bvd _greater1_flag[ lineIdx] is not present, it is inferred to be equal to 0.

coded_ libc_bvd _minus2[ lineIdx] plus 2 specifies the coded value of a motion vector component difference BvdC[lineIdx].

When coded_ libc_bvd _minus2 [ lineIdx] is not present, it is inferred to be equal to −1.

If lineIdx is equal to 0, Bvd[lineIdx] is set equal to BvdC[lineIdx].

Otherwise, Bvd[lineIdx] is set equal to ( BvdC[lineIdx] > 2 * Bv[lineIdx-1] ) ? ( BvdC[lineIdx]- Bv[lineIdx-1] ) :    (( ( BvdC[lineIdx]+1 )>>1) * (2*(BvdC[lineIdx]&1)-1)).

The variable Bv[lineIdx] is set equal to Bv[lineIdx-1] + Bvd[lineIdx] if lineIdx is greater than 0. Otherwise, Bv[lineIdx] is set equal to Bvd[lineIdx].

If line_based_intra_copying_dir is equal to 0, the reconstructed Bv[lineIdx] corresponds to the vertical component of the intra BV, while the horizontal component of the intra BV is set to 0.

Otherwise, if line_based_intra_copying_dir is equal to 1, the reconstructed Bv[lineIdx] corresponds to the horizontal component of the intra BV, while the vertical component of the intra BV is set to 0.

## Palette syntax (scm_info_coding)

| scm_info_coding(x0, y0, CbWidth, CbHeight) { |  |
| --- | --- |
| triplet_index_rotation_flag[ x0 ][ y0 ] | ae(v) |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| If(ChromaArrayType != 0) { |  |
| triplet_share_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if(palette_share_flag[ x0 ][ y0 ]){ |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ]= major_color_num_pred[ cIdx ] |  |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ]= major_color_value_pred[ cIdx ][ n ] |  |
| } |  |
| } |  |
| } else { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ]= major_color_value_pred[ cIdx ][ n ] |  |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| major_color_value_pred[ cIdx ][ n ]= major_color_value[ x0 ][ y0 ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 0) { |  |
| if(triplet_share_flag[ x0 ][ y0 ]){ |  |
| triplet_color_num[ x0 ][ y0 ]= triplet_color_num_pred |  |
| for( n = 0; n < triplet_color_num[ x0 ][ y0 ]; n++ ) { |  |
| for(cIdx = ( ChromaArrayType == 3 )?0:1; cIdx < 3; cIdx++) { |  |
| triplet_color[ x0 ][ y0 ][ cIdx ][ n ]= triplet_color_pred[ cIdx ][ n ] |  |
| } |  |
| } |  |
| } else { |  |
| triplet_color_num[ x0 ][ y0 ] | ae(v) |
| for( n = 0; n < triplet_color_num[ x0 ][ y0 ]; n++ ) { |  |
| triplet_color_pred_flag[ x0 ][ y0 ][ n ] | ae(v) |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++){ |  |
| if(triplet_color_pred_flag[ x0 ][ y0 ][ n ]) { |  |
| triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ]= triplet_color_value_pred[ cIdx ][ n ] |  |
| } else { |  |
| color_index[ x0 ][ y0 ][ cIdx ] | ae(v) |
| triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] = color_index[ x0 ][ y0 ][ cIdx] |  |
| triplet_color_value_pred[ cIdx ][ n ]= triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 3) { |  |
| cIdx = 0; |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ 0 ] | ae(v) |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] = color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i-1 ][ j ] |  |
| } |  |
| } |  |
| } else { //NOR_MODE |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] == NO_PREDICTION) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(color_index[ x0 ][ y0 ][ cIdx ][i][j] == major_color_num[ x0 ][ y0 ][ cIdx ]) { |  |
| quant_escape_value[ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 0) { |  |
| for(cIdx = ( ChromaArrayType == 3 )?0:1; cIdx < 1; cIdx++) { |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx][ i ][ 0 ] | ae(v) |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx][ i ][ j ] = triplet_index[ x0 ][ y0 ][ cIdx][ i-1 ][ j ] |  |
| } |  |
| } else { //NOR_MODE |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] == NO_PREDICTION) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:2; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][j]= triplet_color_value[ x0 ][ y0 ][ cIdx + cSubIdx] [ triplet_index[ x0 ][ y0 ][ cIdx+ cSubIdx ][ i ][ j ] ] |  |
| if(color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][j] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ]) { |  |
| quant_escape_value[ cIdx + cSubIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

triplet_index_rotation_flag[ x0 ][ y0 ] equal to 1 specifies that, in triplet index coding, the horizontal and vertical axis of sample position is switched.

palette_share_flag[ x0 ][ y0 ] equal to 1 specifies that major_color_num and major_color_value is direved from major_color_num_pred and major_color_value_pred directly.

triplet_share_flag[ x0 ][ y0 ] equal to 1 specifies that triplet_color_num and triplet_color_value is direved from triplet _color_num_pred and triplet _color_value_pred directly.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the predictor. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_num[ x0 ][ y0 ] specifies the number of triplet colors. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_pred_flag[ x0 ][ y0 ][ n ] equal to 1 specifies n-th triplet color value could be predicted from the predictor. triplet_color_pred_flag[ x0 ][ y0 ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th triplet color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_mode[ x0 ][ y0 ][ i ] equal to HOR_MODE specifies that the line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][i] equal to VER_MODE specifies that the line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ i ] equal to NOR_MODE specifies that the line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current sample in position ( i, j ). index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to NEW_INDEX specifies that index value of current sample cannot be derived from the coded sample.

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape sample value for color component cIdx in at the location ( i, j ). The variable bitEscapeQuant is derived from a pre-defined look-up table indexed for each QP value

- escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] is set equal to

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ]<<(bitDepth – bitEscapeQuant)

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the sample at the location( i, j ) for color component cIdx.

triplet_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the triplet index value of the sample at the location( i, j ) for color component cIdx.

## Cross-component prediction syntax

| cross_comp_pred( x0, y0, c ) { | Descriptor |
| --- | --- |
| if(scale_parameter_coding_mode = = 0) { |  |
| log2_res_scale_abs_plus1[ c ] | ae(v) |
| if( log2_res_scale_abs_plus1[ c ] != 0 ) |  |
| res_scale_sign_flag[ c ] | ae(v) |
| } |  |
| else { |  |
| log2_res_scale_pred_4minus[ c ] | ae(v) |
| if(log2_res_scale_4minus [ c ] == 5) |  |
| log2_res_scale_neg [ c ] | ae(v) |
| } |  |
| } |  |

log2_res_scale_pred_4minus [ c ] minus 4 specifies the negative value of base 2 logarithm of the magnitude of the scaling factor ResScaleVal used in luma-chroma residual prediction unless log2_res_scale_pred_4minus [ c ] is equal to 0 or 5. When not present, log2_res_scale_abs_plus1 is inferred equal to 0.

- If log2_res_scale_pred_4minus [ c ] is equal to 0, the corresponding ResScaleVal has a 0 value;

- Otherwise if log2_res_scale_pred_4minus [ c ] is equal to 5, the corresponding ResScaleVal has a negative value;

- Otherwise, the corresponding ResScaleVal has a positive value.

log2_res_scale_neg [ c ] multiplies -1 specifies the value of base 2 logarithm of the magnitude of the scaling factor ResScaleVal used in luma-chroma residual prediction when log2_res_scale_pred_4minus [ c ] is equal to 5.

The variable ResScaleVal[ cIdx ][ x0 ][ y0 ] is derived as follows:

if scale_parameter_coding_mode is equal to 0

If log2_res_scale_abs_plus1[ cIdx − 1 ] is equal to 0, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = 0		(7-6)

Otherwise (log2_res_scale_abs_plus1[ cIdx − 1 ] is not equal to 0), the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = ( 1  <<  ( log2_res_scale_abs_plus1[ cIdx − 1 ] − 1 ) ) *	(7-7)
			( 1 − 2 * res_scale_sign_flag[ cIdx − 1 ] )

Otherwise (scale_parameter_coding_mode is not equal to 0),

If log2_res_scale_pred_4minus[ cIdx − 1 ] is equal to 0, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = 0		(7-8)

Otherwise, if log2_res_scale_4minus[ cIdx − 1 ] is not equal to 5, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = ( 1  <<  (4 − log2_res_scale_abs_4minus[ cIdx − 1 ]) ) 	(7-9)
Otherwise,

ResScaleVal[ cIdx ][ x0 ][ y0 ] = − ( 1  <<  (log2_res_scale_neg[ cIdx − 1 ]) ) 	(7-10)

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process (in Section 9.3.2.2)

| … | Syntax element | ctxTable | InitType=0 | InitType=1 | InitType=2 |
| --- | --- | --- | --- | --- | --- |
| luma_chroma_pred( ) | log2_res_scale_abs_plus1[ ] | Error: Reference source not found | 0..7 | 8..15 | 16..23 |
|  | res_scale_sign_flag[ ] | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | Log2_res_scale_pred_4minus[ ] | Error: Reference source not found | 0..8 | 9..17 | 18..26 |

Table 9-xx – Values of initValue for ctxIdx of log2_res_scale_pred_4minus[ ][ ][ ] (in Section 9.3.2.2)

| Initialization variable | ctxIdx of log2_res_scale_abs_plus1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations (in Section 9.3.3.1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … |  |  |  | … |  | … |  |  |  |  |
| inter_component_resi_pred( ) | log2_res_scale_pred_4minus |  |  |  | TR |  | cMax =5, cRiceParam = 0 |  |  |  |  |
|  | log2_res_scale_neg |  |  |  | FL |  | cMax =3 |  |  |  |  |
| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins (in Section 9.3.4.2.1) |  |  |  |  |  |  |  |  |  |  |  |
| … |  | … | … | … |  | … |  | … | … |  |  |
| log2_res_scale_abs_plus1[ c ] |  | 4 * c + 0 | 4 * c + 1 | 4 * c + 2 |  | 4 * c + 3 |  | na | na |  |  |
| res_scale_sign_flag[ c ] |  | c | na | na |  | na |  | na | na |  |  |
| log2_res_scale_pred_4minus |  | 5 * c + 0 | 5 * c + 1 | 5 * c + 2 |  | 5 * c + 3 |  | 5 * c + 4 | na |  |  |
| log2_res_scale_neg |  | bypass | bypass | na |  | na |  | na | na |  |  |

# Compression performance discussion

## Lossy test condition

## Mathematically lossless test condition

# Complexity analysis

## Encoding time and measurement methodology

The encoding time of the anchor is the run time of encoding CfP sequences by using HM-12.1+RExt-5.1. The encoding time of the proposed SCC video encoder is the run time of encoding CfP sequences by using the proposed encoder software.

The encoding time starts after parsing the configuration file at the beginning and ends after recording coded bits for the last picture. For the proposed encoder software, the reconstructed RGBs or YUVs are outputted. Also for the anchor, the reconstructed RGBs or YUVs are outputted.

The run times of the proposed encoder software are shown below.

The encoding time of the proposed encoder software compared with HM-12.1+RExt-5.1 anchor

|  | Lossy AI | Lossy AI | Lossy AI | Lossless AI | Lossless RA | Lossless LD |
| --- | --- | --- | --- | --- | --- | --- |
| Encoding Time | 314% | 151% | 141% | 472% | 175% | 163% |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

The decoding time of the anchor is the run time of decoding anchor bitstreams by using HM-12.1+RExt-5.1. The decoding time of the proposed SCC video decoder is the run time of decoding the generated bitstreams by using the proposed decoder software, where the generated bitstreams are encoded by the proposed encoder.

The decoding time starts after parsing bitstream at the beginning and ends after recoding the last reconstructed RGBs or YUVs output before releasing all memories. For the proposed decoder software, the reconstructed RGBs or YUVs are also outputted.

The run times of the proposed decoder software are shown in Table below.

The decoding time of the proposed decoder software compared with HM-12.1+RExt-5.1 anchor

|  | Lossy AI | Lossy AI | Lossy AI | Lossless AI | Lossless RA | Lossless LD |
| --- | --- | --- | --- | --- | --- | --- |
| Decoding Time | 81% | 99% | 101% | 87% | 93% | 96% |

## Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2

Three kinds of computing platforms are used. The computing platforms are summarized as below.

Summary of computing platform

|  | CPU | Multi-core | Memory | OS | Storage |
| --- | --- | --- | --- | --- | --- |
| Platform 1 | Intel Xeon X5570 (4 cores * 2 , 2.93GHz) | No | 8GB/12GB/24GB | RHEL 5.5/RHEL5.8 64-bits | Local HDD |
| Platform 2 | Intel Xeon X5670 (6 cores * 2 , 2.93GHz) | No | 24GB | RHEL 5.5/RHEL5.8 64-bits | Local HDD |
| Platform 3 | Intel Xeon X5675 ( 6 cores * 2 , 3.06GHz) | No | 48GB | RHEL 5.5/RHEL5.8 64-bits | Local HDD |

## Expected memory usage of encoder

As compared to the anchor HEVC Range Extensions Draft 5 [4], there are three additional memory usage components:

- Additional lines of CTU samples due to intra block-copying with extended search range: As shown Figure 3, additional CTU samples, before in-loop filtering, have to be stored for intra block-copying. Since the decoding proceeds in a raster scanning order, the line-buffer size to facilitate the range illustrated in Figure 3 is (2 CTU lines + 2 CTU + reconstructed CUs in the current CTU) of before-in-loop-filtering samples.

Note that, in a practical implementation, the 2 CTU lines can be stored off-chip (e.g., in DRAM), and the search area of the intra block-copying can be stored on-chip (e.g., in SRAM, registers, or cache memory) to significantly reduce the cost in comparison with using on-chip memory to store the 2 CTU lines and etc. The additional off-chip access bandwidth (including storing non-blocked samples from on-chip to off-chip and loading search area samples from off-chip to on-chip) is usually affordable because two adjacent CTUs have significant overlap in search areas, which means only a small portion of the search area samples needs to be loaded for each non-first-column CTU.

Note that, depending on the actual implementation, this part might not be an “additional memory”. For example, in the HEVC and RExt reference software, the in-loop filtering process is only performed after the entire picture is encoded and reconstructed. Thus when encoding the current CTU, all causal CTUs are before-in-loop filtering, and can directly be used for intra block-copying.

- Storage of major color palettes and triplet index table: To enable the palette copy mode, the last palettes and triplet index table has to be stored. The size of major color palette is 15 × 8 or 10 bits for each component, and the size of the triplet index table is 63 × 3 × 4bits.

- Storage of the transition-copy table (TCT): To enable transition-copy prediction, the transition-copy table has to be stored. The size of transition-copy table is 64 × 6bits.

## Expected memory usage of decoder

As compared to the anchor HEVC Range Extensions Draft 5 [4], there are three additional memory usage components:

- Additional lines of CTU samples due to intra block-copying with extended search range: As shown Figure 3, additional CTU samples, before in-loop filtering, have to be stored for intra block-copying. Since the decoding proceeds in a raster scanning order, the line-buffer size to facilitate the range illustrated in Figure 3 is (2 CTU lines + 2 CTU + reconstructed CUs in the current CTU) of before-in-loop-filtering samples.

Note that, in a practical implementation, the 2 CTU lines + 2 CTU + reconstructed CUs in the current CTU can be stored in off-chip memory (e.g. in DRAM) instead of on-chip memory to reduce the cost. The additional bandwidth (including storing non-blocked samples from on-chip to off-chip and loading necessary samples from off-chip to on-chip for intra compensation) is usually affordable (much smaller than motion compensation due to no fractional sample interpolation).

- Storage of major color palettes and triplet index table: To enable the palette copy mode, the last palettes and triplet index table has to be stored. The size of major color palette is 15 × 8 or 10 bits for each component, and the size of the triplet index table is 63 × 3 × 4bits.

- Storage of the transition-copy table (TCT): To enable transition-copy prediction, the transition-copy table has to be stored. The size of transition-copy table is 64 × 6bits.

## Complexity characteristics of encoder motion estimation and partitioning selection

As compared to the anchor HEVC Range Extensions Draft 5 [4], additional motion estimation is performed for intra block copying with extended search range. Additional area of 11 CTUs is evaluated with integer-pel motion estimation. Fast search algorithm is used in this proposal. As for the partitioning, it uses the same PU-based intra block copying partitions as in HEVC Range Extensions Draft 6 [6]: 2N×2N, 2N×N, N×2N, and N×N.

As for the line-based (2N×1/1×2N) intra copying, additional motion estimation is performed in the area of left CTU, above CTU, and already reconstructed CUs in the current CTU. The motion estimation is on the integer-pel only, with fixed partitioning of 2N vertical or horizontal lines.

Both the aforementioned motion estimation processes increase the encoding time.

## Complexity characteristics of decoder motion compensation

The complexity characteristics of the decoder motion compensation and PU-based intra block-copying is the same as those in HEVC Range Extensions Draft 6 [6]. As for the line-based (2N×1/1×2N) intra copying, line-by-line compensation and reconstruction is performed.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection

This proposal does not modify the complexity characteristics of the encoder intra-frame prediction type and partitioning selection.

## Complexity characteristics of decoder intra-frame prediction operation

As compared to the anchor HEVC Range Extensions Draft 5 [4], this proposal simplifies the decoder complexity of intra-frame prediction operation since the boundary smoothing operations are disabled.

## Complexity characteristics of encoder inter-component prediction tools

## As for the cross-component residual prediction, this proposal only modifies the entropy coding of the alpha parameter.

## Complexity characteristics of decoder inter-component prediction tools

As for the cross-component residual prediction, this proposal only modifies the entropy coding of the alpha parameter.

## Complexity characteristics of transforms specific in screen content coding

In this proposal, the palette coding mode does not use transforms and residual coding. This particular aspect will reduce decoding time as compared to performing reconstruction of the residue.

## Complexity characteristics of encoder related to other tools specific in screen content coding

Encoder complexity characteristics of PU-based intra block copying and line-based intra copying have been discussed in Section 5.6.

For palette coding mode, the encoder determines the major sample values in each component independently, and stored the major sample values in the palette. Then, representative combinations of the three components are selected. The combinations are stored in the form of triplet sample value indices. The major sample values and triplet sample value indices are sorted. Encoder evaluates between using the last palette (copy mode) and using a new palette (modification mode). Once the current CU is converted from sample values to palette index map, the encoder evaluates among the three index line modes. There is no residual coding in palette coding mode, thus the quantization, transform, and RQT processes are all skipped for palette-coded CUs.

It is asserted that the most significant increase in the encoding runtime comes from the PU-based intra block-copying together with extended search range.

## Complexity characteristics of decoder related to other tools specific in screen content coding

Decoder complexity characteristics of PU-based intra block-copying and line-based intra copying have been discussed in Section 5.7.

For palette coding mode, the decoder constructs the major color palette using the previous palette / triplet palette index or parsing the new sample values. The index map is constructed by parsing the corresponding the syntax. The sample values are simply converted from the index map. There is no residual coding, thus the de-quantization and inverse-transform process are all skipped.

## Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding

## The entropy encoding method used in this proposal is the same as that is used in the anchor HEVC Range Extensions Draft 5 [4], i.e., context-based adaptive binary arithmetic coding (CABAC). The complexity characteristics of encoder entropy coding are similar to those of HEVC Range Extensions Draft 5 [4].

## Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding

The entropy decoding method used in this proposal is the same as that is used in the anchor HEVC Range Extensions Draft 5 [4], i.e., context-based adaptive binary arithmetic coding (CABAC). The complexity characteristics of encoder entropy coding are similar to those of HEVC Range Extensions Draft 5 [4].

## Degree of capability for encoder parallel processing

## The degree of capability for encoder parallel processing is similar to that in the anchor HEVC Range Extensions Draft 5 [4].

## Degree of capability for decoder parallel processing

The degree of capability for decoder parallel processing is similar to that in the anchor HEVC Range Extensions Draft 5 [4].

# Software implementation description

The proposed coding tools were implemented using standard C++ programming language. HM-12.1_RExt-5.1 was used as the reference software for implementing single layer coding modules. The software is modular and suitable for further tool experiments.

# Highlighted aspects discussion

All important techniques of this proposal have been described in above sections.

# Closing remarks

This contribution presents MediaTek’s response to the Joint Call for Proposals for Coding of Screen Content issued by ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG). A number of coding tools are proposed. These include intra block copying (IntraBC) with extended search range, triplet palette mode, line-based (2N×1/1×2N) intra copying, modified coding of inter-component residual prediction, single color mode. The proposed screen content video codec reportedly achieves BD-rate savings summarized as the following:

For RGB sequences, G-component, lossy condition:

- text & graphics with motion, 1080p	: (AI) 46.0%,	(RA) 31.4%,	(LB) 26.0%;

- text & graphics with motion,720p 	: (AI) 28.6%,	(RA) 23.7%, 	(LB) 20.3%;

- mixed content, 1440p           	: (AI) 20.2%,	(RA) 14.1%, 	(LB) 13.0%;

- mixed content, 1080p           	: (AI) 18.8%,	(RA) 13.1%, 	(LB) 8.7%;

- animation, 720p               	: (AI) 1.7%, 	(RA) 1.3%, 	(LB) 1.1%.

For YUV sequences, Y-component, lossy condition:

- text & graphics with motion, 1080p	: (AI) 44.1%,	(RA) 27.4%,	(LB) 21.2%;

- text & graphics with motion,720p 	: (AI) 25.1%,	(RA) 20.0%, 	(LB) 14.7%;

- mixed content, 1440p           	: (AI) 20.0%,	(RA) 14.0%, 	(LB) 11.5%;

- mixed content, 1080p           	: (AI) 17.9%,	(RA) 13.4%, 	(LB) 8.1%;

- animation, 720p               	: (AI) 0.3%, 	(RA) 0.1%, 	(LB) 0.3%.

average encoding time for the proposed encoder compared against the anchor RExt 5.1 are 314%, 151%, 141% for lossy AI, RA, and LD configurations, and 472%, 175%, 163% for lossless AI, RA, and LD configurations, respectively. The corresponding average decoding time compared against anchor are 81%, 99%, and 101% for lossy and 87%, 93%, and 96% for lossless, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG), “Joint Call for Proposals for coding of screen content,” Jan. 2014

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1003, Jan. 2013.

- M. Naccari, C. Rosewarne, and G. Sullivan, “HEVC Range extensions test model 5 encoder description,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1013, Nov. 2013.

- D. Flynn, J. Sole, G. J. Sullivan, and T. Suzuki, “HEVC Range Extensions Draft 5,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1005, Nov. 2013.

- X. Zhang, K. Zhang, J. An, and S. Lei, “AHG5 and AHG8: Alpha parameter coding methods for inter-component residual prediction in HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0154, Jan. 2014

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, and T. Suzuk, “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014.

- M. Naccari, C. Rosewarne, K. Sharman, and G. J. Sullivan, “HEVC Range Extensions Test Model 6 Encoder Description,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1013, Jan. 2014