| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0034-r1 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma, Wei Wang, Meng Xu, Xian Wang, Haoping Yu 2330 Central Expy, Santa Clara, CA 95050, United States | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution presents a screen content coding solution proposed in response to the Joint Call for Proposal for coding of screen content [1] issued by ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6 at San Jose meeting in January 2014. An additional “color table and index map coding mode” is introduced in intra frames, on top of the HEVC Range Extensions (RExt) Draft 5 [2], for coding of screen content. Meanwhile, a luma-based chroma intra prediction method is added to further exploit correlations between color components. The proposed solution is implemented in the HEVC range extension reference software HM12.1+RExt-5.1 [3] (RExt5.1). The simulation results have shown an average bit-rate reduction over the CfP anchors for lossless coding by 14.0%, 8.9%, and 7.8% for All Intra (AI), Low-Delay with B-picture (LB), and Random Access (RA), respectively. For the lossy coding mode, BD rate reduction of 10.3% for AI, 7.9% for RA, and 5.3% for LB have been observed. The complexity is measured using the encoding and decoding times. However, due to the heterogeneous computing nodes utilized, the running times presented in the test results may not accurately reflect the relative complexity.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	CTU/TU/PU partitioning	4

2.2	Inter-component prediction	4

2.3	Inter-picture prediction	4

2.4	Intra-picture prediction	4

2.5	In-loop filtering	4

2.5.1	De-blocking filter	4

2.5.2	SAO	4

2.5.3	Other	4

2.6	Spatial transforms	4

2.7	Other specific compression tools for screen content	4

2.8	Quantization	4

2.9	Motion/mode/parameter coding	5

2.9.1	Merge/skip	5

2.9.2	Motion vector coding	5

2.9.3	Intra prediction mode coding	5

2.9.4	Other parameter coding	5

2.10	Entropy coding of residual / transform coefficients	5

2.11	High-layer syntax	5

2.12	Additional algorithmic description discussion topic #1	5

2.13	Additional algorithmic description discussion topic #2	5

2.14	…	5

3	Syntax and semantics description	5

4	Compression performance discussion	5

5	Complexity analysis	5

5.1	Encoding time and measurement methodology	5

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	5

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	6

5.4	Expected memory usage of encoder	6

5.5	Expected memory usage of decoder	6

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	6

5.7	Complexity characteristics of decoder motion compensation	6

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	6

5.9	Complexity characteristics of decoder intra-frame prediction operation	6

5.10	Complexity characteristics of encoder inter-component prediction tools	6

5.11	Complexity characteristics of decoder inter-component prediction tools	6

5.12	Complexity characteristics of transforms specific in screen content coding	6

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	6

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	6

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	7

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	7

5.17	Degree of capability for encoder parallel processing	7

5.18	Degree of capability for decoder parallel processing	7

5.19	Additional complexity discussion topic #1	7

5.20	Additional complexity discussion topic #2	7

5.21	…	7

6	Software implementation description	7

7	Highlighted aspects discussion	7

8	Closing remarks	7

9	Patent rights declaration(s)	7

# Introduction

Screen content coding imposes new challenges for the video compression society because of its distinct signal characteristics compared with the conventional camera captured video. So far, we have observed a few techniques with promising coding performance in coding screen content, e.g., pseudo string matching [5], color palette coding [6, 7, 8] and intra motion compensation or intra block copy [9, 10, 11]. Among these techniques, the pseudo string matching method shows the highest gain for lossless coding, but with significant complexity overhead and difficulties in the lossy coding mode. The color-palette based-method is developed for coding of screen content under the assumption that non-camera captured content typically contains a limited few distinct colors, rather than the continuous color tone often seen in camera captured video. In addition, the intra block copy method, recently adopted in the study draft of HEVC range extensions, has also shown good coding gain for the screen content. However, its coding performance is rather limited because of the fixed block structure. On the other hand, performing block matching in intra picture, something similar to motion estimation in inter pictures, also brings up the encoder complexity significantly in computation as well as in memory access.

In this contribution, we propose an advanced screen content coding solution that outperforms the current HEVC Range Extension Draft 5 significantly as shown in the experimental simulation section. This new solution consists of several key algorithms that are designed specifically for coding screen content. These algorithms include pixel representation using color table, color table compression, color index map compression, 1-D string search, to name a few. More technical details will be discussed in the following sections. This technology is developed, harmonized, and integrated with the High-Efficiency Video Coding (HEVC) ranger extension (RExt) Draft 5 [12] to support efficient screen content coding.

# Algorithm description

An additional color table and index map (colorTableIdx) coding mode is introduced in intra frame to efficiently compress the screen content on top of the current HEVC RExt Draft 5 design. This colorTableIdx tool is implemented at CU level. Besides, luma-based chroma intra prediction (LMChroma) is added to further exploit the correlations between color components. This modified LMChroma mode is essentially the same as the algorithm implemented in HEVC reference mode HM8.1 with a modification so that luma component is no longer required to do resolution sub-sampling for 4:4:4 content.

## CTU/TU/PU partitioning

This part is the same as the HEVC.

## Inter-component prediction

A modified LMChroma method is added. The algorithm is essentially the same as the implementation in HM 8.1 except the removal of luma component resolution sub-sampling to construct the linear prediction method.

## Inter-picture prediction

This part is the same as the HEVC.

## Intra-picture prediction

An additional mode, colorTableIdx mode, as shown in Figure 1, is introduced in intra frames to efficiently compress the screen content. It is implemented at CU level. This mode mainly contains two parts: color table processing and the corresponding color index map processing, as illustrated in Figure 2.  For the residual produced by the original signal and reconstructed signal from decoded color index map and color table, HEVC residual engine is applied without any change.  These two main parts are described in details below.

Figure 1: Additional color table & index map coding mode added to HEVC RExt Draft 5. It is fully harmonized with the HEVC recursive quad-tree structure.

Figure 2: Two major processing units for the proposed colorTableIdx mode

### Color Table Processing

- For each CU, it can use the color table derived from itself or from its left or upper neighboring CU.

- When the color table is derived from its neighboring CU, the color table itself doesn’t need to be included in the bitstream. Instead, two bins are carried in the bistream to inform the decoder (one for merge flag, the other for merge direction). In this proposal document, this mode is called as “color table merge”.

- For the color table derived from the current CU, either a color sharing scheme or a differential color coding scheme is applied to encode each entry in the color table sequentially for each color component. Inter-table color sharing is performed between the current table and a reference table, while the differential color coding is conducted within the current table (so called intra-table color DPCM).

  - Reference table could be inferred from reconstructed neighboring CU/CTU or from a global table at slice or sequence level. In current response implementation, reference table is derived from the left CTU.

#### Color Table Derivation

- For each CU, as shown in Figure 3, it first creates the color histogram using 3 color components together, i.e., packed G, B, R or packed Y, Cb, Cr, according to its frequency of occurrence in descending order;

- Then it applies the neighboring color grouping according to a pre-defined color error allowance (where lossless is 0, lossy is 9), illustrated in Figure 4;

- After neighboring color grouping, it will pick N packed colors with the highest frequency of occurrence. N is bounded by the pre-defined maximum number of colors (max_num_of_colors). In current solution, max_num_of_colors = 128, i.e., N <= 128.

- For these N chosen colors, they are re-ordered by sorting them in a ascending order based on the value of packed colors, i.e., (g<<16)+(b<<8)+(r) or (Y<<16)+(Cb<<8)+(Cr).

Figure 3: planar color components are interleaved to create a basic “packed” processing unit with G, B, R or Y, Cb, Cr components at same pixel location. (GBR is used as example here).

Figure 4: Illustration of color table derivation, including color ordering (based on frequency of occurrence), color grouping, and color re-ordering (based on the intensity of packed colors, i.e., (g<<16)+(b<<8)+(r) or . (Y<<16)+(Cb<<8)+(Cr).). Please note that this derivation process is not only applied to current CU, but also neighboring CU reconstructed pixels for merge process.

As aforementioned, neighboring color grouping will be utilized upon histogram ordered color table to obtain more compact color table representation, in particular at lossy coding scenario.  Given a histogram H with n colors, where each color contains three components, the algorithm is described using the following pseudo code. For lossy coding mode, the max iteration number T equals to the grouping error allowance 9.

Order H[0] to H[n-1] in descending order according to the frequency of occurence;

for ( iter = 0; iter < T; iter++ )

{

for ( idxCur = 1; idxCur < n; idxCur++ )

{

for ( idxMergeCandidate = 0; idxMergeCandidate < idxCur; idxMergeCandidate++ )

{

if ( d( H[idxCur], H[idxMergeCandidate] ) < iter )

{

group H[idxCur] to H[idxMergeCandidate];

Reorder H[0] to H[idxMergeCandidate] in descending order;

break;

}

}

}

}

#### Color Table Coding

When a color table is carried in a bitstream explicitly, it is coded sequentially for each color component. The inter-table color sharing or intra-table color DPCM is applied to code each entry sequentially for all three color components.

##### Inter-table color sharing

Even though color table merge is not used, there are still same colors between current table and reference table. Therefore, applying the inter-table color sharing entry-by-entry could further improve the coding efficiency.  Here, the reference table is derived from the reconstructed left CTU, noted as R[3][Nr]. Let c(i) and r(j) represent i-th and j-th entries in current color table and reference table respectively. Please note that each entry contains three color components.

- For each color entry c(i), i<=N, in current table T[3][N],  it finds the matched identical r(j), j<=Nr from the reference table.

- Instead of signaling c(i),  j is encoded predicatively.  The predictor is determined as the smallest index k that is greater than previously reconstructed j with satisfying r(k)[0] > = c(i – 1)[0]. The prediction difference (j – k) is signalled in the bitstream. Note that (j – k) is non-negative so that no sign bit is needed..

##### Intra-table color DPCM

If current i-th entry couldn’t find its identical match from the reference table, it will be subtracted with the previous color entry and coded the absolute difference (|d(i)|) using color DPCM for each component.  In general, few bits for the absolute predictive difference and a sign bit will be signalled.  However, sign bit could be hidden for the following cases:

- Given that the color table is already ordered in ascending order as discussed in item 4) of Section 2.4.1.1, Y (or G) component difference doesn’t require sign bit at all

  - U difference doesn’t need sign bit if corresponding Y difference is zero ;

  - V difference doesn’t need the sign bit if both Y and U difference are zeros.

- Sign bit will be completely hidden if absolute difference is zero;

- Sign bit could be hidden as well if satisfying the following boundary condition

  - c[i-1] – |d(i)| < 0 or c[i-1] + |d(i)| > 255.

For the first entry of current table c(0), if it is not using the inter-table color sharing, each component of c(0) is coded using fixed 8-bit bypass context model.

Color table coding is summarized in the following pseudo code.

for ( i = 0; i < N; i++ )

{

if exist j such that r(j) = = c(i) // inter-table color sharing

{

if ( j = = 0 )

k = 0;

else

k = minimum x satisfying x > prev_j and r(k)[0] >= c(i – 1)[0];

prev_j = k;

signal j – k;

}

else // intra-table color DPCM

{

if ( i = = 0 )

signal c(i);

else

signal c(i) – c(i – 1);

}

}

Figure 5 and Table 1 gives an example to code the color table using above inter-table sharing and intra-table DPCM method.

Figure 5: Exemplary intra- and inter-color table coding

Table 1: Coding method for exemplary table in Figure 5

| i (current table index) | Coding method | j (matched index in reference table) | k (predicted matched index) | Coding element |
| --- | --- | --- | --- | --- |
| 0 | Intra-table |  |  | (0, 0, 192) |
| 1 | Intra-table |  |  | (0, 0, 48) |
| 2 | Inter-table | 1 | 0 | 1 |
| 3 | Inter-table | 2 | 2 | 0 |
| 4 | Inter-table | 3 | 3 | 0 |
| 5 | Intra-table |  |  | (0, 0, 2) |
| 6 | Intra-table |  |  | (60, 10, -12) |
| 7 | Inter-table | 8 | 7 | 1 |
| 8 | Intra-table |  |  | (0, 30, -30) |
| 9 | Intra-table |  |  | (20, -50, 0) |
| 10 | Inter-table | 9 | 9 | 0 |
| 11 | Intra-table |  |  | (30, 0, 0) |
| 12 | Inter-table | 15 | 11 | 4 |

As shown in Figure 5, reference table (left) is constructed from the left CTU of current CU. Inter-table color sharing and intra-table color DPCM are utilized to encode the color table of current CU.  Starting from the first entry of current color table, i.e., (idx, G, B, R) = (0, 0, 0, 192), it doesn’t have any exactly matched item in reference table, therefore it is coded independently.  For the 2nd entry, it doesn’t have matched item in reference table either. Given that we have coded first entry already, only prediction difference should be carried in the bitstream, i.e., (0, 0, 240) – (0, 0, 192 ) = ( 0, 0, 48 ). For the 3rd entry of current table, it finds the exact match at reference table when j = 1, meanwhile the predictive index using previously coded color entry is 0, therefore, only (1- 0) needs to be encoded.  The similar pattern is applied continuous until the last entry of current table (i.e., idx = 12 in this example) is coded.  In addition, Table 1 also gives the step by step illustration on how to apply inter-table sharing and intra-table DPCM on current table using available reference table.

#### Color Table Merge

As aforementioned, a color table from neighbouring CUs can be used for by the current CU to derive its index map, as depicted in Figure 6. The color table itself is not included in the bitstream in this case.

- Either left or upper color table is allowed to be used in the color table merge.  If the neighbouring CU is out of current CTU boundary, a color table of the left or upper CTU is used instead..

- Otherwise, the reconstructed neighbour blocks of the current CU will be used to derive the color table on-the-fly at both encoder and decoder. Please note that the neighbour block should be in the same size as the current CU.

Figure 6: Color table merge: (left) – reconstructed block neighbors of current CU will be used to derive the color table for merge (where neighbor reconstructed block shares the same size of current CU); (right) – if current CU is at the boundary of current CTU, such as current CU 0 at upper boundary and current CU 1 at left boundary, color tables of upper CTU and left CTU are used for merge decision respectively.

Note: the test results shown in this contribution were generated using only the left merge..

### Color Index Map Processing

After obtaining the color table, each and everyone of the basic processing units (three components packed together) will be translated to the corresponding index in the table, as depicted in Figure 7. As the packed components are used for each pixel to derive the proper index, there is only one index map to be encoded for each CU. The index assignment is based on the minimum error criteria.

Figure 7: Index map derivation illustration

#### Adaptive horizontal and vertical scan

Index map of each CU could be seen as the block with reduced dynamic range, but it still contains the dominant local feature such as the directions.  Thus, adaptive horizontal and vertical scanning is introduced to process the index map considering the local direction feature.  Figure 8 exemplifies this adaptive index map scan for an index map from a corresponding 8x8 CU.

Figure 8: Illustration of adaptive index map scan

Note: the vertical scan was skipped while generating the test results shown in this contribution.

#### 1-D string search

For each scanning pattern, 1-D string search is applied from the first index (the most top left) to the last index (the most bottom right) to derive the matched pairs.   Previously processed indices will be used as the reference in the successive index search.

- If current index couldn’t find its match from previously processed indices, it is encoded as the unmatched pair by sending a flag (signaling no-match) and the index value itself.

- If current index finds one or more matches from the reference buffer, the search continues for the next pixel, and so on. The matched pairs are encoded as by sending a flag (signaling matched string), the distance (offset between the current position and reference position) and the length of successive matched indices.

  - If there are multiple matches found for the current index, the pair providing the best R-D performance should be chosen. (To reduce the complexity, the current implementation uses a heuristic manner to select the pair with the longest matched length).

##### To improve the performance, additional one-line buffer for horizontal scan and one-column buffer for vertical scan are provided for the index map processing of each CU, hence, the first index in current CU could possibly find its match from the line buffer instead of encoding it directly as unmatched index.  Since these upper and left line buffers are already preserved to store the reconstructed pixels for intra prediction, this proposal reuses these two buffers without introducing additional buffers.  To use it for index search, all pixels in the line buffers are converted to the index by using the color table for the current CU.

Figure 9: A piece of 1-D segment after horizontal scanning

To well understand the 1-D string match proposed in this contribution, Figure 9 gives a piece of 1-D segment after a horizontal scanning of a 2-D index map. On top of this 1-D color index vector, 1-D string match will be applied.

- For the first position of each index map, say 14 as shown in Figure 9. Since there is no buffered reference yet, this very first index is treated as the “unmatched pair”, where we give the -1 and 1 to its corresponding distance and length, noted as (dist, len) = (-1, 1).

- For the 2nd index, again another “14”, we have the first index coded as reference, therefore the dist=1. Because there is another “14” at 3rd position, the length should be 2, i.e., len=2, (given the every proceed index could be served as the reference immediately for the subsequent index).

- Moving forward to the 4th position, we encounter the “17” which has not been seen before. Hence, we will encode it as an unmatched pair again, i.e., (dist, len) = (-1, 1).

As we can see, for the unmatched pair, we need to encode the flag (such as the “dist == -1”) followed by the real value of the index (like first appeared “14”, “17”, “6” and etc). On the other hand, for the matched pairs, we still need to encode the flag (such as the “dist != -1” ), followed by the distance offset, and the length of the matched string.  Here is a summary for the encoding procedure using the exemplified index shown in Figure 7.

dist = -1, len = 1, idx=14 (unmatched)

dist= 1, len = 2 (matched)

dist = -1, len = 1, idx=17 (unmatched)

dist= 1, len = 3 (matched)

dist = -1, len = 1, idx= 6 (unmatched)

dist= 1, len = 25 (matched)

dist= 30, len = 4 (matched) /*for the “17” which appeared before*/

….

For the matched pairs, the total number of bins to represent the offset distance and the match length can change adaptively along the course of the encoding of the current CU. In this proposal, neither the matched distance nor the length would be larger than 64x(64+1). In addition, the maximum distance and the maximum length would not come together. For every matched position, if both distance and length are not signaled as “significant” items, the matched distance is bounded by the distance between the current position and the very first position in the reference buffer (such as the first position in current CU as an example), say L. Therefore the maximum bins for this distance binarization is log2(L)+1 (instead of a fixed length), and maximum bins for the length binarization is log2(cuSize-L)+1 with cuSize=cuWidth*cuHeight.

.

## In-loop filtering

These parts (deblocking, SAO and other filtering) are all the same as the HEVC.

### De-blocking filter

### SAO

### Other

## Spatial transforms

This part is the same as the HEVC.

## Other specific compression tools for screen content

This part is the same as the HEVC.

## Quantization

This part is the same as the HEVC.

## Motion/mode/parameter coding

These parts are all the same as the HEVC.

### Merge/skip

### Motion vector coding

### Intra prediction mode coding

### Other parameter coding

## Entropy coding of residual / transform coefficients

This part is the same as the HEVC.

## High-layer syntax

This part is the same as the HEVC.

# Syntax and semantics description

Here is the syntax for the proposed color table and index map coding method.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| color_table_idx_flag | ae(v) |
| if( color_table_idx_flag ) { |  |
| color_table_size | ae(v) |
| nCbS = 1 << log2CbSize |  |
| color_table(x0, y0) |  |
| if ( color_table_size > 1 ) |  |
| color_index_map(x0, y0, nCbS, nCbS) |  |
| } else { |  |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] && color_table_idx_flag[x0][y0] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

| color_table( x0, y0) { | Descriptor |
| --- | --- |
| color_table_merge_flag | ae(v) |
| if( color_table_merge_flag ) { |  |
| color_table_merge_dir | ae(v) |
| }else { |  |
| for( ch = 0; ch < 3; ch++ ) { |  |
| num_bits_for_max_color_diff | ae(v) |
| for( i = 0; i < color_table_size; i++ ) { |  |
| if( ch == 0 && color_table_share_enabled ) { |  |
| color_table_share_flag | ae(v) |
| if( color_table_share_flag ) |  |
| color_table_share_idx_diff | ae(v) |
| } |  |
| if( color_table_idx_ref[ i ] < 0 ) { |  |
| if( i = = 0 ) { |  |
| color_value | u(8) |
| } else { |  |
| abs_color_diff | ae(v) |
| if( code_color_diff_sign ) |  |
| color_diff_sign | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| color_index_map(x0, y0, nCbS, nCbS ) { | Descriptor |
| --- | --- |
| color_idx_map_scan_dir | ae(v) |
| for( i = 0; i < nCbS * nCbS) { |  |
| idx_match _flag | ae(v) |
| if( idx_match_flag) { |  |
| unmatched_idx | ae(v) |
| } else { |  |
| sig_dist_flag | ae(v) |
| if( sig_dist_flag ) { |  |
| dist_equal_one_flag | ae(v) |
| } else { |  |
| distance_minus2 | ae(v) |
| } |  |
| sig_len_ flag | ae(v) |
| if ( sig_len_ flag ) { |  |
| length_minus1 | ae(v) |
| i += length_minus1 + 1; |  |
| } else { |  |
| length_minus_cuWidth | ae(v) |
| i += length_minus_cuWidth + cuWidth; |  |
| } |  |
| } |  |
| } |  |
| } |  |

color_table_idx_flag[x0][y0]: equal to 1 specifies that current coding unit is coded in color table and index map mode. color_table_idx_flag[x0][y0] equal to 0 specifies that current coding unit is not coded in color table and index map mode. The array indices x0, y0 specify the location (x0, y0) of the top-left luma sample of the current coding unit relative to the top left luma sample of the picture. When Color_table_idx_flag[x0][y0] is not presented, it is inferred to be equal to 0.

color_table_size[x0][y0]: specifies the number of colors in color table

color_table_merge_flag[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from neighbor blocks. color_table_merge_flag[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from itself.

color_table_merge_dir[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from its left neighbor block, while color_table_merge_dir[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from its upper neighbor block.

num_bits_for_max_color_diff[x0][y0][n]: specifies the number of bits required to represent the maximum color difference of color component n in color table.

color_table_share_flag[x0][y0][n][cIdx]: equal to 1 specifies that current color entry of current color table uses the color for sharing from its reference table. color_table_share_flag[x0][y0][n][cIdx] equal to 0 specifies that current color entry is not shared from the reference table.

color_table_share_idx_diff[x0][y0][n][cIdx]: specifies the color index difference for shared color entry.

color_value[x0][y0][n][0]: specifies the value of the first color entry of current color table.

abs_color_diff[x0][y0][n][cIdx]: specifies the absolute of color difference for successive entries in current color table.

color_diff_sign[x0][y0][n][cIdx]: specifies the sign of color difference for successive entries in current color table.

color_idx_map_scan_dir[x0][y0]: equal to 1 specifies that the index map of current coding unit uses the horizontal scanning; color_idx_map_scan_dir[x0][y0] equal to 0 specifies that the index map of current coding unit uses the vertical scanning;

idx_match _flag[x0][y0][cIdx]: equal to 1 specifies that current index could find its match from reference buffer; idx_match _flag[x0][y0][cIdx]: equal to 0 specifies that current index could not find its match from reference buffer;

unmatched_idx[x0][y0][cIdx]: specifies the value of current index which could not find its match from reference buffer. After parsing unmatched_idx [x0][y0][cIdx], cIdx = cIdx + 1;

sig_dist_flag[x0][y0][cIdx]: equal to 1 specifies that the distance of current matched pair is significant distance; sig_dist_flag[x0][y0][cIdx] equal to 0 specifies that the distance of current matched pair is not significant distance;

dist_equal_one_flag[x0][y0][cIdx]: equal 1 specifies that the distance equals to 1; dist_equal_one_flag[x0][y0][cIdx] equal 0 specifies that the distance equals to the width of current coding unit

distance_minus2[x0][y0][cIdx]: specifies the distance minus 2 of current matched pair.

The variable distance is derived as follows:

distance = distance_minus2[x0][y0][cIdx] + 2;

sig_len_flag[x0][y0][cIdx]: equal to 1 specifies that the length of current matched pair is significant length; sig_len_flag[x0][y0][cIdx] equal to 0 specifies that the length of current matched pair is not significant length;

length_minus1[x0][y0][cIdx]: specifies the length minus 1 of current matched pair.

The variable length is derived as follows:

Length = length_minus [x0][y0][cIdx] + 1;

cIdx   = cIdx + length;

length_minus_cuWidth[x0][y0][cIdx]:: specifies the length minus the width of current coding unit for current matched pair.

The variable length is derived as follows:

Length = length_minus_cuWidth[x0][y0][cIdx] + cuWidth;

cIdx   = cIdx + length;

# Compression performance discussion

Proposed technical solution is implemented on top of the HEVC RExt 5.1 for comparison. Both lossless and lossy cases are performed to demonstrate the coding efficiency.  For lossless case, bit rate reduction is measured.  For lossy case, bit rate matching is first conducted and then the BD-Rate is calculated using the QPs derived from the bit rate matching.

## Lossy test condition

A summary of lossy BD-Rate reduction for both with PSNR clipping and without PSNR clipping in comparison to the RExt 5.1 is shown below. More detailed simulation data is enclosed as excel attachment.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -27.3% | -28.1% | -27.3% |
| RGB, text & graphics with motion,720p | -16.8% | -15.9% | -16.2% |
| RGB, mixed content, 1440p | -7.2% | -7.0% | -6.9% |
| RGB, mixed content, 1080p | -7.5% | -7.7% | -7.5% |
| RGB, Animation, 720p | -2.7% | -4.3% | -3.4% |
| YUV, text & graphics with motion, 1080p | -22.2% | -25.0% | -25.3% |
| YUV, text & graphics with motion,720p | -9.2% | -16.1% | -19.0% |
| YUV, mixed content, 1440p | -5.2% | -10.8% | -10.3% |
| YUV, mixed content, 1080p | -4.6% | -10.3% | -9.5% |
| YUV, Animation, 720p | -0.4% | -3.2% | -2.7% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 124% |  |  |

(a)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -16.4% | -17.2% | -16.5% |
| RGB, text & graphics with motion,720p | -16.6% | -14.8% | -15.3% |
| RGB, mixed content, 1440p | -7.7% | -6.8% | -6.9% |
| RGB, mixed content, 1080p | -5.8% | -5.8% | -5.6% |
| RGB, Animation, 720p | -1.4% | -2.5% | -1.8% |
| YUV, text & graphics with motion, 1080p | -13.6% | -20.4% | -20.0% |
| YUV, text & graphics with motion,720p | -9.1% | -16.6% | -20.4% |
| YUV, mixed content, 1440p | -5.0% | -14.1% | -13.7% |
| YUV, mixed content, 1080p | -3.6% | -10.2% | -9.6% |
| YUV, Animation, 720p | -0.1% | -3.4% | -2.5% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 145% |  |  |

(b)

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -10.1% | -11.0% | -10.4% |
| RGB, text & graphics with motion,720p | -12.0% | -10.6% | -11.1% |
| RGB, mixed content, 1440p | -6.6% | -5.7% | -5.8% |
| RGB, mixed content, 1080p | -3.7% | -3.7% | -3.5% |
| RGB, Animation, 720p | -0.5% | -1.0% | -0.7% |
| YUV, text & graphics with motion, 1080p | -8.0% | -12.9% | -13.0% |
| YUV, text & graphics with motion,720p | -5.0% | -10.9% | -14.3% |
| YUV, mixed content, 1440p | -4.6% | -12.9% | -12.6% |
| YUV, mixed content, 1080p | -2.3% | -6.6% | -5.8% |
| YUV, Animation, 720p | -0.1% | -1.4% | -0.8% |
| Enc Time[%] | 136% |  |  |
| Dec Time[%] | 161% |  |  |

(c)

Figure 10: summary of BD-Rate reduction (without PSNR clip) for (a) AI, (b) RA, and (c) LB.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, text & graphics with motion,720p | -16.8% | -15.9% | -16.2% |
| RGB, mixed content, 1440p | -7.2% | -6.9% | -6.9% |
| RGB, mixed content, 1080p | -7.5% | -7.7% | -7.5% |
| RGB, Animation, 720p | -2.7% | -4.3% | -3.4% |
| YUV, text & graphics with motion, 1080p | -22.2% | -25.0% | -25.3% |
| YUV, text & graphics with motion,720p | -9.2% | -16.1% | -19.0% |
| YUV, mixed content, 1440p | -5.2% | -10.7% | -10.4% |
| YUV, mixed content, 1080p | -4.6% | -10.3% | -9.5% |
| YUV, Animation, 720p | -0.4% | -3.2% | -2.7% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 124% |  |  |

(a)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -16.4% | -17.2% | -16.5% |
| RGB, text & graphics with motion,720p | -16.6% | -14.8% | -15.3% |
| RGB, mixed content, 1440p | -7.7% | -6.8% | -6.9% |
| RGB, mixed content, 1080p | -5.8% | -5.9% | -5.6% |
| RGB, Animation, 720p | -1.4% | -2.5% | -1.9% |
| YUV, text & graphics with motion, 1080p | -13.6% | -20.3% | -20.0% |
| YUV, text & graphics with motion,720p | -9.1% | -16.7% | -20.4% |
| YUV, mixed content, 1440p | -5.0% | -14.1% | -13.7% |
| YUV, mixed content, 1080p | -3.6% | -10.3% | -9.6% |
| YUV, Animation, 720p | -0.1% | -3.3% | -2.5% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 145% |  |  |

(b)

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -10.1% | -11.1% | -10.5% |
| RGB, text & graphics with motion,720p | -12.1% | -10.6% | -11.1% |
| RGB, mixed content, 1440p | -6.6% | -5.8% | -5.8% |
| RGB, mixed content, 1080p | -3.7% | -3.7% | -3.5% |
| RGB, Animation, 720p | -0.5% | -1.1% | -0.7% |
| YUV, text & graphics with motion, 1080p | -8.0% | -12.9% | -13.0% |
| YUV, text & graphics with motion,720p | -5.0% | -10.9% | -14.4% |
| YUV, mixed content, 1440p | -4.6% | -12.9% | -12.6% |
| YUV, mixed content, 1080p | -2.3% | -6.6% | -5.8% |
| YUV, Animation, 720p | 0.0% | -1.4% | -0.7% |
| Enc Time[%] | 136% |  |  |
| Dec Time[%] | 161% |  |  |

(c)

Figure 11: summary of BD-Rate reduction (with PSNR clip) for (a) AI, (b) RA, and (c) LB.

## Mathematically lossless test condition

A summary of lossless bit rate reduction in comparison to the RExt 5.1 is shown below. More detailed simulation data is enclosed as excel attachment.

|  | AI |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 32.9% | 36.0% | 21.6% | 49.6% |
| RGB, text & graphics with motion,720p | 20.2% | 20.1% | 2.0% | 35.0% |
| RGB, mixed content, 1440p | 6.2% | 6.3% | 5.7% | 6.9% |
| RGB, mixed content, 1080p | 9.6% | 8.7% | 6.4% | 11.0% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, text & graphics with motion, 1080p | 35.3% | 38.6% | 20.1% | 53.2% |
| YUV, text & graphics with motion,720p | 19.1% | 20.2% | 1.9% | 40.9% |
| YUV, mixed content, 1440p | 3.6% | 4.0% | 1.3% | 6.7% |
| YUV, mixed content, 1080p | 6.5% | 6.1% | 5.3% | 6.9% |
| YUV, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 182% |  |  |  |
| Dec Time[%] | 127% |  |  |  |
|  | RA |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 16.1% | 28.9% | 14.5% | 45.8% |
| RGB, text & graphics with motion,720p | 4.6% | 12.6% | 1.0% | 33.7% |
| RGB, mixed content, 1440p | 1.1% | 1.1% | 0.7% | 1.6% |
| RGB, mixed content, 1080p | 4.0% | 2.7% | 1.2% | 4.2% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | 14.2% | 28.0% | 11.9% | 42.8% |
| YUV, text & graphics with motion,720p | 4.2% | 13.5% | 1.1% | 39.2% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.4% | 0.7% |
| YUV, mixed content, 1080p | 2.1% | 1.6% | 1.0% | 2.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 129% |  |  |  |
|  | LB |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 15.1% | 27.3% | 14.0% | 43.3% |
| RGB, text & graphics with motion,720p | 3.5% | 11.0% | 0.9% | 30.9% |
| RGB, mixed content, 1440p | 0.5% | 0.5% | 0.2% | 0.8% |
| RGB, mixed content, 1080p | 3.7% | 2.2% | 0.5% | 3.9% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | 12.1% | 24.1% | 10.4% | 34.7% |
| YUV, text & graphics with motion,720p | 2.9% | 11.6% | 0.7% | 35.5% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.2% | 0.3% |
| YUV, mixed content, 1080p | 1.7% | 1.1% | 0.4% | 1.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  |  |
| Dec Time[%] | 140% |  |  |  |

Figure 12: lossless bit rate reduction summary

# Complexity analysis

## Encoding time and measurement methodology

Both encoding and decoding time are collected from the encoder and decoder log file. However, due to the heterogeneous computing nodes utilized in the simulation, the timing information is not reliable.

## Additional memory

Additional memory/buffer requirement is estimated for worst case.

Color table merge

- Additional 128*3*PicWidth/maxCUWidth bytes for left/upper CTU color table line buffer

Index map processing

- Same size of intra prediction left and upper line buffer at encoder, reuse intra prediction line buffer at decoder

## Complexity characteristics of encoder related to other tools specific in screen content coding

Intra block copy has shown the noticeable gain for coding the screen content, thus it is adopted into the HEVC Range Extension Draft 4.  For intra block copy, SAD based block match is the key factor for its implementation complexity; while string match is the core factor for the proposed advanced screen content coding tool.From the hardware’s point of view, the difference between exact-match based string search and SAD based algorithm is the processing core.

In SAD based algorithm, the processing core is ABS(a-b).  In 8bpp system, it translates to:

tmp[8:0] = a[7:0] – b[7:0];

SAD[8:0] = (tmp[8]) ? (~tmp[7:0])+1 : tmp[8:0];

The hardware resource for this operation is two 8-bit adder, eight NOT gates, and eight 2:1 MUX; the hardware latency is two adder (sequential or parallel) plus one 2:1 MUX.

In exact-match based string search, the processing core is (a==b).  In 8bpp system, it translates to:

tmp[7:0] = a[7:0] ^ b[7:0];

SS = | tmp[7:0];

The hardware resource for this operation is eight XOR gates and one 8:1 OR gates; the hardware latency is one XOR plus one 8:1 OR.

Exact-match based string search algorithm offers approximately 4 time area reduction and 12 times clock speed increase. If the same clock speed is used, the area for one SAD engine equals to the area for 48 string search engine.

# Software implementation description

Proposed technical solution is implemented on top of the HEVC Range Extension reference software 5.1, i.e., HM12.0+RExt5.1.  Simulations are conducted in comparison to the anchor generated using RExt5.1.

# Highlighted aspects discussion

# Closing remarks

An additional “color table and index map coding mode” is introduced in intra frames, on top of the HEVC Range Extensions (RExt) Draft 5 [2], for coding of screen content. Meanwhile, a luma-based chroma intra prediction method is added to further exploit correlations between color components. The proposed solution is implemented in the HEVC range extension reference software HM12.1+RExt-5.1 [3] (RExt5.1). The simulation results have shown an average bit-rate reduction over the CfP anchors for lossless coding by 14.0%, 8.9%, and 7.8% for All Intra (AI), Low-Delay with B-picture (LB), and Random Access (RA), respectively. For the lossy coding mode, BD rate reduction of 10.3% for AI, 7.9% for RA, and 5.3% for LB have been observed

We suggest the committee adopts the color table and index map coding into CE or TE for future study.

Reference

[1] Joint Call for Proposal for coding of screen content, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6, Doc. MPEG2014/N14175, San Jose, USA, Jan. 2014.

[2] D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, HEVC Range Extensions Draft 5, Joint Collaborative Team on Video Coding (JCT-VC) of ITU SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JCTVC-O1005, Geneva, CH, Oct. 2013.

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.1

[4] G. Bjontegaard, Calculation of average PSNR differences between RD-curves, ITU-T SG 16 Q.6, Doc. VCEG-M33, Austin, USA, April 2001.

[5] T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC”, JCT-VC Document, JCTVC-K0133, Shanghai, China, October 2012.

[6] W. Zhu, J. Xu, W. Ding, “RCE3 Test 2: Multi-stage Base Color and Index Map”, JCT-VC Document, JCTVC-N0287, Vienna, Austria, July 2013.

[7] L. Guo, M. Karczewicz, J. Sole, “RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding”, JCT-VC Document, JCTVC-N0247, Vienna, Austria, July 2013.

[8] L. Guo, M. Karczewicz, J. Sole, R. Joshi, “Non-RCE3: Modified Palette Mode for Screen Content Coding”, JCT-VC Document, JCTVC-N0249, Vienna, Austria, July 2013.

[9] D.-K. Kwon, M. Budagavi, “RCE3: Results of test 3.3 on Intra motion compensation, JCT-VC Document, JCTVC-N0205, Vienna, Austria, July 2013.

[10] C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCT-VC Document, JCTVC-N0256, Vienna, Austria, July 2013

[11] C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Pipeline Friendly Intra Motion Compensation”, JCT-VC Document, JCTVC-N0254, Vienna, Austria, July 2013

[12] D. Flynn, J. Soel and T. Suzuki, “Range Extension Draft 4”, JCTVC-L1005, August 2013

# Patent rights declaration(s)

Huawei Technologies (USA), may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).