| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0035_r1 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by Microsoft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Xun Guo Gary J. Sullivan | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document describes the screen content coding technology proposed by Microsoft in response to the joint Call for Proposals. In this proposal, hash based search, 1-D dictionary mode, adaptive color space coding, modifications to intra BC mode, and palette mode, etc. are introduced to improve the coding efficiency and reduce the coding complexity for screen contents. Experimental results reportedly show that for lossy coding, using lower bit rates than the anchor, the proposed scheme achieves 12.47 dB, 9.67 dB, and 9.12 dB on Y-component (or G-component if the input is of RGB format) PSNR improvement on average for AI, RA, and LB respectively. The encoding time for RA and LB are reported to be less than 90% of that of the anchor. The decoding time is similar to the anchor (saving about 10% for AI case). For lossless coding, about 29.9%, 24.0% and 23.0% bit rate savings are reportedly achieved for AI, RA and LB respectively. Moreover, different trade-off points between the encoding complexity and coding efficiency can be achieved; e.g., the proposed scheme with a low complexity setting reportedly shows about 3x faster encoding than the anchor, with average bit-rate savings of 19.5% and 18.6% for RA and LB lossless coding, respectively.

# Contents

Abstract	1

Contents	1

1	Introduction	3

2	Algorithm description	3

2.1	CTU/TU/PU partitioning	3

2.2	Inter-component prediction	3

2.3	Inter-picture prediction	3

2.4	Intra-picture prediction	4

2.5	In-loop filtering	4

2.5.1	De-blocking filter	4

2.5.2	SAO	4

2.5.3	Other	4

2.6	Spatial transforms	4

2.7	Other specific compression tools for screen content	5

2.7.1	1-D dictionary mode	5

2.7.2	Modifications to Intra BC mode	6

2.7.3	Palette mode	6

2.8	Quantization	7

2.9	Motion/mode/parameter coding	7

2.9.1	Merge/skip	7

2.9.2	Motion vector coding	7

2.9.3	Intra prediction mode coding	7

2.9.4	Other parameter coding	7

2.10	Entropy coding of residual / transform coefficients	7

2.11	High-layer syntax	7

2.12	Adaptive color space coding	8

3	Syntax and semantics description	8

4	Compression performance discussion	16

4.1	Lossy test condition	16

4.2	Mathematically lossless test condition	20

5	Complexity analysis	22

5.1	Encoding time and measurement methodology	22

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	22

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	22

5.4	Expected memory usage of encoder	22

5.5	Expected memory usage of decoder	22

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	22

5.7	Complexity characteristics of decoder motion compensation	22

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	23

5.9	Complexity characteristics of decoder intra-frame prediction operation	23

5.10	Complexity characteristics of encoder inter-component prediction tools	23

5.11	Complexity characteristics of decoder inter-component prediction tools	23

5.12	Complexity characteristics of transforms specific in screen content coding	23

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	23

5.13.1	Complexity of hash table maintenance	23

5.13.2	Complexity of intra BC search	23

5.13.3	Complexity of dictionary search	24

5.13.4	Complexity of palette mode	24

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	24

5.14.1	Complexity of modifications to intra BC mode	24

5.14.2	Complexity of dictionary mode	24

5.14.3	Complexity of palette mode	24

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	24

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	24

5.17	Degree of capability for encoder parallel processing	24

5.18	Degree of capability for decoder parallel processing	24

5.19	Complexity characteristics of decoder for adaptive color space coding.	24

5.20	Complexity characteristics of encoder for adaptive color space coding	25

5.21	Trade-off between encoding complexity and coding efficiency	25

6	Software implementation description	26

7	Highlighted aspects discussion	26

8	Closing remarks	26

9	References	27

10	Patent rights declaration(s)	27

# Introduction

In January 2014, a joint Call for Proposals for Coding of Screen Content [1] was issued by ITU-T Q6/16 and ISO/IEC JTC1/SC29/WG11, which targets at enhancing the coding capability of HEVC for mixed content, text and graphics with motion and animation content for various applications [2].

This document presents our response to the CfP. The proposed scheme is based on HM-12.1+RExt-5.1, with several modifications adopted into HM-13.0+RExt-6.0. In the proposed schemes, hash based search (previous presented as 2-D dictionary [4][5]), 1-D dictionary mode [3][5], adaptive color space coding, modifications to intra BC mode, and palette mode [3][6][7], etc., are introduced to improve the coding efficiency or reduce the coding complexity for screen contents.

The proposed scheme significantly reduces the bit rate for coding of screen content, while with less encoding time than the anchor for RA and LB settings. It also shows capability to trade coding efficiency for encoding complexity. For example, the proposed scheme can run about 3x faster than the anchor, while the coding performance is still much better than the anchor on average. It should be noted that our design meets the low extra complexity required for all use cases and applications described in the screen content coding requirement document [2].

Except for improvement of the current modes and introductions of several modes in intra coding, we try to keep other designs consistent with the current HEVC range extension. For example, the block structure, entropy coding, transform, quantization, deblocking, SAO, intra and inter prediction in the current RExt design remain the same. New modes are introduced with low complexity at the decoder and fast decision algorithms at the encoder, so that even with less encoding time, the proposed scheme can achieve significant coding gains.

# Algorithm description

The term of anchor used in this section means HM-12.1+RExt-5.1 if not explicitly stated. Thus, the algorithm modifications described below are relative to the anchor, including some modifications that have been adopted into HM-13.0+RExt-6.0. It should be noted that we have exactly the same picture types (I, B) and reference structure as the anchor. For each intra picture, the encoding is totally independent from other pictures.

## CTU/TU/PU partitioning

Same as the anchor.

## Inter-component prediction

The changes introduced by r3815 (RExt__P0154_ADDITIONAL_CONTEXT_FOR_CCP) and r3841 (Fixed single-component-loop inter search for lossless coding) in HM-range-extensions are incorporated in the software used to response call for proposal. Please note that these changes are also included in HM-13.0+RExt-6.0.

## Inter-picture prediction

The decoding algorithm used in inter-picture prediction is the same as the anchor. The encoding algorithm used in inter-picture prediction (motion estimation strategy) is modified.

- Hash based motion estimation. It was previous presented as 2-D dictionary [4][5]. In the implementation of the software, hash tables are built for every 8x8, 16x16, 32x32, and 64x64 block based on the original pixel. When performing motion estimation for block with size 2Nx2N, hash based checking is performed first. If an exact match is found by hash matching, the integer motion estimation process and the fractional motion estimation process are skipped. Additional memory is required to build hash table. The memory requirement depends on the picture size. 18-bit hash is used in the implementation. The higher two bits are related to the block size, and the low 16 bits are generated from the CRC of original pixels. In the current implementation, the additional memory cost will be 4 (four different sizes) * width * height * 8 (2 bytes for x position of the current block, 2 bytes for y position of the current block, 4 bytes another hash value used to reduce the risk of confliction) bytes to store the detailed hash results and (1<<18) * 8 = 2M bytes to store the lookup table are require for every reference picture. For example, for 1080p sequences using 4 reference pictures, additional (4 * 1920 * 1080 * 8 + 2) * 4 ≈ 273M bytes memory is required.

- Early termination if exact hash match is found in a reference picture. If the current CU is CTU (64x64), and an exact hash match is found in a reference picture, and the quality of reference picture is not worse than the expected quality of the current picture (the QP of the reference picture is equal to or smaller than the QP of current picture), all the RDO process after checking 2Nx2N partition is skipped (including checking other partition mode and splitting the current CU into smaller CUs).

- The motion estimation start point. In HM-12.1+RExt-5.1, one of the MVPs will be used as the start point of motion estimation. In this proposal, for every reference picture, the MV found by motion estimation for the PU with size of 2Nx2N will be stored at the encoder side. When performing motion estimation for the child PU sizes, the MV of parent PU (2Nx2N PU) using a same reference picture, will be used as a candidate motion estimation start point. The encoder will compare the new motion estimation start point candidate and the original MVP and then choose the one leading to smaller prediction cost as the motion estimation start point. This modification introduces additional memory requirement at the encoder side. For example, in the current implementation, the maximum additional memory cost will be about 4 (mvx and mvy, two bytes for each) * 2 (two reference picture lists) * 16 (at most 16 reference pictures in each list) = 128 Bytes.

## Intra-picture prediction

The spatial intra-picture prediction is the same as the anchor. The new added intra picture prediction tools will be introduced in Section 2.7.

## In-loop filtering

### De-blocking filter

Same as the anchor.

### SAO

No modifications at decoder side are introduced.

At the encoder side, if no CTU selects to enable SAO, the encoder will disable SAO at the slice header (bit saving is about 40 bits for one picture of 720p sequences).

### Other

Same as the anchor.

## Spatial transforms

Same as the anchor.

## Other specific compression tools for screen content

### 1-D dictionary mode

A 1-D dictionary mode [3][5] is introduced in the proposal. The 1-D dictionary mode is enabled for all CUs; and both the horizontal scanning and vertical scanning are supported. Three components (YUV or GBR) are processed together. There are two types of 1-D dictionary mode, the first one needs to maintain a dictionary for prediction, like coding a file using Lempel-Ziv, and the second one uses all the previously reconstructed pixels in the same picture (slice and tile) for prediction.

In the first mode, which is called normal 1-D dictionary mode, all the previous coded pixels using 1-D dictionary mode are kept in the dictionary (unless the maximum dictionary size is achieved) and may be used for prediction. The basic dictionary size is 1<<18 pixels. When the dictionary reaches 150% of basic dictionary size, the oldest 50% pixels are removed from the dictionary. The removing process is only invoked after encoding/decoding an entire CTU. In this mode, prediction mode and direct mode are allowed. In prediction mode, an offset (the offset relative to the position of the current pixel in the dictionary) and a matching length are signaled. In direct mode, the pixel value is signaled directly. Additional memory to maintain dictionaries is required at the decoder side. In the implementation of the CfP response, about 2 (horizontal and vertical scanning) * (1<<18) * 2 (maximum size is set to 2x of basic size in the current implementation, actually 4096 more than 1.5x of the basic size should be enough) * 3 (three color components) = 3M Bytes memory is required at the decoder side.

In the second mode, which is called reconstruction based 1-D dictionary mode, all the previously reconstructed pixels can be used for prediction. Prediction mode and direct mode are also allowed. In prediction mode, two offsets (X offset and Y offset relative to the position of the current pixel in the picture) and a matching length are signaled. In direct mode, the pixel value is also signaled directly. When the current region starts a new row or column, the pixel used for prediction also starts a new row or column, as shown in the following figure. The below example is an 8x8 CU uses reconstruction based 1-D dictionary mode with horizontal scanning. First a matching length of three and two offsets are signaled. And then a matching length of 17 and two offset are signaled. There is no additional memory requirement at the decoder side. Figure 1 shows an example of this mode.

Figure 1. An example of reconstruction based 1-D dictionary coding.

Encoder strategy for dictionary mode. Hash based search is applied to dictionary mode for both normal dictionary mode and reconstruction based dictionary mode.

For normal dictionary mode, the encoder tries to find the maximum matching length from the current encoding pixel. The encoder firstly checks the hash value for 1 pixel and tries to get the maximum matching length from the current pixel. If matching for two or more than two pixels is found, the encoder begins to check hash matching for 2 pixels. It is similar for the encoder to check 4 and 8 pixels matching in hash table. Additional memory is required at the encoder side. Similar to the decoder side, 3M bytes memory is required to buffer the dictionary. 2 (horizontal and vertical) * (1<<18) * 2 * 4 (an integer size) * 4 (1, 2, 4, and 8, a total of 4 different matching length) = 16M Bytes memory to save the information of previous matching. If the dictionary size is reduced, the memory cost will also be reduced. 2 (horizontal and vertical) * 4 (an integer size) * 4096 (12-bit hash is used) * 4 (a total of 4 different matching length) = 128K bytes of hash table. To sum up, about 20M additional memory cost is required at the encoder side.

For reconstruction based dictionary mode, the encoder also tries to find the maximum matching length from the current encoding pixel. There are also 4 hash tables for the matching length of 1, 2, 4, and 8. The additional memory requirement is related to the picture size. E.g., for 1080p sequence, about 2 (horizontal and vertical) * 8 (save two integers, x and y position) * 4 (4 different matching length) * 1920 * 1080 ≈ 133M Bytes will be used to save previous matching. And about 2 * 8 * 4 * 64K (16-bit hash is used) = 4M Bytes memory will be used to save hash table. To sum up, about 140M additional memory is used for 1080p sequence.

### Modifications to Intra BC mode

- Intra BC skip mode

A new mode (Intra BC skip mode) is introduced. Intra BC skip mode means the current CU uses Intra BC prediction and the partition size is 2Nx2N and there is no residue.

- Intra BC merge mode

Intra BC merge mode is used to signal the block vector. Intra BC merge mode is only enabled for Intra BC CU with partition size of 2Nx2N. Two merge candidates are enabled, the BV from the left block and the BV from the above block.

- Intra BC with flipping

Vertical flipping is enabled for the CU using Intra BC prediction. One flag is signaled to distinguish flipping or not for every PU using Intra BC prediction (except the CU in Intra BC merge mode).

- Intra BC for 2NxN, Nx2N, and NxN

The changes introduced by r3822 (RExt__PRCE3_D2_INTRABC_ADDITIONAL_PU_CONFIGURATIONS) in HM-range-extensions are incorporated in the software used to response call for proposal. Please note that these changes are also included in HM-13.0+RExt-6.0.

- Intra BC mode using hash based search in large range

The search range for Intra BC mode is extended to the whole slice and tile (in the response of CfP, the whole picture). The search is performed by block hash matching based on original pixels. The normative changes here is only the removing of the constraints on the range of block vectors. The technique was previously presented as 2-D dictionary [4].

Additional memory is required to build the hash table at the encoder. Hash tables are maintained for every possible 4x4, 8x8, and 16x16 block. The amount of additional memory cost is similar to the additional memory required by hash based motion estimation (e.g. about 273M bytes additional memory is required for 1080p sequence).

### Palette mode

The palette mode described in JCTVC-P0108 (i.e., REC4 test 1 at the 16th JCT-VC meeting), with color table sharing (JCTVC-P0153), four-neighbor major color index prediction (JCTVC-P0098) and transition copy mode (JCTVC-P0115) as described in JCTVC-P0249, are incorporated.

In this method, pixels of a CU are classified into major colors and escape pixels. Major colors are the most representative pixel values in a CU like a palette, which cover the majority pixels of the CU. Escape pixels are those ones that do not belong to any of the major color sets. For each pixel, a color index is transmitted to indicate whether it is a major-color pixel or not, and, if it is, which major color set it belongs to. If it is an escape pixel, the pixel value is transmitted. A flag is signaled in CU level to indicate if this method is used or not.

To effectively code the major colors and color indices, predictive coding is used. For the major colors, each color element can be predicted from the left CU and above CU. Meanwhile, a color table sharing method is also applied which uses the entire major color tables from previously coded CU for the current CU. A flag is signaled to switch the two prediction methods. If color sharing is used, no major colors are transmitted for the current CU. Otherwise, the major colors are transmitted by using aforementioned element-wise prediction. For the color indices, a multiple direction predictive coding is used. In specific, for each pixel line, a flag is signalled to indicate the line mode: horizontal mode, vertical mode or normal mode. In horizontal mode, the current pixel line shares the same color index. In this case, only the color index is transmitted. In vertical mode, the current pixel line is the same with the above pixel line. In this case, nothing is transmitted. At the decoder, the current line just copies the color indices from the above line. In normal mode, one flag is transmitted for each pixel position to indicate whether it is the same with one of the five candidates from left, top, top-left, top-right and transition copy, i.e. the last occurred two-pixel pattern stored in a mapping table. If not, the color index for this position is transmitted.

The additional memory requirement for the decoder is expected to be marginal. The complexity influence is expected to be marginal.

## Quantization

Same as the anchor.

## Motion/mode/parameter coding

Same as the anchor.

### Merge/skip

Same as the anchor.

### Motion vector coding

Same as the anchor.

### Intra prediction mode coding

Same as the anchor.

### Other parameter coding

Same as the anchor.

## Entropy coding of residual / transform coefficients

The changes introduced by r3816 (RExt__PRCE2_A1_GOLOMB_RICE_PARAMETER_ADAPTATION) in HM-range-extensions are incorporated in the software used to response call for proposal. Please note that these changes are also included in HM-13.0+RExt-6.0.

## High-layer syntax

Same as the anchor except that several SPS or PPS flags are added to control the on/off of the new added tools.

## Adaptive color space coding

Adaptive color space coding is introduced to improve the RGB (GBR) video coding efficiency, as shown in figure 2. Besides GBR, coding in YCoCg, RGB, and BGR spaces are enabled for Intra predicted lossy coded CU. As GBR to YCoCg conversion is not reversible without a bit-depth increase, it is not used in lossless coding. The pixel values are stored in the original color space (e.g. GBR), and when needed, the necessary pixels (at most 4N+1 pixels used for intra prediction) are converted to a specific color space. The reconstructed pixels are converted back to the main color space (e.g. GBR) before storing. Thus, no additional storage memory is required. The conversion is in the prediction loop so that it requires decoding process changes.

Figure 2. Adaptive color space coding at the decoder.

Please note that in the original implementation, normalization during YCoCgRGB conversion was not considered. A fix to consider this is presented in a separate document JCTVC-Q0213, which shows significant coding gains.

# Syntax and semantics description

All the modifications are based on JCTVC-P1005_v1.

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) { |  |
| intra_bc_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_skip_flag[ x0 ][ y0 ] ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( !intra_bc_flag[ x0 ][ y0 ] && palette_coding_mode_enabled_flag ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && !palette_mode_flag[ x0 ][ y0 ] ) |  |
| dictionary_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] && !palette_mode_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( intra_bc_flag[ x0 ][ y0 ] && !intra_bc_skip_flag[ x0 ][ y0 ] ) ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && !palette_mode_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( PartMode == PART_2Nx2N ) { |  |
| intra_bc_merge_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_bc_merge_flag[ x0 ][ y0 ]) |  |
| intra_bc_merge_index[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( !intra_bc_merge_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } |  |
| } else if (palette_mode_flag[ x0 ][ y0 ] ) { |  |
| palette_mode( x0, y0, nCbs, nCbs) |  |
| } else if (dictionary_mode_flag[ x0 ][ y0 ] ) { |  |
| dictionary_mode( x0, y0, log2CbSize) |  |
| } else { |  |
| if( adaptive_color_space_enable_flag) |  |
| cu_color_space[ x0 ][ y0 ] | ae(v) |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] && !palette_mode_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ !( PartMode = = PART_2Nx2N && intra_bc_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.9 Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| if( refList == 2 ) |  |
| intra_bc_flip_flag[ x0 ][ y0 ] | ae(v) |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

7.3.8.X Palette mode syntax

| palette_mode( x0, y0, CbWidth, CbHeight ) { | Descriptor |
| --- | --- |
| for( cIdx = 0; cIdx < ( ChromaArrayType == 0 ) ? 1 : 3; cIdx++ ) { |  |
| color_table_share_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( !color_table_share_flag[ x0 ][ y0 ][ cIdx ] ) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ x0 ][ y0 ][ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] ) { |  |
| major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType == 3 ) ? 1 : 3; cIdx++ ) { |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][ i ] | ae(v) |
| if( line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE ) { |  |
| ( cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 ) ? 3 : 1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ 0 ] | ae(v) |
| } |  |
| } else if( line_mode[ x0 ][ y0 ][ cIdx ][ i ] == VER_MODE ) { |  |
| for( j = 0; j < CbWidth; j++ ) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 ) ? 3 : 1; cSubIdx++ ) { |  |
| ColorIndex[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] = ColorIndex[ x0 ][ y0 ][ cIdx + cSubIdx ][ i-1 ][ j ] |  |
| } |  |
| } |  |
| } else { //NOR_MODE |  |
| for( j = 0; j < CbWidth; j++ ) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| if( indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] != TOP_PRED && indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] != LEFT_PRED && indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] != TOP_LEFT_PRED && indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] != TOP_RIGHT_PRED &&) { |  |
| index_pred_transition[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| if( !index_pred_transition[ x0 ][ y0 ][ cIdx ][ i ][ j ] ) { |  |
| for(cSubIdx=0; cSubIdx < ( ChromaArrayType == 3 ) ? 3 : 1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| for( j = 0; j < CbWidth; j++ ) { |  |
| for( cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 ) ? 3 : 1; cSubIdx++ ) { |  |
| if( color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ] ) { |  |
| quant_escape_value[ cIdx + cSubIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.X Dictionary mode syntax

| dictionary_mode( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| nCbS = ( 1 << log2CbSize ) |  |
| totalPixel = nCbS*nCbS |  |
| currPixel = 0 |  |
| currIndex = 0 |  |
| dictionary_type_flag[ x0 ][ y0 ] | ae(v) |
| dictionary_scan_flag[ x0 ][ y0 ] | ae(v) |
| while (currPixel < totalPixel ) { |  |
| dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] | ae(v) |
| if( dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] ) { |  |
| if( dictionary_type_flag[ x0 ][ y0 ] == 0 ) { |  |
| dictionay_pred_offset_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } else { |  |
| dictionay_pred_offsetX[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dictionay_pred_offsetY[ x0 ][ y0 ][ currIndex ] | ae(v) |
| } |  |
| dictionary_pred_length_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| currPixel += (dictionary_pred_length_minus1 [ x0 ][ y0 ][ currIndex ] + 1) |  |
| } else { |  |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 0 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 1 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 2 ] | ae(v) |
| currPixel += 1 |  |
| } |  |
| currIndex += 1 |  |
| } |  |
| } |  |

7.4.9.5 Coding unit semantics

intra_bc_skip_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in skipped intra block copying mode (intra block copying with PartMode equaling to PART_2Nx2N and without any residue). intra_bc_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in skipped intra block copying mode. When not present, the value of intra_bc_skip_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in intra block copying mode. intra_bc_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded according to pred_mode_flag. When not present, when intra_bc_skip_flag[ x0 ][ y0 ] is equal to 1, the value of intra_bc_flag is inferred to be equal to 1. Otherwise, the value of intra_bc_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in palette mode. When palette_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_mode_flag [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode. dictionary_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in dictionary mode. When palette_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_merge_flag[ x0 ][ y0 ] equal to 1 specifies that the block vector current prediction unit is coded in merge mode (merge with the left block vector or the above block vector). intra_bc_merge_flag[ x0 ][ y0 ] equal to 0 specifies that the block vector of current prediction unit is not coded in merge mode. When not present, the value of intra_bc_merge_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_merge_index[ x0 ][ y0 ] equal to 1 specifies that the block vector of current prediction unit is the same as the above block vector. intra_bc_merge_index[ x0 ][ y0 ] equal to 0 specifies that the block vector of current prediction unit is the same as the left block vector. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

cu_color_space[ x0 ][ y0 ] specifies the color space used for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

7.4.9.9 Motion vector difference semantics

intra_bc_flip_flag [ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is predicted in vertically flipped mode. intra_bc_flip flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is predicted in normal mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

7.4.9.X Palette mode semantics

color_table_share_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that the current coding unit share the same color table with the last decoded coding unit for color component cIdx. color_table_share_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the color table should be transmitted for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The array index cIdx specify the color components.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies the n-th major color value for color component cIdx could be predicted from the above or left CU. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] specifies the prediction direction of the n-th major color for color component cIdx. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies that the current major color is the same as the corresponding major color in the above CU. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies that the current major color is the same as the corresponding major color in the left CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_mode[ x0 ][ y0 ][ cIdx ][ i ] equal to HOR_MODE specifies that the i-th pixel line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][ cIdx ][ i ] equal to VER_MODE specifies that the i-th pixel line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ cIdx ][ i ] equal to NOR_MODE specifies that the i-th pixel line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current pixel in position ( i, j ). index_pred_symbol[ x0 ][ y0 ][ i ][ j ] equal to TOP_PRED specifies that index value of the current pixel is the same as the above one. index_pred_symbol[ x0 ][ y0 ][ i ][ j ] equal to LEFT_PRED specifies that index value of the current pixel is the same as the left one. index_pred_symbol[ x0 ][ y0 ][ i ][ j ] equal to TOP_LEFT_PRED specifies that index value of the current pixel is the same as the top-left one. index_pred_symbol[ x0 ][ y0 ][ i ][ j ] equal to TOP_RIGHT_PRED specifies that index value of the current pixel is the same as the top-right one.

index_pred_transition[ x0 ][ y0 ][ cIdx ][ i ][ j ] equal to 1 specifies that transition copy is used for the prediction of component cIdx at the location ( i, j ). index_pred_transition[ x0 ][ y0 ][ cIdx ][ i ][ j ] equal to 0 specifies that transition copy is not used.

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape pixel value for color component cIdx at the location ( i, j ).

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the pixel at the location( i, j ) for color component cIdx.

7.4.9.X Dictionary mode semantics

dictionary_type_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the dictionary maintained by the decoder. dictionary_type_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in dictionary mode and the prediction comes from the previously reconstructed pixels belonging to the same picture. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_scan_flag[ x0 ][ y0 ] equal to 0 specifies that the horizontal scanning is applied to current coding unit. dictionary_scan_flag[ x0 ][ y0 ] equal to 1 specifies that the vertical scanning is applied to current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 1 specifies the prediction mode is applied to the i-th decoding sub-string in the current CU. dictionay_pred_mode_flag[ x0 ][ y0 ][ i ] equal to 0 specifies the direct mode is applied to the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offset_minus1[ x0 ][ y0 ][ i ] plus 1 specifies the offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetX[ x0 ][ y0 ][ i ] specifies the horizontal offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionay_pred_offsetY[ x0 ][ y0 ][ i ] specifies the vertical offset of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_pred_length_minus1[ x0 ][ y0 ][ i ] plus 1 specifies the length of the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

dictionary_direct_component[ x0 ][ y0 ][ i ][ comp ] specifies the pixel value of the comp-th color component in the i-th decoding sub-string in the current CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

It should be noted that all the new added tools can have corresponding enabling/disabling flags at the higher level (SPS/PPS/slice header).

# Compression performance discussion

The implementation is based on HM-12.1+RExt-5.1. The executable files were compiled by Microsoft Visual Studio 2013, 64 bit. The decoding were performed on a single machine with the same configuration. When determining the decoding time, the MD5 checking and file writing are disabled. The running time is measured with the same method used in HM-12.1+RExt-5.1. The QP refinement according to lambda value in HM-12.1+RExt-5.1 is enabled for lossy coding cases.

## Lossy test condition

The bit rate of the anchor is used as the target of the test. The (basic) QP (and the associated lambda value) is changed at most once (from QP to QP+1) in the encoding process (according to encoding order) to match the anchor bit rate. However, we observed that there are many points with far lower bit rates than the anchor because lossless coding is already achieved at a lower bit-rate. The PSNR value is clipped with the maximum value of 58.92 dB on a picture by picture basis. The average results provided by the CfP lossy coding template is shown below.

Table 1. Summarized BD-Rate results of the proposed scheme compared with the anchor for lossy coding.

It should be noted that there are many “#VALUE!” entries in the table, which means that the template fails to report a valid number for the given input. There are some cases with all four points (or three points, two points) are lossless coded, which means that all the four points are the same. In this case, the template fails to report a valid BD-Rate number, e.g., Desktop RGB under AI coding structure. Even there are numbers shown, we don’t think that the numbers reported by the template are reliable, because

- When matching the bit rate specified in the CfP, the PSNRs of some points are improved significantly. Thus, the PSNR overlapping range between the anchor the test is small, if there is an overlap.

- The BD-Rate results reported by the template are 0.0% for some obviously bit saving cases. For example, the BD-Rate for the R-D curves shown in figure 3 is reported as 0.0%.

Figure 3. R-D curves of the Y component of WebBrowsing 444 under LB coding structure.

We report and analyze these issues in a separate contribution, JCTVC-Q0053.

According to the above observations, we believe that the current calculation of BD-Rate is not proper. However, considering that all the test points are targeting at the corresponding anchor bit rates, it might be more reasonable to calculate the PSNR difference between the anchor and the test. We show the summarized results by measuring PSNR difference for each test point.

Table 2. Summarized PSNR difference results of the proposed scheme compared with the anchor for lossy coding.

We note that screen contents may be captured without any noises; and coding errors can concentrate on several pixels. Thus, it is more reasonable to use a higher number than the maximum potential PSNR (e.g. only one pixel with only one pixel level’s difference, e.g. 10*log10(255*255*1920*1080) = 111.30 dB for 1080p sequences) as the PSNR if one picture is lossless coded. We suggest to use 114.31dB to clip PSNR numbers. The derivation this number and reasons are described in JCTVC-Q0053. The summarized PSNR difference results using 114.31dB for clipping are shown in the table below.

Table 3. Summarized PSNR difference results of the proposed scheme compared with the anchor for lossy coding (clipped with 114.31dB).

## Mathematically lossless test condition

The summarized lossless coding results are shown in the following table.

Table 4. Summarized rate-saving results of the proposed scheme compared with the anchor for lossless coding.

# Complexity analysis

The analysis in this section is relative to the design of HM-12.1+RExt-5.1. Besides the scheme we show in above sections, we also developed several versions of fast encoders (without changes to the decoder), which show a flexible trade-off between encoding complexity and coding efficiency for screen contents. A summary is illustrated in subsection 5.21 and details can be seen in another document JCTVC-Q0052.

## Encoding time and measurement methodology

The encoding time is measured using the same method as HM-12.1+RExt-5.1.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

The decoding time is measured using the same method as HM-12.1+RExt-5.1. The MD5 checking and the reconstruction output are disabled during decoding.

## Description of computing platform used to determine encoding and decoding times reported in sections  and 5.2

The executable files were compiled by Microsoft Visual Studio 2013, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory |
| --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB |

The decoding were performed on a single machine with the same configurations.

## Expected memory usage of encoder

Additional memory to build hash tables is required at the encoder side. The additional memory requirement has been analyzed in Section 2 for each individual tool. For 1080p sequence with 4 reference pictures, about additional 1.5G Bytes memory is required at the encoder side.

## Expected memory usage of decoder

Additional memory to maintain dictionaries is required at the decoder side. In the implementation of the CfP response, about 2*(1<<18)*2*3 = 3M Bytes memory is required. If the size of the dictionary is reduced, the additional memory requirement will also be reduced. The additional memory required by the other tools is expected to be marginal.

## Complexity characteristics of encoder motion estimation and partitioning selection

Some encoder optimizations are included. The start point of motion estimation is refined and hash matching based fast encoding algorithms are used. In short, if an exact match is found in previous pictures, the following mode checking process may be skipped to save then encoding time. Please find more details in Section 2.3.

## Complexity characteristics of decoder motion compensation

Same as the anchor.

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection

Same as the anchor.

## Complexity characteristics of decoder intra-frame prediction operation

Same as the anchor.

## Complexity characteristics of encoder inter-component prediction tools

The changes introduced by r3815 (RExt__P0154_ADDITIONAL_CONTEXT_FOR_CCP) and r3841 (Fixed single-component-loop inter search for lossless coding) in HM-range-extensions are incorporated in the software used to response call for proposal. The complexity influence is expected to be marginal.

## Complexity characteristics of decoder inter-component prediction tools

The changes introduced by r3815 (RExt__P0154_ADDITIONAL_CONTEXT_FOR_CCP) and r3841 (Fixed single-component-loop inter search for lossless coding) in HM-range-extensions are incorporated in the software used to response call for proposal. The complexity influence is expected to be marginal.

## Complexity characteristics of transforms specific in screen content coding

Same as the anchor.

## Complexity characteristics of encoder related to other tools specific in screen content coding

### Complexity of hash table maintenance

To support hash based Intra BC search and hash based motion estimation, a hash table needs to be built. For Intra BC, the hash table is built for every 16x16, 8x8, and 4x4 blocks. For inter motion estimation, the hash table is built for every 64x64, 32x32, 16x16, and 8x8 blocks. The hash table is built based on original pixels, so the hash table for 8x8 and 16x16 blocks used for intra BC can be reused for inter motion estimation.

For every block, the hash values (CRC) for every row are calculated first. Then the hash values for every row are grouped together and then a new hash value is calculated, which is used as the hash value of the whole block. Thus, when the block moves one row below, most of the intermediate hash values could be reused.

To avoid too many blocks having a same hash value, some blocks are excluded from the hash table. The blocks which are uniform in horizontal or vertical are excluded from the hash table. Because these blocks may be predicted very well using horizontal prediction or vertical prediction.

### Complexity of intra BC search

Besides lossy intra BC estimation, hash based search is also applied to find exact matching blocks based on original pixels.

### Complexity of dictionary search

Hash based encoder decision is applied. Four hash tables with different matching length (1, 2, 4, and 8) are built for the searching process of dictionary mode. More details could be found in Section 2.7.1

### Complexity of palette mode

Two modes are introduced in palette mode. The first one is share mode, the color table of the last CU coded in palette mode will be reused. The second one is a new mode, the color table will be calculated according to the color histogram. The encoding time increased by palette mode is not significant.

## Complexity characteristics of decoder related to other tools specific in screen content coding

### Complexity of modifications to intra BC mode

No significant change of decoding complexity on intra BC mode. If vertical flipping is applied, the decoder should use the flipped region as prediction.

### Complexity of dictionary mode

Only pixel copy is necessary at the decoder for the CUs using dictionary mode.

### Complexity of palette mode

The complexity influence is expected to be marginal.

## Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding

Similar as the anchor (some new syntax elements are added using CABAC coding).

## Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding

Similar as the anchor (some new syntax elements are added using CABAC coding).

## Degree of capability for encoder parallel processing

Same as the anchor.

## Degree of capability for decoder parallel processing

Same as the anchor.

## Complexity characteristics of decoder for adaptive color space coding.

If the color space is used for the current (Intra) CU is not the main color space (e.g., GBR), the decoder needs to convert the surrounding pixels used for prediction (at most 4N+1) pixels to the current color space. Except using YCoCg color space, only reordering operation is introduced here. After the reconstruction process (before loop filters) of current CU, the decoder needs to convert the pixels back to the main color space before storing.

## Complexity characteristics of encoder for adaptive color space coding

Similar conversion operations are required at the encoder side. The encoder performs RDO on different color space to select the best color space for the current CU.

## Trade-off between encoding complexity and coding efficiency

Various encoding strategies are developed to provide different trade-off between encoding complexity and efficiency. The details are described in a separate contribution, JCTVC-Q0052. To illustrate the trade-off, we use fixed QP coding with different complexity settings for the proposed scheme. The QP is the same as that of the anchor. Figure 3 and 4 summarize the trade-off for lossy and lossless coding. It can be seen that the proposed scheme can either provide a high coding efficiency, e.g., about 30% bit-saving compared with the anchor, or show a low encoding complexity, e.g., about 1/3 encoding time of the anchor, while still with significant coding gains. For reference, we also show RExt6.0 in the figures. It should also be noted that for RA and LB cases, the proposed scheme always shows much less encoding time compared with the anchor.

Figure 3. Encoding time vs. bit-saving for lossy coding.

Figure 4. Encoding time vs. bit-saving for lossless coding.

# Software implementation description

The software was implemented based on HM-12.1+RExt-5.1. The software was written in C++ and compiled by Visual Studio 2013, in 64-bit. The software is also compliable using gcc 4.6.3.

# Highlighted aspects discussion

We present our design for screen content coding in this document. The proposed design has the following features.

- High coding efficiency. The proposed scheme can achieve lossless coding even with lower bit rate than that of any lossy points of the anchor.

- Low encoding complexity. For inter coding, the proposed scheme shows less encoding time.

- Trade-off between encoding complexity and coding efficiency. Different trade-off settings have been show. An example illustrates significant coding gains can also be achieved with about 1/3 encoding time compared with the anchor.

- HEVC range extension based design and implementation. Thus, the proposed scheme is consistent with the HEVC design.

# Closing remarks

This document describes our coding scheme for screen content. In addition, the contribution JCTVC-Q0052 shows several encoding settings to achieve different trade-offs between encoding complexity and coding efficiency. JCTVC-Q0053 reports some issues of comparison with the anchor during our development of the proposed scheme. It is suggested to use the proposed scheme or design factors for further development of potential extension of HEVC for screen content.

# References

[1] ISO/IEC JTC 1/SC 29/WG 11 document N14175 and ITU-T Q6/16 document VCEG-AW90, “Joint Call for Proposals for Coding of Screen Content”, Jan. 2014, San Jose, USA.

[2] ISO/IEC JTC 1/SC 29/WG 11 document N14174, “Requirements for an extension of HEVC for coding of screen content”, Jan. 2014, San Jose, USA.

[3] JCT-VC document JCTVC-E145, “Intra and inter coding tools for screen contents,” Mar. 2011, Geneva, Switzerland.

[4] JCT-VC document JCTVC-P0214, “Screen content coding using dictionary based mode,” Jan. 2014, San Jose, USA.

[5] JCT-VC document JCTVC-O0357, “Screen content coding using 2-D dictionary mode,” Oct. 2013, Geneva, Switzerland.

[6] JCT-VC document JCTVC-O0182, “AHG8: Major-color-based screen content coding,” Oct. 2013, Geneva, Switzerland.

[7] JCT-VC document JCTVC-P0108, “RCE4: Test1. Major-color-based screen content coding,” Jan. 2014, San Jose, USA.

# Patent rights declaration(s)

Microsoft Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).