| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0036_r2 |
| --- | --- |

| Title: | Description of screen content coding technology proposal by Mitsubishi Electric Corporation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Xingyu Zhang Anthony Vetro Kazuo Sugimoto Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: Tel: Email: | +1-617-621-7500 cohen@merl.com xzhang@merl.com avetro@merl.com sugimoto@merl.com +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp Murakami.Tokumichi@eb.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This document presents specifications of a new video coding algorithm developed for submission as a response to the Joint Call for Proposals for Coding of Screen Content issued by ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6. The proposed algorithm builds upon HEVC Range Extensions Draft 6 by adding several coding tools targeting objective and subjective performance for coding screen content video. These new tools include block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, an independent uniform prediction mode, and a palette mode. Specific examples of subjective improvements are presented. For objective performance, for lossy test conditions, average gains over the anchor for various content types are up to 27% for the G component of RGB sequences and up to 26% for the Y component of YCbCr, for All Intra conditions. For Random Access conditions, the corresponding averages were up to 17% for G and 14% for Y, and for Low Delay conditions, the averages were up to 10.2% for G and 8.1% for Y. The gains on individual sequences were up to 32%, 29% and 19% for the G component for AI, RA, and LD conditions respectively, and up to 31%, 25% and 11% for the Y component. For lossless conditions, average bit-rate savings for RGB sequences were up to 32%, 28% and 28% for AI, RA, and LD conditions respectively, and for YCbCr sequences average bit-rate savings were up to 30%, 20% and 17%. Maximum bit-rate savings were up to 43%, 40% and 40% for RGB AI, RA, and LB conditions, and 46%, 32% and 24% for YCbCr.

# Contents

Abstract	1

Contents	2

1	Introduction	3

2	Algorithm description	3

2.1	Inter-component prediction	3

2.2	Histogram correction mode for Sample Adaptive Offset	6

2.3	Independent uniform prediction mode	8

2.4	Palette mode	11

3	Syntax and semantics description	12

4	Compression performance discussion	18

4.1	Performance for bitstreams submitted for subjective testing	18

4.2	Objective performance results	19

4.2.1	Lossy test condition	19

4.2.2	Mathematically lossless test condition	20

5	Complexity analysis	21

5.1	Encoding time and measurement methodology	21

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM 12.1_RExt 5.1	22

5.3	Description of computing platform used to determine encoding and decoding times reported in sections 5.1 and 5.2	22

5.4	Expected memory usage of encoder	22

5.5	Expected memory usage of decoder	22

5.6	Complexity characteristics of encoder motion estimation and partitioning selection	22

5.7	Complexity characteristics of decoder motion compensation	22

5.8	Complexity characteristics of encoder intra-frame prediction type and partitioning selection	22

5.9	Complexity characteristics of decoder intra-frame prediction operation	22

5.10	Complexity characteristics of encoder inter-component prediction tools	23

5.11	Complexity characteristics of decoder inter-component prediction tools	23

5.12	Complexity characteristics of transforms specific in screen content coding	23

5.13	Complexity characteristics of encoder related to other tools specific in screen content coding	23

5.13.1	Inter-component prediction	23

5.13.2	Histogram correction mode for Sample Adaptive Offset	23

5.13.3	Independent uniform prediction mode	23

5.14	Complexity characteristics of decoder related to other tools specific in screen content coding	23

5.14.1	Inter-component prediction	23

5.14.2	Histogram correction mode for Sample Adaptive Offset	24

5.14.3	Independent uniform prediction mode	24

5.15	Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding	24

5.16	Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding	24

5.17	Degree of capability for encoder parallel processing	24

5.18	Degree of capability for decoder parallel processing	24

6	Software implementation description	24

7	Highlighted aspects discussion	24

8	Closing remarks	24

9	Patent rights declaration(s)	25

References	25

Appendix	26

# Introduction

In this contribution, a new video coding algorithm is proposed as a response to the Joint Call for Proposals for Coding of Screen Content (SCC CfP) issued by ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6 [1]. The proposed algorithm is developed on top of HEVC Range Extensions Draft 6 [2] specification by adding several coding tools targeting objective and subjective performance improvement for coding screen content video. These new tools, which target both the objective and subjective improvement of the coding system, include block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, an independent uniform prediction mode, and a palette mode. HEVC Range Extensions Draft 6 also contains several tools which improve the performance over HEVC Range Extensions Draft 5 [3] when coding screen content material. These tools are enabled along with the proposed new tools.

This document is organized as follows. Section 2 describes the detailed algorithm of our newly added coding tools. It should be noted that other coding tools which are not described in Section 2 are identical to HEVC Range Extensions Draft 6. Section 3 shows the changes in syntax and semantics for the proposed tools. Section 4 reports the coding performance of the proposed algorithm under both lossy and lossless conditions defined in the Call for Proposals. Section 4 discusses aspects related to the complexity of the new tools. Section 5 briefly summarizes how the software was implemented, followed by the remaining sections highlighting points for discussion and making closing remarks.

# Algorithm description

The HM-13.0_RExt-6.0 software [8] was used as a starting point for this proposal. Compared to the HM-12.1_RExt-5.1 software used to generate the anchor data for the SCC CfP, RExt-6.0 already contains tools that improve the performance when coding screen content material, such as the NxN, 2NxN, and Nx2N modes for intra block copy, and categorized Rice parameter initialization. Additionally, the tools already in HEVC Range Extensions Draft 5 that target screen content material remain enabled. Some examples of these tools include Transform Skip improvements, cross-component prediction, the rotation of Transform Skip blocks, and RDPCM. We used the configuration files that accompanied the HM-13.0_RExt-6.0 software without changing any parameters associated with the existing tools. This proposal includes four additional tools: Inter-component prediction (ICP), histogram correction (HC) mode for SAO, independent uniform prediction (IUP), and palette mode.

## Inter-component prediction

In the current specification of the Range Extensions, cross-component prediction for the residual signal is adopted. This prediction reduces inter-component redundancy in the residual domain. Figure 1 shows an example of the usage distribution of cross-component prediction (CCP) for low QP and high QP cases of the CfP anchor. In this figure, green blocks indicate the use of CCP for both B (Cb) and R (Cr) components. Blue blocks and red blocks indicate the use of CCP only for B (Cb) and R (Cr) components, respectively. According to this figure, the occupancy rate of CCP-selected areas is lower in high QP case. This result shows the prediction performance of CCP highly depends on the degree of quantization distortion. On the other hand, we also have proposed an inter-component prediction technique that may relax such quantization dependencies by exploiting inter-component correlation in the prediction domain rather than the residual domain [4], [5]. Therefore, to further reduce inter-component redundancy for screen content material, the proposed scheme introduces a chroma prediction mode using the correlation between two components. The proposed prediction is available for PUs applying intra prediction or intra block copy for luma component and conducts for both B (Cb) and R (Cr) components. Figure 2 shows the PU-level decoding process of the proposed scheme for B (Cb) and R (Cr) components.

(a) QP=27                                         (b) QP=42

Figure 1. Distribution of cross-component prediction in CfP anchor

Figure 2. Decoding process of the proposed scheme for each PU

According to a flag for each PU indicating whether to apply intra prediction or intra block copy, the proposed scheme selects whether the inter-component prediction (ICP) is turned on or not. Moreover, to select the best reference block from decoded color components, a flag indicating which color component is selected as a reference for prediction is introduced for the R (Cr) component.

ICP generates the prediction samples predC[x, y] using the following formula which is known as “LM chroma” prediction described in JCTVC-M0097 [6]:

predC[x,y]=α×PR[x,y]+β,	(1)

where PR indicates reference component samples. The scaling parameter α and the offset β is derived as follows:

α=R(PR,PC)R(PR,PR)β=M(PC)−α×M(PR).	(2)

PC indicates reconstructed samples around the block to be predicted, and M(A) represents the mean of A, and R(A,B) is defined as follows:

R(A,B)=M(A−M(A))×(B−M(B) .	(3)

The prediction sample generation process shown above is conducted for each TU and PU for the intra prediction CU and intra block copy CU, respectively. It is noted that the derivation process of the scaling parameter α and the offset β is performed for each CU not PU for intra block copy CU.

When combined with CCP for residual samples, the proposed scheme only uses either ICP or CCP at same position to reduce the amount of coding bits related to these predictions, as well as to reduce the complexity at the decoder side.

To illustrate the effects of the proposed ICP method, the distributions of ICP and CCP are shown in Figure 3 and Figure 4, respectively. In these figures, green blocks indicate to use the corresponding prediction for both B (Cb) and R (Cr) component. Blue blocks and red blocks indicate to use the corresponding prediction only for B (Cb) and R (Cr) component, respectively. These figures show that ICP works better than CCP for some sequences using the existing RDO (Rate Distortion Optimization)-based mode decision framework. Moreover, ICP tends to be selected in plane areas and soft edge texture regions, whereas CCP tends to be used at sharp edge areas like text.

(a) The proposed inter-component prediction            (b) Cross-component prediction in RExt

Figure 3. Distribution of selected prediction mode for sc_SlideShow_1280x720_20_8bit_500.rgb (QP=37)

(a) The proposed inter-component prediction              (b) Cross-component prediction in RExt

Figure 4. Distribution of selected prediction mode for sc_robot_1280x720_30_8bit_300.rgb (QP=37)

## Histogram correction mode for Sample Adaptive Offset

One of the most distinguishable characteristics of screen content as compared to camera-captured content is that the histograms of pixel values for screen content have distinct and sparse peaks respectively, as shown in Figure 5 (a). Figure 5 (b) also shows a histogram of decoded screen content, which has some small peaks around the original peak. These small peaks correspond to coding distortion such as mosquito noise, which should be removed for improving the decoded image quality.

Figure 5. Histogram of screen content calculated from (a) uncompressed image, and (b) compressed image

In order to remove the noise which can be observed in a histogram without deviating from the current RExt 6, we modified the existing Sample Adaptive Offset (SAO) process. We added a new mode, Histogram Correction (HC), as an alternative to the existing Band Offset (BO) and the Edge Offset (EO) modes, to recover the sparsity of each peak in a histogram. Figure 6 illustrates the noise removal mechanism in HC. Let p be the pixel value of a peak which is to be corrected. To recover the sparsity of the peak, we replace all the pixel values between (p - w) and (p + w) with p. Note that w, which is the most important parameter for controlling the effectiveness of HC, represents the pixel value range which is replaced with p. If we set w too small, the image quality improvement by HC becomes negligible. If we set w too large, the HC may cause severe artifacts. So in the HC mode, the encoder optimizes the pixel value range w for each distinct peak in the histogram, and then it transmits the parameters to the decoder. Although the number of peaks to be corrected is also an important parameter, we decided to apply the noise removal to the highest four peaks in the histogram to align with the current SAO implementation. That is, the encoder decides the optimal w for each of the four highest histogram peaks and then encodes the four values in the same manner as how the current SAO encodes the four offsets of BO or EO. The proposed encoder optimizes w in the range of 0 to 15 by minimizing the difference between the original and decoded images, and it selects the best mode from BO, EO and HC based on RDO.

Figure 6. Histogram correction

Figure 7. Filtering process of SAO HC at a decoder

The basic filtering process of SAO HC at a decoder is shown in Figure 7. The decoder extracts the pixel value ranges from the bitstream and then calculates the histogram of the decoded block. The highest peak in the histogram is first corrected as shown in Figure 6. Then, the decoder moves to the next peak, and repeats the correction process until four peaks are corrected.

If the current CTU reuses the SAO parameters of a neighboring CTU (sao_merge_left_flag = 1 or sao_merge_up_flag = 1), and if the neighboring CTU selects the HC, then the current CTU refers four pairs of peak positions and their ranges for noise removal (i.e. set of pi and wi in Figure 7) used in the neighboring CTU, and then replaces the pixel values around the four peaks. Since the peak positions are already known in this case, it is not necessary to generate the histogram at a decoder. It should be noted that Figure 7 implies that a decoder always needs the histogram calculation, since it only shows the case when the new SAO parameters are sent from an encoder, for simplicity.

Figure 8. Decoded images using the original SAO (left) and the proposed SAO (right)

Figure 8 compares the subjective quality of decoded images using the original SAO and using the proposed SAO with HC. The left and right images are both encoded by the proposed encoder under all intra condition where QP is set to 42, but only SAO is reverted to the original one (i.e. RExt 6) in the left image. These comparisons clearly demonstrate that the proposed SAO with HC can significantly reduce the coding distortion and improve the image quality.

## Independent uniform prediction mode

The prediction modes currently available in HEVC-RExt are based primarily on data available from nearby previously-decoded blocks. Intra-block directional prediction is based upon adjacent pixels from neighboring blocks. Intra block copy mode and motion estimation for Inter-block coding applies a limited-range search to nearby pixels in the current frame or in a picture reference buffer. Unlike typical camera-captured videos, video from a computer, or screen content video, often follows a consistent theme. This theme defines graphical elements such as background color, window colors and border shapes, etc. Most of the existing Intra prediction schemes, due to their localized prediction process, cannot exploit the commonalities in a theme globally over the entire picture or sequence of pictures, due to the localized nature of the Intra and Inter prediction process. Recently proposed palette modes exploit the reduced number of colors that screen content pictures typically contain, however, the palette is adapted while coding the picture, and in recent implementations, is reset when starting each row of CTBs of a slice. This local adaptation is useful when the palette or set of used colors is consistent over an entire row of CTBs or picture, but when a picture comprises several similar but distant regions, the palette may not be optimal when entering each region. The purpose of the proposed independent uniform prediction mode is to compute a small set of global block predictors to target regions of a picture that have a limited number of commonly-occurring colors that are common in a computing environment, e.g. foreground and background colors. This mode applies to blocks in both Inter and Intra slices.

In the slice encoding process, before CTUs in a slice are being encoded, a histogram of the pixel values (RGB or YCbCr triplets) is maintained. To limit the complexity of this process, only the center pixel from each CTU is used to compute the histogram. Before encoding the slice, these triplets form the set S1. The num_uniform_colors most frequently occurring triplets are saved as set S2. The number of colors num_uniform_colors and the color triplet values are signaled in the slice header.

In the slice decoding process, the num_uniform_colors and the color triplet values are decoded from slice header. The CU-level decoding process is shown in Figure 9. A flag independent_uniform_pred_flag is parsed from the bitstream to indicate whether the independent uniform prediction mode is used for the current CU. If independent_uniform_pred_flag is equal to 1 and num_uniform_colors >1, then uniform_color_index is parsed from the bitstream to indicate which color triplet from set S2 is used in the current CU. If independent_uniform_pred_flag is true and num_uniform_colors ==1, then uniform_color_index is inferred to be 1.

Figure 9. CU decoding process for independent uniform prediction

For predicting a CU using the independent uniform prediction mode, a uniform predictor block is formed for each color component, where all pixels in the predictor block have the same value as the corresponding element in the selected color triplet. This predictor block is then treated the same way as in other prediction methods, where the predictor is added to the reconstructed residual block to form the reconstructed block.

Because the independent uniform prediction mode targets very commonly used colors used over an entire picture, we limited num_uniform_colors in this proposal to 2. The palette mode, also included in this proposal, includes a larger number of colors for a more localized coding approach. Examples of independent uniform prediction when used with two colors are shown in Figure 10 and Figure 11. The blocks that predicted by independent uniform prediction mode are highlighted in pink, blocks that predicted by the Intra Block Copy mode are highlighted in blue, and blocks that predicted by Palette mode are highlighted in yellow. Figure 8 is from the Desktop sequence. It has large areas with a predominantly white background, and other large areas with a black background. The two color triplets selected during the encoding process successfully matched these two background colors, so this mode was used for many blocks over different regions in the picture. Figure 9 is from WebBrowsing sequence. For this particular sequence, the white background in the RGB format picture is (254, 255, 254) instead of (255, 255, 255). Based on a slice-level estimation of the frequently occurring color triplets, the independent uniform prediction mode is able to adapt to the sequence content. For reference, thumbnails of the Desktop and WebBrowsing sequences are shown in Figure 12.

Figure 10. Example of two-color independent uniform prediction on Desktop, 1st frame

Figure 11. Example of two-color independent uniform prediction on WebBrowsing, 1st frame

Desktop

WebBrowsing

Figure 12. Thumbnails of Desktop and WebBrowsing sequences

## Palette mode

The proposed encoder includes a palette mode based on JCTVC-P0303. For details, please refer to JCTVC-P0303 [7]. The software and algorithms included in JCTVC-P0303 are currently being studied in an ad hoc group. For this proposal, we integrated the palette mode software mostly unchanged. Thus, any future improvements to the palette mode algorithms and software can be easily incorporated into the software used for this proposal.

# Syntax and semantics description

This section provides the combined changes to the HEVC Range Extensions Draft 6 syntax and semantics, for the block-level inter-component prediction, the histogram correction mode for Sample Adaptive Offset, and the independent uniform prediction mode. Text for the palette mode can be found in JCTVC-P0303.

7.3.2.2 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| inter_comp_pred_enabled_flag | u(1) |
| independent_uniform_pred_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| If(independent_uniform_pred_enabled_flag) { |  |
| num_uniform_colors | u(v) |
| for( i = 0; i < num_uniform_colors; i++) |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType != 0 ? 3 : 1 ); cIdx++ ) |  |
| uniform_colors[ cIdx ][ i ] | ae(v) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

7.3.8.3 Sample adaptive offset syntax

| sao( rx, ry ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSliceSeg = CtbAddrInRs > SliceAddrRs |  |
| leftCtbInTile = TileId[ CtbAddrInTs ] = = TileId[ CtbAddrRsToTs[ CtbAddrInRs − 1 ] ] |  |
| if( leftCtbInSliceSeg && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSliceSeg = ( CtbAddrInRs − PicWidthInCtbsY ) >= SliceAddrRs |  |
| upCtbInTile = TileId[ CtbAddrInTs ] = = TileId[ CtbAddrRsToTs[ CtbAddrInRs − PicWidthInCtbsY ] ] |  |
| if( upCtbInSliceSeg && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType != 0 ? 3 : 1 ); cIdx++ ) |  |
| if( ( slice_sao_luma_flag && cIdx = = 0 ) ¦ ¦ ( slice_sao_chroma_flag && cIdx > 0 ) ) { |  |
| if( cIdx = = 0 ) |  |
| sao_type_idx_luma | ae(v) |
| else if( cIdx = = 1 ) |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| if( ( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 1 ) ¦ ¦ ( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 2 ) ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| if( ( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) ¦ ¦ ( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 2 ) ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 1 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } else { |  |
| if( cIdx = = 0 ) |  |
| sao_eo_class_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_eo_class_chroma | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( independent_uniform_pred_enabled_flag && num_uniform_colors > 0) |  |
| independent_uniform_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( independent_uniform_pred_ flag && num_uniform_colors > 1 ) |  |
| uniform_color_index[ x0 ][ y0 ] | ae(v) |
| else { |  |
| ... |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| if( ChromaArrayType = = 3 ¦¦ inter_comp_pred_enabled_flag) { |  |
| pbOffsetX = ( PartMode = = PART_NxN ¦¦ PartMode = = PART_Nx2N) ? ( nCbS / 2 ) : nCbS |  |
| pbOffsetY = ( PartMode = = PART_NxN ¦¦ PartMode = = PART_2NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffsetY ) |  |
| for( i = 0; i < nCbS; i = i + pbOffsetX ) { |  |
| inter_comp_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if(inter_comp_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| inter_comp_ref_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| { |  |
| if(inter_comp_pred_enabled_flag) |  |
| inter_comp_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if(inter_comp_pred_flag[ x0 + i ][ y0 + j ]) |  |
| inter_comp_ref_flag[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| ... |  |
| } |  |
| ... |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

inter_comp_pred_enabled_flag equal to 1 specifies that inter-component prediction may be invoked in the decoding process for intra prediction. inter_comp_pred_enabled_flag equal to 0 specifies that inter-component prediction is not applied. When not present, the value of inter_comp_pred_enabled_flag is inferred to be equal to 0.

independent_uniform_pred_enabled_flag equal to 1 specifies that independent uniform prediction may be used in coding tree blocks. independent_uniform_pred_enabled_flag equal to 0 specifies that independent uniform prediction cannot be used in coding tree blocks.

7.4.7.1 General slice segment header semantics

num_uniform_colors specifies the total number of colors that may be used by the independent uniform prediction mode in in the current slice. When num_uniform_colors is not present, it is inferred to be equal to 0.

uniform_colors[ cIdx ][ i ] specifies the i-th element of the color value for color component cIdx. When uniform_colors[ cIdx ][ i ] is not present, it is inferred to be equal to 0.

7.4.3.4 Coding unit semantics

independent_uniform_pred_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the independent uniform prediction mode. independent_uniform_pred equal to 0 specifies that the current coding unit is code coded using the independent uniform prediction mode.

uniform_color_index[ x0 ][ y0 ] is an index to the color values that will be used in the independent uniform prediction mode to the current coding unit. When uniform_color_index[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_comp_pred_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current chroma prediction block is predicted in inter-component prediction. The array indices x0 + i, y0 + j specify the location ( x0 + i, y0 + j ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When inter_comp_pred_flag[ x0 + i ][ y0 + j ] is not present, it is inferred to be equal to 0.

inter_comp_ref_flag[ x0 + i ][ y0 + j ] equal to 0 specifies that the current Cr prediction block is predicted by referring luma component in inter-component prediction. inter_comp_ref_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current Cr prediction block is predicted by referring Cb component in inter-component prediction. The array indices x0 + i, y0 + j specify the location ( x0 + i, y0 + j ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When inter_comp_ref_flag[ x0 + i ][ y0 + j ] is not present, it is inferred to be equal to 0.

7.4.9.3 Sample adaptive offset semantics

Table 7-8 – Specification of the SAO type

| - SaoTypeIdx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - Band offset |
| - 2 | - Edge offset Histogram correction |
| - 3 | - Edge offset |

sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] specifies the offset value of i-th category for band offset and edge offset, or specifies the noise removal range of i-th histogram peak for histogram correction for the coding tree block at the location ( rx, ry ) for the colour component cIdx.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of the offset value of i-th category for band offset and edge offset, or specifies the most significant bit of noise removal range of i-th histogram peak for histogram correction for the coding tree block at the location ( rx, ry ) for the colour component cIdx when SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows:

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal 0.

The variable offsetSign is derived as follows:

- If SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 23 and i is equal to 2 or 3, offsetSign is set equal to −1.

- Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 23 and i is equal to 0 or 1, offsetSign is set equal to 1.

- Otherwise, (if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1), the following applies:

- If sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is equal to 0, offsetSign is set equal to equal to 1.

- Otherwise, offsetSign is set equal to equal to −1.

- Otherwise (SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2), the following applies:

- If sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is equal to 0, offsetSign is set equal to equal to 0.

- Otherwise, offsetSign is set equal to equal to 1.

The variable log2OffsetScale is derived as follows:

- If cIdx is equal to 0, log2OffsetScale is set equal to log2_sao_offset_scale_luma.

- Otherwise (cIdx is equal to 1 or 2), log2OffsetScale is set equal to log2_sao_offset_scale_chroma.

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

- If SaoTypeIdx[ cIdx ][ rx ][ ry ] is not equal to 2, the following applices:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  log2OffsetScale

- Otherwise (SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2), the following applies:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =
		offsetSign << 3 + sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]

# Compression performance discussion

The results reported in here use the templates and measurement methodologies specified by the CfP. After the CfP was issued, proponents were asked to clip each picture’s PSNR to 58.9226 dB, given that with screen content material some pictures under lossy conditions may still be coded losslessly. In this document and the accompanying spreadsheets, both the anchor and proposed per-picture PSNR values were clipped to 58.9226 dB prior to averaging.

## Performance for bitstreams submitted for subjective testing

For subjective testing, a subset of RGB sequences was coded, at three rates per sequence. Since BD-Rate computations require four rate points, we explicitly report in Table 1 the detailed rate, PSNR, and file size information. The QP values used to encode each sequence were selected to bring the file size as close as possible to and below the file size limit. To ensure and offer proof of encoder/decoder match, the bitstreams submitted for subjective testing include the MD5 hashes for each picture. The file sizes reported for the submitted bitstreams, which are all below the file size limits, include the space used by the MD5 hashes.

Table 1. Performance for bitstreams submitted for subjective testing

|  | RExt5.1 SCC Anchor |  |  |  |  | JCTVC-Q0036 subjective testing bitstreams |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | kbps | Y psnr | U psnr | V psnr | FileSize (Bytes) | kbps | Y psnr | U psnr | V psnr | FileSize (Bytes) |
| Flyinggraphicstext | 94841 | 38.62 | 38.39 | 38.26 | 118553768 | 94506 | 42.98 | 42.74 | 42.53 | 118169526 |
|  | 67232 | 33.80 | 33.56 | 33.51 | 84041897 | 66950 | 37.09 | 37.02 | 36.87 | 83724609 |
|  | 43748 | 29.20 | 28.94 | 28.94 | 54687407 | 43404 | 31.33 | 31.27 | 31.22 | 54291356 |
| Desktop | 86787 | 41.03 | 40.50 | 40.20 | 108486105 | 85760 | 57.50 | 57.01 | 56.31 | 107236761 |
|  | 69539 | 35.40 | 35.15 | 35.03 | 86926385 | 69356 | 45.80 | 45.33 | 44.92 | 86730997 |
|  | 50506 | 29.88 | 29.99 | 29.62 | 63135032 | 50353 | 34.86 | 34.87 | 34.35 | 62977775 |
| Console | 47765 | 42.84 | 42.36 | 41.95 | 59708771 | 47728 | 54.36 | 54.71 | 54.05 | 59697068 |
|  | 37959 | 36.97 | 36.96 | 36.63 | 47451071 | 37787 | 45.09 | 45.57 | 44.99 | 47270211 |
|  | 28011 | 30.38 | 30.90 | 30.45 | 35015906 | 27970 | 34.20 | 34.68 | 34.15 | 34998794 |
| WebBrowsing | 15372 | 42.39 | 41.66 | 41.32 | 19216678 | 15354 | 51.92 | 51.18 | 51.02 | 19210953 |
|  | 10820 | 35.70 | 35.28 | 35.11 | 13526650 | 10798 | 40.83 | 40.99 | 40.84 | 13515780 |
|  | 7354 | 31.10 | 30.65 | 30.99 | 9193551 | 7333 | 34.85 | 34.34 | 34.53 | 9184282 |
| Map | 50468 | 42.74 | 41.91 | 42.40 | 63087006 | 50414 | 45.09 | 44.29 | 44.73 | 63054316 |
|  | 32576 | 38.38 | 37.66 | 38.34 | 40722859 | 32534 | 39.88 | 39.28 | 39.96 | 40704118 |
|  | 20695 | 34.35 | 33.77 | 34.57 | 25871022 | 20658 | 35.52 | 35.06 | 35.88 | 25858714 |
| Programming | 54868 | 43.14 | 42.34 | 42.58 | 68587520 | 54797 | 45.31 | 44.36 | 44.44 | 68532772 |
|  | 38597 | 38.86 | 37.92 | 38.22 | 48249060 | 38567 | 40.91 | 39.74 | 40.10 | 48245662 |
|  | 25586 | 34.43 | 33.51 | 33.79 | 31984402 | 25534 | 36.26 | 35.05 | 35.53 | 31953833 |
| SlideShow | 3385 | 43.77 | 43.38 | 43.49 | 10579588 | 3372 | 44.95 | 44.56 | 44.75 | 10567678 |
|  | 2198 | 40.00 | 39.71 | 39.79 | 6871897 | 2188 | 41.22 | 40.92 | 41.12 | 6866929 |
|  | 1374 | 35.92 | 35.62 | 35.75 | 4295851 | 1363 | 37.11 | 36.85 | 37.15 | 4289739 |
| MissionControlClip3 | 99342 | 42.93 | 42.40 | 42.42 | 124179415 | 99237 | 44.70 | 44.13 | 44.16 | 124082816 |
|  | 70599 | 38.81 | 38.52 | 38.42 | 88251062 | 70440 | 40.62 | 40.30 | 40.18 | 88086317 |
|  | 48389 | 34.68 | 34.49 | 34.34 | 60488288 | 48312 | 36.24 | 36.11 | 35.97 | 60426326 |
| SocialNetworkMap | 365666 | 38.70 | 38.31 | 38.31 | 457084503 | 365003 | 40.55 | 40.02 | 40.04 | 456289901 |
|  | 255891 | 34.36 | 34.11 | 34.06 | 319866730 | 243814 | 35.31 | 35.12 | 35.07 | 304803754 |
|  | 167056 | 30.07 | 29.90 | 29.76 | 208822841 | 167008 | 31.20 | 31.10 | 30.98 | 208796128 |
| Robot | 36835 | 38.15 | 38.44 | 37.78 | 46044456 | 36697 | 38.65 | 39.02 | 38.23 | 45889092 |
|  | 16928 | 34.68 | 35.29 | 34.42 | 21161162 | 16848 | 34.95 | 35.65 | 34.70 | 21077709 |
|  | 7821 | 32.13 | 32.83 | 31.80 | 9777473 | 7777 | 32.31 | 33.09 | 32.02 | 9739349 |

## Objective performance results

### Lossy test condition

Table 2 shows the simulation results of the proposed algorithm under lossy test condition defined in the CfP [1]. For the spreadsheets accompanying this document, the bitstreams do not include MD5 hashes for each picture, as was the case for the anchors. The resulting bitstreams were all below the specified file size limits. From the table, it can be said that the proposed algorithm achieves significant bitrate reduction compared with the CfP anchor of HEVC Range Extensions software HM-12.1_RExt-5.1. Average gains for various content types as shown in Table 2 are up to 27% for the G component of RGB sequences and up to 26% for the Y component of YCbCr, for All Intra conditions. For Random Access conditions, the corresponding averages were up to 17% for G and 13% for Y, and for Low Delay conditions, the averages were up to 10.2% for G and 8.1% for Y. The gains on individual sequences were up to 32%, 29% and 19% for the G component for AI, RA, and LD conditions respectively, and up to 31%, 24% and 11% for the Y component. Regarding run times, note that some of the increases in encoder run times are due to the tools already in HM-13.0_RExt-6.0 as compared to HM-12.1_RExt-5.1. For example, the encoder run time ratio for AI conditions when going from HM-12.1_RExt-5.1 to HM-13.0_RExt-6.0 alone is about 150%.

Table 2. The simulation results under lossy test conditions

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y/G | U/Cb | V/Cr |
| RGB, text & graphics with motion, 1080p | -27.0% | -28.0% | -27.6% |
| RGB, text & graphics with motion,720p | -19.1% | -19.3% | -20.0% |
| RGB, mixed content, 1440p | -11.0% | -10.9% | -11.0% |
| RGB, mixed content, 1080p | -13.4% | -13.6% | -13.4% |
| RGB, Animation, 720p | -8.2% | -10.4% | -8.0% |
| YUV, text & graphics with motion, 1080p | -25.5% | -27.7% | -27.6% |
| YUV, text & graphics with motion,720p | -14.6% | -19.6% | -22.4% |
| YUV, mixed content, 1440p | -8.3% | -12.8% | -12.5% |
| YUV, mixed content, 1080p | -11.5% | -13.8% | -13.7% |
| YUV, Animation, 720p | -1.8% | -6.4% | -5.8% |
| Enc Time[%] | 301% (262%, 237%)* |  |  |
| Dec Time[%] | 87% |  |  |
| *Two methods for IUP encoder speed-up subsequently developed; < 1% impact on BD-Rate measurements |  |  |  |
|  | Random Access |  |  |
|  | Y/G | U/Cb | V/Cr |
| RGB, text & graphics with motion, 1080p | -15.0% | -16.0% | -15.5% |
| RGB, text & graphics with motion,720p | -16.9% | -17.1% | -18.1% |
| RGB, mixed content, 1440p | -7.7% | -8.1% | -8.2% |
| RGB, mixed content, 1080p | -7.5% | -7.7% | -7.5% |
| RGB, Animation, 720p | -4.3% | -5.9% | -4.6% |
| YUV, text & graphics with motion, 1080p | -13.8% | -17.2% | -17.4% |
| YUV, text & graphics with motion,720p | -12.5% | -17.2% | -20.9% |
| YUV, mixed content, 1440p | -5.5% | -12.1% | -11.9% |
| YUV, mixed content, 1080p | -7.3% | -10.8% | -10.4% |
| YUV, Animation, 720p | -0.7% | -5.1% | -4.8% |
| Enc Time[%] | 152% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | Y/G | U/Cb | V/Cr |
| RGB, text & graphics with motion, 1080p | -9.4% | -10.2% | -9.7% |
| RGB, text & graphics with motion,720p | -10.2% | -10.1% | -10.7% |
| RGB, mixed content, 1440p | -5.0% | -5.1% | -5.2% |
| RGB, mixed content, 1080p | -6.3% | -6.0% | -6.0% |
| RGB, Animation, 720p | -2.3% | -2.9% | -2.1% |
| YUV, text & graphics with motion, 1080p | -8.1% | -10.4% | -10.6% |
| YUV, text & graphics with motion,720p | -7.3% | -10.3% | -13.5% |
| YUV, mixed content, 1440p | -3.6% | -10.2% | -10.1% |
| YUV, mixed content, 1080p | -4.4% | -7.2% | -6.8% |
| YUV, Animation, 720p | 0.0% | -2.4% | -2.3% |
| Enc Time[%] | 140% |  |  |
| Dec Time[%] | 111% |  |  |

### Mathematically lossless test condition

Table 3. Table 3 shows the simulation results of the proposed algorithm under lossless test conditions defined in the CfP [1]. As in the lossy conditions, these results also show that the coding performance of the proposed algorithm is much higher than that of CfP anchor. For RGB sequences, average bit-rate savings were up to 32%, 28% and 28% for AI, RA, and LD conditions respectively, and for YCbCr sequences average bit-rate savings were up to 30%, 20% and 17%. Maximum bit-rate savings were up to 43%, 40% and 40% for RGB AI, RA, and LB, and 46%, 32% and 24% for YCbCr.

Table 3. The simulation results under lossless test conditions

|  | AI |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 30.0% | 31.8% | 21.8% | 43.3% |
| RGB, text & graphics with motion,720p | 16.3% | 17.2% | 3.5% | 32.0% |
| RGB, mixed content, 1440p | 5.4% | 5.7% | 4.6% | 6.8% |
| RGB, mixed content, 1080p | 10.7% | 9.7% | 7.1% | 12.3% |
| RGB, Animation, 720p | 4.5% | 4.5% | 4.5% | 4.5% |
| YUV, text & graphics with motion, 1080p | 28.9% | 30.4% | 17.5% | 45.8% |
| YUV, text & graphics with motion,720p | 14.1% | 16.3% | 1.8% | 37.0% |
| YUV, mixed content, 1440p | 3.2% | 3.5% | 1.9% | 5.0% |
| YUV, mixed content, 1080p | 8.7% | 7.7% | 5.3% | 10.1% |
| YUV, Animation, 720p | 2.6% | 2.6% | 2.6% | 2.6% |
| Enc Time[%] | 259% |  |  |  |
| Dec Time[%] | 92% |  |  |  |
|  | RA |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 20.1% | 28.5% | 19.1% | 41.3% |
| RGB, text & graphics with motion,720p | 7.9% | 14.0% | 5.8% | 28.2% |
| RGB, mixed content, 1440p | 8.2% | 8.2% | 7.1% | 9.2% |
| RGB, mixed content, 1080p | 11.1% | 9.9% | 8.6% | 11.3% |
| RGB, Animation, 720p | 7.3% | 7.3% | 7.3% | 7.3% |
| YUV, text & graphics with motion, 1080p | 12.6% | 20.9% | 11.6% | 34.6% |
| YUV, text & graphics with motion,720p | 4.9% | 11.2% | 1.6% | 31.7% |
| YUV, mixed content, 1440p | 1.8% | 1.8% | 1.4% | 2.1% |
| YUV, mixed content, 1080p | 6.8% | 4.5% | 1.8% | 7.1% |
| YUV, Animation, 720p | 2.0% | 2.0% | 2.0% | 2.0% |
| Enc Time[%] | 119% |  |  |  |
| Dec Time[%] | 105% |  |  |  |
|  | LB |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 20.5% | 28.3% | 19.9% | 40.7% |
| RGB, text & graphics with motion,720p | 8.2% | 12.6% | 6.3% | 21.2% |
| RGB, mixed content, 1440p | 9.2% | 9.2% | 8.2% | 10.2% |
| RGB, mixed content, 1080p | 11.1% | 10.3% | 9.4% | 11.2% |
| RGB, Animation, 720p | 7.7% | 7.7% | 7.7% | 7.7% |
| YUV, text & graphics with motion, 1080p | 11.8% | 17.7% | 11.3% | 26.6% |
| YUV, text & graphics with motion,720p | 4.2% | 8.2% | 1.6% | 22.6% |
| YUV, mixed content, 1440p | 1.9% | 1.8% | 1.3% | 2.4% |
| YUV, mixed content, 1080p | 6.8% | 4.2% | 1.4% | 7.1% |
| YUV, Animation, 720p | 1.9% | 1.9% | 1.9% | 1.9% |
| Enc Time[%] | 122% |  |  |  |
| Dec Time[%] | 105% |  |  |  |

# Complexity analysis

Because HEVC Range Extensions Draft 6 and the corresponding HM-13.0_RExt-6.0 software were used as a foundation for this proposal, this section describes aspects related to the architectural complexity of the newly proposed tools described in Section 2. Details related to the existing RExt 6 tools and the palette tool of JCTVC-P0303 have been studied extensively in previous JCT-VC meetings and in associated documents, and thus will not be reported again here.

## Encoding time and measurement methodology

The encoding times measured and output in the same way as in the HM-13.0_RExt-6.0 software platform. The three newly proposed tools and the palette tool all use the rate-distortion optimization (RDO) process in the encoder to decide which modes are used. The RDO process is the main reason for the encoder time increases shown in Section 4. The HC and IUP tools were implemented in a straightforward manner, so no encoder speed-up techniques were applied to these new tools. Additional improvements in the encoder run times are therefore possible. Under some conditions, the encoder will skip the ICP mode evaluation in the RDO loop, which provides some speed-up over the straightforward implementation.

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM 12.1_RExt 5.1

The decoding times measured and output in the same way as in the HM-13.0_RExt-6.0 software platform. There is some variation in decoder times due to the way that the multicore processors used for these simulations increase the clock speed for some cores when not all cores on one processor are being used.

## Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

Most encoding and decoding runs were done on a 64-bit Linux compute cluster comprising several Intel® Xeon® E5620 2.4GHz processors. The makefiles for compiling the software were identical to those used in HM-13.0_RExt-6.0, so no parallel processing or special optimization was used.

## Expected memory usage of encoder

Please refer to Section 4.13 for a description of the additional encoder architectural complexity of the new tools. Computations are generally done for each CU or PU, so the additional memory needed over HM-13.0_RExt-6.0 and palette modes are on the order of a block of pixels for the corresponding CU or PU.

## Expected memory usage of decoder

Please refer to Section 4.13 and 4.14 for descriptions of the additional architectural complexity of the new tools. Computations are generally done for each CU or PU, so the additional memory needed over HM-13.0_RExt-6.0 and palette modes are on the order of a block of pixels for the corresponding CU or PU. However, the additional complexity from the Histogram Correction mode is less than in the encoder, as the decoder does not always have to compute the histogram. Also, RDO measurements are not performed for these tools in the decoder, so the additional encoder storage needed to store intermediate computation results are not needed in the decoder.

## Complexity characteristics of encoder motion estimation and partitioning selection

No changes are made to the motion estimation and partitioning selection already in HM-13.0_RExt-6.0.

## Complexity characteristics of decoder motion compensation

No changes are made to the motion compensation already in HM-13.0_RExt-6.0

## Complexity characteristics of encoder intra-frame prediction type and partitioning selection

No changes are made to the existing intra-frame prediction types and partitioning selection in HM-13.0_RExt-6.0. The independent uniform prediction and palette modes each add another intra-frame prediction type to the system.

## Complexity characteristics of decoder intra-frame prediction operation

No changes are made to the existing intra-frame prediction types and partitioning selection in HM-13.0_RExt-6.0. The independent uniform prediction and palette modes each add another intra-frame prediction type to the system. For the independent uniform prediction mode, Figure 9 shows the steps needed to decode a CU. For that mode, the additional complexity involves generating a predictor block in which all pixels have the same value. In a practical implementation, it is not necessary to actually populate a block memory with those values; the same value can be added to each pixel in the residual as part of the existing reconstruction process.

## Complexity characteristics of encoder inter-component prediction tools

Please see Section 4.13.1 for the encoder complexity characteristics of the new proposed inter-component prediction tool. The encoder complexity of existing inter-component prediction tools is unchanged from HM-13.0_RExt-6.0.

## Complexity characteristics of decoder inter-component prediction tools

Please see Section 4.14.1 for the decoder complexity characteristics of the new proposed inter-component prediction tool. The decoder complexity of existing inter-component prediction tools is unchanged from HM-13.0_RExt-6.0.

## Complexity characteristics of transforms specific in screen content coding

No changes were made to the transforms already in HM-13.0_RExt-6.0.

## Complexity characteristics of encoder related to other tools specific in screen content coding

Characteristics of the three new tools are described here. For palette mode, please refer to JCTVC-P0303 and its cited documents.

### Inter-component prediction

ICP described in Section 2.1 requires scaling parameter and offset derivation process for each TU for intra prediction or for each CU for intra block copy.

### Histogram correction mode for Sample Adaptive Offset

SAO HC mode described in Section 2.2 requires the pixel value histogram calculation and the pixel value range decision to remove the noise around histogram peaks for each CTU. The noise removal is done by replacing the pixel values within the range by the pixel value of the peak.

### Independent uniform prediction mode

At slice level, the independent uniform prediction mode described in Section 2.3 requires pixel value histogram calculation for a set of subsampled pixels of the slice before encoding the coding tree units in the slice. num_uniform_colors of the most frequently-used colors are selected to build an num_uniform_colors-by-3 look-up table for each slice. In our implementation, the value of num_uniform_colors is restricted to be equal to or less than 2.

At the CU level, IUP mode is added as a prediction mode for any CU in either I-slice or P/B-slice. This mode will go through the RDO process as do other prediction modes for best RD cost check. The prediction process of IUP mode is independent on neighboring reconstructed reference pixels, and can be taken for all pixels in the block simultaneously.

## Complexity characteristics of decoder related to other tools specific in screen content coding

### Inter-component prediction

ICP described in Section 2.1 requires a scaling parameter and offset derivation process for each TU for intra prediction or for each CU for intra block copy. ICP requires that prediction and reconstruction processes for three components are serially conducted for each TU for intra prediction. It also requires that the prediction and reconstruction processes for the three components are serially conducted for each PU for intra block copy. If ICP is enabled for the current PU, the inter-component dependency for the residual signal generation process is resolved due to disabling CCP for TU(s) in the PU performing ICP.

### Histogram correction mode for Sample Adaptive Offset

SAO HC mode described in Section 2.2 requires the pixel value histogram calculation for each CTU. The noise removal is done by replacing the pixel values within the decoded range by the pixel value of the peak. Note that if a current CTU reuses the HC parameters of a neighboring CTU, the histogram calculation is not necessary.

### Independent uniform prediction mode

At the slice level, IUP mode requires a num_uniform_colors-by-3 look-up table for each slice. In our implementation, the value of num_uniform_colors is restricted to be equal to or less than 2. Note that the histogram calculation is not necessary at the decoder. The colors that may be used by IUP mode are signaled in slice header.

At the CU level, IUP mode is added as a prediction mode for any CU in either I-slice or P/B-slice. The prediction process of IUP mode is independent on neighboring reconstructed reference pixels, and can be taken for all pixels in the block simultaneously.

## Complexity characteristics of encoder entropy coding operation related to tools/parameters specific in screen content coding

No changes were made to the entropy coder from HM-13.0_RExt-6.0.

## Complexity characteristics of decoder entropy decoding operation related to tools/parameters specific in screen content coding

No changes were made to the entropy coder from HM-13.0_RExt-6.0.

## Degree of capability for encoder parallel processing

The overall encoder framework of HM-13.0_RExt-6.0 was not changed. As described in 4.13.1, Inter-component prediction, as in the case of the existing cross-component prediction, predicts across the components. Otherwise, the tools operate with dependencies similar to the existing tools.

## Degree of capability for decoder parallel processing

The capability description in Section 4.17 for the encoder applies to the decoder as well.

# Software implementation description

As described at the start of Section 2, the HM-13.0_RExt-6.0 software [8] was used as a starting point for this proposal. Tools were added consistent with the way typically done for JCT-VC proposals, in which #define macros are used to identify sections of the code modified by any new tool. For this proposal, four macros were defined.

# Highlighted aspects discussion

This proposal adds several new tools to the existing RExt 6 framework, for improving the objective and subjective performance of the codec, especially on screen content material. Given these improvements and the consistency with the current HEVC Range Extensions Draft 6, these tools are well-suited for either immediate adoption into the proposed extension or for study in core experiments either separately or in combination with similar tools proposed by others.

# Closing remarks

This document proposed a new video coding algorithm developed for submission as a response to the Joint Call for Proposals for Coding of Screen Content issued by ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6. The proposed algorithm builds upon HEVC Range Extensions Draft 6 by adding several coding tools targeting objective and subjective performance for coding screen content video. These new tools include block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, an independent uniform prediction mode, and a palette mode. For lossy test conditions, average gains for various content types are up to 27% for the G component of RGB sequences and up to 26% for the Y component of YCbCr, for All Intra conditions. For Random Access conditions, the corresponding averages were up to 17% for G and 14% for Y, and for Low Delay conditions, the averages were up to 10.2% for G and 8.1% for Y. The gains on individual sequences were up to 32%, 29% and 19% for the G component for AI, RA, and LD conditions respectively, and up to 31%, 25% and 11% for the Y component. Subjective improvements, which are not always reflected by quantitative metrics, were also achieved. The experiments under both lossy and lossless conditions clearly demonstrate that the proposed algorithm shows much higher coding performance than that of the CfP anchor for coding of screen content. Given these improvements and the consistency with the current HEVC Range Extensions Draft 6, these tools are well-suited for either immediate adoption into the proposed extension or for study in core experiments either separately or in combination with similar tools proposed by others.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Joint Call for Proposals for Coding of Screen Content,” ISO/IEC JTC1/SC29/WG11, MPEG2014/N14175, San José, US, January, 2014.

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. Sullivan and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 6,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P1005, 16th Meeting: San José, US, January, 2014.

- D. Flynn, J. Sole, G. Sullivan and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 5,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O1005, 15th Meeting: Geneva, CH, October/November, 2013.

- A. Minezawa, S. Sekiguchi and T. Murakami, “AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0116, 14th Meeting: Vienna, AT, July, 2013.

- A. Minezawa, K. Miyazawa, S. Sekiguchi and T. Murakami, “AHG5/AHG8: Improvement of MV-HEVC-based RGB coding for screen contents,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O0085, October 2013, 15th Meeting: Geneva, CH, October/November, 2013.

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M0097, 13th Meeting: Incheon, KR, April, 2013.

- W. Pu, X. Guo, P. Onno, P. Lai and J. Xu, “Suggested Software for the AHG on Investigation of Palette Mode Coding Tools,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0303, 16th Meeting: San José, US, January, 2014.

- HM RExt software, available at https://hevc.kw.bbc.co.uk/svn/jctvc-hm/branches/HM-range-extensions

# Appendix

The first revision of this document contained BD-Rate results for the Random Access lossy coding condition that used one QP value per test point. That table is shown below so as to be available for cross-checking purposes.

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -14.6% | -15.5% | -15.2% |
| RGB, text & graphics with motion,720p | -16.6% | -16.6% | -17.7% |
| RGB, mixed content, 1440p | -7.7% | -8.1% | -8.3% |
| RGB, mixed content, 1080p | -7.7% | -8.1% | -7.9% |
| RGB, Animation, 720p | -4.2% | -5.8% | -4.5% |
| YUV, text & graphics with motion, 1080p | -13.2% | -16.8% | -16.9% |
| YUV, text & graphics with motion,720p | -11.9% | -17.1% | -20.7% |
| YUV, mixed content, 1440p | -5.6% | -12.3% | -12.0% |
| YUV, mixed content, 1080p | -6.4% | -10.0% | -9.7% |
| YUV, Animation, 720p | -0.7% | -5.1% | -4.8% |
| Enc Time[%] | 147% |  |  |
| Dec Time[%] | 124% (93%)* |  |  |
| *Decode time ratios may vary due to differences in computing platforms and the relatively short decode times. Multiple results are reported if available. |  |  |  |