| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0042v1 |
| --- | --- |

| Title: | AHG15: Support of hybrid scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jianle Chen Ying Chen Hendry Adarsh K. Ramasubramonian Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | -858-651-8345 yekuiw@qti.qualcomm.com cjianle@qti.qualcomm.com cheny@qti.qualcomm.com fhendry@qti.qualcomm.com aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document (JCTVC-Q0042v1) is a follow-up of JCTVC-P0084v4 and can be potentially considered as JCTVC-P0084v5.

JCTVC-Q0042v1 proposes a definition of two hybrid scalable profiles (for 8 bits and 10 bits, respectively) as well as a definition of bitstream and decoder conformance. The text is provided in the attachment, with new text marked by the user name "Q0042v1", in relative to the text provided in JCTVC-P0084v4.

The remainder of this umbrella document is basically the same as the umbrella document of JCTVC-P0084v4.

JCTVC-P0084 proposes a way for the support of AVC base layer in SHVC that is asserted to be the simplest in terms of the changes needed to the SHVC specification. The two key aspects of the proposed design are: 1) no encapsulation, meaning decoded base layer pictures are provided by external means; and 2) output of base layer pictures, including the synchronization with output of enhancement layer pictures, is controlled by external means. The spec text changes for the design are provided in the attachment of this document, with changes marked in relative to the latest SHVC spec text in JCTVC-O1008v3.

JCTVC-P0184v3 provides a text (in the attachment) implementing the preliminary decision made at the first Hybrid codec scalability (AHG15) conference call on Thursday 2 February 2014 (see the notes sent by Gary Sullivan to the JCT-VC/3V reflectors on Friday 3 February 2014). The text also includes some easy bug fixes or straightforward minor improvements for places where bugs or minor issues were seen during the implementation of the text. The text is based on the SHVC text output in JCTVC-P1008v4.

JCTVC-P0184v4 fixes a couple of bugs in the text provided in JCTVC-P0184v3.

# Proposal

The propose design follows the following design rules:

- For the current access unit, either no information is provided (meaning no base layer picture is used for inter-layer prediction for the current access unit, regardless whether there was a base layer picture in this access unit in the base layer bitstream) or the following information of the base layer picture is provided by external means:

  - The decoded sample values of the base layer decoded picture

  - The representation format of the base layer decoded picture, including the width and height in luma samples, the colour format, the the separate colour plane flag, the luma bit depth, and the chroma bit depth

  - Whether the base layer picture is an IRAP picture or not, and if yes, the IRAP NAL unit type, which may specify an IDR picture, a CRA picture, or a BLA picture

  - Optionally, whether the picture is a frame or a field, and when a field, the field parity (a top field or a bottom field). When not provided, the decoded picture is inferred to be a frame picture

- Output of base layer pictures is the responsibility of the base layer decoder, and output synchronization between a base layer picture and an enhancement layer picture in the same access unit is externally controlled (e.g. by using presentation timestamps).

- The (multi-standard) SHVC decoder would only need to keep one decoded picture store of memory for the base layer in the sub-DPB for the base layer, and after the decoding of each access unit the sub-DPB for the base layer is emptied.

- Association of a base layer decoded picture to an access unit is the responsibility of external means (e.g. the base layer decoder or other external means).

- The nuh_layer_id of the base layer decoded picture is inferred to be equal to 0.

- The picture order count of the base layer decoded picture is set equal to the picture order count of any enhancement layer picture, if present, in the same access unit. Note that in this case the actual picture order count of a base layer picture decoded by the base layer decoder in such a scalable or multiview codec might be different than the picture order count value of the same picture when it is decoded by an AVC decoder. When no enhancement layer picture is present for the access unit, the base layer decoded picture is not used and can be discarded.

- The base layer decoded picture is marked as "used for long-term reference".

- For CPB operations, the base layer is considered as having zero bits.

- For DPB operations, only decoded pictures of enhancement layers are considered.

- Inter-layer motion prediction from the base layer is disallowed.

- When the base layer is non-HEVC, general_profile_space in the first profile_tier_level( ) syntax structure in a VPS is set equal to 1. In this case, the codec type is signalled, and when AVC is indicated, the three-byte AVC profile and level information is signalled, and the rest of the bits in the profile_tier_level( ) syntax structure are all reserved.

- When the base layer is non-HEVC, it is required that the signalling of the three DPB parameters (max sub-DPB size, max reorder, and max latency) use the least amount of bits, and it is required that none of the hrd_parameters( ) syntax structures in the VPS applies to layer set 0 (the base layer only).

- When the base layer is non-HEVC, unnecessary signalling for the base layer in the VPS extension, including VPS VUI, is avoided, e.g. sub_layers_vps_max_minus1[ 0 ], max_tid_il_ref_pics_plus1[ 0 ][ j ], and direct_dependency_type[ i ][ 0 ].

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).