| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0046 |
| --- | --- |

| Title: | Complexity analysis of an optimized SHVC decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | W. Hamidouche, M Raulet and O. Deforges IETR/INSA de Rennes 20 avenue des buttes de Coesmes 35043 Rennes Cedex, France | Tel: Email: | +33 2 23 23 82 83 Wassim.Hamidouche@insa-rennes.fr Mickael.Raulet@insa-rennes.fr |
| Source: | IETR/INSA Rennes |  |  |

_____________________________

# Abstract

This contribution provides a complexity assessment of an optimized Scalable High efficiency Video Coding (SHVC) decoder. The SHVC decoder is based on a real time and parallel HEVC decoder developed under the open source OpenHEVC project. The main time-consuming operations introduced in the SHVC standard including the up-sampling of the lower layer picture and the up-scaling of its Motion Vectors (MVs) are optimized in Single Instruction Multiple Data (SIMD) operations.

The optimised SHVC decoder archives a real time decoding of 1080p50 enhancement layer on single core of an Intel i7 processor running at 2.6 GHz (Macbook Pro Retina, end 2013, turbo boost 3.4GHz). The SHVC decoder decoding two layers introduces in average 50%-80% extra complexity in respect to the single layer HEVC decoder. Moreover, the up-sampling and up-scaling operations, in the case of spatial scalability and inter coding configuration, represent around 16% of the whole decoding time. Finally, the low level optimizations jointly with a hybrid parallelism approach enable the decoding of 1600p enhancement layer at 88 frames per second on a 4 cores i7 processor.

# Introduction

The Scalable High efficiency Video Coding (SHVC) standard provides tools to enable spatial and quality scalability. In addition to the technologies defined in the HEVC standard, SHVC uses inter-layer predictions to further improve the rate-distortion coding efficiency. The SHVC standard is based on an in-loop coding approach, which requires the whole decoding of all intermediate layers. The complexity of the encoding and decoding processes becomes then a real issue especially to reach a real time encoding/decoding of high definition videos (4K, 8K).

This contribution provides a complexity assessment of an optimized SHVC decoder. The performance of the SHVC decoder is compared to the singe layer HEVC decoder for a simulcast configuration. The SHVC decoder is based on the open source OpenHEVC decoder [2], which implements a conforming HEVC decoder. The SHVC decoder consists of L instances of the OpenHEVC decoder, one instance to decode each layer, with L the number of layers. In the case of spatial scalability, the prediction unit (PU) from lower layer used for inter-layer prediction is up-sampled and the corresponding MVs are up-scaled at the enhancement layer decoder. The 8-tap and 4-tap filters used for the up-sampling of the luma and chroma samples, respectively, are optimized in SSE operations.

The SHVC decoder supports several parallel processing solutions for parallel decoding. The video frames can be decoded in parallel thanks to the high level parallel processing solutions defined in the HEVC standard including wavefront and tile solutions. The frame-based parallelism is also supported enabling the decoding of a set of frames at each layer in parallel under the constrain that inter and inter-layer predictions are satisfied. Finally, a hybrid parallelism approach combining the frame-based and the wavefront solutions is also supported in the SHVC decoder [3].

# Results and analysis

The performance of the SHVC decoder is carried out on a laptop fitted with a 4 cores Intel i7 processor running at 2.6 GHz. The common test conditions defined in the SHVC standard [1] are considered.

belowTable 1 shows the decoding time performance of the SHVC decoder for all considered video sequences in different scalability configurations. In single core configuration, the SHVC decoder enables a real time decoding of low frame rate video sequences including Kimono and ParkScene in all scalability configurations. The last row in Table 1 summarizes the additional complexity of the no optimized HEVC decoder and the optimized SHVC decoder in respect to the optimized HEVC decoder. The low level optimizations in the single layer HEVC decoder enable a speedup of 3. This gain allows to reach a real time decoding of the high frame rate video sequences such as BQTerrace and BasketBallDrive.

On the other hand, the SHVC decoder decoding two layers in x2 and x1.5 spatial scalability introduces 43%  and 77% additional complexity compared to the simulcast configuration, respectively. This additional complexity is caused by the decoding of the base layer and the up-sampling and up-scaling operations performed by the SHVC decoder in the spatial scalability configuration. In the case of SNR scalability, the SHVC decoder introduces 80% additional complexity compared to the single layer decoder decoding only the enhancement layer. This additional complexity is mainly related to the decoding of the base layer, which is at the same resolution as the enhancement layer.

| Sequences | Decoding time (second) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | HEVC |  | SHVC (SSE “on”) |  |  |
|  | SSE | no SSE | SNR | x2 | x1.5 |
| Kimono | 3.53 | 12.35 | 5.78 | 5.06 | 5.86 |
| ParkScene | 4.03 | 12.08 | 7.24 | 5.86 | 6.92 |
| Cactus | 6.52 | 18.04 | 12.29 | 9.51 | 12.41 |
| BasketBallDrive | 8.02 | 26.56 | 14.74 | 11.26 | 14.27 |
| BQTerrace | 9.60 | 29.53 | 18.02 | 12.92 | 16.56 |
| Traffic | 4.14 | 11.27 | 7.83 | 6.22 | - |
| PeopleonStreet | 6.70 | 15.90 | 10.42 | 9.73 | - |
| Complexity ( %) | 0 | 195 | 80 | 43 | 77 |

Table 1 – Decoding time performance of the optimized SHVC decoder: Random Access, QPEL=26, QPBL=26 and 30 for spatial and SNR scalability, respectively  (turbo boost “on”)

Figure 1 shows the time repartition (in %) of different decoding steps in the optimized SHVC decoder. The up-sampling of the lower layer frame and the up-scaling of its MVs (inter-layer predictions) represent around 15 % and 18 % of the whole decoding time in x2 and x1.5 spatial scalability configurations, respectively. In the case of SNR scalability, the inter-layer prediction is reduced to 6% since the lower layer picture is only copied in the inter-layer reference picture without being up-sampled.

(a) Optimized HEVC (8.02 seconds)                          (b) Optimized SHVC SNR (14.74 seconds)

(c) Optimized SHVC x2 (11.26)                                (d) Optimized SHVC x1.5 (14.27)

Figure 1 – Time distribution (%) in the optimized SHVC decoder: BasketBallDrive, Random Access and QPEL=26, QPBL=26 and 30 for spatial and SNR scalability, respectively

We consider n the number of threads allocated for the wavefront parallelism and m the number of frames decoded in parallel for frame based parallelism.  Table 2 gives the performances of the SHVC decoder in terms of speedup, decoding frame rate and latency in different decoding configurations. We can notice that the hybrid parallelism solution (n, m)=(2, 2) decoding two layers in parallel (frame-based) and each frame is decoded in wavefront by 2 threads allows the best speedup performance. This hybrid parallelism solution enables to reach the decoding of 1600p video sequences in 88 frames per second (fps) for x2 spatial scalability and 65 fps for SNR scalability. The latency of the hybrid parallelism is lower than the latency in the frame-based parallelism hanks to the two threads used for wavefront decoding.

| Configurations |  |  | Decoding configurations (n, m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (1, 1) | (4, 1) | (1, 4) | (2, 2) |
| Class B | Speedup | x2 | 1 | 2.99 | 2.02 | 3.24 |
|  |  | x1.5 | 1 | 3.13 | 2.21 | 3.36 |
|  |  | SNR | 1 | 2.90 | 2.50 | 3.34 |
|  |  | HEVC | 1 | 3.10 | 2.64 | 3.05 |
|  | Decoding frame rate (fps) | x2 | 52 | 156 | 107 | 169 |
|  |  | x1.5 | 44 | 138 | 99 | 149 |
|  |  | SNR | 41 | 124 | 108 | 138 |
|  |  | HEVC | 72 | 226 | 195 | 224 |
|  | Decoding time per frame (ms) | x2 | 20.54 | 6.88 | 24.04 | 16.08 |
|  |  | x1.5 | 24.11 | 7.76 | 27.82 | 19.27 |
|  |  | SNR | 29.47 | 10.28 | 33.36 | 24.81 |
|  |  | HEVC | 16.07 | 5.20 | 24.05 | 10.66 |
| Class A | Speedup | x2 | 1 | 3.6 | 2.07 | 3.81 |
|  |  | SNR | 1 | 3.49 | 2.47 | 3.67 |
|  |  | HEVC | 1 | 3.49 | 2.83 | 3.22 |
|  | Decoding frame rate (fps) | x2 | 23 | 84 | 47 | 88 |
|  |  | SNR | 17 | 62 | 43 | 65 |
|  |  | HEVC | 33 | 120 | 96 | 110 |
|  | Decoding time per frame (ms) | x2 | 47.85 | 13.65 | 55.00 | 32.65 |
|  |  | SNR | 63.92 | 18.76 | 73.67 | 50.95 |
|  |  | HEVC | 34.56 | 10.25 | 48.65 | 21.90 |

Table 2 – Performance of the SHVC decoder: Random Access, all QP values

# References

- V. Seregin and Y. He, “Common SHM test conditions and software reference configurations,” in document JCTVC-O1009. Geneva, Switzerland, November 2013.

- OpenHEVC decoder: https://github.com/OpenHEVC/openHEVC

- W. Hamidouche, M. Raulet, O. Déforges, “Update of JCTVC-O0115 on a pipeline and parallel architecture of an SHVC decoder” , document JCTVC-P0169. Saint Jose, January 2014.

# Patent rights declaration(s)

INSA Rennes does not have any current or pending patent rights relating to the technology described in this contribution.