| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0047-r1 |
| --- | --- |

| Title: | AHG10: Simplification of Palette Based Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Feng Zou, Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 858-651-7749 Wei.Pu@qti.qualcomm.com fzou@qti.qualcomm.com rajanj@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document presents encoder improvements and codec simplifications in palette based coding. First, improvements on palette generation are proposed, which achieves better coding efficiency without any syntax or semantic change. Second, the CABAC context used in escape pixel coding is removed by change fixed length code to truncated binary code. Index is also coded using truncated binary codes without any CABAC context to align with escape pixel coding.

# Introduction

Run-length based palette mode coding was studied in [4][1]. The general procedure is summarized as follows:

- Transmission of the palette: An entry-wise prediction scheme is used to encode the current palette based on the palette of the previous CU. After that, no predicted entries are transmitted.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order using 3 modes.

  - “Copy Left Run” mode: A palette index is first signaled followed by value M, which means the following M palette indexes are the same as the one signaled.

  - “Copy Above Run” mode: A value N is transmitted to indicate that for the following N palette indexes are the same as their above neighbors, respectively.

  - “Escape” mode: The quantized pixel value is transmitted directly.

# Algorithm description

## BugFix on Suggested Software [1]

Three bugs in AHG10 suggested software were identified since San Jose meeting. 2.1.1 was identified and fixed by Qualcomm. The following two of them were identified and fixed by Canon.

### Non444 Palette Prediction

In [1], when calculation which palette predictors are used to predict current palette, the current palette size was mistakenly set to 0, which invalidates the palette prediction. I.e. the Boolean palette prediction flag vector was always an all zero vector.

### Logic Bug in Deriving Copy Left Run

In [1], from the syntax and semantic description, when copy left run is used, run length equals to 0 is a valid case. However, the software mistakenly disabled this case.

### Bit Counting Bug

In [1], there are redundancy calls in encodePLTModeInfo() and encodePredMode() which results in inaccurate RD cost calculation.

## Improvement on Palette Generation (Encoder Only)

### New Major Color Derivation Method

The major procedure of the new major color derivation method is:

- Generating histogram using three color component triplet;

- Sorting the histogram;

- Scanning from the beginning of the sorted histogram and include triplet into the palette if it is not covered by previous palette colors. The covering range depends on QP;

- The derived palette is refined to each item’s centroid. The refined palette item is mapped to its nearest physically existed pixel value in the CU.

### RD Check between Escape and Index Coding

Rate-Distortion checking is used to determine whether a pixel should be coded as escape pixel or as index.

### Escape Pixel and Index Coding

The number of context coded bins for each pixel has high impact on the decoder’s throughput and the hardware implementation complexity. Therefore, such number should be well controlled. In [1], to code the escape pixel, the most signification bin in each color component is context coded to improve coding efficiency. In this proposal, such context coded bin is removed by replacing the fixed length code to truncated binary code [3]. Index is also coded using truncated binary code to align with escape pixel coding so that the pixel coding method in palette is unified without requiring any context, i.e. coded as CABAC bypass.

When coding the index, if the left pixel exists and not escape pixel, we assume its index is I. Also assuming that the current palette size is L. Then I can be chosen from {0, 1, …, L-1}. For the current index C, if its left pixel is the end of copy above run, it cannot be equal to its above pixel. If its left pixel is the end of copy left run, C cannot be the same as I. Using such prior information, the maximum possible codeword can be reduced by one to improve coding efficiency.

# Experimental Results

Simulations are performance under AHG 8 test condition based on RExt 6.0 reference software. Note that in the configuration, “Level” is set to 6.2 to match the reference results in [6]. Maximum palette size for each CU is set to 64. AHG10 Software [5] is used as anchor. We appreciate Canon for cross checking.

## Final Results

Table 1. JCTVC-Q0047 v.s. RExt6.0 Based AHG10 (AI Lossy).

### Palette Size Limit = 64

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.0% | -3.1% | -3.4% | -1.9% | -2.4% | -2.6% | -1.7% | -1.8% | -2.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -5.9% | -4.8% | -4.9% | -6.2% | -5.2% | -5.4% | -6.3% | -5.7% | -6.0% |
| RGB 4:4:4 Animation | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | -3.6% | -5.8% | -6.9% | -4.2% | -5.7% | -6.7% | -5.0% | -5.8% | -6.6% |
| YCbCr 4:4:4 Animation | 0.0% | -0.4% | -0.3% | 0.0% | -0.4% | -0.3% | 0.0% | -0.3% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -7.8% | -7.7% | -7.7% | -8.3% | -8.1% | -8.1% | -9.1% | -9.2% | -9.2% |
| YCbCr 4:4:4 SC (Optional) | -5.6% | -5.3% | -5.2% | -6.2% | -5.7% | -5.5% | -6.9% | -6.9% | -7.0% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 106% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 96% |  |  |

Step by Step Results

### AHG10 [5] + BugFix

Table 2. RExt6.0 Based AHG10 + BugFix v.s. RExt6.0 Based AHG10 (AI Lossy).

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.6% | -0.4% | -0.5% | -0.9% | -0.7% | -0.8% | -1.2% | -1.0% | -1.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | -0.2% | -0.4% | -0.3% | -0.4% | -0.8% | -0.7% | -0.8% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.2% | -0.2% | -0.2% | -0.4% | -0.3% | -0.3% | -0.6% | -0.7% | -0.7% |
| YCbCr 4:4:4 SC (Optional) | -0.2% | -0.1% | -0.1% | -0.4% | -0.3% | -0.3% | -0.5% | -0.5% | -0.6% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |

AHG10 [5] + BugFix – P0108

Table 3. RExt6.0 Based AHG10 + BugFix – P0108 v.s. RExt6.0 Based AHG10 (AI Lossy).

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.7% | -0.6% | -0.4% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.9% | -0.7% | -0.8% | -1.4% | -1.1% | -1.2% | -1.8% | -1.5% | -1.7% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.3% | -0.3% | -0.3% | -0.6% | -0.6% | -0.6% | -1.1% | -1.0% | -1.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -2.0% | -1.9% | -1.9% | -2.2% | -2.1% | -2.1% | -2.6% | -2.6% | -2.7% |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.0% | -1.0% | -1.4% | -1.2% | -1.2% | -1.7% | -1.6% | -1.8% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |

### AHG10 [5] + BugFix – P0108 + Encoder Only Improvements

Table 4. RExt6.0 Based AHG10 + BugFix – P0108 v.s. RExt6.0 Based AHG10 (AI Lossy).

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -1.8% | -2.0% | -1.0% | -1.4% | -1.5% | -1.0% | -1.0% | -1.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -3.3% | -2.4% | -2.5% | -3.8% | -2.9% | -3.1% | -4.2% | -3.6% | -3.9% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | -1.8% | -3.3% | -4.1% | -2.1% | -3.0% | -3.9% | -2.8% | -3.2% | -3.9% |
| YCbCr 4:4:4 Animation | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -2.9% | -2.9% | -2.8% | -3.5% | -3.3% | -3.3% | -4.4% | -4.4% | -4.5% |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.7% | -1.8% | -2.1% | -1.7% | -1.6% | -2.8% | -2.9% | -2.9% |
| Enc Time[%] | 108% |  |  | 107% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |

# References

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “Suggested software for the AHG on investigation of palette mode coding tools,” JCTVC-P0303, San Jose, USA, Jan. 2014.

- Truncated Binary Codes. http://en.wikipedia.org/wiki/Truncated_binary_encoding

- W. Pu, F. Zou, J. Sole, M. Karczewicz, and R. Joshi, “Non-RCE4: Refinement of the palette in RCE4 Test 2,” JCTVC-P0231, San Jose, USA, Jan. 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “AHG10: Suggested software for palette based on RExt6.0,” JCTVC-Q0094, Valencia, ES, Jan. 2014.

- C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-P1006, San Jose, USA, Jan. 2014.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).