| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0048 |
| --- | --- |

| Title: | SCE1.2: Color gamut scalability with asymmetric 3D LUT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Jianle Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Yuwen He, Yan Ye, Jie Dong 9710 Scranton Rd, #250 San Diego, CA 92121, USA Philippe Bordes, Pierre Andrivon, Edouard François, Franck Hiron, 975 avenue des champs blancs CS 17616, 35576, Cesson-Sévigné Cedex, France | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com +1-858-210-4819 yuwen.he@interdigital.com yan.ye@interdigital.com jie.dong@interdigital.com +33-2-99-27-32-42 philippe.bordes@technicolor.com, pierre.andrivon@technicolor.com |
| Source: | Qualcomm Inc., InterDigital Communications, Inc., Technicolor |  |  |

_____________________________

# Abstract

This proposal reports the results of SCE1.2 on color gamut scalability with asymmetric 3D LUT. Two cases are tested: With and without phase alignment. When phase alignment is on, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-9.9%, -15.9%,-18.8%}, and {-8.2%, -10.3%, -13.2%}, {-10.4%, -16.3%, -21.0%}, and {-6.0%, -7.8%, -11.5%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively. When phase alignment is off, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-8.9%, -15.6%, -18.5%}, and {-7.6%, -10.0%, -12.8%}, { -9.9%, -16.2%, -20.8%}, and {-5.7%, -7.6%, -11.2%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively.

# Introduction

In this proposal, a combination of JCTVC-P0063, JCTVC-P0186 and JCTVC-0128 is investigated. Color gamut scalability is achieved with asymmetric 3D LUT. To keep low computational complexity, color gamut conversion is conducted before up-sampling. To further improve the coding performance, luma and chroma phases are aligned for luma and chroma 3D LUT interpolation.

# Color mapping at decoder

The input to this process is a (y,u,v) triplet in base layer color gamut. The output of the process is the triplet (Y,U,V) in enhancement layer color gamut.

First, following frame level varibles are derived based on signaled parameters as follows.

yShift2Idx = InputBitDepth - cur_octant_depth - cur_y_part_num_log2

uShift2Idx = InputBitDepth - cur_octant_depth

vShift2Idx = InputBitDepth - cur_octant_depth

nMappingShift = yShift2Idx + uShift2Idx

nMappingOffset = 1 << (nMappingShift - 1)

## (y, u, v) triplet adjustment when phase alignment is enabled

When phase alignment is enabled, triplet (y, u’, v’) instead of (y,u,v) is used for luma component color gamut mapping as the input where u’ and v’ are derived based on positions as follows. c indicates chroma signal u or v. L4, L5, L8 and L9 indicate the luma positions. C0, C1, C2 and C3 indicate the chroma positions.

c(L4) = ((c(C0)<<1)+c(C0) + c(C2) + 2)>>2

c(L5) = (((c(C0)+c(C1))<<1)+ (c(C0)+c(C1)) + (c(C2)+c(C3)) + 4)>>3

c(L8) = (c(C0) + (c(C2)<<1)+c(C2) + 2)>>2

c(L9) = ((c(C0)+c(C1)) + ((c(C2)+c(C3))<<1)+(c(C2)+c(C3)) + 4)>>3

When calculating chroma components in 4:2:0 format, the luma component is derived by averaging 2 luma positions to align with the choma position. That is, triplet (y’, u, v) is used, where y’ is derived as follows, where Y indicates the derived luma signal.

y(C0) = (y(L0) + y(L4) + 1)>>1

y(C1) = (y(L2) + y(L6) + 1)>>1

y(C2) = (y(L8) + y(L12) + 1)>>1

y(C3) = (y(L10) + y(L14) + 1)>>1

## Intermediate variables

With input (y, u, v), intermediate variables are derived as follows.

yIdx = y >> yShift2Idx

uIdx = u >> uShift2Idx

vIdx = v >> vShift2Idx

deltaY = y – (yIdx << yShift2Idx)

deltaU = u – (uIdx << uShift2Idx)

deltaV = v – (vIdx << vShift2Idx)

## Color mapping

Y = lutY[yIdx][uIdx][vIdx][0]

+((((deltaY *((lutY[yIdx][uIdx][vIdx][3] – lutY[yIdx][uIdx][vIdx][2]) << uShift2Idx)

+ ((deltaU *((lutY[yIdx][uIdx][vIdx][1] – lutY[yIdx][uIdx][vIdx][0]) << yShift2Idx)

+ ((deltaV *((lutY[yIdx][uIdx][vIdx][2] – lutY[yIdx][uIdx][vIdx][1]) << yShift2Idx)

+ nMappingOffset) >> nMappingShift)

U = lutU[yIdx][uIdx][vIdx][0]

+((((deltaY *((lutU[yIdx][uIdx][vIdx][3] – lutU[yIdx][uIdx][vIdx][2]) << uShift2Idx)

+ ((deltaU *((lutU[yIdx][uIdx][vIdx][1] – lutU[yIdx][uIdx][vIdx][0]) << yShift2Idx)

+ ((deltaV *((lutU[yIdx][uIdx][vIdx][2] – lutU[yIdx][uIdx][vIdx][1]) << yShift2Idx)

+ nMappingOffset) >> nMappingShift)

V = lutV[yIdx][uIdx][vIdx][0]

+((((deltaY *((lutV[yIdx][uIdx][vIdx][3] – lutV[yIdx][uIdx][vIdx][2]) << uShift2Idx)

+ ((deltaU *((lutV[yIdx][uIdx][vIdx][1] – lutV[yIdx][uIdx][vIdx][0]) << yShift2Idx)

+ ((deltaV *((lutV[yIdx][uIdx][vIdx][2] – lutV[yIdx][uIdx][vIdx][1]) << yShift2Idx)

+ nMappingOffset) >> nMappingShift)

# Complexity analysis

Complexity of the proposal is analysed as recommended by SCE1 description JCTVC-P1101. For the detailed number of multipliers and bit-depth of multiplier, please see attached excel sheets.

## Number of multipliers and bit-depth

| Bit-depth of base layer | Bit-depth of multiplier | Number of luma multiplier per base layer pixel | Number of chroma multiplier per base layer pixel | Total number of multiplier per base layer pixel | Total number of multiplier per enh. layer pixel (2x case) |
| --- | --- | --- | --- | --- | --- |
| 8-bit | 8-bit x 12-bit | 3 | (3+3)/4 | 4.5 | 1.125 |
| 10-bit | 10-bit x 12-bit | 3 | (3+3)/4 | 4.5 | 1.125 |

Operations of re-sampling are excluded here. Please note that only shift and addition operations are needed for intermediate variable derivation in phase alignment.

## The sizes of 3D LUT

The table has up to 8x2x2x4x3=384 entries, where 8x2x2 represents the asymmetric partitioning of Y-U-V space, 4 indicates that for each (y, u, v) triplet we have 4 vertexes to be used in interpolation, 3 denotes 3 color components.

## Summary of processing stages

There are 2 stages in color conversion and up-sampling. In the first stage, base layer reconstructed pixels are mapped to new color gamut and the bit-depth at enhancement layer based on the formulas listed in section 2.

In the second stage, the mapped pixels are up-sampled to enhancement layer resolution which is the same process as what it is in the current SHVC specification.

## Whether resampling is used when reporting the number of multiplications

Re-sampling is not included in the numbers reported in section 3.1.

## Whether cross color dependency is being used.

As shown in the formulas in section 2, cross-color dependency is used.

# Description of CGS information derivation at encoder side

In general, CGS information is derived at encoder side for each picture.

First, the partition number of CGS LUT is derived based on the bit cost of CGS LUT in the previous same-type-same-temporal-level picture. Then the CGS LUT for the current picture is derived by minimizing the SSE between the base layer construction and the down-sampled enhancement input picture. Subsequently, the distortion resulted by the CGS LUT is estimated based on models. If the estimated distortion is smaller than that by CGS LUT signaled in PPS, the CGS LUT is updated in PPS.

# Simulation results

To verify the performance of the proposed method, tests were conducted according to the test conditions defined in JCTVC-P1101. The results are summarized as follows.

## Phase alignment on with se-3(v) for LUT residues

## Phase alignment off with se-3(v) for LUT residues

## Phase alignment on with se (v) for LUT residues

## Phase alignment off with se (v) for LUT residues

# Conclusions

In this proposal, a combination of JCTVC-P0063, JCTVC-P0186 and JCTVC-0128 is investigated. Color gamut scalability is achieved with asymmetric 3D LUT. To keep low computational complexity, color gamut conversion is conducted before up-sampling. To further improve the coding performance, luma and chroma phases are aligned for luma 3D LUT interpolation. Simulations under SCE1 test conditions show that significant BD-rate savings are achieved by the proposed method.

# Proposed text

The proposed text change has been integrated in P1008 as attached. The text change is ~3 pages.

## Syntax and semantics

- Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).