| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0049 |
| --- | --- |

| Title: | RCE1.2: Adaptive MV Precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports the results of RCE1 test 2 on adaptive MV precision. With the proposed method, a flag is signaled at CU level to indicate the MV precision of the CU. For the AhG8 test conditions, it is reported that 0.9%, 1.0%, 1.2% and 1.3% luma BD-rate reduction is achieved on average for RA-MT, RA-HT, LB-MT and LB-HT, respectively.

# Introduction

It is observed that sub-pixel MV precision is not always efficient for screen content and some natural content. Therefore, adaptive MV precision was investigated in RCE 1. This proposal reports the results of RCE1 test 2.

# Proposed method

It is proposed to signal a flag at CU level to indicate whether all PUs in the CU have integer-precision MVs. When the flag is on, MV predictors of non-skip and non-merge PUs are rounded to integer precision and MV differences are signaled in integer precision. For PUs in merge or skip mode, MVs are rounded only before motion compensation so that fractional MVs are saved and can be used as MV predictors for neighboring PUs. When the flag is off, sub-pixel motion compensation is allowed.

# Simulation results and discussions

The proposed method is implemented on top of HM13.0-RExt6.0. The test sequences and conditions defined by AhG 8 are used. The coding performance is summarized as follows.

## Proposed method

## Encoder-only method

In this test, the encoder decides whether MVs of a CU are at integer precision in the same way as proposed above. However, all syntax elements are kept unchanged and no CU level flag is signaled. The results below show that some coding gain can be obtained by this encoder-only method. However, the gain by the encoder-only method is only about 50% of that of the proposed method, especially for the RA configuration.

## Discussion on coding complexity

At the encoder side, the coding time is increased. The reason is that in the current implementation whether a CU only has integer MVs is decided by additional RD checking and motion estimation. It is believed that the encoding speed can be much improved by a better fast algorithm while the coding gain can be kept at a similar level.

At the decoder side, the decoding time is decreased on average. because the reason is that more integer MVs are employed and so the interpolation process in sub-pixel motion compensation is invoked less frequently.

# Conclusions

In this contribution, an adaptive MV precision method is proposed. Simulations show that 0.9%, 1.0%, 1.2% and 1.3% luma BD-rate reduction is achieved on average for RA-MT, RA-HT, LB-MT and LB-HT, respectively.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ){ |  |
| cu_imv_flag[ x0 ][ y0 ] | ae(v) |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| } |  |
| else { |  |
| … |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| cu_imv_flag[ x0 ][ y0 ] | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |

cu_imv_flag[ x0 ][ y0 ] equal to 1 specifies that motion vectors of all prediction units belonging to the current coding unit are in integer-pixel precision. cu_imv_flag[ x0 ][ y0 ] equal to 0 specifies that motion vectors of all prediction units belonging to the current coding unit are in quarter-pixel precision. When cu_imv_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.