| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0050 |
| --- | --- |

| Title: | 4K real time streaming with SHVC decoder and GPAC player |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | W. Hamidouche, M. Raulet and J. Le Feuvre IETR/INSA de Rennes 20 avenue des buttes de Coesmes 35043 Rennes Cedex, France | Tel: Email: | +33 2 23 23 82 83 Wassim.Hamidouche@insa-rennes.fr Mickael.Raulet@insa-rennes.fr |
| Source: | IETR/INSA and Telecom ParisTech |  |  |

_____________________________

# Abstract

This contribution provides a 4K end-to-end video streaming demonstration with using an optimized SHVC decoder under the openHEVC/FFmpeg library (openHEVC project) and the GPAC player. On the one hand, the optimized and parallel SHVC decoder enables a real time decoding 2160p30 video sequence associated with low frame latency. The SHVC decoder was integrated in the openHEVC/FFmpeg library enabling this open source library to support the decoding of a conforming SHVC bitstream. On the other hand, the GPAC player implements tools to encapsulate the conforming SHVC bitstream into the mp4 file format.

At the server side, the GPAC player broadcasts the mp4 content in MPEG2-TS. At the receiver side, the GPAC player feeds the openHEVC/FFmpeg SHVC decoder with the received video packets. The FFmpeg library through the SHVC decoder decodes the requested SHVC video layers and pushes YUV pictures corresponding the highest decoded layer for rendering by the GPAC layer. This demonstration also provides at the receiver side an interactive interface to switch between the decoded SHVC layers: HD and 4K for x2 spatial scalability.

# Introduction

The contribution shows the first 4K end-to-end video streaming demonstration with using an open source software including SHVC decoder [1], openHEVC/FFmpeg library [2] and GPAC player [3]. The first step in this demonstration is to generate through the SHVC reference software model version 4.1 (SHM4.1) [4] the SHVC bitsreams. We encode two 4K video sequences (CrowdRun and ParkJoy) from the STV High Definition Multi Format Test Set. The video sequences are encoded in two x2 spatial scalability layers (1080p base layer and 2160p enhancement layer) and low delay coding configuration with enabling the wavefront tool. The encoding command line is provided Table 1.

The MP4Box tool of the GPAC software allows to encapsulate the SHVC conforming elementary streams into an MP4 file format. The two SHVC video sequences are encapsulated into one MP4 file format. The GPAC executable compiled with the FFmpeg library integrating the SHVC decoder [2] can be downloaded from [5]. The command line encapsulating the SHVC bitsreams is provided in Table 1. The next step consists in the streaming of the MP4 content through the UDP transport protocol with GPAC player. Finally, the GPAC player allows to receive the SHVC packets and displays the YUV pictures decoded with FFmpeg library through the optimized SHVC decoder.

This demonstration enables to switch in real time between the base layer  (ctr+l) for HD resolution and the enhancement layer (ctr+h) for 4K resolution.

| Task | Software | Command line |
| --- | --- | --- |
| Generate SHVC video sequences | SHM4.1 | TAppEncoderStatic -c /cfg/encoder_lowdelay_main.cfg -c /cfg/per-sequence-svc/ParkJoy-2x.cfg -q0 30 -q1 26 --NumLayers=2 --WaveFrontSynchro=1 |
| Encapsulate the SHVC elementary streams into MP4 file format | GPAC | MP4Box -new -add ParkJoy_2160p.shvc ParkJoy2160.mp4 -fps 30 |
| Broadcast the MP4 content | GPAC | mp42ts -prog= ParkJoy2160.mp4 -dst-udp=224.1.1.1:1234 |
| Receive and decode the MP4 content | GPAC/FFmpeg/SHVC | MP4Client mpegts-udp://224.1.1.1:1234 -fs |

Table 1 - Command lines of different steps required to run the 4K streaming demonstration with SHVC decoder within FFmpeg and the GPAC player

# References

- W. Hamidouche, M. Raulet and O. Deforges, “Complexity analysis of an optimized SHVC decoder,” in document JCTVC-Q0046. Valencia, Spain, March 2014.

- Fork of FFmpeg library branch shvc_ext: https://github.com/OpenHEVC/FFmpeg

- GPAC multimedia open source project: http://gpac.wp.mines-telecom.fr

- SHVC reference software model version 4.1: https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/branches/SHM-4.1-dev/

# Patent rights declaration(s)

INSA/IETR does not have any current or pending patent rights relating to the technology described in this contribution.