| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0051 |
| --- | --- |

| Title: | AHG5: Recommended profiling of range extension coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Park Elena Alshina Chanyul Kim Ken McCann | Email: | sunil.lee@samsung.com cy1205.park@samsung.com elena_a.alshina@samsung.com dionism@samsung.com ken@zetacast.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides recommendations on the process that should be used in the profiling of range extension coding tools. It is suggested that the decision on profiling for the HEVC range extensions should follow a similar process to that used for HEVC v1, i.e. it should be based on an assessment of the trade-off between coding gain and complexity of the additional range extension coding tools, evaluated under the common test conditions of HEVC range extensions.

This contribution provides such an assessment. It is asserted that the results indicate that cross component de-correlation is the only additional coding tool in the draft RExt text that provides a beneficial trade-off between performance and complexity with generic video content at less than 16 bits. It is therefore proposed that the Main 4:4:4 10/12 profiles should include cross component de-correlation, as well as CU-adaptive chroma QP offset for encoder-side rate control in 4:4:4, but not the other coding tools present in the draft text, which are asserted to provide a beneficial performance/complexity trade-off only with screen content. In addition, it is suggested to include Golomb-Rice parameter adaptation and CABAC bit alignment in the 4:4:4 16b all-intra profile, but not the Main 4:4:4 10/12 profiles.

It is also suggested that Screen Content (SC) 4:4:4 Profiles, including SC coding tools, should be developed based on the evaluation results of the responses to the call for proposals for coding of screen content. It is suggested that all of the coding tools in the draft RExt text should be included in the initial test model for this new activity.

# Introduction

This contribution is an update of documents submitted to the 15th JCT-VC meeting in Geneva [12] and the 16th JCT-VC meeting in San Jose [13]. Unfortunately, it appeared to be impossible to find time to allow the presentation of these documents at either of the meetings.

At the 14th JCT-VC meeting in Vienna, five profiles of the HEVC range extensions were defined: Main 12, Main 4:2:2 10/12, and Main 4:4:4 10/12. However, there was no discussion on which tools it would be appropriate in any of these profiles. At the 15th JCT-VC meeting in Geneva, there was some discussion on the profiling of range extension coding tools and it was agreed to exclude the coding tools effective only for screen content coding (SCC) from Main 12 and Main 4:2:2 10/12. However, the consensus on tools for Main 4:4:4 10/12 could not be reached, since sufficient time was not allocated to discuss all input documents related to the profiles of HEVC range extensions. At the 16th JCT-VC meeting in San Jose, it was agreed to add a 4:4:4 16b all-intra profile, but the consideration of other 4:4:4 profiles was deferred to the 17th JCT-VC meeting in Valencia.

In addition to the design improvements to properly support high bit-depth and extended colour format, the range extension coding tools which have been adopted so far include:

- implicit residual DPCM (RDPCM) [1][2]

- explicit RDPCM [3]

- residual rotation [4]

- single context model for significance map coding in transform-skipped (TS) block [5]

- intra block copy [6]

- Golomb-Rice parameter adaptation [7]

- cross component de-correlation [8]

- CU-adaptive chroma QP offset [9]

- CABAC bit alignment [10]

In this contribution, the coding gain and complexity of these additional coding tools are evaluated under the common test condition of HEVC range extensions, and a profiling of the coding tools is recommended based on the test results. Please note that CU-adaptive chroma QP offset could not be tested since it has not been integrated into the reference software in time for testing in this contribution. Instead, bit-portion of chroma in each colour format is analysed to estimate the effectiveness of the CU-adaptive chroma QP offset for the encoder-side rate control. CABAC bit alignment is not tested either, since it has been already agreed at the last meeting to enable this tool only in 4:4:4 16b all-intra profile.

# Test results in AHG5 CTC

The coding gain and complexity of the additional coding tools are evaluated under the AHG5 common test condition (CTC) of the HEVC range extensions [11]. In all the tests, the anchor is HM13.0-RExt6.0 and the tested condition is “coding tool off”. Table 1 to Table 7 summarize the test results for implicit RDPCM, explicit RDPCM, residual rotation, single context model for significance map coding in TS block, intra block copy, Golomb-Rice parameter adaptation, and cross component de-correlation, respectively.

The results show that the coding gain provided by the additional range extension coding tools is very minor for the test sequences in the AHG5 CTC. This had been already confirmed when those coding tools were reviewed and adopted, since they were mainly for SCC. Only one exception is cross component de-correlation, which provides significant coding gain even for camera-captured video sequences in RGB 4:4:4 colour format. The complexity of the coding tools measured in terms of encoding and decoding time was not significant for implicit RDPCM, residual rotation, single context model for significance map coding in TS block, Golomb-Rice parameter adaptation. However, the encoding time could be reduced by up to 13% (RA-MT) by disabling explicit RDPCM and by up to 8% (AI-SHT) by disabling cross component de-correlation. The test results also show that the encoding time could be significantly reduced (60% in AI and 10% in RA and LB) by disabling intra block copy. Considering that the coding gain of intra block copy is minor (less than 0.5% in all coding configurations on average), the trade-off between the complexity and the coding gain that intra block copy provides is not beneficial to the profiles which will be mainly used for camera-captured natural video sequences. Please note that intra block copy is also known to increase the cost of practical decoder implementations, since the samples within the search range (currently two CTUs) should be kept in intra slices.

Table 1. Performance of cross component de-correlation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 25.6% | 19.6% | 13.9% | 18.6% | 14.4% | 16.1% | 12.5% |
| YCbCr 4:4:4 | 1.5% | 1.9% | 2.1% | 0.6% | 1.0% | 0.2% | 0.7% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 9.0% | 7.2% | 5.3% | 6.4% | 5.1% | 5.4% | 4.4% |
| Enc Time[%] | 94% | 93% | 92% | 95% | 93% | 95% | 93% |
| Dec Time[%] | 101% | 101% | 101% | 101% | 100% | 99% | 99% |

Table 2. Performance of explicit residual DPCM off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% | 95% | 96% | 87% | 89% | 89% | 90% |
| Dec Time[%] | 99% | 99% | 99% | 100% | 100% | 99% | 98% |

Table 3. Performance of Golomb-Rice group adaptation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 99% | 99% | 99% | 100% | 99% | 99% | 98% |

Table 4. Performance of implicit residual DPCM off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 100% | 99% | 99% | 100% | 100% | 98% | 98% |

Table 5. Performance of intra block copy off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.7% | 0.4% | 0.2% | 0.5% | 0.2% | 0.3% | 0.1% |
| YCbCr 4:4:4 | 0.5% | 0.3% | 0.2% | 0.2% | 0.2% | 0.0% | 0.1% |
| YCbCr 4:2:2 | 0.4% | 0.3% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| Overall | 0.5% | 0.3% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 44% | 40% | 40% | 91% | 87% | 91% | 87% |
| Dec Time[%] | 100% | 100% | 99% | 100% | 99% | 98% | 98% |

Table 6. Performance of residual rotation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 99% | 99% | 99% | 98% | 99% | 98% | 97% |

Table 7. Performance of single significance map context off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 99% | 99% | 99% | 99% | 100% | 99% | 98% |

# Test results in AHG8 CTC

The coding gain and complexity of the additional coding tools are also evaluated under the AHG8 CTC of the HEVC range extensions [11]. In all the tests, the anchor is HM13.0-RExt6.0 and the tested condition is “coding tool off”. Table 8 to Table 21 summarize the test results for implicit RDPCM, explicit RDPCM, residual rotation, single context model for significance map coding in TS block, intra block copy, Rice parameter adaptation, and cross component de-correlation, respectively, in both lossy and lossless coding configurations.

The results show that the coding gain provided by the additional range extension coding tools is higher in AHG8 CTC which includes SC test sequences than their coding gain in AHG5 CTC without any SC sequences. However, the coding gain of the additional range extension tools is still minor in lossy coding even for SC (mostly less than 1.0% in all coding configurations), except cross component de-correlation and intra block copy. Implicit/explicit RDPCM and Golomb-Rice parameter adaptation provide high coding gain in lossless coding, while the coding gain of residual rotation and single context model for significance map coding in TS block is still minor (less than 1.0% in all coding configurations) even in lossless coding. As compared with the results reported in [13], the coding gain of Golomb-Rice parameter adaptation was slightly improved since more complicated statistics-accumulation-based initialization method was adopted at the last meeting [7]. However, the additional coding gain on top of the previous Golomb-Rice fast adaptation is marginal. The coding gain of intra block copy was also improved due to the adoption of PU splitting for intra block copy [14]. However, the encoding complexity also increases significantly, so that the encoding time can be now reduced by 45% in AI by disabling intra block copy, while the reduction was around 25% in [13].

Table 8. Performance of cross color de-correlation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 33.3% | 30.6% | 27.8% | 29.0% | 28.1% | 27.5% | 26.7% |
| RGB 4:4:4 Animation | 37.7% | 37.0% | 32.3% | 25.9% | 27.3% | 24.0% | 25.1% |
| YCbCr 4:4:4 SC | 4.3% | 4.5% | 4.1% | 3.6% | 3.9% | 2.9% | 3.4% |
| YCbCr 4:4:4 Animation | 2.7% | 3.3% | 3.6% | 1.2% | 1.6% | 0.5% | 1.2% |
| RangeExt | 0.9% | 1.0% | 1.0% | 0.3% | 0.5% | 0.2% | 0.3% |
| RGB 4:4:4 SC (Optional) | 25.4% | 27.0% | 28.3% | 25.7% | 28.2% | 23.2% | 26.3% |
| YCbCr 4:4:4 SC (Optoinal) | 7.1% | 6.5% | 5.2% | 5.4% | 4.7% | 6.2% | 5.2% |
| Enc Time[%] | 96% | 95% | 94% | 97% | 96% | 97% | 96% |
| Dec Time[%] | 101% | 102% | 102% | 99% | 99% | 100% | 99% |

Table 9. Performance of cross color de-correlation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 21.4% | 18.7% | 17.7% |
| RGB 4:4:4 Animation | 14.9% | 8.8% | 8.5% |
| YCbCr 4:4:4 SC | 2.4% | 2.1% | 2.2% |
| YCbCr 4:4:4 Animation | 1.7% | 1.5% | 1.5% |
| RangeExt | 0.3% | 0.3% | 0.3% |
| RGB 4:4:4 SC (Optional) | 35.6% | 38.4% | 37.6% |
| YCbCr 4:4:4 SC (Optoinal) | 1.6% | 1.3% | 1.1% |
| Enc Time[%] | 95% | 96% | 97% |
| Dec Time[%] | 103% | 100% | 102% |

Table 10. Performance of explicit residual RDPCM off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 1.0% | 0.9% | 0.8% | 0.4% | 0.5% | 0.8% | 0.6% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| YCbCr 4:4:4 SC | 0.6% | 0.5% | 0.5% | 0.3% | 0.2% | 0.1% | 0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 5.9% | 7.1% | 7.3% | 6.9% | 7.8% | 9.9% | 8.7% |
| YCbCr 4:4:4 SC (Optoinal) | 2.1% | 1.9% | 1.7% | 2.2% | 2.0% | 3.1% | 2.6% |
| Enc Time[%] | 97% | 97% | 97% | 87% | 88% | 89% | 90% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 98% | 99% | 99% |

Table 11. Performance of explicit residual RDPCM off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.6% | 4.1% | 6.0% |
| Class B | 0.0% | 0.5% | 0.6% |
| RGB 4:4:4 SC | 1.1% | 4.1% | 5.2% |
| RGB 4:4:4 Animation | 0.1% | 0.9% | 1.1% |
| YCbCr 4:4:4 SC | 0.9% | 3.0% | 4.5% |
| YCbCr 4:4:4 Animation | 0.0% | 0.7% | 0.8% |
| RangeExt | 0.0% | 0.2% | 0.3% |
| RGB 4:4:4 SC (Optional) | 3.0% | 4.4% | 8.5% |
| YCbCr 4:4:4 SC (Optoinal) | 2.2% | 3.3% | 6.0% |
| Enc Time[%] | 98% | 93% | 94% |
| Dec Time[%] | 100% | 100% | 102% |

Table 12. Performance of Golomb-Rice group adaptation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.3% | 0.6% | 1.0% | 0.3% | 0.4% | 0.0% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.4% | 0.9% | 1.8% | 0.3% | 0.8% | 0.3% | 0.7% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | 0.3% | 0.7% | 1.2% | 0.3% | 0.6% | 0.1% | 0.3% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 4.5% | 7.1% | 10.3% | 4.7% | 7.4% | 3.6% | 7.5% |
| YCbCr 4:4:4 SC (Optoinal) | 1.2% | 2.5% | 4.4% | 1.0% | 1.9% | 1.2% | 2.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 98% | 99% | 99% |

Table 13. Performance of Golomb-Rice group adaptation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 1.2% | 0.7% | 0.3% |
| Class B | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 5.7% | 5.3% | 5.1% |
| RGB 4:4:4 Animation | 0.2% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 3.7% | 3.2% | 3.2% |
| YCbCr 4:4:4 Animation | 0.3% | 0.1% | 0.1% |
| RangeExt | 3.7% | 2.8% | 2.8% |
| RGB 4:4:4 SC (Optional) | 18.7% | 18.8% | 18.8% |
| YCbCr 4:4:4 SC (Optoinal) | 11.1% | 10.9% | 10.4% |
| Enc Time[%] | 101% | 100% | 100% |
| Dec Time[%] | 101% | 99% | 102% |

Table 14. Performance of implicit residual DPCM off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.9% | 1.0% | 1.1% | 0.5% | 0.4% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.4% | 0.5% | 0.5% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | 0.5% | 0.5% | 0.5% | 0.4% | 0.3% | 0.0% | -0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.7% | 0.9% | 0.8% | 0.3% | 0.3% | -1.4% | -1.4% |
| YCbCr 4:4:4 SC (Optoinal) | 1.3% | 1.4% | 1.5% | 0.4% | 0.6% | -0.4% | -0.4% |
| Enc Time[%] | 100% | 100% | 100% | 99% | 100% | 100% | 100% |
| Dec Time[%] | 99% | 100% | 100% | 99% | 98% | 99% | 99% |

Table 15. Performance of implicit residual DPCM off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 7.2% | 3.4% | 2.3% |
| Class B | 3.0% | 0.4% | 0.3% |
| RGB 4:4:4 SC | 7.8% | 4.2% | 3.4% |
| RGB 4:4:4 Animation | 8.3% | 1.5% | 0.9% |
| YCbCr 4:4:4 SC | 6.6% | 3.7% | 2.9% |
| YCbCr 4:4:4 Animation | 6.9% | 1.3% | 0.8% |
| RangeExt | 1.3% | 0.3% | 0.3% |
| RGB 4:4:4 SC (Optional) | 7.6% | 5.3% | 4.2% |
| YCbCr 4:4:4 SC (Optoinal) | 5.7% | 4.5% | 3.5% |
| Enc Time[%] | 102% | 100% | 100% |
| Dec Time[%] | 102% | 99% | 100% |

Table 16. Performance of intra block copy off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 16.1% | 13.5% | 11.0% | 11.9% | 10.1% | 6.6% | 6.1% |
| Class B | 0.4% | 0.3% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 51.5% | 51.0% | 49.5% | 39.6% | 39.2% | 25.3% | 26.4% |
| RGB 4:4:4 Animation | 0.9% | 0.7% | 0.4% | 0.5% | 0.3% | 0.2% | 0.1% |
| YCbCr 4:4:4 SC | 48.9% | 48.6% | 46.6% | 37.1% | 37.3% | 22.5% | 24.6% |
| YCbCr 4:4:4 Animation | 0.8% | 0.7% | 0.5% | 0.3% | 0.3% | 0.1% | 0.1% |
| RangeExt | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 105.5% | 109.8% | 113.8% | 84.8% | 89.7% | 55.2% | 58.9% |
| YCbCr 4:4:4 SC (Optoinal) | 120.0% | 125.0% | 128.5% | 101.4% | 107.3% | 74.1% | 85.4% |
| Enc Time[%] | 57% | 55% | 55% | 95% | 94% | 96% | 95% |
| Dec Time[%] | 113% | 114% | 114% | 101% | 100% | 100% | 100% |

Table 17. Performance of intra block copy off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 4.8% | 2.7% | 1.2% |
| Class B | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 33.8% | 28.0% | 21.5% |
| RGB 4:4:4 Animation | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 31.8% | 25.2% | 20.3% |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 86.4% | 74.1% | 68.3% |
| YCbCr 4:4:4 SC (Optoinal) | 109.6% | 99.7% | 98.8% |
| Enc Time[%] | 49% | 90% | 91% |
| Dec Time[%] | 111% | 102% | 102% |

Table 18. Performance of residual rotation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.4% | 0.4% | 0.3% | 0.3% | 0.2% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | 0.3% | 0.3% | 0.2% | 0.4% | 0.3% | 0.1% | 0.0% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.7% | 0.1% |
| YCbCr 4:4:4 SC (Optoinal) | 0.1% | 0.0% | 0.1% | 0.3% | 0.2% | 0.3% | 1.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 99% | 99% | 100% | 99% | 99% | 99% | 99% |

Table 19. Performance of residual rotation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.6% | 0.2% | 0.1% |
| Class B | 0.4% | 0.1% | 0.1% |
| RGB 4:4:4 SC | 0.3% | 0.2% | 0.1% |
| RGB 4:4:4 Animation | 0.4% | 0.1% | 0.1% |
| YCbCr 4:4:4 SC | 0.3% | 0.1% | 0.1% |
| YCbCr 4:4:4 Animation | 0.4% | 0.1% | 0.1% |
| RangeExt | 0.3% | 0.1% | 0.1% |
| RGB 4:4:4 SC (Optional) | 0.1% | -0.2% | 1.5% |
| YCbCr 4:4:4 SC (Optoinal) | 0.1% | 0.1% | -0.4% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 100% | 98% | 101% |

Table 20. Performance of single significance map context off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.7% | 0.7% | 0.7% | 0.6% | 0.7% | 0.3% | 0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.6% | 0.6% | 0.7% | 0.4% | 0.5% | 0.5% | 0.6% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% |
| YCbCr 4:4:4 SC | 0.5% | 0.5% | 0.6% | 0.7% | 0.8% | 0.5% | 0.5% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% |
| RGB 4:4:4 SC (Optional) | 0.7% | 0.5% | 0.4% | 0.6% | 0.6% | 0.0% | 0.2% |
| YCbCr 4:4:4 SC (Optoinal) | 0.4% | 0.3% | 0.4% | 0.7% | 0.5% | -0.9% | -1.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 99% | 100% | 100% | 99% | 99% | 98% | 99% |

Table 21. Performance of single significance map context off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.7% | 0.6% | 0.4% |
| Class B | 0.2% | 0.1% | 0.1% |
| RGB 4:4:4 SC | 0.6% | 0.6% | 0.6% |
| RGB 4:4:4 Animation | 0.5% | 0.5% | 0.5% |
| YCbCr 4:4:4 SC | 0.6% | 0.2% | 0.4% |
| YCbCr 4:4:4 Animation | 0.5% | 0.5% | 0.5% |
| RangeExt | 0.1% | 0.1% | 0.1% |
| RGB 4:4:4 SC (Optional) | 0.1% | 0.0% | 0.1% |
| YCbCr 4:4:4 SC (Optoinal) | 0.2% | 0.2% | 1.1% |
| Enc Time[%] | 104% | 104% | 103% |
| Dec Time[%] | 102% | 101% | 103% |

# Test results in AHG18 CTC

The coding gain and complexity of the additional coding tools are also evaluated under the AHG18 CTC of the HEVC range extensions [11]. In all the tests, the anchor is HM13.0-RExt6.0 and the tested condition is “coding tool off”. Table 22 to Table 35 summarize the test results for implicit RDPCM, explicit RDPCM, residual rotation, single context model for significance map coding in TS block, intra block copy, Rice parameter adaptation, and cross component de-correlation, respectively, in both lossy and lossless coding configurations.

The test results show that cross component de-correlation still provides the significant coding gain for high bit-depth (7.5~19.8% on average in lossy coding and 3.5~5.9% on average in lossless coding). However, intra block copy does not provide any coding benefit in both lossy and lossless coding configurations. The coding gain of explicit RDPCM, implicit RDPCM, residual rotation, and single context model for significance map coding in TS block is almost 0.0% in lossy coding. However, explicit and implicit RDPCM provide the coding gain in lossless coding (explicit RDPCM: 1.2~1.8% on average in LB, implicit RDPCM: 2.4~3.2% on average in AI), while the coding gain of residual DPCM and single context model for significance map coding in TS block is still minor (less than 0.5% in all coding configurations) even in lossless coding. Golomb-Rice parameter adaptation provides the significant coding gain for 16 bits, and it shows that this method is effective for handling large absolute values of transform coefficients at bit-depths higher than 12 bits.

Table 22. Performance of cross component de-correlation off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.6% | 1.3% | 0.3% | 0.5% |
| RExt RGB | 3.4% | 7.5% | 1.8% | 4.0% |
| HDR | 32.9% | 61.7% | 42.8% | 72.3% |
| 4K from HDR | 4.8% | 11.2% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 4.9% | 13.2% | 7.3% | 18.4% |
| Synthetic RGB HDR (+4 MSBs) | 8.0% | 23.1% | 13.5% | 28.6% |
| Synthetic RGB HDR (+6 MSBs) | 13.1% | 40.7% | 18.4% | 34.8% |
| All Classes | 7.5% | 17.6% | 10.5% | 19.8% |
| Enc Time[%] | 89% | 90% | 85% | 89% |
| Dec Time[%] | 103% | 104% | 105% | 104% |

Table 23. Performance of cross component de-correlation off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.5% | 0.8% | 0.2% | 0.4% |
| RExt RGB | 2.4% | 3.8% | 1.2% | 1.7% |
| HDR | 13.8% | 24.8% | 13.5% | 24.0% |
| 4K from HDR | 1.9% | 3.2% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 3.4% | 5.1% | 3.5% | 5.8% |
| Synthetic RGB HDR (+4 MSBs) | 4.1% | 7.5% | 4.2% | 6.9% |
| Synthetic RGB HDR (+6 MSBs) | 5.3% | 7.7% | 5.9% | 2.8% |
| All Classes | 3.5% | 5.9% | 3.6% | 5.2% |
| Enc Time[%] | 90% | 95% | 90% | 92% |
| Dec Time[%] | 103% | 104% | 104% | 105% |

Table 24. Performance of explicit residual DPCM off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.0% | 0.0% | -0.1% | 0.0% |
| HDR | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | -0.1% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.0% | 0.0% | -0.1% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.0% | -0.1% | 0.1% |
| All Classes | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% | 96% | 96% | 95% |
| Dec Time[%] | 100% | 100% | 100% | 101% |

Table 25. Performance of explicit residual DPCM off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.0% | 0.0% | 0.9% | 1.7% |
| HDR | 0.0% | 0.1% | 2.4% | 3.8% |
| 4K from HDR | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.9% | 1.4% |
| 4:0:0 Medical | 0.0% | 0.2% | 3.1% | 5.4% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.5% | 1.1% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.0% | 0.6% | 0.9% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.0% | 1.1% | 0.2% |
| All Classes | 0.0% | 0.0% | 1.2% | 1.8% |
| Enc Time[%] | 101% | 98% | 95% | 95% |
| Dec Time[%] | 101% | 100% | 104% | 105% |

Table 26. Performance of Golomb-Rice group adaptation off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 32.0% | 4.4% | 27.8% | 3.0% |
| RExt RGB | 17.0% | 0.8% | 12.0% | 0.2% |
| HDR | 6.0% | 0.6% | 3.7% | 0.1% |
| 4K from HDR | 18.7% | 1.4% |  |  |
| HDR 4:0:0 Medical | 1.5% | 0.0% | 1.2% | 0.2% |
| 4:0:0 Medical | 5.7% | 0.8% | 4.0% | 0.2% |
| Synthetic RGB HDR (+2 MSBs) | 6.6% | 0.1% | 2.2% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 1.2% | 0.0% | 0.1% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 0.1% | 0.0% | 0.0% | 0.0% |
| All Classes | 9.9% | 0.9% | 6.4% | 0.5% |
| Enc Time[%] | 105% | 101% | 104% | 101% |
| Dec Time[%] | 112% | 102% | 108% | 101% |

Table 27. Performance of Golomb-Rice group adaptation off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 51.8% | 24.4% | 51.4% | 22.8% |
| RExt RGB | 44.9% | 11.2% | 41.6% | 7.8% |
| HDR | 22.4% | 2.6% | 18.0% | 1.5% |
| 4K from HDR | 46.0% | 12.5% |  |  |
| HDR 4:0:0 Medical | 13.3% | 0.5% | 12.1% | 0.4% |
| 4:0:0 Medical | 31.8% | 3.8% | 27.5% | 2.7% |
| Synthetic RGB HDR (+2 MSBs) | 30.3% | 3.1% | 19.9% | 0.7% |
| Synthetic RGB HDR (+4 MSBs) | 12.5% | 0.1% | 6.2% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 3.2% | 0.0% | 0.8% | 0.0% |
| All Classes | 28.5% | 6.5% | 22.2% | 4.5% |
| Enc Time[%] | 103% | 108% | 103% | 103% |
| Dec Time[%] | 110% | 128% | 112% | 110% |

Table 28. Performance of implicit residual DPCM off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.0% | 0.0% | 0.0% | 0.0% |
| HDR | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical | -0.1% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 101% | 100% |

Table 29. Performance of implicit residual DPCM off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.3% | 0.5% | 0.0% | 0.0% |
| RExt RGB | 4.5% | 4.7% | 1.3% | 0.8% |
| HDR | 3.8% | 6.2% | 0.3% | 0.7% |
| 4K from HDR | 1.9% | 2.5% |  |  |
| HDR 4:0:0 Medical | 0.8% | 1.8% | 0.1% | 0.4% |
| 4:0:0 Medical | 2.7% | 2.7% | 0.4% | 0.2% |
| Synthetic RGB HDR (+2 MSBs) | 2.7% | 2.0% | 0.7% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) | 2.3% | 3.1% | 0.4% | 0.4% |
| Synthetic RGB HDR (+6 MSBs) | 2.1% | 5.0% | 0.1% | 1.0% |
| All Classes | 2.4% | 3.2% | 0.4% | 0.5% |
| Enc Time[%] | 103% | 107% | 101% | 101% |
| Dec Time[%] | 106% | 105% | 101% | 102% |

Table 30. Performance of intra block copy off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.1% | 0.1% | 0.0% | 0.0% |
| HDR | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR | 0.1% | 0.0% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.1% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.1% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.1% | 0.0% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.3% | 0.0% | 0.2% |
| All Classes | 0.1% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 38% | 41% | 82% | 85% |
| Dec Time[%] | 100% | 100% | 101% | 100% |

Table 31. Performance of intra block copy off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.0% | 0.1% | 0.0% | 0.0% |
| HDR | 0.0% | 0.1% | 0.0% | 0.0% |
| 4K from HDR | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.1% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.2% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 45% | 101% | 93% |
| Dec Time[%] | 100% | 100% | 101% | 103% |

Table 32. Performance of residual rotation off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.0% | 0.0% | 0.0% | 0.0% |
| HDR | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.0% | 0.0% | 0.1% |
| All Classes | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 99% | 100% |

Table 33. Performance of residual rotation off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.1% | 0.1% | 0.0% | 0.0% |
| RExt RGB | 0.1% | 0.2% | 0.1% | 0.1% |
| HDR | 0.4% | 0.5% | 0.0% | 0.0% |
| 4K from HDR | 0.2% | 0.2% |  |  |
| HDR 4:0:0 Medical | 0.4% | 0.6% | 0.1% | 0.1% |
| 4:0:0 Medical | 0.4% | 0.7% | 0.1% | 0.1% |
| Synthetic RGB HDR (+2 MSBs) | 0.1% | 0.3% | 0.0% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) | 0.2% | 0.4% | 0.0% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) | 0.3% | 0.2% | 0.1% | 0.0% |
| All Classes | 0.2% | 0.4% | 0.0% | 0.1% |
| Enc Time[%] | 102% | 101% | 101% | 101% |
| Dec Time[%] | 101% | 100% | 100% | 101% |

Table 34. Performance of single significance map context off vs. on (AHG18 CTC, Lossy)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB | 0.1% | 0.0% | 0.1% | 0.0% |
| HDR | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR | 0.1% | 0.1% |  |  |
| HDR 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) | 0.0% | 0.1% | 0.0% | 0.3% |
| Synthetic RGB HDR (+6 MSBs) | 0.0% | 0.1% | 0.0% | 0.1% |
| All Classes | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% | 100% | 99% | 99% |
| Dec Time[%] | 100% | 100% | 99% | 99% |

Table 35. Performance of single significance map context off vs. on (AHG18 CTC, Lossless)

| BD-rate Y | AI 16bit | AI 12bit | LB 16bit | LB 12bit |
| --- | --- | --- | --- | --- |
| SVT | 0.0% | 0.1% | 0.1% | 0.1% |
| RExt RGB | 0.1% | 0.1% | 0.1% | 0.1% |
| HDR | 0.0% | 0.2% | 0.1% | 0.1% |
| 4K from HDR | 0.1% | 0.1% |  |  |
| HDR 4:0:0 Medical | 0.1% | 0.2% | 0.1% | 0.2% |
| 4:0:0 Medical | 0.1% | 0.2% | 0.2% | 0.2% |
| Synthetic RGB HDR (+2 MSBs) | 0.1% | 0.1% | 0.1% | 0.2% |
| Synthetic RGB HDR (+4 MSBs) | 0.1% | 0.1% | 0.1% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) | 0.1% | 0.3% | 0.1% | 0.3% |
| All Classes | 0.1% | 0.2% | 0.1% | 0.2% |
| Enc Time[%] | 107% | 105% | 106% | 107% |
| Dec Time[%] | 103% | 104% | 102% | 104% |

# Comments on CU-adaptive chroma QP offset

CU-adaptive chroma QP offset was adopted at the 15th JCT-VC meeting in Geneva [9]. It allows finer QP control for chroma component than slice-level chroma QP offset. However, its effectiveness has never been tested actually, since it has not been integrated into the reference software until this contribution is submitted. As described in [9], this functionality is particularly useful for 4:4:4 video sequences, especially when natural and synthetic videos are mixed together in one video sequence. However, its effectiveness for 4:2:2 colour format is not clear, since no such mixed content is handled in 4:2:2 colour format. Furthermore, as summarized in Table 36, the bit-portion of syntax elements for chroma components in 4:2:2 is just slightly higher than 10% at QP ranges used in practical applications. This bit-portion, significantly lower than that in 4:4:4, means that the encoder-side rate control by adjusting the CU chroma QP offset is very limited in 4:2:2.

Table 36. Bit-portion of syntax elements for chroma components in 4:4:4, 4:2:2, and 4:2:0

| Sequence | All Intra |  |  |  | Random Access |  |  |  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 | 22 | 27 | 32 | 37 |
| Traffic_444_10bit | 45% | 37% | 28% | 20% | 61% | 51% | 35% | 18% | 57% | 46% | 33% | 22% |
| Kimono_444_10bit | 67% | 40% | 28% | 19% | 68% | 33% | 21% | 14% | 67% | 38% | 25% | 18% |
| EBULupoCandlelight_444_10bit | 19% | 17% | 16% | 17% | 17% | 13% | 13% | 14% | 20% | 16% | 16% | 16% |
| EBURainFruits_444_10bit | 21% | 18% | 15% | 14% | 12% | 10% | 7% | 6% | 15% | 12% | 10% | 9% |
| VenueVu_444_10bit | 43% | 40% | 36% | 33% | 34% | 32% | 30% | 28% | 37% | 35% | 33% | 31% |
| BirdsInCage_444_10bit | 67% | 55% | 32% | 28% | 91% | 86% | 42% | 33% | 87% | 78% | 40% | 32% |
| CrowdRun_444_10bit | 47% | 34% | 26% | 21% | 42% | 28% | 20% | 15% | 42% | 30% | 24% | 19% |
| Traffic_422_10bit | 31% | 24% | 17% | 12% | 37% | 27% | 15% | 8% | 36% | 27% | 18% | 12% |
| Kimono_422_10bit | 45% | 27% | 18% | 12% | 38% | 18% | 12% | 7% | 40% | 22% | 15% | 10% |
| EBUHorse_422_10bit | 25% | 16% | 11% | 9% | 31% | 13% | 10% | 8% | 30% | 17% | 12% | 9% |
| EBUGraphics_422_10bit | 37% | 29% | 25% | 23% | 32% | 26% | 24% | 22% | 34% | 29% | 25% | 23% |
| EBUWaterRocksClose_422_10bit | 32% | 20% | 13% | 7% | 25% | 12% | 6% | 3% | 23% | 11% | 6% | 3% |
| EBUKidsSoccer_422_10bit | 13% | 6% | 4% | 3% | 5% | 3% | 3% | 4% | 8% | 5% | 3% | 3% |
| Seeking_422_10bit | 31% | 23% | 16% | 12% | 23% | 16% | 13% | 10% | 25% | 19% | 15% | 12% |
| BasketballDriveClassB | 10% | 11% | 13% | 15% | 8% | 10% | 10% | 13% | 9% | 11% | 12% | 16% |
| BQTerraceClassB | 6% | 5% | 5% | 6% | 2% | 2% | 3% | 3% | 2% | 3% | 4% | 5% |
| CactusClassB | 16% | 13% | 13% | 16% | 13% | 9% | 9% | 10% | 13% | 10% | 10% | 13% |
| KimonoClassB | 19% | 14% | 11% | 11% | 13% | 8% | 6% | 7% | 14% | 10% | 8% | 9% |
| ParkSceneClassB | 16% | 12% | 10% | 11% | 10% | 8% | 7% | 7% | 12% | 10% | 9% | 10% |
| 4:4:4 Average | 44% | 34% | 26% | 22% | 46% | 36% | 24% | 18% | 46% | 36% | 26% | 21% |
| 4:2:2 Average | 31% | 21% | 15% | 11% | 27% | 17% | 12% | 9% | 28% | 18% | 13% | 10% |
| 4:2:0 Average | 13% | 11% | 10% | 12% | 9% | 7% | 7% | 8% | 10% | 9% | 9% | 11% |

# Recommended profiling of range extension coding tools

The profiling of the range extension coding tools is recommended to follow a similar process to that used for HEVC v1. As shown in the test results, the coding gain of all the additional tools except cross component de-correlation is minor for natural video sequences, even in 4:4:4 colour format.

There remains a market requirement for HEVC range extensions which fulfill the planned purpose of HEVC Amd 1, which was specified as follows in the Request for Amendment [15]:

“The purpose of this amendment of ISO/IEC 23008-2 is to provide the capability to support high fidelity video signals. In high end consumer and professional environment, larger sample accuracy and alternative chroma sampling structures such as 4:4:4 and 4:2:2 are used. This amendment is intended to extend the capability of ISO/IEC 23008-2 for those applications.”

Considering that the primary goal of HEVC range extensions is to support bit-depth higher than 10 bits and non-4:2:0 colour formats, it is suggested to include only cross component de-correlation and CU-adaptive chroma QP offset in Main 4:4:4 10/12 to be included in Amd1, which reaches FDAM in April 2014. CU-adaptive chroma QP offset is proposed to be included only in 4:4:4 profiles, since its benefit in terms of fine encoder-side rate control for chroma is limited in 4:2:2, as shown in Section 5. It is also suggested to include Golomb-Rice parameter adaptation only in 4:4:4 16b all-intra profile, since its major benefit in terms of coding efficiency is observed only at high bit-depth. Please note that CABAC bit alignment is included only in 4:4:4 16b all-intra profile, since it was the agreement at the last meeting.

Regarding SC coding tools, it is suggested to develop SC 4:4:4 Profiles, based on the evaluation results of the responses to the call for proposals for coding of screen content. It is suggested that all of the coding tools in the draft RExt text should be included in the initial test model for this new activity.

The recommended mapping between profiles and coding tools is summarized in Table 37.

Table 37. Mapping of coding tools onto Main profiles and initial TM for SC profiles

| Range extension coding tools | Main 12 | Main 4:2:2 |  | Main 4:4:4 |  | 4:4:4 All-Intra 16 | TM1 for SC 4:4:4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 | 12 | 10 | 12 |  | 8 | 10 |
| Implicit RDPCM |  |  |  |  |  |  |  |  |
| Explicit RDPCM |  |  |  |  |  |  |  |  |
| Residual rotation |  |  |  |  |  |  |  |  |
| Single ctx for TS |  |  |  |  |  |  |  |  |
| Intra block copy |  |  |  |  |  |  |  |  |
| Rice parm. adapt. |  |  |  |  |  |  |  |  |
| Cross comp. de-cor. |  |  |  |  |  |  |  |  |
| CU chroma QP offset |  |  |  |  |  |  |  |  |
| CABAC bit alignment |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, it is suggested that the decision process for profiling of range extension coding tools should follow a similar process to that used for HEVC v1.

Based on the coding gain and complexity trade-off for the additional range extension coding tools, evaluated under HEVC range extension common test condition, it is suggested that only cross component de-correlation and CU-adaptive chroma QP offset are included in Main 4:4:4 10/12 profiles to be included in Amd 1 which reaches FDAM in April 2014. It is suggested that this proposed mapping between HEVC range extension profiles and coding tools is adopted into the FDAM of HEVC range extension text and the corresponding reference software.

It is also suggested that SC 4:4:4 profiles should be developed, based on the evaluation results of the responses to the call for proposals for coding of screen content. It would be appropriate to include all the coding tools considered in this proposal in the initial test model for this new activity.

# References

- M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “RCE2 subtest C.2: Extension of residual DPCM to lossy coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0052, Jul. 2013.

- M. Naccari, M. Mrak, “RCE2: Experimental results for Test C.1,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0074, Jul. 2013.

- D. He, J. Wang, G. Martin-Cocher, “Rotation of Residual Block for Transform Skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0093, Jun. 2012.

- T. Tsukuba, T. Yamamoto, “Constant coefficient context for intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0069, Jun. 2012.

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

- M. Karczewicz, L. Guo, J. Sole, R. Joshi, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test 1 on Rice Parameter Initialization,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0199, Jan. 2014.

- W. Pu, W.-S. Kim, J. Chen, J. Sole, M. Karczewicz, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0202, Oct. 2013.

- D. Flynn, N. Nguyen, D. He, A. Tourapis, G. Cote, D. Singer, “RExt: CU-adaptive chroma QP offsets,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0044, Oct. 2013.

- K. Sharman, N. Saunders, J. Gamei, “RCE1: Results for tests B1, B2 and B3a,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0060, Jan. 2014.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1006, Jan. 2014.

- S. Lee, E. Alshina, C. Kim, K. McCann, “AHG5 and AHG8: Recommended profiling of range extension coding tools”, JCTVC-O0144, Oct. 2013.

- S. Lee, E. Alshina, C. Kim, K. McCann, “AHG5: Recommended profiling of range extension coding tools,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0106, Jan. 2014.

- T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou, C. Pang, J. Sole, L. Guo, M. Karczewicz, “RCE3: Results of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0180, Jan. 2014.

- MPEG Video and JCT-VC, “Request for ISO/IEC 23008-2:201X/Amd.1”, ISO/IEC document JTC1/SC29/WG11/N13344, Jan. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).