| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0057 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0039 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Comments on IRAP picture alignment and POC value derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jing Young Lee Sunil Lee Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Several restrictions on the IRAP picture alignment and the POC value derivation are proposed. With the proposed restrictions for IRAP alignments, the behaviour of POC value derivation can be simplified.

- A flag layerwise_start_up_disable_flag disabling a layerwise start-up is proposed.

- When all_layers_idr_aligned_flag is equal to 1, vps_poc_lsb_aligned_flag shall be equal to 1 and poc_reset_info_present_flag shall be equal to 0.

- Replacing the restriction that “the value of poc_reset_idc of all pictures in an access unit shall be the same” with “the value of poc_reset_idc of all slices in a layer shall be the same” is proposed.

- In MV-HEVC profile (stereo main profile), cross_layer_pic_type_aligned_flag and cross_layer_irap_aligned_flag are equal to 1 by default.

# Proposal

In this contribution, we propose several restrictions on the IRAP picture alignment and the POC value derivation. When the proposed restrictions for IRAP alignments are imposed, the behaviour of POC value derivation can be simplified. Each proposal is described below.

  - layerwise_startup_disable_flag

To notify a layerwise start-up is disabled in some bitstreams, a flag layerwise_startup_disable_flag in VPS extension is proposed.

- Discussion: A layerwise start-up is a useful feature to enable unequal IRAP periods for multiple layers. However, if this feature is never used for some bitstreams, e.g. stereo video, this flag will be helpful for simplifying the decoder behaviour, especially for the POC value derivation. Furthermore, this is helpful for session negotiation, because some simple systems may not be compatible with a layerwise start-up.

Proposed specification text:

vps_extension( ) {; Descriptor. …. layerwise_startup_disable_flag; u(1). vps_poc_lsb_aligned_flag; u(1). …. }. layerwise_startup_disable_flag equal to 1 indicates a layerwise start-up is disallowed in the current bitstream. When layerwise_startup_disable_flag is equal to 1, cross_layer_irap_aligned_flag, and vps_poc_lsb_aligned_flag shall be equal to 1, and poc_reset_info_present_flag shall be equal to 0. layer_wise_startup_disable_flag equal to 0 specifies that the above restriction may or may not apply. When layerwise_startup_disable_flag is equal to 1, the variables LayerInitializedFlag[ i ] and FirstPicInLayerDecodedFlag[ i ] shall have the same value for all values of i from 0 to vps_max_layer_id.

1.2. all_layers_idr_aligned_flag

When all_layers_idr_aligned_flag is equal to 1, vps_poc_lsb_aligned_flag shall be equal to 1 and poc_reset_info_present_flag shall be equal to 0.

- Discussion: The proposed restriction will be helpful for checking errors in conformance bitstreams, and simplifying the POC value derivation in decoders.

  - poc_reset_idc

It is proposed to replace the restriction that “the value of poc_reset_idc of all pictures in an access unit shall be the same” with “the value of poc_reset_idc of all slices having the same value of nal_unit_type in an access unit shall be the same”.

- Discussion: This relaxation can avoid unnecessary POC value reset of decoded pictures in DPB. When a POC value of a layer is equal to a POC value of a base layer without POC reset, the redundant POC reset process can be skipped even if the POC values of other layers are reset.

  - Restrictions for MV-HEVC

In MV-HEVC profile (stereo main profile), cross_layer_pic_type_aligned_flag and cross_layer_irap_aligned_flag are equal to 1 by default.

Proposed specification text:

cross_layer_pic_type_aligned_flag equal to 1 specifies that within a CVS that refers to the VPS, all VCL NAL units that belong to an access unit have the same value of nal_unit_type. cross_layer_pic_type_aligned_flag equal to 0 specifies that within a CVS that refers to the VPS, all VCL NAL units in each access unit may or may not have the same value of nal_unit_type. When not present, the value of cross_layer_pic_type_aligned_flag is inferred to be equal to 1 in MV-HEVC profiles. cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply. When not present, the value of cross_layer_irap_aligned_flag is inferred to be equal to 1 in MV-HEVC profiles. For other profiles, the value of cross_layer_irap_aligned_flag is inferred to be equal to vps_vui_present_flag when not present.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).