| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0062_r1 |
| --- | --- |

| Title: | AhG5: On the displacement vector prediction scheme for Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrice Onno Guillaume Laroche Christophe Gisquet Tangi Poirier Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33(0)299876800 patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the displacement vector prediction scheme for the Intra Block Copy method. In the current design of IBC, the displacement vector predictor corresponds to the latest displacement vector used for the last CU coded with the IBC mode. In this contribution, the displacement vector predictor scheme is changed to take into account the new 2NxN, Nx2N and NxN IBC partitions adopted at the last meeting. In average, it is reported that the new proposed prediction method based on the three last displacement vectors gives -0.3% gain for the AI and -0.2% for the RA/LB configurations. It is also reported that the proposed scheme shows some gains for the Lossless case as well.

# Introduction

At the 15th JCTVC meeting, the method of JCTVC-O0122 [2] was adopted in order to predict the displacement block vector of the current CU by using the displacement vector of the previously IBC-coded CU. This method was considered as the best tradeoff between complexity and coding efficiency performance. However, with the adoption during the 16th JCTVC meeting of the 2NxN, Nx2N and NxN IBC partition, the current displacement vector prediction scheme based on the latest IBC block is not the best performing in terms of BD-Rate solution due to this new partition. This contribution is presenting a simple modification of the current IBC displacement vector prediction design to get higher performance.

# Proposed method

In this contribution, we are proposing to use a predictor list which includes the 3 last decoded IBC displacement vectors and indicate through a syntax element the predictor index to select the right predictor in that list. This is similar to the “ping-pong” method introduced in contribution [3] where the two last decoded IBC vectors were used and the selected one was signaled through a flag.

With the new IBC partition, three predictors is the best trade-off between performance and storage memory compared to other prediction schemes like in [4] where they are using the displacement vectors of the top and left CUs. In that particular case of [4], it is necessary to store a line and a column of IBC displacement vectors. With the new 2NxN, Nx2N and NxN partitions of IBC, it is necessary to store up to 31 vectors for the worst case. Compared to the current design of using only the last displacement vector, the proposed method with the 3 last decoded vectors provides a coding efficiency gain since there is a better match of the current IBC block due to the new IBC partition and its associated scan order. This is not the case with the last decoded displacement vector which is unique. In terms of memory, the additional memory to store two additional vectors is negligible.

# Results

The proposed method was implemented in the Range extension software version 6.0. The results were generated with the conditions as described in the Common Test Conditions [5] for the Range Extensions. Error: Reference source not found gives the results for Lossy configuration.

## Lossy results

The proposed method gives an average gain (excluding the optional sequences) of -0.3% for the AI, and 
-0.2% for the Random Access and Low Delay cases as depicted in Table 1.

Table 1: Lossy results of the proposed method

## Lossless results

Table 2 shows the results of the methods for the lossless configuration. The modification of the prediction scheme shows improvement for the lossless case as well.

Table 2: Lossless results of the proposed method

# Syntax change for the prediction

### Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

- a variable bvpIndex specifying the predictor index for the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array of block vectors bvIntra.

The variables nCbS, nCbSw, nCbSh are derived as follows:

nCbS = 1  <<  log2CbSize		(8-1)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-2)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-3)

The variable BvpIntra[ bvpIndex ] [ compIdx ] specifies a list of  block vector predictor where bvpIndex is ranging from 0 to  2 . The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. The variable BvpIntra[ bvpIndex ] [ compIdx ] is initialized as follows:

BvpIntra[ 0 ] [ 0 ] = - nCbS, BvpIntra[ 0 ] [ 1 ] = 0.

BvpIntra[ 1 ] [ 0 ] = - 2*nCbS, BvpIntra[ 1 ] [ 1 ] = 0.

BvpIntra[ 2 ] [ 0 ] = - 3*nCbS, BvpIntra[ 2 ] [ 1 ] = 0.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-4)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-5)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ bvpIndex ][ 0 ]	(8-6)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ bvpIndex ][ 1 ]	(8-7)

- Depending of the value of bvIntra[ xPb ][ yPb ][ compIdx ], the following update is perfomed

  - If  (bvIntra[ xPb ][ yPb ][ compIdx ] != BvpIntra[ 1 ][ compIdx ]) and (bvIntra[ xPb ][ yPb ][ compIdx ] != BvpIntra[ 0 ][ compIdx ]):

The value of BvpIntra[ 2 ][ compIdx ] is updated to be equal to BvpIntra[ 1 ][ compIdx ]. 
The value of BvpIntra[ 1 ][ compIdx ] is updated to be equal to BvpIntra[ 0 ][ compIdx ].
The value of BvpIntra[ 0 ][ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]

#### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| bvp_idx | ae(v) (TR) |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |

# Conclusion

In this contribution, we propose to modify the IBC displacement vector prediction scheme to take into account the new partitions (2NxN, Nx2N and NxN) of Intra Block Copy. The proposed method consists in using the three last decoded IBC displacement vectors to build a predictor list and then in signaling the selected displacement vector by an index in the bitstream.

From a coding efficiency point of view, this method provides an average gain of -0.3% by using the Common Test Conditions of RExt 6.0. Moreover, the additional memory needed to store these two additional vectors compared to the current scheme is negligible.

# References

- D . Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan and T. Suzuki, “HEVC Range Extensions Draft 6”, JCTVC-P1005, 16th meeting, San-Jose, January 2014.

- G. Laroche, T. Poirier and C. Gisquet, “AHG5: Vector prediction for Intra Block Copy”, JCTVC-O0122, 15th meeting, Geneva, October/November 2013.

- L. Zhu, J. Xu, G. J. Sullivan, S. Sankuratri, B. A. Kumar and F. Wu, “Initialization of block vector predictor for intra block copy”, JCTVC-P0217, 16th meeting, San-Jose, January 2014.

- C. Pang, J. Sole, R. Joshi and M. Karczewicz “Non-RCE3: Block vector signaling for intra block copy”, JCTVC-P0149, 16th meeting, San-Jose, January 2014. (Qualcomm)

- C. Rosewarne, K. Sharman and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th meeting, San-Jose, January 2014.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).