| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0064 |
| --- | --- |

| Title: | AhG10: palette index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

Palette mode in AhG10 uses FLC for coding the index. This proposition instead uses two codelengths depending on the index, as well as removing impossible values from the set. It is reported that the use of one such prediction mode provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 software, of more than 1.6%/1.7%/1.6% for SC classes, and up to 6.1%/6.2%/6.4% on optional classes.

# Proposal

There are actually 3 things that are combined and together affect the index coding.

## Impossible values

Currently, when the mode on the left is copy-left, the index to code cannot be the index on the left, otherwise the copy left run would have been incremented by one. Similarly, when the mode on the left is copy up, the index to code cannot be the same as the one above, otherwise the copy up run would have been incremented by one.

This is cannot be exploited alone because of the FLC used for the index.

## Pseudo VLCs for index coding

When the palette size is smaller than allowed by the binary FLC coding of the index, inefficiency occurs. A typical solution is to use VLCs, but unless they produce easily definable codewords (e.g. Exp/Rice-Golomb coding), they require the use of important value/length pair tables. For palette coding, this represent from 465 (if palette sizes that are powers of 2 are excluded) to 527 pairs.

Therefore, it was proposed in [1] to code index+run pairs within the unused index values. This represented only a minimal improvement.

The index values, once the prediction mode are used, are still not equiprobable, but it is difficult to reorder them as the palette prediction already enforces some restriction. As a consequence, it is both simpler and safer to just assume equiprobable symbols. In that case, it is easy to design simple VLCs not requiring additional tables.

One such example is, for a palette size of P = 2N - R with N ≥ 0 and 0 < R ≤ 2N integers, one can code and index “idx” as follows:

- If idx < R, idx is coded on N-1 bits;

- Otherwise, idx+R is coded on N bits.

## Escape signalling

AhG10 reserves the last palette index to signal the use of an escape-coded pixel. This method was considered to be a simplification, albeit with a non-negligible coding loss.

However, most of the loss can be overcome by employing a trick described in [2]: the bitstream can be made to guarantee that, whenever the palette size is below the maximum, there is no escape-coded pixel. Indeed, during palettization, such kind of pixels can be added to the palette until there is no such pixel left, or the maximum palette size has been reached. This is actually what happens with AhG10 palettization method.

As a consequence, the reservation of the last palette index can be only performed for palette sizes of 31 (i.e., index 31 represents the escape index). This again affects the index coding.

# Results

Using AhG10 software as reference, we have for the AI lossy configurations:

The most noticeable part is how well it performs on class F, which is otherwise little affected by palette coding.

For the lossless configuration, we have:

# Conclusion

It is proposed a relatively simple VLC coding for the indexes of the palette mode, in particular not requiring important tables. In light of the coding gains, it is suggested to further study this during any further palette coding investigations.

# References

- C. Gisquet, G. Laroche, P. Onno (Canon), “Non-RCE4: combined coding of run and index for RCE4 Test2”, JCTVC-P0117, 16th Meeting: San Jose, USA, Jan. 2014

- W. Pu, F. Zou, J. Sole, M. Karczewicz, R. Joshi (Qualcomm), “Non-RCE4: Refinement of the palette in RCE4 Test 2”, JCTVC-P0231, 16th Meeting: San Jose, USA, Jan. 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).