| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0066 |
| --- | --- |

| Title: | AhG10:Run coding for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the palette mode of the AhG10 software. This modification relates to the run coding and was proposed in JCTVC-P0113. The principle is to limit the number of runs coded after a palette index of the run mode. A syntax element is transmitted for each palette CU to decode or not the run. The proposed modification of the AhG10 software gives 0.4% gain on top of AhG10 software for screen content sequences for Intra configurations.

# Introduction

The palette mode coding of the AhG10 software uses 2 modes to code the palette index: the “Run mode” and the “Copy above mode”. For the Run mode, a palette index is transmitted, followed by a run value, the “Palette run”, which indicates the number of following palette indexes with the same value. For the Copy above mode, the “Copy_run” is transmitted to indicate that, for the following positions (including the current one), the palette index is equal to the palette index of the one that is at the same location in the row above.

# Proposed Method

The proposed modification is applied only for the Run mode. The method consists in indicating the maximum palette index value (Limit_run) for which the run is coded. Thus, for all palette index values superior to this Limit_run, the run is not coded.

The syntax element Limit_run is decoded just after the palette_entries for each Palette CU. It is coded with a unary max code with the maximum value equal to the Palette_size.

On encoder side, the Limit_run is determined a posteriori. It means that, during the selection between the Run mode and the Copy above mode, the rates of using or not the proposed method are estimated. At the end of this process, the best Limit_run value is determined and the syntax of the current palette CU is updated.

# Experimental results

The proposed method was implemented on top of the AhG10 software. This software is the RExt6.0 where the palette as described in JCTVC-P0303 was integrated.

The modification gives 0.4% gain in average for Class F and non-optional SC 4:4:4 (RGB & YUV).

Table 1 Results of the proposed modification for Lossy configuration compared to the AhG10 software (Rext6.0 + P0303)

# Conclusion

This contribution proposes a modification to improve the coding of the run of the Palette Mode Coding software of the AhG10. According to the interesting gains compared to the small increase of complexity, we recommend to consider these modifications if the Palette Mode coding is adopted.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).