| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0067 |
| --- | --- |

| Title: | Non-RCE1: On MV resolution and motion vector predictor number |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The HEVC Range Extensions Core Experiment 1 is dedicated to the adaptive Motion vector precision method. In this RCE1, 2 methods to signal the motion vector precision are investigated. One signals the MV precision at slice level and the second one at CU level. Both methods modify the Merge and Inter/AMVP modes. In this contribution, the Motion vector precision is signaled at PU level for Inter mode only. Moreover, when the precision is set equal to the full-pel, the number of predictors for Inter/AMVP is reduced to 1. Several complexity compromises are reported. The proposed modification can reach 2.1% gain over RExt6.0 for RA/ LDB configurations. When the encoding complexity is reduced to 110% the average gain for all RA/LDB configurations is 1.7%.

# Introduction

For the first method tested in RCE1 [1], 2 bits are transmitted in the slice header to indicate the MV precision between the ¼, the ½ and the full-pel resolutions. When the ½ or the full-pel resolution is selected, the motion vector predictor, and the motion vector residual are changed compared to the current range extension.

In the second method [2], a flag is transmitted at CU level to indicate whether or not all PU in the CU have an integer precision. When this flag is on, the motion vector predictors are rounded to the integer precision before motion compensation and the motion vector differences are signaled in integer precision.

In this contribution, the motion vector precision is adapted at PU level for the AMVP mode only but not for the Merge mode. In addition, the number of motion vector predictor is modified.

# Proposed Method

## Adaptive motion vector resolution for Inter/AMVP PU

Our analysis of the adaptive motion vector resolution at CU level or slice level showed that the gains come from the Inter/AMVP mode.

In our proposed method, a flag pu_imv_flag is transmitted for each Inter/AMVP mode PU. When this flag is on, the integer precision is enabled for current Inter/AMVP PU. The motion vector predictors are rounded to the integer precision before motion compensation. Except when the mvd_l1_zero_flag is set equal to 1, the motion predictors of L1 for a bi-directional Inter mode are not rounded.

When the flag pu_imv_flag is equal to 1, the motion vector differences are signaled in integer precision.

## One predictor for Integer precision

The motion vector resolution is not the only parameter which can be reduced for some Inter/AMVP PU. Indeed, the reduction of the number of predictors for AMVP from 2 to 1 predictor improves the coding efficiency when it is combined with the reduction of the vector resolution.

In the proposed method, when the pu_imv_flag is set to 1, the number of predictors for the current Inter/AMVP PU is set equal to 1 and no predictor index (mvp_l0_flag, mvp_l1_flag) is decoded.

## Fast estimation

To limit the impact on complexity when setting the best value for pu_imv_flag, we are performing twice the Inter/AMVP motion estimation at integer precision only. In our proposed fast estimation algorithm, after the motion estimation of the Inter/AMVP PU for the quarter-pel estimation, a second motion estimation for integer precision only (no sub-pel refinement) is carried out and this second estimation is performed with only one predictor. In addition, to limit the number of motion estimations when the first motion estimation leads to a motion vector equal to (0, 0), the second motion estimation is not performed and pu_imv_flag is set equal to 0.

As opposed to RCE1 Test2 [2], the residual coding is not computed twice as the motion estimation for the Merge.

# Experimental results

The proposed method is implemented on top of Rext6.0 by using the testing conditions of [3]. In this section, the results of 4 experiments are provided for a better analysis of the proposed modifications. In the first experiment, Test 1, the adaptive motion vector resolution for Inter/AMVP PU (MV res) is combined with 1 motion vector predictor for full-pel resolution (1 Pred). The second one, Test 2, is the same as Test 1 with the proposed fast estimation. Test 3 is the same as Test 2 without 1 predictor for full-pel resolution. Test 4 is the fast estimation without change of the Rext6.0 syntax.

Table 1 Summary of results of the proposed modifications  summarizes the results of the 4 proposed experiments. The maximum average gain is obtained by Test 1 with 1.6% over both Inter configurations. But this gives the highest encoding run time with 120%. When the fast estimation is enabled, the encoding time decreases to 110%, yet the average gain is 1.1% for classical RExt sequences and 1.7% with optional sequences.

Test 3 gives the results of the use of the adaptive motion vector resolution for Inter/AMVP PU only. This gives 0.7% average gain with the same complexity as Test2 where this adaptive MV resolution is combined with the use of 1 predictor for full-pel resolution. Yet, Test 2 gives better results with 0.4% to 0.6% additional gains compared to Test 3. For tests 1, 2 and 3, the decoding time decreases compared to Rext6.0 and reaches 94% for Test 2.

Test 4, gives only 0.1% BDR gains with the same encoding time complexity as Test 2 and Test 3. This confirms that the gain doesn’t come from the estimation.

Table 1 Summary of results of the proposed modifications

|  | MV RES Inter/ AMVP PU |  | 1 Pred |  | Fast Estimation |  | Average BDR |  | Average BDR +optional |  | Enc Time |  | Dec Time |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 |  | ✓ |  | ✓ |  |  |  | -1.6% |  | -2.1% |  | 120% |  | 95% |  |
| Test 2 |  | ✓ |  | ✓ |  | ✓ |  | -1.1% |  | -1.7% |  | 110% |  | 94% |  |
| Test 3 |  | ✓ |  |  |  | ✓ |  | -0.7% |  | -1.1% |  | 110% |  | 98% |  |
| Test 4 |  |  |  |  |  | ✓ |  | -0.1% |  | -0.1% |  | 110% |  | 99% |  |
| RCE1 Test2 MV1 |  |  |  |  |  |  |  | -0.6% |  | -0.7 |  | 148% |  | 101% |  |
| RCE1 Test2 MV2 |  |  |  |  |  |  |  | -0.9% |  | -1.2% |  | 142% |  | 95% |  |

Table 1 Summary of results of the proposed modifications  gives also the results of RCE1 Test2 MV1 and MV2. RCE1 Test2 MV1 is only an encoding choice (No RExt Syntax change) where the MV resolution is compared at CU level. RCE1 Test2 MV2 is the adaptive motion vector resolution at CU level. Test 1, Test 2 and Test 3 give better complexity compromises than the 2 experiments RCE1 Test2.

Table 2 gives the detailed results of Test 2. The proposed method gives systematic gain over all Classes.

The results of Test 1, 2, 3 and 4 were generated for the lossless configurations and are attached to this contribution. The conclusions are equivalent to the conclusions for lossy configurations.

Table 2 Results of Test 2 (MV RES PU Inter/AMVP + 1 Pred) for the Inter lossy configurations compared to Rext6.0.

# Conclusion

The results provided in that contribution is above the results provided by the two tests in the RCE1 both in terms of run time and coding efficiency. We thus recommend considering this contribution when discussing RCE1 results and before any change of the motion accuracy signaling based on RCE1 contributions.

# References

- Y. Zhou, B. Li, J. Xu, G. J. Sullivan and B. Lin, “Motion Vector Resolution Control for Screen Content Coding”, JCTVC-P0277, 16th JCTVC meeting, San-Jose, January 2014.

- X. Li, J. Sole, M. Karczewicz ,“Adaptive MV precision for Screen Content Coding”, JCTVC-P0283, 16th JCTVC meeting, San-Jose, January 2014.

- C. Rosewarne, K. Sharman and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th JCTVC meeting, San-Jose, January 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).