| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0069 |
| --- | --- |

| Title: | AhG10: Combination of palette coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

Several palette coding tools were proposed in the scope of AhG10. This proposal studies their combination, together with several bugfixes and more limited tools. It is reported that the combination provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 software, of more than 5.3%/7.0%/8.0% for SC classes, and up to 11.6%/12.6%/13.8% on optional classes.

# Proposal

The combination consists in:

- A mode predicting the index value, based on transition memory (JCTVC-Q0065);

- A palette prediction method that merges the last coded palette and last predictor to form the predictor for next palette-coded CU, as well as additional flags to stop early the signalling of reused elements from the predictor (JCTVC-Q0063);

- A method to signal which index values when in “copy left” prediction mode are not run-length coded (JCTVC-Q0066);

- An index coding method employing VLCs and the knowledge of cases where an index is not possible (JCTVC-Q0064);

- An palettization algorithm inspired from [1], which essentially is a form of Kmeans++ algorithm

- Bugfixes:

  - The “copy up” mode was sometimes incorrectly evaluated as invalid;

  - The encoder estimation sometimes performed twice the palette syntax writing, most often repeating a flag set to 0, but more particularly, writing twice the complete syntax for a palette-coded CU in inter slices – bitstream writing is fine.

# Results

The test uses the settings employed by each other contribution.

## Compared to AhG10

Using AhG10 software as reference, we have for the AI lossy configurations:

For the AI lossless configuration, we have:

## Compared to HM13.0+RExt6.0

The results are for the AI configuration:

# Conclusion

This contribution demonstrates a high margin of gain in the palette coding mode. It is suggested to continue studying its different elements in the scope of Screen Content coding.

# References

- C. Gisquet, G. Laroche, T. Poirier, P. Onno, “Non-RCE4: combination of improvements for Palette mode”, JCTVC-P0119, 16th Meeting: San Jose, USA, Jan. 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).