| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0070 |
| --- | --- |

| Title: | Consistent usage of intra boundary filter disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC range extensions, the horizontal and vertical gradient filters are disabled when implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, from JCTVC-O0147. In addition to the gradient filters, the DC filter is another boundary filter to smooth the intra block boundary. However, the variable disableIntraBoundaryFilter only controls the gradient filters. Therefore, this contribution firstly proposes method 1 to consistently utilize of boundary-filter disabling in lossless coding by disabling and enabling the gradient filters and DC filter together. Considering the video characteristics that some videos do not need to smooth the boundary in intra prediction, in method 2, the boundary filters are also suggested to be optionally disabled for lossy coding. Experimental results reportedly show the consistent usage of intra boundary filters (method 1) does not change the bit-rate noticeably and can achieve up to 0.2% bit-savings for AHG8 lossless coding conditions.

# Introduction

In HEVC range extensions [1], for intra DC, horizontal, and vertical modes, boundary filters (or smoothing filters) are applied, including the DC filter and the horizontal and vertical gradient filters. The boundary prediction samples of DC mode will be smoothed with an [1, 3] or [1, 2, 1] filter to reduce the blocking artifact, as shown in Fig. 1. For vertical and horizontal intra prediction directions, a gradient based boundary filter is applied. Fig. 2 shows an example for the gradient based boundary smoothing filter for vertical intra prediction direction.

.

Fig. 1.  DC filter

Fig. 2.  Gradient filter for vertical prediction

In the current text of HEVC range extensions, however, the variable disableIntraBoundaryFilter only controls the gradient horizontal and vertical filters, as proposed in JCTVC-O0147 [2].

The variable disableIntraBoundaryFilter is derived as follows:

–	If implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

The disableIntraBoundaryFilter controls the gradient filters as follows:

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-1)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32 and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-2)

# Proposed method

Method 1:

In this contribution, it is proposed to calculate a variable disableIntraBoundaryFilter also for DC filtering process, in the way of calculating disableIntraBoundaryFilter for gradient filters. And then, the DC filtering process is disabled when disableIntraBoundaryFilter equals to 1.

Method 2:

It is proposed to add an optional flag in PPS and slice header, which controls the disabling of two boundary filters.

# Experimental results

The proposed methods are integrated on HM-13.0+RExt-6.0 [3].	Table 1 shows the results for lossless under the AHG8 common test condition [4]. Because lossy coding never disables the boundary filters, this contribution has no effect on lossy coding. Experimental results show, there is 0.2% average bit saving on YUV444 SC set in Random Access conditions for Method 1.

Table 1.  Results of Method 1 for lossless coding under AHG8 CTC

Table 2.  Results of Method 1 for lossless coding under SCC_CFP CTC

# WD Change

Method 1

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-3)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-4)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-5)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-6)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-7)

Method 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| If(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |
| ... |  |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

- Depending on the value of the colour component index cIdx, the following applies:

- If IntraBoundaryFilterSliceDisable is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-8)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-9)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-10)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-11)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-12)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

Method 1+Method 2( the same performance as Method 2)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| If(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |
| ... |  |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-13)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-14)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-15)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-16)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-17)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

# Conclusion

This contribution proposes to consistently disable the DC boundary filter with gradient filters for lossless coding. Experimental results reportedly show there is 0.2% average bit saving on YUV444 SC set in Random Access conditions in Method 1, without loss for other sets of videos.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014

- M. Zhou, “RExt: Proposed changes in the horizontal and vertical gradient filtering control for intra prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0147, Oct. 2013

- HM-13.0+RExt-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1006, Jan. 2014