| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0070_r1 |
| --- | --- |

| Title: | Consistent usage of intra boundary filter disabling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC Range Extension Draft 6, the gradient filters in Intra horizontal and vertical modes are disabled when the derived value of disableIntraBoundaryFilter is 1. This occurs only when implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1. In this contribution, it is proposed to: First, using the same derivation of disableIntraBoundaryFilter as in HEVC Range Extension Draft 6, also disable the boundary filters in Intra DC mode when the derived value of disableIntraBoundaryFilter is 1. Second, conditionally disable the boundary filters for Intra horizontal, vertical, and DC modes altogether for lossy coding, conditioned under flag control in PPS and slice header. Experimental results reportedly show that the first method does not produce any noticeable changes in coding efficiency. As for the second method when the Intra boundary filters are always disabled, the results demonstrate the following under AHG8 test conditions:

The numbers below are for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC.

- AI Main-Tier: 0.1%, 1.6%, 0.5%, 1.2%, and 1.3% BD-rate savings.

- RA Main-Tier: 0.0%, 1.3%, 0.2%, 0.8%, and 0.9% BD-rate savings.

- LD Main-Tier: 0.1%, 0.6%, 0.2%, 0.2%, and 0.8% BD-rate savings.

- A class-wise BD-rate loss ranging from 0.1% to 0.5% is reported for Class B and RangeExt.

The test results under the test conditions of Joint Call for Proposals for coding of screen content are also provided in this contribution.

# Introduction

In HEVC Range Extensions Draft 6[1], for intra DC, horizontal, and vertical modes, boundary filters (or smoothing filters) are applied. The boundary prediction samples of DC mode will be smoothed with an [1, 3] or [1, 2, 1] filter to reduce the blocking artifact, as shown in Fig. 1. For vertical and horizontal intra prediction directions, a gradient based boundary filter is applied. Fig. 2 shows an example for the gradient based boundary smoothing filter for vertical intra prediction direction.

.

Fig. 1.  DC filter

Fig. 2.  Gradient filter for vertical prediction

In the current text of HEVC range extensions, however, the variable disableIntraBoundaryFilter only controls the gradient horizontal and vertical filters, as proposed in JCTVC-O0147 [2].

The variable disableIntraBoundaryFilter is derived as follows:

–	If implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

The disableIntraBoundaryFilter controls the gradient filters as follows:

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-1)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32 and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-2)

# Proposed method

Method 1: Also disabling Intra DC boundary filter when disableIntraBoundaryFilter = 1

In this contribution, it is proposed to also disable the Intra DC boundary filtering process, under the same derivation of disableIntraBoundaryFilter for Intra horizontal and vertical gradient filters. That is, the DC filtering process is also disabled when disableIntraBoundaryFilter equals to 1.

Method 2: Disabling Intra boundary filters using PPS and slice-level control

It is proposed to add a flags in PPS and slice header, which control the disabling of two boundary filters (Intra DC and Intra gradient filters). This provides the capability of disabling these filters under lossy conditions (cu_transquant_bypass_flag = 0). In the lossy test results provided in this contribution, the boundary filters are always disabled using these control flags. Note that it is an encoder choice to turn on or off the Intra boundary filters.

# Experimental results

The proposed methods are integrated on HM-13.0+RExt-6.0 [3].

Method 1: Also disabling Intra DC boundary filter when disableIntraBoundaryFilter = 1

Under the current test conditions, cu_transquant_bypass_flag is 1 only for lossless coding. Thus the proposed first method only affects lossless coding results. Table 1 shows the results under the AHG8 common test condition [4] and Table 2 shows the results under the test conditions of Joint Call for Proposals for coding of screen content.

Table 1.  Results of Method 1 for lossless coding under AHG8 CTC

Table 2.  Results of Method 1 for lossless coding under SCC_CFP CTC

Method 2: Disabling Intra boundary filters using Picture / slice level control

With the introduced flags, Intra boundary filters are always disabled for lossy conditions in the following tests. Table 3 shows the results under the AHG8 common test condition [4] and Table 4 shows the results under the test conditions of Joint Call for Proposals for coding of screen content.

Table 3.  Results of Method 2 for lossy coding under AHG8 CTC

Table 4.  Results of Method 2 for lossy coding under SCC_CFP CTC

# WD Change

Method 1

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-3)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-4)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-5)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-6)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-7)

Method 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| If(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |
| ... |  |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_pic_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

- Depending on the value of the colour component index cIdx, the following applies:

- If IntraBoundaryFilterSliceDisable is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-8)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-9)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-10)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-11)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-12)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

Method 1+Method 2( the same performance as Method 2)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| If(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |
| ... |  |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_pic_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-13)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-14)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-15)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-16)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-17)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

# Conclusion

This contribution proposes to consistently disable the DC boundary filter with gradient filters for lossless coding. Experimental results reportedly show there is 0.2% average bit saving on YUV444 SC set in Random Access conditions in Method 1, without loss for other sets of videos.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014

- M. Zhou, “RExt: Proposed changes in the horizontal and vertical gradient filtering control for intra prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0147, Oct. 2013

- HM-13.0+RExt-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1006, Jan. 2014