| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0071 |
| --- | --- |

| Title: | AHG10: Improvements on palette coding |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Döne Bugdayci, Jani Lainema, Kemal Ugur, Miska Hannuksela | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes three tools to improve coding efficiency of palette based coding of screen content. The tools; (i) long-term palette prediction, (ii) restricted run-length coding and (iii) adaptive scanning of palette coded CUs; are implemented on top of the RExt 5.1 based JCTVC-P0303 software. The luma gain in main tier with respect to that software model is reportedly 2.2%, 1.4% and 0.1% for the lossy all intra RGB 4:4:4 SC, YUV 4:4:4 SC and Class F categories, respectively.

# Introduction

This contribution proposes three tools to improve coding efficiency of palette based coding of computer screen content. The tools are described in detail in the following subsections.

## Long-term palette prediction

Proposed method signals a long-term palette at the slice header to be used in the coding of the palette of a CU that is coded with palette mode within that slice. Long term palette information signaled at the slice header consists of the size and entries of the long term palette. At CU level, while coding the palette of a CU, for each palette entry that is not predicted from a previous slice, one bit is signaled to indicate whether this entry is copied from the long term palette or explicitly signaled. If the indication is that the entry is copied from the long term palette, then the index of the entry within the long term palette is signaled.

The encoder-side algorithm implemented in the proposal constructs the long term palette using statistics of the previous slice. Therefore, for the first slice in a sequence (or after an IDR frame), the size of the long term palette is signaled to be zero and no palette entries are coded. The frequency of each explicitly coded (the ones that are not predicted from the last decoded CU’s palette) palette entry is obtained in the previous slice. Long term palette is constructed using the N most frequently occurring colors, where N is the long term palette size and set to 16 in this proposal. If there are less than 16 explicitly coded palette entries in previous slice, the long term palette size is set to that number. It should be noted that the performance of the encoder algorithm described here varies for different predefined list sizes in a content dependent way. We have observed that the optimum predefined list size is different for different content. Therefore a more intelligent encoder algorithm that would capture the content specific properties (such as counts of colors in the list, frequency of palette mode being used and frequency of long-term palette being used in the previous slice) would allow even better gains than the ones we show here.

## Syntax

PPS:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_long_term_palette_enabled_flag | u(1) |
| … |  |
| } |  |

Slice Header:

| slice_segment_header( ) { |  |
| --- | --- |
| …. |  |
| if(pps_long_term_palette_enabled_flag){ |  |
| for (cIdx=0; cIdx<3; cIdx++){ |  |
| long_term_palette_size[cIdx] | u(v) |
| for ( i=0; i< long_term_palette_size[cIdx]; i++){ |  |
| long_term_palette_entries[ cIdx ] [ i] | u(5) |
| } |  |
| } |  |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteSize; i++ ) { |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| if ( previous_palette_entry_flag[ i ] ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ i ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| if( numPredPreviousPalette < 31 ) |  |
| palette_num_signalled_entries | ae(v) |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| for( i = 0; i < palette_num_signalled_entries; i++ ){ |  |
| if( long_term_palette_size > 0 ) |  |
| copy_from_long_term_palette_flag[cIdx][i] | u(1) |
| if(copy_from_long_term_palette_flag[cIdx][i] ){ |  |
| index_in_long_term_palette [cIdx ][ i ] | u(v) |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] = long_term_palette_entries[ cIdx ] [index_in_long_term_palette [cIdx ][ i ] ] |  |
| } |  |
| else |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| } |  |
| … |  |
| } |  |

## Semantics

pps_long_term_palette_enabled_flag: equal to 1 specifies that long term palette is used.

long_term_palette_size[cIdx] specifies the size of long term palette for component cIdx at the slice header. If pps_long_term_palette_enabled_flag is 0 then this syntax element is not present in the bistream and it is set to 0.

long_term_palette_entries[ cIdx ] [ i ] specifies ith long term palette entry for component cIdx at the slice header.

copy_from_long_term_palette_flag [cIdx ][ i ] equal to 1 specifies that the ith palette entry for color component indicated by cIdx is copied from the long term palette for color component indicated by cIdx. copy_from_long_term_palette_flag [cIdx ][ i ] equal to 0 specifies that the ith palette entry for color component indicated by cIdx is not copied from the long therm palette for color component indicated by cIdx but explicity signaled. If copy_from_long_term_palette_flag [cIdx ][ i ] is not present in the bitstream, then it is value is set to 0.

index_in_long_term_palette [cIdx ][ i ] specifies the long term palette entry index for reconstructing the current coding units palette entry indicated by palette entry index i and color component cIdx. This syntax element is coded using fixed length coding where number of bits used in fixed length coding is log2(long_term_palette_size.

## Restricted run-length coding

Proposed method signals which palette indexes have “palette_run” associated with them in “run mode” and which don’t. For the palette indexes for which the palette_run is not signalled those are assumed to be 0. This avoids signalling unnecessary run-length information for “stand-alone” samples that are not repeating consequently within the block. A single CU level syntax element is included in the bitstream to indicate the maximum palette index for which the “palette_run” is indicated. Example below illustrates the proposed coding.

(a)                        (b)

|  | Max_ind | Palette index – run-length pairs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-P0303: |  | 0, 3 | 1, 1 | 0, 2 | 1, 1 | 0, 2 | 1, 1 | 0, 2 | 1, 1 | 0, 3 |
| Proposed method: | 0 | 0, 3 | 1 | 0, 2 | 1 | 0, 2 | 1 | 0, 2 | 1 | 0, 3 |

(c)

Figure 1: Example of a palette coded block (a), mapping to palette indexes (b) and resulting syntax elements in both P0303 approach and proposed method (c). Proposed method signals first that only palette index 0 is associated with run information. When coding the palette index – run-length pairs, the proposed method omits run-length information from the palette indexes larger than 0.

## Syntax

| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| … |  |
| palette_index_max_run_coding | ae(v) |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| if ( yC != 0 && previous_run_type_flag != COPY_ABOVE_RUN_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = INDEX_RUN_MODE |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_RUN_MODE ) { |  |
| palette_index | ae(v) |
| if( palette_index = = palette_size ) { /* ESCAPE_MODE */ |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| scanPos++ |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } else { |  |
| if( palette_index > palette_index_max_run_coding ) |  |
| palette_run = 0 |  |
| else |  |
| palette_run | ae(v) |
| previous_run_type_flag = palette_run_type_flag |  |
| runPos = 0 |  |
| while ( runPos <= palette_run ) { |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } |  |
| } else { /* COPY_ABOVE_RUN_MODE */ |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC − 1 ] |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| samples_array[ cIdx ][ xC ][ yC ] = palette_entries[ cIdx ][ paletteMap[ xC ][ yC ]] |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

Palette_index_max_run_coding: specifies the maximum palette index that has run-length coding associated with it.

## Scanning indicator for the palette coded CUs

Proposed method signals whether the palette coded CU is scanned in either horizontal or vertical scan order. Proposed method does not change the coding/decoding of the remaining syntax elements of the palette coded CU. However, the reconstruction process is changed in the decoder.

## Syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ){ |  |
| palette_scan_direction_flag[ x0 ][ y0 ] |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| } |  |
| if(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY)) && ( !palette_mode_flag[ x0 ][ y0 ] ) |  |
| part_mode | ae(v) |
| … |  |

## Semantics

palette_scan_direction_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using vertical scan order whereas equal to 0 means that the current coding unit is coded using the horizontal scan order.

# Objective coding efficiency of the proposed tools

Proposed methods are tested against JCTVC-P0303. Simulation results are provided both for RExt set and Screen Content Coding Call for Proposal set. Detailed information on the simulation results can be found in the attached excel sheets where the naming is accordingly:

test 100 : Long-term palette prediction only

test 010 : Restricted run-length coding only

test 001 : Scanning indicator for the palette coded CUs only

test 111 : Combination of the three methods

Encoder/decoder run times are not measured accurately due to heterogeneous cluster.

Figure 2-1 Lossy all intra results for RExt set against JCTVC-P0303 for the combination of the three methods (test 111)

Figure 2-2 Lossless all intra results for RExt set against JCTVC-P0303 for the combination of the three methods (test 111)

Figure 2-3 Lossy all intra results for SCC Cfp set against JCTVC-P0303 for the combination of the three methods (test 111)

Figure 2-4 Lossless all intra results for SCC Cfp set against JCTVC-P0303 for the combination of the three methods (test 111)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).