| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0073 |
| --- | --- |

| Title: | AHG18: Worst-case Escape Code Length Mitigation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution identifies the worst-case code length that may occur when coding an escape code for a coefficient as 46. The contribution proposes an alternative scheme to the exponential-Golomb part of escape coding that claims to reduce the worst-case escape code length to 32, which is the same as the worst-case escape code length in HEVC version 1. All-tier BD-rate changes of 0.0% are reported for AHG18 lossy and lossless common test conditions.

# Introduction

In the current HEVC standard, escape codes (any part of a coefficient’s magnitude not described by a CABAC-coded syntax element) are coded using a two-part Golomb-Rice/exponential-Golomb method. When coding 16-bit video using extended precision, this method results in a maximum escape code length of 46 bits, which could be a considerable burden to decode for a single coefficient.

By limiting the prefix length (and introducing a corresponding maximum suffix length) in the exponential-Golomb part of the escape coding scheme, the worst case can be reduced to 32 bits without significant adverse effect on coding efficiency or run time.

HEVC Version 1 was designed such that the worst-case escape code length is 32 bits, and this scheme continues that design methodology.

# Proposed Scheme

When coding 16-bit video using extended precision, the dynamic range of the transform and quantiser and therefore of coefficients entering the entropy coder (maxTrDynamicRange) is equal to 16 + 6 = 22; this results in a maximum coefficient magnitude of 222 = 4194304. When adjusted by subtraction of 1 for the significance map entry and 3 for the Golomb-Rice code (the Rice parameter rParam is 0 in the worst case), the maximum value that can be coded using exponential-Golomb coding is 4194300.

It can be shown that, to code this value using exponential-Golomb in HEVC, a 21-bit suffix (and therefore a 21-bit prefix) is required in addition to a one-bit separator and three extra prefix bits to signal that exponential-Golomb coding is in use; this results in a worst-case total code length of 46.

However, rather than simply allow prefix length to extend without limit, an alternative scheme can be used when extended-precision processing is enabled (via the syntax element extended_precision_processing_enabled_flag) that limits the number of prefix bits required to signal the worst case. The maximum prefix length is given by:

maximumPrefixLength = 32 – (3 + MAX_TR_DYNAMIC_RANGE)

When this prefix length is reached, the corresponding suffix length is then given by:

suffixLength = MAX_TR_DYNAMIC_RANGE – rParam

Furthermore, since the maximum prefix length is known at the start of encoding/decoding, the prefix can be coded using truncated-unary coding – omitting the separator if the maximum prefix length is reached.

The worst case escape code length is hence reduced to:

(32 – (3 + MAX_TR_DYNAMIC_RANGE))		(prefix)

+	(MAX_TR_DYNAMIC_RANGE – rParam)		(suffix)

+	rParam							(fixed bits)

+	3								(not-a-Golomb-Rice-code bits)

= 	32

This allows any escape code to fit into a single 32-bit integer and renders maximum escape code length the same as that in HEVC version 1 (see JCTVC-J0142), which may have to be supported by a Range Extensions codec. A number of example escape code codewords are illustrated in Figure 1.

Figure 1 – Codewords for various escape codes that may occur when processing 14-bit video with extended-precision processing enabled

A similar scheme using a mapping table was previously presented in JCTVC-P0061. It is observed that the scheme presented here achieves the same worst-case mitigation and the same coding performance without the need for a lookup table.

# Results

The proposed scheme has been simulated under the AHG18 common test conditions. The results for lossless and lossy test conditions are presented in Table 1 and Table 2 respectively:

Table 1 – Bit rate changes and coding times vs. HM13.0_RExt6.0 anchor under AHG18 lossless test conditions

Table 2 - BD-rate changes and coding times vs. HM13.0_RExt6.0 anchor under AHG18 lossy test conditions

# Conclusion

An alternative scheme to exponential-Golomb for escape coding has been presented for use when extended-precision processing is enabled that reduces the worst case escape code length from 46 to 32, bringing the maximum length in line with that of HEVC version 1. All-tier BD-rate changes of 0.0% are observed for AHG18 lossy and lossless common test conditions.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Working Draft Text

(Based on JCTVC-O1005)

- 9.3.3.X	Limited exponential-Golomb-order-k binarization process

Input to this process is a request for a limited exponential-Golomb binarization for a syntax element with unsigned value synVal, the Rice parameter riceParam and a flag isChroma indicating whether or not the current syntax element relates to a chroma coefficient.

Output of this process is the limited exponential-Golomb-order-k binarization of the syntax element synVal.

The variable maxTrDynamicRange is derived as follows:

–	If isChroma is equal to 0,

maxTrDynamicRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15

–	Otherwise,

maxTrDynamicRange = extended_precision_processing_flag ? Max( 15, BitDepthC + 6 ) : 15

The variable maximumPrefixLength is set equal to (29 – maxTrDynamicRange).

The bin string of the limited exponential-Golomb-order-k binarization process of a syntax element synVal is specified as follows, where each call of the function put( X ), with X being equal to 0 or 1, adds the binary value X at the end of the bin string:

codeValue = synVal >> riceParam
prefixLength = 0

while( (prefixLength < maximumPrefixLength) && (codeValue > ( (2 << prefixLength) – 2) ) )
{
  prefixLength++
  put(1)
}

if (prefixLength = = maximumPrefixLength)
{
  totalSuffixLength = maxTrDynamicRange
  put(1)
}
else
{
  totalSuffixLength = prefixLength + riceParam
  put(0)
}

synVal = synVal – ( ( (1 << prefixLength) – 1) << riceParam)

while ( (totalSuffixLength – –) > 0)
{
  put( (synVal >> totalSuffixLength) & 1)
}

#### 9.3.3.9	Binarization process for coeff_abs_level_remaining

…

The variable cMax is derived from cRiceParam as:

cMax = 43  <<  cRiceParam

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause 9.3.3.2 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 43 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] – cMax

- When extended_precision_processing_flag is equal to 1, the suffix bin string is specified by invoking the limited exponential-Golomb-order-k binarization process as specified in subclause 9.3.3.X for suffixVal with the variable isChroma set equal to 1 if cIdx is greater than 0 and 0 otherwise.

- Otherwise (extended_precision_processing_flag is equal to 0), the suffix bin string is specified by invoking the EGk binarization process as specified in subclause 9.3.3.3 for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

The minimum negative value is -222 = -4194304; the maximum positive value is 222 – 1 = 4194303