| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0074-r3 |
| --- | --- |

| Title: | SEI message for Colour Mapping Information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Philippe Bordes, Edouard François Technicolor 975 avenue des champs blancs CS 17616-35576 Cesson-Sévigné Cedex, France | Tel: Email: | +33 (0)2 99 27 38 07 pierre.andrivon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes colour mapping side information in an SEI message that guarantees smooth colour space transition with regards to upcoming HDTV and multi-phases UHDTV services deployment. It is stated that the proposed mapping helps preserving artistic intent of the content produced by studios while maintaining differentiation between TV sets manufacturers.
This idea was first exposed in JCTVC-N0180. The proposed SEI message intent was clarified in JCTVC-O0363. Besides, complexity concerns have been dealt with a simplification of the colour mapping model in JCTVC-P0126. Finally, editorial issues and synchronization aspects are addressed in this proposal.
It is stated that a software is supplied to identify proposed model parameters. Implementation is provided in HM-13.0+RExt-6.0 encoders and decoders. When the proposed colour mapping information SEI message is present, colour mapping is applied to the decoded output pictures.

# Introduction

As stated in JCTVC-P0126 ([4]), upcoming UHD video services consider delivering content in a new colour encoding system defined in Rec. ITU-R BT.2020 ([5]). Rec. 2020 gamut is wider than Rec. 709 gamut by 30% (CIE 1931) and 24% (CIE 1976) (Figure 1).

Figure 1: Rec. 709 (left) and Rec. 2020 (right) gamut in CIE 1931– courtesy from BSkyB

Considering both Rec. 709 legacy displays and Rec. 2020 video signals, this significant gamut difference may lead to colour space conversions (gamut mapping) that will either distort the artistic creation of movies in terms of colour fidelity to the masterpiece (in comparison with an original Rec. 709 grade, cf. Figure 2) or the likelihood of live scenario memory colours (the blue of the sky, the green of the grass, the tone of the skin…).

Figure 2: Preserving artistic intent with proposed colour mapping SEI

Top-left: Studio colour graded BT.709, top-right: Colour graded BT.2020 converted to BT.709 using SMPTE RP177, bottom-left: BT.2020 colour mapped to BT.709 using proposed colour mapping SEI message (generated with supplied tool software), bottom-right: Studio colour graded BT.2020 displayed as is on BT.709 display

Transmitted colour mapping metadata provides the information to reconstruct the Rec. 709 grade from the Rec. 2020 transmitted grade (with some limitations according to the selected mapping model) (cf. Figure 3).

Figure 3: Colour mapping metadata conveyance principle and benefits

# Inversely, the same reasoning can be applied to a Rec. 709 video signal to be rendered on display that can interpret Rec. 2020 signal (wide colour gamut display).

# Thus, colour mapping metadata enable preservation of the artistic intent and colour fidelity by providing information for mapping transmitted content grade to the content grade supported by the renderer.

Nonetheless, in the suggested use cases, colour mapping is performed between both grades; not between the content colour space and the display primaries. Hence, the incoming video signal is considered as a usually supported Rec. 709 or Rec. 2020 video signal by the display and display processing may be applied on top. Complementary use cases or applications that can be considered comprise forward compatibility between multi-phases UHD-1 TV services, high quality stored content colorist intent preservation (Blu-Ray discs…), non-destructive content personalization (colour mapping metadata as a colour layer)… (cf. [1], [2], [3], [4]).

On the relationship with existing SEI messages:

Mastering display colour volume SEI message: this SEI message provides metadata related to the colour volume of the display that was used in mastering video content. For instance a video signal using Rec. 2020 colour encoding system may have been graded on a reference screen that is not Rec. 2020 (e.g. P3). This “wrapping” information may be conveyed by the mastering display colour volume SEI message. It is complementary to the proposed colour mapping information SEI message and it does not provide information on how mapping from one grade to another.

Tone mapping SEI message: this SEI message has a different purpose as it is related to non-linear transfer function mapping and considered models are mono-dimensional (one 1D LUT) without any optional cross-component correlation.

On synchronization of colour mapping metadata and decoded pictures:

Regarding synchronization of colour mapping information with related pictures, as a first order approximation, one unique SEI set of colour mapping information, optimized for a whole long sequence (unlike a shot-based sequence) or movie, results in a consistent and coherent look. Indeed the artistic intent is generally coherent between different grades (prepared for known reference screens/mastering displays) in order to preserve the storytelling consistency. Considering digital cinema to broadcast/storage media conversion, generally content is firstly graded (“trim pass”) with the most limited gamut mastering display (e.g. Rec. 709-compliant) then it is converted with a Look Up-Table to a wider gamut (e.g. DCI-P3-compliant) display with visual control and fine-tuning. In other scenarios, a very short session (couple of days) of colour grading is performed to adapt the existing grade to targeted mastering display. The process is artistically supervised by the director of photography (DoP) and the producer that validate the versioning. Note: conforming to post-production modus operandi, optimal colour mapping parameters assessment may be shot-based.

Software supply:

A modified version of the HM-13.0+RExt-6.0 implementing the decoding of the SEI message for Colour Mapping Information (added macro SEI_COLOR_MAPPING) are provided.

At the encoder, a new parameter in the configuration file named “SEIColorMappingFile” allows:

- to load an ASCII file containing the SEI Colour Mapping data as specified in the syntax described in section 2.2 and

- to encode it in the bitstream.

At the decoder, when a Colour Mapping Information SEI message is present, it is parsed and the colour mapping is applied to the decoded pictures before output.

Additionally, a standalone tool software “ColorMapping_1DLutMat1DLut” is provided allowing to generate an ASCII file containing the Colour Mapping Information SEI message parameters model using two input images.

Example of use:

ColorMapping_1DLutMat1DLut.exe -sx0 sizeX0 -sy0 sizeY0 -imaIn0 seq0.yuv -sx1 sizeX1 -sy1 sizeY1 -imaIn1 seq1.yuv -imaOut seqColorMapped.yuv -inputBitDepth 10 -outputBitDepth 10 -lutNbp nbpCode  -sei seiColorMappingFile.txt

# Proposal

The proposed model claims leveraging today’s displays and CE devices computational resources. It is based on a composition of 1D LUT per component, 3x3 matrix, 1D LUT per component (cf. [4]). By construction, it is stated that the proposed syntax and semantics are inspired from:

- tone mapping SEI message model tone_map_model_id = 3 to describe the per component 1D LUT model (piece-wise linear function),

- some VUI/Film Grain SEI parameters to describe the output (mapped) colour space.

Proposed text changes are on top of JCTVC-P1005_v1.

## Proposed text changes

  - SEI payload syntax

    - General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| picture_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| structure_of_pictures_info( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 131 ) |  |
| temporal_sub_layer_zero_index( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| no_display( payloadSize ) |  |
| else if( payloadType = = 136 ) |  |
| temporal_motion_constrained_tile_sets( payloadSize ) |  |
| else if( payloadType = = 137 ) |  |
| chroma_resampling_filter_hint( payloadSize ) |  |
| else if( payloadType = = 138 ) |  |
| time_code( payloadSize ) |  |
| else if( payloadType = = 139 ) |  |
| knee_function_info( payloadSize ) |  |
| else if( payloadType = = 140 ) |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 141 ) |  |
| colour_mapping_info( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

D.2.3132 Reserved SEI message syntax

D.3.3132 Reserved SEI message semantics

  - SEI payload semantics

    - General SEI payload semantics

[…]

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Buffering period | - The remainder of the bitstream |
| Picture timing | - The access unit containing the SEI message |
| Pan-scan rectangle | - Specified by the syntax of the SEI message |
| Filler payload | - The access unit containing the SEI message |
| User data registered by Rec. ITU-T T.35 | - Unspecified |
| User data unregistered | - Unspecified |
| Recovery point | - Specified by the syntax of the SEI message |
| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message |
| Picture snapshot | - The access unit containing the SEI message |
| Progressive refinement segment start | - Specified by the syntax of the SEI message |
| Progressive refinement segment end | - The access unit containing the SEI message |
| Film grain characteristics | - Specified by the syntax of the SEI message |
| Post-filter hint | - The access unit containing the SEI message |
| Tone mapping information | - Specified by the syntax of the SEI message |
| Frame packing arrangement | - Specified by the syntax of the SEI message |
| Display orientation | - Specified by the syntax of the SEI message |
| Structure of pictures information | - The set of access units in the CVS that correspond to entries listed in the SEI message |
| Decoded picture hash | - The access unit containing the SEI message |
| Active parameter sets | - The CVS containing the SEI message |
| Decoding unit information | - The decoding unit containing the SEI message |
| Temporal sub-layer zero index | - The access unit containing the SEI message |
| Scalable nesting | - Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested |
| Region refresh information | - The set of VCL NAL units within the access unit starting from the VCL NAL unit following the SEI message up to but not including the VCL NAL unit following the next SEI NAL unit containing a region refresh information SEI message (if any) |
| No display | - The access unit containing the SEI message |
| Temporal motion-constrained tile sets | - The CVS containing the SEI message |
| Chroma sampling filter hint | - The CVS containing the SEI message |
| Time code | - The access unit containing the SEI message |
| Knee function information | - Specified by the syntax of the SEI message |
| Mastering display colour volume | - Specified by the syntax of the SEI message |
| Colour mapping information | - Specified by the syntax of the SEI message |

[…]

For a non-nested SEI message, depending on the value of payloadType, the following applies:

–	If payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the non-nested SEI message applies to the operation point that has OpTid equal to the greatest value of nuh_temporal_id_plus1 among all VCL NAL units in the bitstream, and that has OpLayerIdList containing all values of nuh_layer_id in all VCL units in the bitstream.

–	Otherwise, when payloadType is equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 134, or 139 or 141 (i.e. one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the non-nested SEI message applies to the layer for which the VCL NAL units have nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on nesting of SEI messages:

–	A scalable nesting SEI message shall not be nested in a scalable nesting SEI message.

–	An active parameter sets SEI message shall not be nested in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a buffering period SEI message, a picture timing SEI message, or a decoding unit information SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When a non-nested SEI message has payloadType equal to 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 131, 134, or 139 or 141 (i.e. one of the SEI messages that have payloadType not equal to 0, 1, or 130, and that are allowed to be nested SEI messages), the SEI NAL unit containing the non-nested SEI message shall have TemporalId equal to the TemporalId of the access unit containing the SEI NAL unit.

It is a requirement of bitstream conformance that the following restrictions apply on the presence of SEI messages between two VCL NAL units of an access unit:

–	When there is a prefix SEI message that has payloadType equal to 0, 1, 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 129, 131, or 139 or 141 (i.e. one of the prefix SEI messages that are not user data registered by Rec. ITU-T T.35 SEI message, user data unregistered SEI message, decoding unit information SEI message, scalable nesting SEI message, or region refresh information SEI message) between two VCL NAL units of an access unit in decoding order, there shall be a prefix SEI message of the same type in the same access unit preceding the first VCL NAL unit of the access unit.

–	When there is a suffix SEI message that has payloadType equal to 3 (filler payload), 17 (progressive refinement segment end), 22 (post filter hint), or 132 (decoded picture hash) between two VCL NAL units of an access unit in decoding order, there shall be a suffix SEI message of the same type in the same access unit succeeding the last VCL NAL unit of the access unit.

It is a requirement of bitstream conformance that the following restrictions apply on repetition of SEI messages:

–	For each of the following payloadType values, there shall be less than or equal to 8 identical sei_payload( ) syntax structures within an access unit: 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 128, 129, 131, 132, 133, and 139 and 141.

–	There shall be less than or equal to 8 identical sei_payload( ) syntax structures with payloadType equal to 130 within a decoding unit.

–	The number of identical sei_payload( ) syntax structures with payloadType equal to 134 in an access unit shall be less than or equal to the number of slice segments in the access unit.

## Proposed new SEI message and semantics

  - SEI payload syntax

[…]

D.2.31 Colour mapping information SEI message syntax

| colour_mapping_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_map_id | ue(v) |
| colour_map_cancel_flag | u(1) |
| if( !colour_map_cancel_flag ) { |  |
| colour_map_persistence_flag | u(1) |
| colour_map_video_signal_type_present_flag | u(1) VUI-like/ Film Grain SEI-like syntax elements |
| if(colour_map_video_signal_type_present_flag ) { |  |
| colour_map_video_full_range_flag | u(1) |
| colour_map_primaries | u(8) |
| colour_map_transfer_characteristics | u(8) |
| colour_map_matrix_coeffs | u(8) |
| } |  |
| colour_map_coded_data_bit_depth | u(5) |
| colour_map_target_bit_depth | u(5) Close from tone mapping SEI model 3 |
| colour_map_model_id | ue(v) |
| if( colour_map_model_id = = 0 ) { |  |
| for( c = 0; c < 3; c++ ) { |  |
| pre_lut_num_pivots_minus1[ c ] | u(6) |
| if( pre_lut_num_pivots_minus1[ c ] > 0 ) { |  |
| for( i = 0; i <= pre_lut_num_pivots_minus1[ c ]; i++ ) { |  |
| pre_lut_coded_pivot_value[ c ][ i ] | u(v) |
| pre_lut_target_pivot_value[ c ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| colour_map_matrix_present_flag | u(1) |
| if( colour_map_matrix_present_flag ) { |  |
| log2_matrix_denom | u(4) |
| for( i = 0; i<3; i++) { |  |
| for( j = 0; j<3; j++) { |  |
| colour_map_coeffs[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| for( c = 0; c < 3; c++ ) { | Close from tone mapping SEI model 3 |
| post_lut_num_pivots_minus1[ c ] | u(6) |
| if( post_lut_num_pivots_minus1[ c ] > 0 ) { |  |
| for( i = 0; i <= post_lut_num_pivots_minus1[ c ]; i++ ) { |  |
| post_lut_coded_pivot_value[ c ][ i ] | u(v) |
| post_lut_target_pivot_value[ c ][ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

  - SEI payload semantics

[…]

D.3.31 Colour mapping information SEI message semantics

This SEI message provides information to enable remapping of the colour samples of the output decoded pictures for customization to particular applications or display environments. The remapped samples are expressed either in the luma and chroma or RGB colour space domain, and should be applied to the reconstructed luma and chroma, or the reconstructed RGB samples.

NOTE 1 – For example, an encoder may use the colour mapping information SEI message by mapping a first original source video material colour-grade to a second colour-grade. Colour mapping on the decoded images for the display process is optional and does not affect the decoding process specified in this Specification.

colour_map_id contains an identifying number that may be used to identify the purpose of the colour mapping information. The value of colour_map_id shall be in the range of 0 to 232 − 2, inclusive.

Values of colour_map_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of colour_map_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour mapping information SEI messages containing a value of colour_map_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

NOTE 2 – The colour_map_id can be used to support colour mapping process that are suitable for different display scenarios. For example, different values of colour_map_id may correspond to different mapped colour space supported by displays.

colour_map_cancel_flag equal to 1 indicates that the colour mapping information SEI message cancels the persistence of any previous colour mapping information SEI message in output order that applies to the current layer. colour_map_cancel_flag equal to 0 indicates that colour mapping information follows.

colour_map_persistence_flag specifies the persistence of the colour mapping information SEI message for the current layer.

colour_map_persistence_flag equal to 0 specifies that the colour mapping information applies to the current decoded picture only.

Let picA be the current picture. colour_map_persistence_flag equal to 1 specifies that the colour mapping information persists for the current layer in output order until any of the following conditions are true:

–	A new CVS begins.

–	A picture picB in the current layer in an access unit containing a colour mapping information SEI message with the same value of colour_map_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB..

colour_map_video_signal_type_present_flag equal to 1 specifies that colour_map_video_full_range,  colour_map_primaries, colour_map_transfer_characteristics and colour_map_matrix_coeffs are present, colour_map_video_signal_type_present_flag equal to 0, specifies that colour_map_video_full_range,  colour_map_primaries, colour_map_transfer_characteristics and colour_map_matrix_coeffs are not present.

colour_map_video_full_range_flag has the same semantic as specified in clause E.2.1 for the video_full_range_flag syntax element, except as follows:

- colour_map_video_full_range_flag specifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CVS.

- When colour_map_video_full_range_flag is not present, the value of colour_map_video_full_range_flag is inferred to be equal to video_full_range_flag.

colour_map_primaries has the same semantic as specified in clause E.2.1 for the colour_primaries syntax element, except as follows:

- colour_map_primaries specifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CVS.

- When colour_map_primaries is not present, the value of colour_map_primaries is inferred to be equal to colour_primaries.

colour_map_transfer_characteristics has the same semantic as specified in clause E.2.1 for the transfer_characteristics syntax element, except as follows:

- colour_map_transfer_characteristics specifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CVS.

- When colour_map_transfer_characteristics is not present, the value of colour_map_transfer_characteristics is inferred to be equal to transfer_characteristics.

colour_map_matrix_coeffs has the same semantic as specified in clause E.2.1 for the matrix_coeffs syntax element, except as follows:

- colour_map_matrix_coeffs specifies the colour space of the remapped reconstructed picture, rather than the colour space used for the CVS.

- When colour_map_matrix_coeffs is not present, the value of colour_map_matrix_coeffs is inferred to be equal to matrix_coeffs

colour_map_coded_data_bit_depth specifies the bitDepth for interpretation of the luma and chroma components or RGB components of the associated pictures for purposes of interpretation of the colour mapping information SEI message. When colour mapping information SEI messages are present that have colour_map_coded_data_bit_depth that is not equal to bitDepth for luma and chroma components or for RGB components, these refer to the hypothetical result of a transcoding operation performed to convert the coded video to the bitDepth corresponding to the value of colour_map_coded_data_bit_depth.

The value of colour_map_coded_data_bit_depth shall be in the range of 8 to 16, inclusive. Values of colour_map_coded_data_bit_depth from 0 to 7 and from 17 to 255 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour mapping SEI messages that contain a colour_map_coded_data_bit_depth in the range of 0 to 7, inclusive, or in the range of 17 to 255, inclusive, and bitstreams shall not contain such values.

colour_map_target_bit_depth specifies the bit depth of the output of each colour mapping function described by the colour mapping information SEI message. The colour mapping functions specified with a particular colour_map_target_bit_depth are suggested to be reasonable for all display bit depths that are less than or equal to the colour_map_target_bit_depth.

The value of colour_map_target_bit_depth shall be in the range of 8 to 16, inclusive. Values of colour_map_target_bit_depth from 0 to 7, inclusive, and in the range of 17 to 255, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour mapping SEI messages that contain a value of colour_map_target_bit_depth from 0 to 7, inclusive, or in the range of 17 to 255, inclusive, and bitstreams shall not contain such values.

NOTE 3 – colour_map_target_bit_depth specifies the ouput sample values bidepth of colour mapping functions then colour_map_target_bit_depth  is representative of the intermediate computation accuracy.

colour_map_model_id specifies the model utilized for mapping the coded data. Values greater than 0 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore all colour mapping information SEI messages that contain a value of colour_map_model_id greater than 0, and bitstreams shall not contain such values.

NOTE 4 – A colour_map_model_id of 0 corresponds to a colour mapping model which applies to the three components of each sample a composition of a first piece-wise linear function applied to each component value, a three by-three matrix applied to the three components values, and a second piece-wise linear function applied to each component value.

pre_lut_num_pivots_minus1[ c ] plus 1 specifies the number of pivot points in the piece-wise linear mapping function for the c-th component. When pre_lut_num_pivots_minus1[ c ] is equal to 0, the two default end points are specified as (0, 0) and (2colour_map_coded_data_bit_depth - 1, 2colour_map_target_bit_depth - 1) for the c-th component. The value of pre_lut_num_pivots_minus1[ c ] shall be in the range of 0 to 32, inclusive.

pre_lut_coded_pivot_value[ c ][ i ] specifies the value in the colour_map_coded_data_bit_depth corresponding to the i-th pivot point for the c-th component. The number of bits used for the representation of the pre_lut_coded_pivot_value[ c ][ i ] is ( ( colour_map_coded_data_bit_depth + 7 ) >> 3 )  <<  3.

pre_lut_target_pivot_value[ c ][ i ] specifies the value in the reference colour_map_target_bit_depth corresponding to the i-th pivot point for the c-th component. The number of bits used for the representation of the pre_lut_target_pivot_value[ c ][ i ] is ( ( colour_map_target_bit_depth + 7 ) >> 3 )  <<  3.

colour_map_matrix_present_flag equal to 1 indicates that syntax elements log2_matrix_denom and colour_map_coeffs[ i ][ j ] are present. colour_map_matrix_present_flag equal to 0 indicates that syntax elements log2_matrix_denom and colour_map_coeffs[ i ][ j ] are not present.

log2_matrix_denom specifies the base 2 logarithm of the denominator for all matrix coefficients. The value of log2_matrix_denom shall be in the range of 0 to 15, inclusive. When colour_map_matrix_present_flag is equal to 0, log2_matrix_denom is inferred to be equal to 0.

colour_map_coeffs[ i ][ j ] specifies the value of the three-by-three matrix coefficients.

The variable matrixOutput[ c ] for c = 0, 1 and 2 is derived as follows:

matrixOutput[ c ] = Clip3( 0, ( 1 << colour_map_target_bit_depth ) – 1, 
  (colour_map_coeffs[ c ][ 0 ] * matrixInput[ 0 ] + colour_map_coeffs[ c ][ 1 ] *  matrixInput[ 1 ]
  + colour_map_coeffs[ c ][ 2 ] * matrixInput[ 2 ] +  (1<< (log2_matrix_denom-1)))
  >> log2_matrix_denom)  	                                                                                                                    (D-XX)

where matrixInput[ c ] represents a sample value of the c-th colour component of the matrixInput, where c equal to 0 refers to the luma or G component, c equal to 1 refers to the Cb or B component, and c equal to 2 refers to the Cr or R component. matrixOutput[ c ] represents a sample value of the c-th colour component of the matrixOutput.

The value of colour_map_coeffs[ i ][ j ] shall be in the range of −215 to 215 − 1, inclusive. When colour_map_matrix_present_flag is equal to 0, the matrix colour_map_coeffs is inferred to be equal to the identity matrix I3.

post_lut_num_pivots_minus1[ c ] has the same semantic as pre_lut_num_pivots_minus1[ c ], with pre replaced by post, except that when post_lut_num_pivots_minus1[ c ] is equal to 0 and pre_lut_num_pivots_minus1[ c ] is not equal to 0 or post_lut_num_pivots_minus1[ c ] is equal to 0 and matrix_present_flag is equal to 1, the two default end points for c-th colour component are (0, 0) and (2colour_map_target_bit_depth - 1, 2colour_map_target_bit_depth - 1). The value of post_lut_num_pivots_minus1[ c ] shall be in the range of 0 to 32, inclusive.

post_lut_coded_pivot_value[ c ][ i ] has the same semantic as pre_lut_coded_pivot_value[ c ][ i ], with pre replaced by post, except that if pre_lut_num_pivots_minus1[ c ] is not equal to 0 or matrix_present_flag is equal to 1, the number of bits used for the representation of the post_lut_coded_pivot_value[ c ][ i ] is ( ( colour_map_target_bit_depth + 7 ) >> 3 )  <<  3.

post_lut_target_pivot_value[ c ][ i ] has the same semantic as pre_lut_target_pivot_value[ c ][ i ], with pre replaced by post.

# References

[1] P. Bordes, P. Andrivon and F. Hiron, “Colour Mapping SEI message”, JCTVC-N0180, 14th JCT-VC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[2] P. Bordes, “Colour Mapping SEI message”, JCTVC-O0363, 15th JCT-VC Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013.

[3] P. Bordes, Y. He, “Colour Mapping SEI message”, m31186, 106th MPEG Meeting: Geneva, CH, 29 Oct. – 1 Nov. 2013.

[4] P. Andrivon, P. Bordes, E. François, “SEI message for Colour Mapping Information”, JCTVC-P0126, 16th JCT-VC Meeting: San José CA, USA, 9 – 17 Jan. 2014.

[5] “Parameter values for ultra-high definition television systems for production and international programme exchange”, Recommendation ITU-R BT.2020, Aug. 2012.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).