| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0075 |
| --- | --- |

| Title: | AHG5: Intra-block-copy in Non-4:4:4 Formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution details two issues encountered when using intra-block-copy for non-2Nx2N PUs in non-4:4:4 chroma formats, both of which stem from the adopted practice of merging the PUs where the chroma PU dimensions would be smaller than 4x4. A rule is proposed to be added to HEVC Range Extensions that would prevent these issues from occurring and also reduce computational complexity in the decoder. The application of this rule is reported to have no effect on coding efficiency.

# Introduction

In the current HEVC Range Extensions working text, the intra-block-copy tool has been extended to allow PU splits other than 2Nx2N (NxN, Nx2N and 2NxN). For 8x8 CUs in chroma formats other than 4:4:4 this can result in PUs with chroma dimensions smaller than 4x4, so in these cases, the PUs are merged, with the bottom-rightmost’s motion vector being used for the entire set.

If the bottom-right motion vector would be out-of-bounds for another PU in the set, it is clipped to the boundary position. This requires the motion vectors’ legality to be checked at the decoder side – a potentially significant computational cost, and also may cause the source regions for separate PUs to overlap – as shown in Figure 1.

Figure 1 – Example of motion vector clipping causing source region overlap

Considering that the purpose of motion vectors is to match a region to be predicted with a similar region elsewhere, it is unlikely that an overlapped source region would be a good match for any possible destination.

In addition, there is an issue when constrained intra-prediction is in use. Though the source motion vector must be valid for the bottom-right PU, it may cause the source region for the other PUs to lie outside the current slice (the maximum horizontal range for an intra-block-copy motion vector is anywhere within the current LCU plus 64 luma samples to the left) or in a CU that is inter-coded, defeating the purpose of using constrained intra-prediction for error propagation control. This is illustrated in Figure 2.

Figure 2 – Example of PU merging causing a violation of constrained intra-prediction – the selected motion vector is legal for the bottom-right PU (and the top-right PU), but not for the left PUs

Both the computational complexity of checking the motion vectors in the decoder and the possibility of source region overlap can be removed simply by requiring that the motion vectors that may be applied to the bottom-right PU of a merged set must be valid for every PU in that set.

# Working Text Change

(based on JCTVC-P1005_v1)

#### 7.4.3.3	Picture parameter set RBSP semantics

…

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded using either intra or inter prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case the general intra sample prediction process only uses residual data and decoded samples from neighbouring coding blocks coded using intra prediction modes.

NOTE 1 – Samples reconstructed from available intra blocks with intra_bc_flag equal to 1 are considered as available for the purposes of constrained intra prediction. However, no restriction is placed on the origin of reference samples used by intra blocks with intra_bc_flag equal to 1. Encoders that employ constrained intra prediction for the purpose of error propagation control should, in regions where error propagation is to be controlled through the use of constrained intra prediction, ensure that intra blocks with intra_bc_flag equal to 1 only use reference samples from neighbouring coding blocks coded using intra prediction modes - noting that, for non-4:4:4 chroma formats, the block vector associated with an intra block where intra_bc_flag is equal to 1 may also be used for other neighbouring blocks (see subclause 8.4.5.2.7).

transform_skip_enabled_flag equal to 1 specifies that transform_skip_flag may be present in the residual coding syntax. transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the residual coding syntax.

- 8.4.5.2.7	Specification of intra block copying prediction mode

…

–	Otherwise, the variable bv representing the block vector for prediction in full-sample units is derived as follows:

–	If cIdx is not equal to 0, trafoDepth is equal to 0, and nTbS is equal to 4, the following applies:

–	If ChromaArrayType is equal to 1, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY + 4 ].

The bitstream shall not contain data such that the value of bvIntra[ xTbY + 4 ][ yTbY + 4 ] is invalid when used as the value of bvIntra[ xTbY ][ yTbY ], where validity is defined by the bitstream conformance requirements specified in subclause 8.4.4.

–	Otherwise, if ChromaArrayType is equal to 2, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY ].

The bitstream shall not contain data such that the value of bvIntra[ xTbY + 4 ][ yTbY ] is invalid when used as the value of bvIntra[ xTbY ][ yTbY ], where validity is defined by the bitstream conformance requirements specified in subclause 8.4.4.

–	bv[ 0 ] = max( -( xTbY % CtbSizeY + CtbSizeY ), bv[0] ) >> ( SubWidthC − 1 )

–	bv[ 1 ] = max( -( yTbY % CtbSizeY ), bv[ 1 ] ) >> ( SubHeightC − 1 )

–	Otherwise, the following applies:

bv[ 0 ] = bvIntra[ xTbY ][ yTbY ][ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-63)

bv[ 1 ] = bvIntra[ xTbY ][ yTbY ][ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-64)

–	The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, is derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-65)

Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

# Results

The proposed scheme has been tested using AHG5, AHG8-lossy and AHG8-lossless test conditions. The results are shown in Table 1, Table 2 and Table 3 respectively.

Table 1 - BD-rate changes and coding times for the proposed scheme under AHG5 test conditions

Table 2 - BD-rate changes and coding times for the proposed scheme under AHG8 lossy test conditions

Table 3 – BD-rate changes and coding times for the proposed scheme under AHG8 lossless test conditions

Under AHG8 lossy test conditions, small losses are observed for class F sequences. These losses are concentrated in the SlideEditing and SlideShow sequences, both of which contain heavy use of sharp features. Where these features are present, a small change to a PU’s motion vector may have a significant impact on the efficiency with which that PU is coded. The losses can therefore be attributed to the additional restriction placed on luma motion vector selection (causing the luma channel data to require more bits to encode while keeping the PSNR for all channels approximately the same). In addition, since the sequences are largely static, they are very easily inter-predicted (e.g. the random access compression ratio for both sequences at Qp 27 is approximately 480:1) and where the video is compressed to this degree, a small change in bit rate or PSNR appears as a more significant change in BD-rate.

# Conclusion

Issues concerning the use of intra-block-copy in non-4:4:4 chroma formats have been highlighted. A simple rule addition to the standard is proposed that resolves these issues while reducing decoder complexity. The application of this rule has been shown to have no significant effect on coding efficiency.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).