| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0077 |
| --- | --- |

| Title: | AHG8: Effectiveness of the HM Inter Search for Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution examines the reasons for the effectiveness of intra block-copy when using inter-predictive coding, noting that intra block-copy is effective for source material with a high degree of spatial repetitiveness, such as screen content. It is asserted that the current HM inter-prediction search is poorly suited for predicting such content and that substantial improvement may be made purely by changing the search algorithm.

It is recommended that the inter search algorithm be considered before accepting any further new tools based solely on their inter performance, and that some test material be re-evaluated for its suitability for testing new tools.

# Introduction

In JCTVC-P0106, as was commonly understood, it was asserted that the intra block-copy tool in the RExt amendment was shown to provide a BD-rate saving of up to 0.5% when coding natural video, whereas when used for the target content (Class F and 4:4:4 screen content), the tool is extremely effective, yielding an average BD-rate saving over those sequences of 26%, 19% and 12% for AHG8 lossy test conditions and 15%, 12% and 9% for AHG8 lossless test conditions for intra, random-access and low delay main tiers respectively.

However, while it is well understood that intra block-copy is very effective on intra-frames, a question was raised outside the meeting about why this tool was so effective for inter-coded sequences, since it was felt that those sequences should be able to be coded using only inter prediction.

# Explanation

## Intra vs. Inter frames

The AHG8 test material on which intra block-copy is the most effective is repetitive, i.e. most frames are identical. This means that inter-prediction can efficiently remove most of the information from each frame where it is used. As a result, the intra-coded frames constitute a large fraction of the total bit count.

Table 1 shows the percentage of the total bit rate that encodes B-frame data. It can be seen that if a tool is extremely effective for intra frames, it will have a large effect on the inter results for certain sequences (especially sc_ppt_doc_xls). This helps to explain the difference of approximately 7% between the random-access results (that contain more intra-coded frames) and the low-delay results.

Table 1 – Analysis of proportions of data attributable to I and B frames under AHG8 lossless test conditions

It can be seen from this table that, for the sc_ppt_doc_xls sequence (and to some extent for the sc_web_browsing sequence), the vast majority of the data is contained within intra frames. This, combined with the extremely high inter compression ratio (as the sequence is largely static), suggests this sequence is unsuitable for study of inter-coding performance. In addition, since all frames within these sequences are near-identical to each other, it should be considered that they are not so much “sequences” as single frames repeated over and over with occasional changes. It is suggested that this test material be moved into the optional test set.

## Inter Search

There are two aspects to the current inter search that affect the reported efficacy of intra block-copy:

- The “fast” inter search is sparse. It was designed to find the best motion vector in a smooth correlation surface, which can be expected for general natural video. However, the three screen content classes of material in the AHG8 test set do not exhibit this quality: the correlation surface is noisy as the features are small and consist of high frequencies. The fast search is therefore not best suited to finding the best matches for the high frequency data being matched.

- The default search area being used is practical for the video being tested in HM and AHG5, although it perhaps should be increased as resolution increases, and reduced as frame rate increases. However, for the screen-content material, the search range is not large enough to cover the large erratic displacements seen in the content.

The fast intra block-copy search is comparatively not as sparse, and is well-suited to finding horizontal features and text with the fast horizontal search, and suited to finding vertical features with the fast vertical search. In addition, the intra block-copy search does not need to cope with large movements, and thus the search range can be significantly smaller (up to 64 pixels to the left of the current CTU).

The fast inter search has been adjusted to extend the search range and add a refining horizontal and vertical scan for motion vectors. Although this has no effect for intra, the effect on B-frames (shown in Table 2 and Table 3) is up to 10% for the target sequences:

Table 2 – BD-rate changes and coding times for modified inter search under AHG8 lossless test conditions (all frames)

Table 3 – BD-rate changes and coding times for modified inter search under AHG8 lossless test conditions (B-frames only)

With intra block-copy disabled, large losses are seen (see Table 4) when looking at all-frame data, due to the effectiveness of this tool for intra-coding. However, looking at the B-frame statistics (Table 5), the modified search can be seen to be at least as effective as intra block-copy. This does not indicate that intra block-copy is ineffective (since it is effective for the intra-frames), only that for B-frames, the current reported BD-rate changes can be achieved using the inter search.

Table 4 – BD-rate changes and coding times for modified inter search with intra block-copy disabled under AHG8 lossless test conditions (all frames)

Table 5 – BD-rate changes and coding times for modified inter search with intra block-copy disabled under AHG8 lossless test conditions (B-frames only)

It is likely that even better results are attainable through further refinement of the inter search, and it may be beneficial to consider calculating the best that a full-frame, non-sparse inter search can achieve to determine an upper limit on the efficiency of inter-coding. This may perhaps require optimisation of the inter search for particular computing platforms in order to process the full area within a reasonable timeframe. In addition, since the type of content that is so effective in these tests is likely to be sourced from a computer (which may have additional processing power available for searching compared with smaller ASIC solutions), there may be perhaps other side information available about the type of content that is present, which may lead to more effective inter-coding. For example, parts of the screen may be known to be window decorations, titles, text, graphics etc.

## Sudden Changes in Image Data

In screen content sequences, it is common for regions of a picture to change significantly within a frame period, such as the appearance or disappearance of a context menu or a newly-rendered web page. In these cases, inter processing may struggle to find a good match for this region, no matter what the search range. Such areas would be best coded with intra-prediction, and if there is a repetitive nature to the image data, the surrounding area would benefit from intra block-copy.

# Other Contributions

Previously (JCTVC-O0337), problems regarding the limited ability of the inter search to take neighbouring PUs into account have led to some sequences being removed from the main test set. In addition (JCTVC-O0066 [JCTVC-O0066_r1.pptx slides]), problems regarding the intra search have been highlighted, where the fast intra search has not been optimised for the use of transform-skip or RDPCM.

# Conclusion

The intra block-copy tool is effective when processing intra-slices or any parts of the picture where intra would have been used due to sudden changes in image data. Results have shown that significant benefits can be achieved by improving the current inter search algorithm, both with and without intra block-copy enabled, and it is recommended that the AHG8 community consider improving the current inter search algorithm before considering any additional tools based solely upon their inter performance.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).