| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0078r3 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0026r3 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The proposal includes the following aspects:

- The following modifications related to layer sets are proposed:

- The sub-bitstream extraction process is appended so that it can be applied to layer sets where an external base layer is in use.

- Additional layer sets (which may be converted to a new bitstream) can be specified in the VPS extension. These additional layer sets are not required to generate conforming bitstreams with the sub-bitstream extraction process of clause 10.

- It is proposed that the independent layers of additional layer sets that will become base layers in the non-base layer subtree extraction process are required to activate SPSs and PPSs with nuh_layer_id equal to 0 only.

- A non-base layer subtree extraction process is specified for converting one or more non-base layer subtrees to a standalone conforming bitstream.

- Profile-tier-level combinations specified for additional layer sets apply to the output bitstream of the non-base layer subtree extraction process:

- Profile-tier-level combinations indicated in the VPS for the output layer sets of additional layer sets apply to the output bitstream of the non-base layer subtree extraction process.

- The profile-tier-level combination indicated in the SPS for a non-base independent layer applies to the base layer of an output bitstream of the non-base layer subtree extraction process.

- It is allowed to include auxiliary picture layers for output layer sets that are indicated to be conforming to the MV-HEVC and SHVC profile specifications (currently Stereo Main, Scalable Main, Scalable Main 10), but auxiliary picture layers are not taken into account in the profile constraints.

Additionally, complementary aspects numbered 1, 2, 3a to 3d, and 4 are proposed in Section 2.7 of the contribution.

Revision 2 of the contribution responds to the comments raised in the first JCT-VC review by some changes in the design.

Revision 3 of the contribution attempts to reflect all the JCT-VC/JCT-3V decisions regarding this contribution in the attached specification text (excluding items 3a to 3d of Section 2.7).

# Introduction

## Background

While the VPS syntax, specifically layer_id_included_flag[ i ][ j ], allows to exclude the base layer, clause 10 constraints the layer sets as follows:

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream of the process specified in this clause with tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget equal to the layer identifier list associated with a layer set specified in the active video parameter set shall be a conforming bitstream.

Therefore the layer sets specified in the VPS mandatorily include the base layer.

## Problem statement

The contribution proposes a solution to the following problems:

- MV-HEVC/SHVC bitstreams with an external base layer do not conform to HEVC version 1. No layer sets according to VPS (layer_id_included_flag[ i ][ j ]) can be specified for them, as it is required that the layer sets can be used in the sub-bitstream extraction process of clause 10 to generate conforming bitstreams. Yet, it is believed that layer sets and output layer sets would be useful concepts also for bitstreams with an external base layer. For example, output layer sets are associated with a profile-tier-level combination in MV-HEVC/SHVC.

- Some indications and syntax choices have been made to facilitate rewriting of independent non-base layers as independent bitstreams. However, a rewriting process has not been specified in MV-HEVC/SHVC, there are no indications whether the independent non-base layers are completely rewritable, and also the profile-tier-level information for the rewritten bitstream cannot be indicated. It is therefore asserted that the support for extracting independent non-base layers as independent bitstreams is not supported to a sufficient amount.

- A problem has been identified that the required decoding capabilities, such as profile-tier-level, cannot be indicated for auxiliary picture layers.

- Both a profile-tier-level combination and HRD parameters (e.g. buffering parameters, DPB size) are associated with an output layer set. It is asserted that in many cases it is desirable to include auxiliary picture layers in output layer sets conforming to a range extension, multiview or scalable profile, because HRD parameter signalled for the output layer set then cover also the auxiliary picture layers and hence indicate the total buffering requirements for the case when decoding of both primary and auxiliary layers is carried out. However, the current profile specifications intend not to require the decoding of auxiliary picture layers, while the possibility of requiring the decoding of auxiliary picture layers should be enabled for potential future profiles. Furthermore, the indicated profile for an output layer set that includes auxiliary picture layers should not constrain the profile which is used for coding the auxiliary picture layers. For example, it should be allowed to have a REXT-coded auxiliary picture layer for a version-1 coded base layer.

# Proposal

## Definitions

The following definitions are used in this contribution:

layer tree: a set of layers connected with inter-layer prediction dependencies. (Note: the proposed definition in the specification is more explicit but has the same spirit.)

base layer tree: a layer tree that includes the base layer.

non-base layer tree: a layer tree that does not include the base layer.

independent layer: a layer that does not have direct reference layers.

independent non-base layer: an independent layer that is not the base layer.

An example of these definitions is provided in Figure 1. The example presents how a 3-view multiview-video-plus-depth MV-HEVC bitstream can allocate nuh_layer_id values. As in MV-HEVC there is no prediction from texture video to depth or vice versa, there is independent non-base layer which contains the "base" depth view. There are two layer trees in the bitstream, one (the base layer tree) containing the layers for the texture video, and another one (the non-base layer tree) containing the depth layers.

Figure 1. An example of a 3-view multiview-video-plus-depth MV-HEVC bitstream.

The following two definitions are also used:

layer subtree: a subset of the layers of a layer tree including all the direct and indirect reference layers of the layers within the subset.

non-base layer subtree: a layer subtree that does not include the base layer.

Referring to Figure 1, a layer subtree can for example consist of layers with nuh_layer_id equal to 0 and 2. An example of a non-base layer subtree consists of layers with nuh_layer_id equal to 1 and 3. A layer subtree can also contain all layers of a layer tree.

A layer tree may contain more than one independent layers. A layer tree partition is therefore defined as follows:

layer tree partition: a subset of the layers of a layer tree including exactly one independent layer and all its direct or indirect predicted layers unless they are included in a layer tree partition with a smaller index of the same layer tree.

Layer tree partitions of a layer tree are derived in ascending nuh_layer_id order of the independent layers of the layer tree.

Figure 2 presents an example of a layer tree with two independent layers. The layer with nuh_layer_id equal to 1 could be e.g. a region-of-interest enhancement of the base layer, whereas the layer with nuh_layer_id equal to 2 could enhance the entire base-layer picture in terms of quality or spatially. The layer tree of Figure 2 is partitioned into two layer tree partitions as shown in the figure.

Figure 2. An example of a layer tree with two independent layers.

A non-base layer subtree can therefore be a subset of the non-base layer tree or a layer tree partition of a base layer tree with partition index greater than 0. For example layer tree partition 1 in Figure 2 is a non-base layer subtree.

Finally, the following layer set related terms are also defined:

additional layer set: a set of layers of one or more non-base layer subtrees.

## Overview

The proposal includes the following aspects:

- The following modifications related to layer sets are proposed:

- The sub-bitstream extraction process is appended so that it can be applied to layer sets where an external base layer is in use.

- Additional layer sets (which may be converted to a new bitstream) can be specified in the VPS extension. These additional layer sets are not required to generate conforming bitstreams with the sub-bitstream extraction process of clause 10.

- It is proposed that the independent layers of additional layer sets that will become base layers in the non-base layer subtree extraction process are required to activate SPSs and PPSs with nuh_layer_id equal to 0 only.

- A non-base layer subtree extraction process is specified for converting one or more non-base layer subtrees to a standalone conforming bitstream.

- Profile-tier-level combinations specified for additional layer sets apply to the output bitstream of the non-base layer subtree extraction process:

- Profile-tier-level combinations indicated in the VPS for the output layer sets of additional layer sets apply to the output bitstream of the non-base layer subtree extraction process.

- The profile-tier-level combination indicated in the SPS for a non-base independent layer applies to the base layer of an output bitstream of the non-base layer subtree extraction process.

- It is allowed to include auxiliary picture layers for output layer sets that are indicated to be conforming to the MV-HEVC and SHVC profile specifications (currently Stereo Main, Scalable Main, Scalable Main 10), but auxiliary picture layers are not taken into account in the profile constraints.

The aspects summarized above are discussed in more details in the following sub-sections, whereas the detailed proposal is indicated with change marks in the accompanying specification text.

It is remarked that some parts of the proposed specification text assume that the external base layer is indicated with a VPS flag internal_base_layer_flag (being equal to 0), as proposed in JCTVC-Q0015. However, the specification text changes are written on top of the SHVC specification JCTVC-P1008v4 and hence internal_base_layer_flag is not present in the syntax.

## Additional layer sets

Additional layer sets are specified by taking advantage of layer dependency information: the highest layer within each layer tree partition is indicated. This design is asserted to reduce the bit count compared to the layer-wise indication whether a layer is included in the layer set in the VPS (layer_id_included_flag[ i ][ j ]). For example, the additional layer set consisting of the entire non-base layer tree of Figure 1 is specified with one 2-bit u(v) codeword (whereas layer-wise signalling would consume 3 bits).

## Version 1 compatible SPS and PPS syntax

(Intentionally void to keep the section numbering unchanged. No changes in SPS or PPS syntax are required, because it is proposed that the independent layers of additional independent layer sets that will become base layers in the non-base layer subtree extraction process are required to activate SPSs and PPSs with nuh_layer_id equal to 0 only.)

## Non-base layer subtree extraction process

The process creates a conforming bitstream from an additional independent layer set.

It was felt that rewriting a VPS using heuristic rules specified in the non-base layer subtree extraction process would be difficult or impossible, and a hence VPS rewriting SEI message is proposed. The SEI message includes a VPS NAL unit for a rewritten bitstream. The non-base layer subtree extraction process turns these VPS rewriting SEI messages to VPS NAL units. VPS rewriting SEI message is required for an additional layer set with two or more layers and may be present for an additional layer set with one layer. When VPS rewriting SEI message is not present for an additional layer set with one layer, the resulting rewritten bitstream is otherwise conforming but does not include a VPS.

In addition to the VPS rewriting SEI message, buffering period, picture timing and decoding unit information SEI messages have to be carried for the output bitstreams of the non-base layer subtree extraction process in order to determine conformance of those output bitstreams. According to HEVC, these HRD related may be carried within the scalable nesting SEI message when they apply to sub-bitstreams resulting from the sub-bitstream extraction process of clause 10. A scalable nesting SEI message is disallowed to contain another scalable nesting SEI message.

For both the VPS rewriting SEI message and the HRD related SEI message applying to rewritten bitstreams, there was a need to have a new type of a nesting SEI message that specifies the scope to be the indicated additional independent layer sets. In order to design a generic SEI message, the SEI message also allows the scope of the nested SEI messages to be any indicated output layer sets. In other words, the scope is specified with one or more indices to the common index space of layer sets, additional layer sets, and output layer sets.

The non-base layer subtree extraction process gets the layer set index lsIdx of an additional independent layer set as input.

The non-base layer subtree extraction process includes the following steps:

- It removes NAL units other than SPS, PPS, end of sequence and end of bitstream with nuh_layer_id not in the layer set.

- It removes SPS, PPS, end of sequence and end of bitstream NAL units with nuh_layer_id not equal to 0 and not in the layer set.

- It rewrites nuh_layer_id equal to the new base layer associated with lsIdx to 0.

- It extracts the VPS from the VPS rewriting SEI message.

- It extracts buffering period, picture timing and decoding unit information SEI messages from the output layer set nesting SEI messages.

- It removes SEI NAL units with nesting SEI messages that may not apply to the rewritten bitstream.

## Auxiliary picture layers in MV-HEVC and SHVC profile specifications

It is allowed to include auxiliary picture layers for output layer sets that are indicated to be conforming to the MV-HEVC and SHVC profile specifications (currently Stereo Main, Scalable Main, Scalable Main 10), but auxiliary picture layers are not taken into account in the profile and level constraints. Output layer sets including auxiliary picture layers are proposed to be constrainted as follows:

- (Intentionally blank. This item was removed from revision 3 of the contribution, as the item was left for AHG study.)

- The specified profile constraints apply to a sub-bitstream consisting of the non-auxiliary picture layers of the output layer set.

- (Intentionally blank. This item was removed revision 3 of the contribution, as the item is an editorial issue until there will be profiles requiring decoding of auxiliary pictures.)

## Other aspects

This sub-section lists other proposed aspects that are somewhat independent of the aspects above:

- The default output layer set mechanism is used for the "normal" layer sets and the external BL layer sets, which enables selecting either the highest non-auxiliary layer as the only output layer or all layers as the highest layer. The default output layer set mechanism is not used for additional independent layer sets, because neither of the default mechanisms was felt to be suitable for typical usages of additional independent layer sets, such as indicating layer sets containing only auxiliary picture layers.

- Bitstream partitions and related HRD parameters may be specified for the "normal" layer sets and the external BL layer sets (but not for additional independent layer sets).

- The indications related to source scanning type (progressive or interlaced) and picture type (frame or field) were clarified for MV-HEVC/SHVC. (This part of the contribution was moved later to JCTVC-Q0247/JCT3V-H0206.)

Presently, it is asserted that the use of indications related to source scanning type (progressive or interlaced) and picture type (frame or field) in MV-HEVC/SHVC is unclear, because:

  - general_progressive_source_flag and general_interlaced_source_flag are included in the profile_tier_level( ) syntax structure. In MV-HEVC/SHVC, the profile_tier_level( ) syntax structure is associated with an output layer set. Yet, the semantics of general_progressive_source_flag and general_interlaced_source_flag refer to the CVS – which supposedly means all layers, not just the layers of the output layer set which the profile_tier_level( ) syntax structure is associated with.

  - In the absence of SPS VUI, general_progressive_source_flag and general_interlaced_source_flag are used to infer the value of frame_field_info_present_flag, which specifies whether the pic_struct, source_scan_type, and duplicate_flag syntax elements are present in the picture timing SEI messages. However, general_progressive_source_flag and general_interlaced_source_flag are absent in SPSs with nuh_layer_id greater than 0, so it is unclear which profile_tier_level( ) syntax structure is in the inference of general_interlaced_source_flag.

It is noted that one of the envisioned types of scalability for SHVC is interlaced-to-progressive scalability. Consequently, it may be common to have interlaced source content represented by the base layer and progressive source content represented by the enhancement layer.

The following are therefore proposed:

- layer_progressive_source_flag and layer_interlaced_source_flag, which are present in the SPS when profile_tier_level( ) is not present in the SPS. These syntax elements specify the source scanning type similarly to how general_progressive_source_flag and general_interlaced_source_flag in the SPS with nuh_layer_id equal to 0 specify the source scanning type for the base layer.

- When general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag appear in an SPS, they apply to the pictures to which the SPS is an active SPS.

- When general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag appear in a profile_tier_level( ) syntax structure associated with an output layer set, they apply to output layers and alternative output layers, if any, of the output layer set.

- The constraints on and the inference of the value of frame_field_info_present_flag (in SPS VUI) is derived on the basis of general_progressive_source_flag and general_interlaced_source_flag, if they are present in the SPS, and on the basis of layer_progressive_source_flag and layer_interlaced_source_flag, otherwise.

- In a couple of occasions, a bug was fixed on the indexing used with layer_id_included_flag[ ][ ].

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).