| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0080 |
| --- | --- |

| Title: | Block vector prediction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC range extensions, the block vector (BV) predictor for each intra block copy (IBC) PU is set to the BV of the previous IBC PU in general. For the 3rd CU in one coding_quadtree and the 3rd PU in one CU, the BV predictor is probably from the 2nd CU in the coding_quadtree and the 2nd PU in the CU. Because the 3rd CU/PU and their corresponding 2nd CU/PU are not adjacent, this contribution proposes to set the BV predictor of the 3rd CU/PU to the 1st CU/PU. Experimental results reportedly show that the proposed method can achieve 0.1%, 0.1%, 0.2%, 0.1%, and 0.3% BD-rate savings respectively for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI Main-Tier condition. For RA Main-Tier, the BD-rate savings are 0.1%, 0.2%, 0.0%, 0.1%, and 0.2%. For LD Main-Tier, the BD-rate savings are 0.1%, -0.2%, 0.1%, 1.7%, and 0.3%.

# Introduction

In the current IBC design [1][2][3][4], there is a variable LastIntraBCMv stored in both encoder and decoder for the BV prediction mechanism of IBC. For each PU except the first IBC PU of one CTU, the BV predictor is set equal to the previous decoded BV. For the first PU in one CTU, the BV predictor is set (-width, 0).

For CU-level IBC, each CU’s BV predictor (i.e., the BV predictor of its first PU) is initialized to be the previously decoded BV. Supposing that all CUs in the current coding_quadtree are IBC ones, as Fig. 1 shows, the BV predictor of the 3rd CU is currently equal to the BV of the 2nd CU’s ending IBC PU. As for PU-level IBC, the 1st PU of one CU follows the above CU-level BV prediction method, whereas the other PUs set the BV predictor be the BV of previous PU, as Fig. 2 shows.

Fig. 1.  CU-level BV prediction.

Fig. 2  PU-level BV prediction

In this way, for the 3rd CU in one coding_quadtree, the BV predictor for the 3rd CU is the 2nd CU’s ending BV in the corresponding coding_quadtree when the 2nd CU is with IBC coding; for the 3rd PU in one CU, the BV predictor for the 3rd PU is the 2nd PU’s BV. However, the 3rd CU/PU and the corresponding 2nd CU/PU are not adjacent, so the BV predictor will not be accurate enough to predict the 3rd CU/PU’s BVs.

# Proposed method

In this contribution, it is proposed to utilize the decoded BV previous to the 2nd decoded CU/PU as the BV predictor for the 3rd IBC CU/PU, when the previous-to-2nd-CU/PU BV exists. In such way, the BV predictor for the 3rd CU is more likely to be an adjacent BV, especially when the 1st CU is with IBC coding; whereas for the 3rd PU, the BV predictor is just to be the up-side 1st PU’s BV. Fig. 3 shows the CU-level and PU-level BV prediction in the proposed method. Please note that the proposed method does not require additional buffers, and it only requires the decoded 2nd CU/PU not to re-write the variable LastIntraBCMv.

| (a) PU-level BV prediction | (b) CU-level BV prediction |
| --- | --- |

Fig. 3.  Predicting BV of the 3rd CU or PU from the 1st CU or PU

# Experimental results

The proposed method is integrated on HM-13.0+RExt-6.0 [5]. Table 1 and Table 2 show the results for lossless and lossy coding respectively under the AHG8 common test condition (CTC) [6]. Experimental results reportedly show that the proposed method can achieve 0.1%, 0.1%, 0.2%, 0.1%, and 0.3% BD-rate savings respectively for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI Main-Tier condition. For RA Main-Tier, the BD-rate savings are 0.1%, 0.2%, 0.0%, 0.1%, and 0.2%. For LD Main-Tier, the BD-rate savings are 0.1%, -0.2%, 0.1%, 1.7%, and 0.3%.

Table 1.  Results for lossy coding under AHG8 CTC

Table 2. Results for lossless coding under AHG8 CTC

Table 3. Results for lossy coding under SCC_CFP CTC

# Table 4. Results for lossless coding under SCC_CFP CTC

# WD Change

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-1)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-2)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ 0 ]	(8-3)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ 1 ]	(8-4)

- If blkIdx is equal to 0, the value of BvpIntra[ compIdx ] is set equal to available BvpCUIntra[ compIdx ]; Otherwise, if numPartitions is equal to 4 and blkIdx is equal to 1, the value of BvpIntra[ compIdx ] is not updated; Otherwise, the value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- If xPb%(nCbS<<1) is equal to nCbS and yPb%(nCbS<<1) is equal to 0 , the available BvpCUIntra[ compIdx ] is not updated. Otherwise, If blkIdx equal to numPartitions minus 1, BvpCUIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]	(8-5)

# Conclusion

This contribution proposes to set the BV predictor of the 3rd CU/PU to the 1st CU/PU. Experimental results reportedly show that the proposed method can achieve 0.1%, 0.1%, 0.2%, 0.1%, and 0.3% BD-rate savings respectively for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI Main-Tier condition. For RA Main-Tier, the BD-rate savings are 0.1%, 0.2%, 0.0%, 0.1%, and 0.2%. For LD Main-Tier, the BD-rate savings are 0.1%, -0.2%, 0.1%, 1.7%, and 0.3%.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014

- C. Pang, J. Sole, L. Guo, M. Karczewicz, “Subtest B.3 - Intra block copy with NxN PU,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0145, Jan. 2014

- T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou, “RCE3: Results of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0180, Jan. 2014

- G. Laroche, C. Gisquet, P. Onno, T. Poirier, L. Zhu, J. Xu, G. J. Sullivan, S. Sankuratri, B. A. Kumar, F. Wu “Text and results for block vector predictor for intra block copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0304, Jan. 2014

- HM-13.0+RExt-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0.

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1006, Jan. 2014